Copyright 2022 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 014000 - QUALITY REQUIREMENTS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

- GENERAL

      - SUMMARY

        - Section includes administrative and procedural requirements for quality assurance and quality control.

        - Testing and inspection services are required to verify compliance with requirements specified or indicated. These services do not relieve Contractor of responsibility for compliance with the Contract Document requirements.

Retain three subparagraphs below to clarify applicability of provisions in this Section. Revise to suit Project. Coordinate revisions with the General Conditions and other Division 01 Sections.

          - Specific quality-assurance and quality-control requirements for individual work results are specified in their respective Specification Sections. Requirements in individual Sections may also cover production of standard products.

          - Specified tests, inspections, and related actions do not limit Contractor's other quality-assurance and quality-control procedures that facilitate compliance with the Contract Document requirements.

          - Requirements for Contractor to provide quality-assurance and quality-control services required by Architect, Owner,[ Commissioning Authority,] [ Construction Manager,] or authorities having jurisdiction are not limited by provisions of this Section.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 012100 "Allowances" for testing and inspection allowances.

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

Revise the number of projects in "Experienced" Paragraph below to suit Project, office policy, and Owner's requirements. Delete if experience requirements are included in individual Specification Sections or are prohibited by law.

        - Experienced: When used with an entity or individual, "experienced," unless otherwise further described, means having successfully completed a minimum of [five] <Insert number> previous projects similar in nature, size, and extent to this Project; being familiar with special requirements indicated; and having complied with requirements of authorities having jurisdiction.

        - Field Quality-Control Tests and Inspections: Tests and inspections that are performed on-site for installation of the Work and for completed Work.

        - Installer/Applicator/Erector: Contractor or another entity engaged by Contractor as an employee, subcontractor, or sub-subcontractor, to perform a particular construction operation, including installation, erection, application, assembly, and similar operations.

          - Use of trade-specific terminology in referring to a Work result does not require that certain construction activities specified apply exclusively to specific trade(s).

Retain "Mockups" Paragraph below if Project requires mockups.

        - Mockups: Physical assemblies of portions of the Work constructed to establish the standard by which the Work will be judged. Mockups are not Samples.

          - Mockups are used for one or more of the following:

            - Verify selections made under Sample submittals.

            - Demonstrate aesthetic effects.

            - Demonstrate the qualities of products and workmanship.

            - Demonstrate successful installation of interfaces between components and systems.

            - Perform preconstruction testing to determine system performance.

          - Product Mockups: Mockups that may include multiple products, materials, or systems specified in a single Section.

          - In-Place Mockups: Mockups constructed on-site in their actual final location as part of permanent construction.

        - Preconstruction Testing: Tests and inspections performed specifically for Project before products and materials are incorporated into the Work, to verify performance or compliance with specified criteria. Unless otherwise indicated, copies of reports of tests or inspections performed for other than the Project do not meet this definition.

        - Product Tests: Tests and inspections that are performed by a nationally recognized testing laboratory (NRTL) in accordance with 29 CFR 1910.7, by a testing agency accredited in accordance with NIST's National Voluntary Laboratory Accreditation Program (NVLAP), or by a testing agency qualified to conduct product testing and acceptable to authorities having jurisdiction, to establish product performance and compliance with specified requirements.

        - Source Quality-Control Tests and Inspections: Tests and inspections that are performed at the source (e.g., plant, mill, factory, or shop).

        - Testing Agency: An entity engaged to perform specific tests, inspections, or both. The term "testing laboratory" has the same meaning as the term "testing agency."

        - Quality-Assurance Services: Activities, actions, and procedures performed before and during execution of the Work, to guard against defects and deficiencies and substantiate that proposed construction will comply with requirements.

        - Quality-Control Services: Tests, inspections, procedures, and related actions during and after execution of the Work, to evaluate that actual products incorporated into the Work and completed construction comply with requirements. Contractor's quality-control services do not include contract administration activities performed by Architect[ or Construction Manager].

      - DELEGATED DESIGN SERVICES

This article directly relates to AIA Document A201 requirement for Contractor to provide professional services where indicated by the Contract Documents. AIA Document A201 refers to "an appropriately licensed design professional," but no other qualifications are included. Definitions of qualifications of design professionals are indicated in "Quality Assurance" Article. Retain this article if required and if authorities having jurisdiction allow Contractor to perform these services.

        - Performance and Design Criteria: Where professional design services or certifications by a design professional are specifically required of Contractor by the Contract Documents, provide products and systems complying with specific performance and design criteria indicated.

          - If criteria indicated are not sufficient to perform services or certification required, submit a written request for additional information to Architect.

        - Delegated Design Services Statement: Submit a statement signed and sealed by the responsible design professional, for each product and system specifically assigned to Contractor to be designed or certified by a design professional, indicating that the products and systems are in compliance with performance and design criteria indicated. Include list of codes, loads, and other factors used in performing these services.

      - CONFLICTING REQUIREMENTS

"Conflicting Standards and Other Requirements" Paragraph below may resolve issues that sometimes arise when using referenced standards and other specific requirements due to conflicting requirements in the Contract Documents. Coordinate with Conditions of the Contract. Revise to suit Project, office policy, and Owner's requirements.

        - Conflicting Standards and Other Requirements: If compliance with two or more standards or requirements is specified and the standards or requirements establish different or conflicting requirements for minimum quantities or quality levels, inform the Architect regarding the conflict and obtain clarification prior to proceeding with the Work. Refer conflicting requirements that are different, but apparently equal, to Architect for clarification before proceeding.

        - Minimum Quantity or Quality Levels: The quantity or quality level shown or specified is the minimum provided or performed. The actual installation may comply exactly with the minimum quantity or quality specified, or it may exceed the minimum within reasonable limits. To comply with these requirements, indicated numeric values are minimum or maximum, as appropriate, for the context of requirements. Refer uncertainties to Architect for a decision before proceeding.

Insert an article on special compliance with a governing code only if necessary. Such provisions typically belong in the Supplementary Conditions.

      - ACTION SUBMITTALS

Retain "Mockup Shop Drawings" Paragraph below if integrated exterior mockup complexity warrants preparation of Shop Drawings, or if Drawings or the Specifications do not adequately describe requirements.

        - Mockup Shop Drawings:

          - Include plans, sections, elevations, and details, indicating materials and size of mockup construction.

          - Indicate manufacturer and model number of individual components.

          - Provide axonometric drawings for conditions difficult to illustrate in two dimensions.

      - INFORMATIONAL SUBMITTALS

Retain "Contractor's Quality-Control Plan" and "Qualification Data" paragraphs below if requiring written plan described in "Contractor's Quality-Control Plan" Article.

        - Contractor's Quality-Control Plan: For quality-assurance and quality-control activities and responsibilities.

        - Qualification Data: For Contractor's quality-control personnel.

Retain "Contractor's Statement of Responsibility" Paragraph below when local code requirements stipulate submittal to authorities having jurisdiction of Contractor's statement of responsibility for seismic- or wind-force-resisting system construction and if an informational submittal is required. See requirements in the 2018 IBC, Sections 1704 and 1705.

        - Contractor's Statement of Responsibility: When required by authorities having jurisdiction, submit copy of written statement of responsibility submitted to authorities having jurisdiction before starting work on the following systems:

          - Seismic-force-resisting system, designated seismic system, or component listed in the Statement of Special Inspections.

          - Primary wind-force-resisting system or a wind-resisting component listed in the Statement of Special Inspections.

        - Testing Agency Qualifications: For testing agencies specified in "Quality Assurance" Article to demonstrate their capabilities and experience. Include proof of qualifications in the form of a recent report on the inspection of the testing agency by a recognized authority.

Retain "Schedule of Tests and Inspections" Paragraph below if retaining "Schedule of Tests and Inspections" Paragraph in "Quality Control" Article. Coordinate with the IBC requirement for the Statement of Special Inspections prepared by Architect, if applicable to Project.

        - Schedule of Tests and Inspections: Prepare in tabular form and include the following:

          - Specification Section number and title.

          - Entity responsible for performing tests and inspections.

          - Description of test and inspection.

          - Identification of applicable standards.

          - Identification of test and inspection methods.

          - Number of tests and inspections required.

          - Time schedule or time span for tests and inspections.

          - Requirements for obtaining samples.

          - Unique characteristics of each quality-control service.

        - Reports: Prepare and submit certified written reports and documents as specified.

        - Permits, Licenses, and Certificates: For Owner's record, submit copies of permits, licenses, certifications, inspection reports, releases, jurisdictional settlements, notices, receipts for fee payments, judgments, correspondence, records, and similar documents established for compliance with standards and regulations bearing on performance of the Work.

      - CONTRACTOR'S QUALITY-CONTROL PLAN

Retain this article for projects where the extent or complexity of Contractor's quality-assurance and quality-control requirements warrants preparation of a contractor's quality-control plan that incorporates the formal planning, management, and execution actions described. Delete this article when overall quality control is the responsibility of a construction manager.

        - Quality-Control Plan, General: Submit quality-control plan within [10] <Insert number> days of [Notice of Award] [Notice to Proceed], and not less than [five] <Insert number> days prior to preconstruction conference. Submit in format acceptable to Architect. Identify personnel, procedures, controls, instructions, tests, records, and forms to be used to carry out Contractor's quality-assurance and quality-control responsibilities and to coordinate Owner's quality-assurance and quality-control activities. Coordinate with Contractor's Construction Schedule.

        - Quality-Control Personnel Qualifications: Engage qualified personnel trained and experienced in managing and executing quality-assurance and quality-control procedures similar in nature and extent to those required for Project.

          - Project quality-control manager [may also serve as Project superintendent] [does not have other Project responsibilities].

          - <Insert qualifications appropriate to Project>.

        - Submittal Procedure: Describe procedures for ensuring compliance with requirements through review and management of submittal process. Indicate qualifications of personnel responsible for submittal review.

        - Testing and Inspection: In quality-control plan, include a comprehensive schedule of Work requiring testing or inspection, including the following:

          - Contractor-performed tests and inspections, including subcontractor-performed tests and inspections. Include required tests and inspections and Contractor-elected tests and inspections. Distinguish source quality-control tests and inspections from field quality-control tests and inspections.

          - Special inspections required by authorities having jurisdiction and indicated on the Statement of Special Inspections.

          - Owner-performed tests and inspections indicated in the Contract Documents[, including tests and inspections indicated to be performed by Commissioning Authority].

        - Continuous Inspection of Workmanship: Describe process for continuous inspection during construction to identify and correct deficiencies in workmanship in addition to testing and inspection specified. Indicate types of corrective actions to be required to bring the Work into compliance with standards of workmanship established by Contract requirements and approved mockups.

        - Monitoring and Documentation: Maintain testing and inspection reports, including log of approved and rejected results. Include Work Architect has indicated as nonconforming or defective. Indicate corrective actions taken to bring nonconforming Work into compliance with requirements. Comply with requirements of authorities having jurisdiction.

      - REPORTS AND DOCUMENTS

Revise "Test and Inspection Reports" Paragraph below to suit Project. In the case of multiple contracts, consider assigning responsibility for coordination of report submittals to one contract. See the Evaluations.

        - Test and Inspection Reports: Prepare and submit certified written reports specified in other Sections. Include the following:

          - Date of issue.

          - Project title and number.

          - Name, address, telephone number, and email address of testing agency.

          - Dates and locations of samples and tests or inspections.

          - Names of individuals making tests and inspections.

          - Description of the Work and test and inspection method.

          - Identification of product and Specification Section.

          - Complete test or inspection data.

          - Test and inspection results and an interpretation of test results.

          - Record of temperature and weather conditions at time of sample-taking and testing and inspection.

          - Comments or professional opinion on whether tested or inspected Work complies with the Contract Document requirements.

          - Name and signature of laboratory inspector.

          - Recommendations on retesting and reinspecting.

        - Manufacturer's Technical Representative's Field Reports: Prepare written information documenting manufacturer's technical representative's tests and inspections specified in other Sections. Include the following:

          - Name, address, telephone number, and email address of technical representative making report.

          - Statement on condition of substrates and their acceptability for installation of product.

          - Statement that products at Project site comply with requirements.

          - Summary of installation procedures being followed, whether they comply with requirements and, if not, what corrective action was taken.

          - Results of operational and other tests and a statement of whether observed performance complies with requirements.

          - Statement of whether conditions, products, and installation will affect warranty.

          - Other required items indicated in individual Specification Sections.

        - Factory-Authorized Service Representative's Reports: Prepare written information documenting manufacturer's factory-authorized service representative's tests and inspections specified in other Sections. Include the following:

          - Name, address, telephone number, and email address of factory-authorized service representative making report.

          - Statement that equipment complies with requirements.

          - Results of operational and other tests and a statement of whether observed performance complies with requirements.

          - Statement of whether conditions, products, and installation will affect warranty.

          - Other required items indicated in individual Specification Sections.

      - QUALITY ASSURANCE

        - Qualifications paragraphs in this article establish the minimum qualification levels required; individual Specification Sections specify additional requirements.

        - Manufacturer Qualifications: A firm experienced in manufacturing products or systems similar to those indicated for this Project and with a record of successful in-service performance, as well as sufficient production capacity to produce required units. As applicable, procure products from manufacturers able to meet qualification requirements, warranty requirements, and technical or factory-authorized service representative requirements.

        - Fabricator Qualifications: A firm experienced in producing products similar to those indicated for this Project and with a record of successful in-service performance, as well as sufficient production capacity to produce required units.

If more detailed requirements are needed in "Installer Qualifications" Paragraph below, add this information to specific individual Specification Sections. Examples include Installer employing workers trained and approved by manufacturer, Installer being acceptable to manufacturer, and Installer being an authorized representative of manufacturer for both installation and maintenance.

        - Installer Qualifications: A firm or individual experienced in installing, erecting, applying, or assembling work similar in material, design, and extent to that indicated for this Project, whose work has resulted in construction with a record of successful in-service performance.

If other design professionals are indicated in Specification Sections, insert additional paragraphs describing qualifications.

        - Professional Engineer Qualifications: A professional engineer who is legally qualified to practice in jurisdiction where Project is located and who is experienced in providing engineering services of the kind indicated. Engineering services are defined as those performed for installations of the system, assembly, or product that is similar in material, design, and extent to those indicated for this Project.

Retain "Specialists" Paragraph below if other Specification Sections assign certain items of Work to preselected subcontractors (specialists). Revise to suit Project. See the Evaluations for discussion on naming parties other than Owner and Contractor in Specifications.

        - Specialists: Certain Specification Sections require that specific construction activities be performed by entities who are recognized experts in those operations. Specialists will satisfy qualification requirements indicated and engage in the activities indicated.

          - Requirements of authorities having jurisdiction supersede requirements for specialists.

        - Testing and Inspecting Agency Qualifications: An NRTL, an NVLAP, or an independent agency with the experience and capability to conduct testing and inspection indicated, as documented in accordance with ASTM E329, and with additional qualifications specified in individual Sections; and, where required by authorities having jurisdiction, that is acceptable to authorities.

        - Manufacturer's Technical Representative Qualifications: An authorized representative of manufacturer who is trained and approved by manufacturer to observe and inspect installation of manufacturer's products that are similar in material, design, and extent to those indicated for this Project.

        - Factory-Authorized Service Representative Qualifications: An authorized representative of manufacturer who is trained and approved by manufacturer to inspect, demonstrate, repair, and perform service on installations of manufacturer's products that are similar in material, design, and extent to those indicated for this Project.

Retain "Preconstruction Testing" Paragraph below if preconstruction testing is required. Indicate in individual Specification Sections whether Owner or Contractor engages testing agency.

        - Preconstruction Testing: Where testing agency is indicated to perform preconstruction testing for compliance with specified requirements for performance and test methods, comply with the following:

          - Contractor's Responsibilities:

            - Provide test specimens representative of proposed products and construction.

            - Submit specimens in a timely manner with sufficient time for testing and analyzing results to prevent delaying the Work.

            - Provide sizes and configurations of test assemblies, mockups, and laboratory mockups to adequately demonstrate capability of products to comply with performance requirements.

First subparagraph below attempts to ensure that tested assemblies will be representative of actual construction. This requirement may complicate testing and add cost.

            - Build site-assembled test assemblies and mockups, using installers who will perform same tasks for Project.

            - When testing is complete, remove test specimens and test assemblies[, and mockups]; do not reuse products on Project.

          - Testing Agency Responsibilities: Submit a certified written report of each test, inspection, and similar quality-assurance service to Architect[ and Commissioning Authority][, through Construction Manager], with copy to Contractor. Interpret tests and inspections, and state in each report whether tested and inspected Work complies with or deviates from the Contract Documents.

        - Mockups: Before installing portions of the Work requiring mockups, build mockups for each form of construction and finish required to comply with the following requirements, using materials indicated for the completed Work:

          - Build mockups of size indicated.

          - Build mockups in location indicated or, if not indicated, as directed by Architect[ or Construction Manager].

          - Notify Architect[ and Construction Manager] [seven] <Insert number> days in advance of dates and times when mockups will be constructed.

          - Employ supervisory personnel who will oversee mockup construction. Employ workers who will be employed to perform same tasks during the construction at Project.

          - Demonstrate the proposed range of aesthetic effects and workmanship.

          - Obtain Architect's[ and Construction Manager's] approval of mockups before starting corresponding Work, fabrication, or construction.

            - Allow [seven] <Insert number> days for initial review and each re-review of each mockup.

          - Promptly correct unsatisfactory conditions noted by Architect's preliminary review, to the satisfaction of the Architect, before completion of final mockup.

          - Approval of mockups by the Architect does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

          - Maintain mockups during construction in an undisturbed condition as a standard for judging the completed Work.

Retain subparagraph below as the default requirement, and add specific requirements in individual Specification Sections if approved mockups may become part of the Work of those Sections.

          - Demolish and remove mockups when directed unless otherwise indicated.

Retain "Specialty Mockups" Paragraph below if detailed specifications are required for specialty mockups.

        - Specialty Mockups: See Section 014339 "Mockups" for additional construction requirements for [integrated exterior mockups] [preconstruction laboratory mockups] [and] [room mockups].

      - QUALITY CONTROL

Retain "Owner Responsibilities" Paragraph below if some Specification Sections designate Owner as responsible for tests and inspections.

        - Owner Responsibilities: Where quality-control services are indicated as Owner's responsibility, Owner will engage a qualified testing agency to perform these services.

          - Owner will furnish Contractor with names, addresses, and telephone numbers of testing agencies engaged and a description of types of testing and inspection they are engaged to perform.

Retain first subparagraph below where cost of quality-control services must be included in the Contract Sum and where Owner wants to retain control of these services over certain construction operations.

          - Payment for these services will be made from testing and inspection allowances specified in Section 012100 "Allowances," as authorized by Change Orders.

          - Costs for retesting and reinspecting construction that replaces or is necessitated by Work that failed to comply with the Contract Documents will be charged to Contractor[, and the Contract Sum will be adjusted by Change Order].

Usually, retain "Contractor Responsibilities" Paragraph below to address testing and inspection and other quality-control activities not explicitly assigned to Owner. See the Evaluations.

        - Contractor Responsibilities: Tests and inspections not explicitly assigned to Owner are Contractor's responsibility. Perform additional quality-control activities, whether specified or not, to verify and document that the Work complies with requirements.

          - Unless otherwise indicated, provide quality-control services specified and those required by authorities having jurisdiction. Perform quality-control services required of Contractor by authorities having jurisdiction, whether specified or not.

Retain first subparagraph below if some Specification Sections require an independent testing agency to perform certain tests and inspections.

          - Engage a qualified testing agency to perform quality-control services.

Retain first subparagraph below to assure validity of agencies' reports.

            - Contractor will not employ same entity engaged by Owner, unless agreed to in writing by Owner.

          - Notify testing agencies at least [24] <Insert number> hours in advance of time when Work that requires testing or inspection will be performed.

          - Where quality-control services are indicated as Contractor's responsibility, submit a certified written report, in duplicate, of each quality-control service.

          - Testing and inspection requested by Contractor and not required by the Contract Documents are Contractor's responsibility.

          - Submit additional copies of each written report directly to authorities having jurisdiction, when they so direct.

Retain "Retesting/Reinspecting" Paragraph below, which is not covered in the General Conditions, and insert special Project requirements. Additional requirements might include Architect's or Construction Manager's right to require use of a different laboratory for retesting and reinspecting.

        - Retesting/Reinspecting: Regardless of whether original tests or inspections were Contractor's responsibility, provide quality-control services, including retesting and reinspecting, for construction that replaced Work that failed to comply with the Contract Documents.

Retain "Testing Agency Responsibilities" Paragraph below if Contractor engages testing agency.

        - Testing Agency Responsibilities: Cooperate with Architect[, Commissioning Authority] [, Construction Manager,] and Contractor in performance of duties. Provide qualified personnel to perform required tests and inspections.

          - Notify Architect[, Commissioning Authority,] [, Construction Manager,] and Contractor promptly of irregularities or deficiencies observed in the Work during performance of its services.

          - Determine the locations from which test samples will be taken and in which in-situ tests are conducted.

          - Conduct and interpret tests and inspections, and state in each report whether tested and inspected Work complies with or deviates from requirements.

          - Submit a certified written report, in duplicate, of each test, inspection, and similar quality-control service through Contractor.

          - Do not release, revoke, alter, or increase the Contract Document requirements or approve or accept any portion of the Work.

          - Do not perform duties of Contractor.

        - Manufacturer's Field Services: Where indicated, engage a factory-authorized service representative to inspect field-assembled components and equipment installation, including service connections. Report results in writing as specified in Section 013300 "Submittal Procedures."

        - Manufacturer's Technical Services: Where indicated, engage a manufacturer's technical representative to observe and inspect the Work. Manufacturer's technical representative's services include participation in preinstallation conferences, examination of substrates and conditions, verification of materials, observation of Installer activities, inspection of completed portions of the Work, and submittal of written reports.

        - Contractor's Associated Requirements and Services: Cooperate with agencies and representatives performing required tests, inspections, and similar quality-control services, and provide reasonable auxiliary services as requested. Notify agency sufficiently in advance of operations to permit assignment of personnel. Provide the following:

          - Access to the Work.

          - Incidental labor and facilities necessary to facilitate tests and inspections.

          - Adequate quantities of representative samples of materials that require testing and inspection. Assist agency in obtaining samples.

          - Facilities for storage and field curing of test samples.

Retain first subparagraph below if required or if common practice in Project vicinity.

          - Delivery of samples to testing agencies.

          - Preliminary design mix proposed for use for material mixes that require control by testing agency.

          - Security and protection for samples and for testing and inspection equipment at Project site.

        - Coordination: Coordinate sequence of activities to accommodate required quality-assurance and quality-control services with a minimum of delay and to avoid necessity of removing and replacing construction to accommodate testing and inspection.

          - Schedule times for tests, inspections, obtaining samples, and similar activities.

Retain "Schedule of Tests and Inspections" Paragraph below if quality-control services by testing agencies are extensive and job progress might be hindered if services are not carefully managed. Include option when retaining "Contractor's Quality-Control Plan" Article. Schedule below is not the statement of special tests and inspections required by some model codes and jurisdictions as a condition for obtaining a building permit. Coordinate Contractor requirements below with responsibilities of Commissioning Authority in Section 019113 "General Commissioning Requirements." See the Evaluations.

        - Schedule of Tests and Inspections: Prepare a schedule of tests, inspections, and similar quality-control services required by the Contract Documents[ as a component of Contractor's quality-control plan]. Coordinate and submit concurrently with Contractor's Construction Schedule. Update and submit with each Application for Payment.

          - Schedule Contents: Include tests, inspections, and quality-control services, including Contractor- and Owner-retained services, commissioning activities, and other Project-required services paid for by other entities.

          - Distribution: Distribute schedule to Owner, Architect,[ Commissioning Authority,] [ Construction Manager,] testing agencies, and each party involved in performance of portions of the Work where tests and inspections are required.

      - SPECIAL TESTS AND INSPECTIONS

Many model codes require both periodic and continuous special inspections for certain types of construction and require Owner or design professional to engage testing agency. The "special inspector" option is consistent with the IBC; change this term to match that used to identify the entity as defined by authorities having jurisdiction. Verify requirements with authorities having jurisdiction. Some areas may allow Construction Manager or Contractor to engage special inspectors; if this applies to Project, revise "Special Tests and Inspections" Paragraph below and add requirements in "Informational Submittals" Article for Contractor to submit qualifications of special inspectors. Statement of Special Inspections is an IBC requirement that may be enforced by jurisdiction; see the Evaluations.

        - Special Tests and Inspections: Owner will engage a qualified [testing agency] [special inspector] to conduct special tests and inspections required by authorities having jurisdiction as the responsibility of Owner[, as indicated in the Statement of Special Inspections attached to this Section], and as follows:

Retain first subparagraph only when retaining "special inspector" option in "Special Tests and Inspections" Paragraph above.

          - Verifying that manufacturer maintains detailed fabrication and quality-control procedures, and reviewing the completeness and adequacy of those procedures to perform the Work.

          - Notifying Architect[, Commissioning Authority,] [, Construction Manager,] and Contractor promptly of irregularities and deficiencies observed in the Work during performance of its services.

          - Submitting a certified written report of each test, inspection, and similar quality-control service to Architect[ and Commissioning Authority][, through Construction Manager,] with copy to Contractor and to authorities having jurisdiction.

          - Submitting a final report of special tests and inspections at Substantial Completion, which includes a list of unresolved deficiencies.

          - Interpreting tests and inspections, and stating in each report whether tested and inspected Work complies with or deviates from the Contract Documents.

          - Retesting and reinspecting corrected Work.

          - <Insert requirements>.

- PRODUCTS (Not Used)

- EXECUTION

      - TEST AND INSPECTION LOG

Coordinate requirements in "Test and Inspection Log" Paragraph below with requirements of authorities having jurisdiction.

        - Test and Inspection Log: Prepare a record of tests and inspections. Include the following:

          - Date test or inspection was conducted.

          - Description of the Work tested or inspected.

          - Date test or inspection results were transmitted to Architect.

          - Identification of testing agency or special inspector conducting test or inspection.

        - Maintain log at Project site. Post changes and revisions as they occur. Provide access to test and inspection log for Architect's[, Commissioning Authority's,][ and Construction Manager's] [and] [authorities' having jurisdiction] reference during normal working hours.

          - Submit log at Project closeout as part of Project Record Documents.

      - REPAIR AND PROTECTION

        - General: On completion of testing, inspection, sample-taking, and similar services, repair damaged construction and restore substrates and finishes.

          - Provide materials and comply with installation requirements specified in other Specification Sections or matching existing substrates and finishes. Restore patched areas and extend restoration into adjoining areas with durable seams that are as invisible as possible. Comply with the Contract Document requirements for cutting and patching in Section 017300 "Execution."

        - Protect construction exposed by or for quality-control service activities.

        - Repair and protection are Contractor's responsibility, regardless of the assignment of responsibility for quality-control services.

END OF SECTION 014000