Copyright 2019 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 018113.17 - SUSTAINABLE DESIGN REQUIREMENTS - LEED v4 ID+C: COMMERCIAL INTERIORS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

Review the Evaluations and applicable USGBC LEED Rating System before revising this Section.

Where practical, terminology used in this Section has been made identical to or at least consistent with terminology used in USGBC's LEED Rating Systems; if questions arise relating to terminology, review the applicable LEED reference guide and the standards referenced by applicable USGBC LEED Rating System for possible explanations.

If Project design team does not include a LEED Accredited Professional (LEED AP), consider engaging a Sustainability Consultant; Innovation: Credit - LEED Accredited Professional applies if at least one principal participant of Project team is a LEED AP. Note that all LEED credits are optional, so delete requirements for credits not sought.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes general requirements and procedures for compliance with USGBC's LEED prerequisites and credits needed for Project to obtain LEED [Certified] [Silver] [Gold] [Platinum] certification based on USGBC's "LEED Version 4 for Interior Design and Construction" (hereafter, LEED v4 ID+C).

          - Specific requirements for LEED are also included in other Sections.

          - Other LEED prerequisites and credits needed to obtain LEED certification depend on product selections and may not be specifically identified as LEED requirements. Compliance with requirements needed to obtain LEED prerequisites and credits may be used as one criterion to evaluate substitution requests and comparable product requests.

If retaining subparagraph below, attach a copy of LEED checklist to end of this Section as information for Contractor.

          - A copy of LEED Project checklist is attached at end of this Section for information only.

            - Some LEED prerequisites and credits needed to obtain indicated LEED certification depend on Architect's design and other aspects of Project that are not part of the Work of the Contract.

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

        - Chain-of-Custody Certificates: Certificates signed by manufacturers certifying that wood used to make products was obtained from forests certified by an FSC-accredited certification body to comply with FSC STD-01-001. Certificates shall include evidence that manufacturer is certified for chain of custody by an FSC-accredited certification body.

        - Cradle-to-Gate Assessment: Analysis of a product's partial life-cycle from extraction (cradle) to gate (factory completion prior to distribution).

        - LEED: USGBC's "LEED Version 4 for Interior Design and Construction." Definitions that are a part of this document apply to this Section.

        - Life-Cycle Assessment: Evaluation of environmental impacts of a product from cradle to gate, defined by ISO 14040 and ISO 14044.

        - Life-Cycle Inventory: Database that defines environmental input and output for each step in a material or assembly's life cycle.

Note that LEED uses the term "preconsumer" rather than "postindustrial." Also note that when manufacturers and trade associations use the term "postindustrial" it often includes spills, scraps, and damaged and surplus materials that are fed back into the same manufacturing process and that these materials are not considered recycled content by UGBC's LEED Rating Systems.

        - Recycled Content: The recycled content value of a material assembly shall be determined by weight. The recycled fraction of the assembly is then multiplied by the cost of assembly to determine the recycled content value.

          - "Postconsumer" material is defined as waste material generated by households or by commercial, industrial, and institutional facilities in their role as end users of the product, which can no longer be used for its intended purpose.

          - "Preconsumer" material is defined as material diverted from the waste stream during the manufacturing process. Reutilization of materials (such as rework, regrind, or scrap generated in a process and capable of being reclaimed within the same process that generated it) is excluded.

        - Regional Materials: Materials that have been extracted, harvested, or recovered, as well as manufactured, within 100 miles (160 km) of Project site. If only a fraction of a product or material is extracted/harvested/recovered and manufactured locally, then only that percentage (by weight) shall contribute to the regional value.

      - PREINSTALLATION MEETINGS

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>. Review LEED requirements and action plans for compliance with requirements.

      - ADMINISTRATIVE REQUIREMENTS

Retain one of two paragraphs in this article. Retain first if Architect or Owner submits LEED documentation to USGBC; retain second if Contractor does.

        - Respond to questions and requests from Architect about USGBC's LEED prerequisites and credits that are Contractor's responsibility, that depend on product selection or product qualities, or that depend on Contractor's procedures, until USGBC has made its determination on Project's LEED certification application.

        - Submit documentation to USGBC and respond to questions and requests from USGBC about its LEED prerequisites and credits that are Contractor's responsibility, that depend on product selection or product qualities, or that depend on Contractor's procedures, until USGBC has made its determination on Project's LEED certification application.

          - Document correspondence with USGBC as informational submittals.

      - ACTION SUBMITTALS

Requirements in this article assume that Product Data and cost information are submitted to Architect, who then either fills out the LEED online forms or forwards the submittals to Owner or Sustainability Consultant to fill out the forms. Submittals include documentation needed to verify compliance with LEED requirements so that Architect, Owner, or Sustainability Consultant can be assured, when filling out the online forms, that requirements have been met. Although USGBC's LEED reference guides do not specifically identify who can certify compliance, submittals could be deleted if requirements in this Section are revised to require Contractor to fill out the online forms and to notify Architect or Owner when completed.

        - General: Submit additional sustainable design submittals required by other Sections.

Retain first paragraph below if separate sustainable design submittals are required.

        - Sustainable design submittals are in addition to other submittals.

          - If submitted item is identical to that proposed to comply with other requirements, include additional copy with other submittal as a record of compliance with indicated LEED instead of separate sustainable design submittal. Mark additional copy "Sustainable design submittal."

Submittals in "Sustainable Design Documentation Submittals" Paragraph below may be necessary to verify compliance with indicated USGBC LEED prerequisites and credits. Additional submittals may be required in other Sections.

        - Sustainable Design Documentation Submittals:

          - Environmental Product Declarations (EPDs) complying with LEED requirements.

          - Documentation for products that comply with LEED requirements for multi-attribute optimization.

            - Include documentation for regional materials, indicating location and distance from Project of material manufacturer and point of extraction, harvest, or recovery for each raw material and costs of regional materials.

          - Sustainability reports for products that comply with LEED requirements for raw material and source extraction reporting.

          - Documentation for products that comply with LEED requirements for leadership extraction practices. Include the following:

            - Product Data and certification letter from product manufacturers, indicating participation in an extended producer responsibility program and statement of costs.

            - Product Data and certification for bio-based materials, indicating that they comply with requirements. Include statement of costs.

            - Product Data and chain-of-custody certificates for products containing certified wood. Include invoices.

            - Receipts for salvaged and refurbished materials used for Project, indicating sources and costs.

            - Product Data and certification letter from product manufacturers, indicating percentages by weight of postconsumer and preconsumer recycled content for products having recycled content. Include statement of costs.

            - Documentation for regional materials, indicating location and distance from Project of material manufacturer and point of extraction, harvest, or recovery for each raw material and costs of regional materials.

          - Material ingredient reports for products that comply with LEED requirements for material ingredient reporting.

          - Documentation for products that comply with LEED requirements for material ingredient optimization.

          - Documentation for products that comply with LEED requirements for product manufacturer supply chain optimization.

            - Include documentation for regional materials, indicating location and distance from Project of material manufacturer and point of extraction, harvest, or recovery for each raw material and costs of regional materials.

          - Documentation complying with Section 017419 "Construction Waste Management and Disposal."

First six subparagraphs below are based on requiring all materials in a category to comply with minimum requirements. An alternative is the budget method. If the budget method is used, add a submittal showing calculations.

          - Product Data for adhesives and sealants used inside weatherproofing system, indicating VOC content and laboratory test reports showing compliance with requirements for low-emitting materials.

          - Product Data for paints and coatings used inside weatherproofing system, indicating VOC content and laboratory test reports showing compliance with requirements for low-emitting materials.

          - Laboratory test reports for flooring, indicating compliance with requirements for low-emitting materials.

          - Laboratory test reports for products containing composite wood or agrifiber products or wood glues, indicating compliance with requirements for low-emitting materials.

          - Laboratory test reports for ceilings, walls, and thermal insulation, indicating compliance with requirements for low-emitting materials.

          - Laboratory test reports for furniture, indicating compliance with requirements for low-emitting materials.

          - Construction Indoor-Air-Quality (IAQ) Management:

            - Construction IAQ management plan.

            - Product Data for temporary filtration media.

            - Product Data for filtration media used during occupancy.

            - Construction Documentation: Six photographs at three different times during construction period, along with brief description of SMACNA approach employed, documenting implementation of IAQ management measures, including protection of ducts and on-site stored or installed absorptive materials.

Delete "IAQ Assessment" Subparagraph below if using Owner-engaged IAQ testing to obtain Credit EQ IAQ Assessment.

          - IAQ Assessment:

Retain first two subparagraphs below if using building air flush-out procedures.

            - Signed statement describing the building air flush-out procedures, including dates when flush-out was begun and completed and statement that filtration media was replaced after flush-out.

            - Product Data for filtration media used during flush-out and during occupancy.

Retain subparagraph below if using Contractor-engaged IAQ testing.

            - Report from testing and inspecting agency, indicating results of IAQ testing and documentation that show compliance with IAQ testing procedures and requirements.

      - INFORMATIONAL SUBMITTALS

        - Qualification Data: For Sustainability Consultant.

        - Project Materials Cost Data: Provide statement indicating total cost for materials used for Project. Costs exclude labor, overhead, and profit. Include breakout of costs for the following categories of items:

          - Plumbing.

          - Mechanical.

          - Electrical.

          - Specialty items, such as elevators and equipment.

          - [Furniture.]

"Sustainable Design Action Plans" Paragraph below requires Contractor to make early submittals, indicating how certain LEED requirements will be met. This action can provide reassurance that Contractor understands LEED requirements and can clear up misunderstandings before they become a problem.

        - Sustainable Design Action Plans: Provide preliminary submittals within [seven] [14] [30] [60] <Insert number> days of date established for [commencement of the Work] [the Notice to Proceed] [the Notice of Award], indicating how the following requirements will be met:

          - List of proposed products with EPDs.

          - List of proposed products complying with requirements for multi-attribute optimization.

          - List of proposed products complying with requirements for raw material and source extraction reporting.

          - List of proposed products complying with requirements for leadership extraction practices.

          - List of proposed products complying with requirements for material ingredient reporting.

          - List of proposed products complying with requirements for material ingredient optimization.

          - List of proposed products complying with requirements for product manufacturer supply chain optimization.

          - Waste management plan complying with Section 017419 "Construction Waste Management and Disposal."

          - Construction IAQ management plan.

        - Sustainable Design Progress Reports: Concurrent with each Application for Payment, submit reports comparing actual construction and purchasing activities with sustainable design action plans.

      - QUALITY ASSURANCE

Requiring that Contractor engage a LEED AP to coordinate LEED requirements may help avoid inadvertent errors that could jeopardize Project's LEED certification if Contractors' personnel have limited experience.

        - Sustainability Consultant: Engage an experienced LEED Accredited Professional to coordinate LEED requirements. Sustainability Consultant may also serve as waste management coordinator.

- PRODUCTS

      - MATERIALS

Usually retain first paragraph below when requirements for salvaged and refurbished materials, recycled content, bio-based materials, or certified wood are included in other Sections. This action will help ensure that credit requirements are met if costs differ from Architect's estimates.

        - Provide products and procedures necessary to obtain LEED credits indicated as Contractor's responsibility. Although other Sections may specify some requirements that contribute to these LEED credits, Contractor shall provide additional materials and procedures necessary to obtain LEED credits indicated.

        - At least 20 different products from at least five different manufacturers shall have EPDs that comply with LEED requirements. Industry-wide (generic) EPDs shall be valued as one-half of a product.

        - At least 50 percent, by cost, of permanently installed products for Project shall comply with LEED requirements for multi-attribute optimization.

        - At least 20 different products from at least five different manufacturers shall have publicly released reports that comply with LEED requirements for raw material source and extraction reporting. Self-declared reports by manufacturers shall be valued as one-half of a product.

        - At least 20 different products from at least five different manufacturers shall comply with LEED requirements for material ingredient reporting.

        - At least 25 percent, by cost, of permanently installed products for Project shall comply with LEED requirements for material ingredient optimization.

        - At least 25 percent, by cost, of permanently installed products for Project shall comply with LEED requirements for product manufacturer supply chain optimization.

Retain first paragraph below and delete remaining paragraphs in this article if Contractor is responsible for complying with credit requirements for leadership extraction practices.

        - Not less than 25 percent of building materials, by cost, shall comply with LEED requirements for leadership extraction practices.

          - Structure and enclosure materials shall not be more than 30 percent, by cost, of materials used to comply with this requirement.

Retain one or more of "Extended Producer Responsibility Program," "Recycled Content," and "Certified Wood" paragraphs below, and possibly supplement by requirements in other Sections, for bio-based materials to comply with credit requirements for leadership extraction practices.

        - Extended Producer Responsibility Program: Not less than <Insert number> percent of building materials, by cost, shall be manufactured by a participant in an extended producer responsibility program.

"Recycled Content" Paragraph below is an alternative to requiring recycled content in Sections where such products are specified.

        - Recycled Content: Building materials shall have recycled content such that postconsumer recycled content plus one-half of preconsumer recycled content for Project constitutes a minimum of <Insert number> percent of cost of materials used for Project.

          - Cost of postconsumer recycled content plus one-half of preconsumer recycled content of an item shall be determined by dividing the weight of postconsumer recycled content plus one-half of preconsumer recycled content in the item by the total weight of the item and multiplying by cost of the item.

          - Do not include [furniture, ]plumbing, mechanical and electrical components, and specialty items, such as elevators and equipment, in the calculation.

"Certified Wood" Paragraph below is an alternative to requiring certified wood in the Specification Sections where wood products are specified.

        - Certified Wood: Not less than <Insert number> percent, by cost, of wood-based materials shall be produced from wood obtained from forests certified by an FSC-accredited certification body to comply with FSC STD-01-001.

      - LOW-EMITTING MATERIALS

This article contains requirements applicable to Credit EQ "Low-Emitting Materials," Option 1: "Product Category Calculations." If retaining this article, coordinate with Sections where relevant products are specified to avoid conflicting requirements in those Sections.

        - Paints and Coatings: For field applications that are inside weatherproofing system, paints and coatings shall comply with VOC content limits of authorities having jurisdiction and the following VOC content limits:

Categories in subparagraphs below are taken from USGBC's LEED Rating Systems and the standards referenced by them; if clarification is required, see those documents or LEED reference guides.

          - Flat Paints and Coatings: 50 g/L.

          - Nonflat Paints and Coatings: 50 g/L.

          - Dry-Fog Coatings: 150 g/L.

          - Primers, Sealers, and Undercoaters: 100 g/L.

          - Rust-Preventive Coatings: 100 g/L.

          - Zinc-Rich Industrial Maintenance Primers: 100 g/L.

          - Pretreatment Wash Primers: 420 g/L.

          - Clear Wood Finishes, Varnishes: 275 g/L.

          - Clear Wood Finishes, Lacquers: 275 g/L.

          - Floor Coatings: 50 g/L.

          - Shellacs, Clear: 730 g/L.

          - Shellacs, Pigmented: 550 g/L.

          - Stains: 100 g/L.

        - Paints and Coatings: For field applications that are inside weatherproofing system, 90 percent of paints and coatings shall comply with requirements of California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

Adhesives and sealants are required in many Sections where, without being specifically mentioned, they may be specified only by requirement to install products according to manufacturer's written instructions. For this reason, it is probably best to retain both "Adhesives and Sealants" paragraphs below if this credit is required.

        - Adhesives and Sealants: For field applications that are inside weatherproofing system, paints and coatings shall comply with VOC content limits of authorities having jurisdiction and the following VOC content limits:

Categories in subparagraphs below are taken from USGBC's LEED Rating Systems and the standards referenced by them; if clarification is required, see those documents or LEED reference guides.

          - Wood Glues: 30 g/L.

          - Metal-to-Metal Adhesives: 30 g/L.

          - Adhesives for Porous Materials (except Wood): 50 g/L.

          - Subfloor Adhesives: 50 g/L.

          - Plastic Foam Adhesives: 50 g/L.

          - Carpet Adhesives: 50 g/L.

          - Carpet Pad Adhesives: 50 g/L.

          - VCT and Asphalt Tile Adhesives: 50 g/L.

          - Cove Base Adhesives: 50 g/L.

          - Gypsum Board and Panel Adhesives: 50 g/L.

          - Rubber Floor Adhesives: 60 g/L.

          - Ceramic Tile Adhesives: 65 g/L.

          - Multipurpose Construction Adhesives: 70 g/L.

          - Fiberglass Adhesives: 80 g/L.

          - Contact Adhesive: 80 g/L.

          - Structural Glazing Adhesives: 100 g/L.

          - Wood Flooring Adhesive: 100 g/L.

          - Structural Wood Member Adhesive: 140 g/L.

          - Single-Ply Roof Membrane Adhesive: 250 g/L.

          - Special-Purpose Contact Adhesive (Used to Bond Melamine-Covered Board, Metal, Unsupported Vinyl, Rubber, or Wood Veneer 1/16 Inch (1.6 mm) or Less in Thickness to Any Surface): 250 g/L.

          - Top and Trim Adhesive: 250 g/L.

          - Plastic Cement Welding Compounds: 250 g/L.

          - ABS Welding Compounds: 325 g/L.

          - CPVC Welding Compounds: 490 g/L.

          - PVC Welding Compounds: 510 g/L.

          - Adhesive Primer for Plastic: 550 g/L.

          - Sheet-Applied Rubber Lining Adhesive: 850 g/L.

          - Aerosol Adhesive, General-Purpose Mist Spray: 65 percent by weight.

          - Aerosol Adhesive, General-Purpose Web Spray: 55 percent by weight.

          - Special-Purpose Aerosol Adhesive (All Types): 70 percent by weight.

          - Other Adhesives: 250 g/L.

          - Architectural Sealants: 250 g/L.

          - Nonmembrane Roof Sealants: 300 g/L.

          - Single-Ply Roof Membrane Sealants: 450 g/L.

          - Other Sealants: 420 g/L.

          - Sealant Primers for Nonporous Substrates: 250 g/L.

          - Sealant Primers for Porous Substrates: 775 g/L.

          - Modified Bituminous Sealant Primers: 500 g/L.

          - Other Sealant Primers: 750 g/L.

        - Adhesives and Sealants: For field applications that are inside weatherproofing system, 90 percent of adhesives and sealants shall comply with requirements of California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

        - Flooring: Shall comply with requirements of California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

If retaining "Composite Wood, Agrifiber Products, and Adhesives" Paragraph below, coordinate with Sections where composite wood products are specified to avoid conflicting requirements in those Sections.

        - Composite Wood, Agrifiber Products, and Adhesives: Shall be made using ultra-low-emitting formaldehyde resins as defined in California Air Resources Board's "Airborne Toxic Control Measure to Reduce Formaldehyde Emissions from Composite Wood Products" or shall me made with no added formaldehyde.

        - Ceilings, Walls, and Thermal Insulation: Shall comply with requirements of California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

- EXECUTION

      - NONSMOKING BUILDING

        - Smoking is not permitted within the building or within 25 ft. (8 m) of entrances, operable windows, or outdoor-air intakes.

      - CONSTRUCTION WASTE MANAGEMENT

        - Comply with Section 017419 "Construction Waste Management and Disposal."

      - CONSTRUCTION INDOOR-AIR-QUALITY (IAQ) MANAGEMENT

        - Comply with SMACNA's "SMACNA IAQ Guideline for Occupied Buildings under Construction."

Coordinate first subparagraph below with Section 015000 "Temporary Facilities and Controls." Identify air handlers and associated return-air inlets authorized by Owner for use during construction period.

          - If Owner authorizes use of permanent heating, cooling, and ventilating systems during construction period as specified in Section 015000 "Temporary Facilities and Controls," install MERV 8 filter media at each return-air inlet for the air-handling system used during construction.

          - Replace air filters immediately prior to occupancy with new filters specified in Section 234100 "Particulate Air Filtration."

      - INDOOR-AIR-QUALITY (IAQ) ASSESSMENT

        - Flush-Out:

Project's mechanical engineer of record should verify that HVAC system design and equipment indicated are capable of delivering flush-out indicated. Provide HVAC system and equipment operating information necessary to achieve credit. If Project HVAC systems and equipment cannot suit requirement, consider requiring temporary systems and equipment.

          - After construction ends, prior to occupancy and with all interior finishes installed, perform a building flush-out by supplying a total volume of 14,000 cu. ft. (4 300 000 L) of outdoor air per sq. ft. (sq. m) of floor area while maintaining an internal temperature of at least 60 deg F (16 deg C) and a relative humidity of no higher than 60 percent.

In first subparagraph below, indicate operating procedure for each HVAC system and piece of equipment and the operating duration required for flush-out.

            - <Insert operating requirements>.

          - If occupancy is desired prior to flush-out completion, the space may be occupied following delivery of a minimum of 3500 cu. ft. (1 070 000 L) of outdoor air per sq. ft. (sq. m) of floor area to the space. Once a space is occupied, it shall be ventilated at a minimum rate of 0.30 cfm per sq. ft. (1.52 L/s per sq. m) of outside air or the design minimum outside air rate, whichever is greater. During each day of the flush-out period, ventilation shall begin a minimum of three hours prior to occupancy and continue during occupancy. These conditions shall be maintained until a total of 14,000 cu. ft./sq. ft. (4 300 000 L/sq. m) of outside air has been delivered to the space.

In subparagraph below, indicate operating procedure for each HVAC system and piece of equipment and the operating duration required for flush-out.

            - <Insert operating requirements>.

        - Air-Quality Testing: [Engage] [Owner will engage] testing agency to perform the following:

The EPA standard referenced in first subparagraph below is available from NTIS, with PB90200288 ordering number; (800) 553-6847.

          - Conduct baseline IAQ testing, after construction ends and prior to occupancy, using testing protocols consistent with the EPA's "Compendium of Methods for the Determination of Air Pollutants in Indoor Air," and as additionally detailed in USGBC's "LEED Reference Guide for Interior Design and Construction v4."

          - Demonstrate that contaminant maximum concentrations listed below are not exceeded:

Note that it may be necessary to comply with all requirements in Credit EQ "Low-Emitting Materials" to comply with limits specified in first seven subparagraphs below.

            - Formaldehyde: 27 ppb.

            - Particulates (PM10): 50 mcg/cu. m.

            - Ozone: 0.075 ppm, according to ASTM D5149.

            - Total Volatile Organic Compounds (TVOC): 500 mcg/cu. m.

            - 4-Phenylcyclohexene (4-PH): 6.5 mcg/cu. m.

            - Carbon Monoxide: 9 ppm and no greater than 2 ppm above outdoor levels.

            - Target Chemicals in California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers," Table 4-1 (except formaldehyde).

First subparagraph below is part of LEED credit requirements but creates requirements that are contingent on field conditions that are unknown at time of bid and could influence bids. Also, requirements may result in claims for extra payment and time. Revise to suit office practice.

          - For each sampling point where maximum concentration limits are exceeded, take corrective action until requirements have been met.

          - Air-sample testing shall be conducted as follows:

            - All measurements shall be conducted prior to occupancy but during normal occupied hours, and with building ventilation system starting at the normal daily start time and operated at the minimum outside airflow rate for the occupied mode throughout the duration of the air testing.

            - Building shall have all interior finishes installed including, but not limited to, millwork, doors, paint, carpet, and acoustic tiles. Nonfixed furnishings, such as workstations and partitions, are required to be in place for the testing.

            - Number of sampling locations varies depending on the size of building and number of ventilation systems. For each portion of building served by a separate ventilation system, the number of sampling points shall not be less than one per 5000 sq. ft. (465 sq. m).

            - Air samples shall be collected between 3 and 6 ft. (0.9 and 1.8 m) from the floor to represent the breathing zone of occupants, and over a minimum four-hour period.

END OF SECTION 018113.17