Copyright 2021 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 061753 - SHOP-FABRICATED WOOD TRUSSES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, LEED v4.1, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Wood products.

          - Preservative-treated lumber.

          - Fire-retardant-treated lumber.

      - ALLOWANCES

Retain this article if an allowance is specified for permanent bracing, and indicate in Section 012100 "Allowances" that allowance includes installation as well as materials. Delete if permanent bracing is shown on Drawings.

        - Provide wood truss bracing under the Metal-Plate-Connected Truss Bracing Allowance as specified in Section 012100 "Allowances."

      - DEFINITIONS

        - Metal-Plate-Connected Wood Trusses: Planar structural units consisting of metal-plate-connected members fabricated from dimension lumber and cut and assembled before delivery to Project site.

      - ACTION SUBMITTALS

        - Product Data: For [wood-preservative-treated lumber,] [fire-retardant-treated lumber,] metal-plate connectors, metal truss accessories, and fasteners.

          - Include data for wood-preservative treatment from chemical treatment manufacturer and certification from treating plant that treated materials comply with requirements. Indicate type of preservative used and net amount of preservative retained.

          - Include data for fire-retardant treatment from chemical-treatment manufacturer and certification from treating plant that treated materials comply with requirements. Include physical properties of treated materials based on testing by a qualified independent testing agency.

          - For fire-retardant treatments, include physical properties of treated lumber both before and after exposure to elevated temperatures, based on testing by a qualified independent testing agency in accordance with ASTM D5664.

          - For products receiving a waterborne treatment, include statement that moisture content of treated materials was reduced to levels specified before shipment to truss fabricator.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for regional materials.>

          - <Double click to insert sustainable design text for certified wood.>

          - <Double click to insert sustainable design text for certified wood.>

        - Shop Drawings: Show fabrication and installation details for trusses.

          - Show location, pitch, span, camber, configuration, and spacing for each type of truss required.

          - Indicate sizes, stress grades, and species of lumber.

In accordance with TPI 1, building designer is responsible for "permanent bracing design for the building, including bracing to resist wind, seismic, or other lateral forces and permanent bracing for all structural elements and trusses," but the truss design must show the "approximate location for or maximum spacing between permanent lateral bracing of truss members." If building designer is responsible for design of bracing to prevent buckling of compression members, this bracing should be detailed on Drawings, subject to possible revision when truss Shop Drawings are received, or an allowance for it must be included in the Contract Sum if a Change Order is to be avoided. See the Evaluations.

          - Indicate locations of permanent bracing required to prevent buckling of individual truss members due to design loads.

According to the Council of American Structural Engineers (CASE), building designer is responsible for design of "primary structural system," but truss designer is responsible for design of "bracing needed to resist buckling of compression truss members," because this depends on truss design. Retain first subparagraph below instead of last subparagraph above if truss designer is required to design bracing of compression members.

          - Indicate locations, sizes, and materials for permanent bracing required to prevent buckling of individual truss members due to design loads.

          - Indicate type, size, material, finish, design values, orientation, and location of metal connector plates.

          - Show splice details and bearing details.

Retain "Delegated Design Submittals" Paragraph below if design services have been delegated to Contractor.

        - Delegated Design Submittals: For metal-plate-connected wood trusses indicated to comply with performance requirements and design criteria, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data: For [metal connector-plate manufacturer] [professional engineer] [and] [fabricator].

Retain "Material Certificates" Paragraph below to require submittal of material certificates from manufacturers.

        - Material Certificates: For dimension lumber specified to comply with minimum specific gravity. Indicate species and grade selected for each use and specific gravity.

Retain "Product Certificates" Paragraph below if certificates are required by authorities having jurisdiction. The IBC waives requirement for special inspection if fabricator is approved and certifies compliance with the approved construction documents.

        - Product Certificates: For metal-plate-connected wood trusses, signed by officer of truss-fabricating firm.

Insert specific model code organization in "Evaluation Reports" Paragraph below or revise if report must be from another source.

        - Evaluation Reports: For the following, from ICC-ES:

Revise list below to retain only those products retained in Part 2.

          - Wood-preservative-treated lumber.

          - Fire-retardant-treated wood.

          - Metal-plate connectors.

          - Metal truss accessories.

      - QUALITY ASSURANCE

        - Metal Connector-Plate Manufacturer Qualifications: A manufacturer that is a member of TPI and that complies with quality-control procedures in TPI 1 for manufacture of connector plates.

          - Manufacturer's responsibilities include providing professional engineering services needed to assume engineering responsibility.

          - Engineering Responsibility: Preparation of Shop Drawings and comprehensive engineering analysis by a qualified professional engineer.

TPI 1 added specific requirements for fabricator quality-assurance programs in the 2002 version, but does not require third-party inspection. Verify that local truss fabricators participate in third-party inspection programs; many do not. Third-party inspection is required by the IBC to be an approved fabricator and to waive requirement for special inspection. Retain third option in "Fabricator Qualifications" Paragraph below if required for LEED.

        - Fabricator Qualifications: Shop that [participates in a recognized quality-assurance program, complies with quality-control procedures in TPI 1, and involves third-party inspection by an independent testing and inspecting agency acceptable to Architect and authorities having jurisdiction] [and] [is certified for chain of custody by an FSC-accredited certification body].

        - Testing Agency Qualifications: For testing agency providing classification marking for fire-retardant-treated material, an inspection agency acceptable to authorities having jurisdiction that periodically performs inspections to verify that the material bearing the classification marking is representative of the material tested.

        - <Double click to insert sustainable design text for manufacturer qualifications.>

        - <Double click to insert sustainable design text for vendor qualifications.>

      - DELIVERY, STORAGE, AND HANDLING

        - Handle and store trusses to comply with recommendations in SBCA BCSI, "Building Component Safety Information: Guide to Good Practice for Handling, Installing, Restraining, & Bracing Metal Plate Connected Wood Trusses."

          - Store trusses flat, off of ground, and adequately supported to prevent lateral bending.

          - Protect trusses from weather by covering with waterproof sheeting, securely anchored.

          - Provide for air circulation around stacks and under coverings.

        - Inspect trusses showing discoloration, corrosion, or other evidence of deterioration. Discard and replace trusses that are damaged or defective.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

        - Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design metal-plate-connected wood trusses.

        - Structural Performance: Metal-plate-connected wood trusses are to be capable of withstanding design loads within limits and under conditions indicated. Comply with requirements in TPI 1 unless more stringent requirements are specified below.

Tabulate minimum load requirements here or on Drawings. Revise "Design Loads" Subparagraph below when design loads are included here. Include applicable live, dead, snow, collateral, seismic, wind, and uplift loads, and load combinations.

          - Design Loads: As indicated.

          - Maximum Deflection under Design Loads:

Retain deflection limits from options in "Roof Trusses" Subparagraph below or insert others as appropriate for floor, roof, and ceiling materials.

            - Roof Trusses: Vertical deflection of [1/180] [1/240] [1/360] of span.

Insert a subparagraph for horizontal (longitudinal) deflection limits if using scissor trusses.

            - Floor Trusses: Vertical deflection of [1/360] [1/480] [1/600] of span.

TPI publications listed below are by title without alphanumeric designations in which the number represents year of issue. Designations in effect when this Section was updated appear in "Referenced Standards" Article in the Evaluations.

        - Comply with applicable requirements and recommendations of TPI 1, TPI DSB, and SBCA BCSI.

        - Wood Structural Design Standard: Comply with applicable requirements in AF&PA's "National Design Specifications for Wood Construction" and its "Supplement."

        - <Double click to insert sustainable design text for regional wood materials.>

        - <Double click to insert sustainable design text for certified wood.>

      - WOOD PRODUCTS

        - Lumber: DOC PS 20 and applicable rules of any rules-writing agency certified by the American Lumber Standard Committee (ALSC) Board of Review. Provide lumber graded by an agency certified by the ALSC Board of Review to inspect and grade lumber under the rules indicated.

          - Factory mark each piece of lumber with grade stamp of grading agency.

Delete first subparagraph below if authorities having jurisdiction require grade stamps on all materials.

          - For exposed lumber indicated to receive a stained or natural finish, omit grade stamp and provide certificates of grade compliance issued by grading agency.

          - Provide dressed lumber, S4S.

Retain one of two options in subparagraph below. Retain the first unless lumber with 15 percent maximum moisture content is unavailable.

          - Provide dry lumber with [15] [19] percent maximum moisture content at time of dressing.

Retain "Minimum Chord Size for Roof Trusses" Paragraph below if requirement for minimum chord sizes is needed to provide stiffer members for nailing.

        - Minimum Chord Size for Roof Trusses: [2 by 6 inches nominal (38 by 140 mm actual) for top chords] [2 by 6 inches nominal (38 by 140 mm actual) for bottom chords] [2 by 6 inches nominal (38 by 140 mm actual) for both top and bottom chords].

Retain "Minimum Specific Gravity for Top Chords" Paragraph below if required for diaphragm construction.

        - Minimum Specific Gravity for Top Chords: [0.50] <Insert value>.

        - Permanent Bracing: Provide wood bracing that complies with requirements for miscellaneous lumber in Section 061000 "Rough Carpentry."

      - PRESERVATIVE-TREATED LUMBER

Preservative treatment is usually limited to wood exposed in wet and humid locations or geographical areas where termite infestation is extensive. See the Evaluations for discussion of treatment chemicals. Some treatment chemicals increase rate of corrosion of galvanized truss plates.

        - Preservative Treatment by Pressure Process: AWPA U1; Use Category UC2[ for interior construction not in contact with the ground, Use Category UC3b for exterior construction not in contact with the ground, and Use Category UC4a for items in contact with the ground].

          - Preservative Chemicals: Acceptable to authorities having jurisdiction and containing no arsenic or chromium.

Delete subparagraph below if not using exposed trusses or if considered unnecessary.

          - For exposed trusses indicated to receive a stained or natural finish, use chemical formulations that do not require incising, contain colorants, bleed through, or otherwise adversely affect finishes.

        - Kiln-dry lumber after treatment to a maximum moisture content of 19 percent. Do not use material that is warped or does not comply with requirements for untreated material.

        - Mark lumber with treatment quality mark of an inspection agency approved by the ALSC Board of Review.

Retain only first option in subparagraph below if authorities having jurisdiction require quality mark on all materials.

          - For exposed trusses indicated to receive a stained or natural finish, [mark end or back of each piece] [or] [omit marking and provide certificates of treatment compliance issued by inspection agency].

Retain first option in "Application" Paragraph below if total treatment is required; otherwise, retain second.

        - Application: Treat [all trusses unless otherwise indicated] [trusses where indicated on Drawings].

      - FIRE-RETARDANT-TREATED LUMBER

See the Evaluations for discussion of fire-retardant-treated wood. Verify availability with truss fabricators.

        - General: Where fire-retardant-treated materials are indicated, use materials complying with requirements in this article, that are acceptable to authorities having jurisdiction, and with fire-test-response characteristics specified as determined by testing identical products according to test method indicated by a qualified testing agency.

        - Fire-Retardant-Treated Lumber by Pressure Process: Products with a flame-spread index of 25 or less when tested in accordance with ASTM E84, with no evidence of significant progressive combustion when the test is extended an additional 20 minutes, and with the flame front not extending more than 10.5 feet (3.2 m) beyond the centerline of the burners at any time during the test.

          - Use treatment that does not promote corrosion of metal fasteners.

Exterior type is suitable for both exterior and interior applications. Interior type is only for interior applications.

          - Exterior Type: Treated materials are to comply with requirements specified above for fire-retardant-treated lumber and plywood by pressure process after being subjected to accelerated weathering in accordance with ASTM D2898. Use for exterior locations and where indicated.

          - Interior Type A: Treated materials are to have a moisture content of 28 percent or less when tested in accordance with ASTM D3201 at 92 percent relative humidity. Use for interior locations where exterior type is not indicated.

Delete option in "Design Value Adjustment Factors" Subparagraph below if not applicable. Revise description of locations to suit Project. Verify adjustment factors with Project's structural engineer.

          - Design Value Adjustment Factors: Treated lumber is to be tested in accordance with ASTM D5664, and design value adjustment factors is to be calculated in accordance with ASTM D6841.[ For enclosed roof framing and framing in attic spaces, and where high-temperature fire-retardant treatment is indicated, provide material with adjustment factors of not less than 0.85 modulus of elasticity and 0.75 for extreme fiber in bending for Project's climatological zone.]

Pressure-treated lumber that is dried after treatment is typically only dried to 19 percent, which may be inadequate for trusses. Usually retain first option in first paragraph below, but only after verifying availability.

        - Kiln-dry lumber after treatment to a maximum moisture content of [15] [19] percent.

        - Identify fire-retardant-treated wood with appropriate classification marking of testing and inspecting agency acceptable to authorities having jurisdiction.

Retain only first option in subparagraph below if authorities having jurisdiction require classification marking on all materials.

          - For exposed trusses and bracing indicated to receive a stained or natural finish, [mark end or back of each piece] [or] [omit marking and provide certificates of treatment compliance issued by inspection agency].

Delete or revise first paragraph below if no exposed trusses or if staining hides colorants.

        - For exposed trusses indicated to receive a stained or natural finish, use chemical formulations that do not bleed through, contain colorants, or otherwise adversely affect finishes.

Retain first option in "Application" Paragraph below and delete subparagraphs if all trusses are required to be fire-retardant treated; otherwise, retain second option and appropriate subparagraphs.

        - Application: Treat [all trusses unless otherwise indicated.] [items indicated on Drawings, and the following:]

Revise list below to suit local code and Project.

          - Floor trusses.

          - Roof trusses.

          - <Insert category of trusses required to be treated>.

      - METAL CONNECTOR PLATES

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Fabricate connector plates to comply with TPI 1.

        - Hot-Dip Galvanized-Steel Sheet: ASTM A653/A653M; Structural Steel (SS), high-strength low-alloy steel Type A (HSLAS Type A), or high-strength low-alloy steel Type B (HSLAS Type B); G60 (Z180) coating designation; and not less than 0.036 inch (0.9 mm) thick.

          - Use for interior locations unless otherwise indicated.

Verify availability of connectors with heavy galvanized coating before retaining "Hot-Dip Heavy-Galvanized-Steel Sheet" Paragraph below.

        - Hot-Dip Heavy-Galvanized-Steel Sheet: ASTM A653/A653M; Structural Steel (SS), high-strength low-alloy steel Type A (HSLAS Type A), or high-strength low-alloy steel Type B (HSLAS Type B); G185 (Z550) coating designation; and not less than 0.036 inch (0.9 mm) thick.

          - Use for wood-preservative-treated lumber and where indicated.

Type 304 stainless steel is usually standard; Type 316 gives better corrosion resistance for exposed applications in coastal environments.

        - Stainless Steel Sheet: ASTM A240/A240M or ASTM A666, [Type 304] [Type 316], and not less than 0.035 inch (0.88 mm) thick.

          - Use for exterior locations[, wood-preservative-treated lumber,] [fire-retardant treated lumber,] and where indicated.

      - FASTENERS

        - Provide fasteners of size and type indicated that comply with requirements specified in this article for material and manufacture.

          - Provide fasteners for use with metal framing anchors that comply with written recommendations of metal framing manufacturer.

          - Where trusses are exposed to weather, in ground contact, made from pressure-preservative treated wood, or in area of high relative humidity, provide fasteners [with hot-dip zinc coating complying with ASTM A153/A153M] [of Type 304 stainless steel].

        - Nails, Brads, and Staples: ASTM F1667.

      - METAL FRAMING ANCHORS AND ACCESSORIES

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Allowable design loads, as published by manufacturer, are to comply with or exceed those [indicated] [of basis-of-design products] [of products of manufacturers listed]. Manufacturer's published values are to be determined from empirical data or by rational engineering analysis and demonstrated by comprehensive testing performed by a qualified independent testing agency. Framing anchors are to be punched for fasteners adequate to withstand same loads as framing anchors.

Galvanized steel is typical for most manufacturers and is suitable for most applications.

        - Galvanized-Steel Sheet: Hot-dip, zinc-coated steel sheet complying with ASTM A653/A653M, G60 (Z180) coating designation.

          - Use for interior locations unless otherwise indicated.

        - Hot-Dip Heavy-Galvanized-Steel Sheet: ASTM A653/A653M; Structural Steel (SS), high-strength low-alloy steel Type A (HSLAS Type A), or high-strength low-alloy steel Type B (HSLAS Type B); G185 (Z550) coating designation; and not less than 0.036 inch (0.9 mm) thick.

          - Use for wood-preservative-treated lumber and where indicated.

Retain "Stainless Steel Sheet" Paragraph below if required. Type 304 is usually standard for stainless steel; Type 316 gives better corrosion resistance for exposed applications in coastal environments.

        - Stainless Steel Sheet: ASTM A240/A240M or ASTM A666, [Type 304] [Type 316].

          - Use for exterior locations and where indicated.

Remaining paragraphs below are examples only. Revise to suit Project or delete all if "Basis-of-Design Products" Paragraph above is used and they are not needed to provide salient characteristics for products.

"Truss Tie-Downs" Paragraph below, including option, is based on Simpson's "H-2."

        - Truss Tie-Downs: Bent strap tie for fastening roof trusses to wall studs below, 1-1/2 inches (38 mm) wide by 0.050 inch (1.3 mm) thick.[ Tie fastens to one side of truss, top plates, and side of stud below.]

"Truss Tie-Downs (Hurricane or Seismic Ties)" Paragraph below is based on Simpson's "H-7."

        - Truss Tie-Downs (Hurricane or Seismic Ties): Bent strap tie for fastening roof trusses to wall studs below, 2-1/4 inches (57 mm) wide by 0.062 inch (1.6 mm) thick. Tie fits over top of truss and fastens to both sides of truss, top plates, and one side of stud below.

        - Roof Truss Clips: Angle clips for bracing bottom chord of roof trusses at non-load-bearing walls, 1-1/4 inches (32 mm) wide by 0.050 inch (1.3 mm) thick. Clip is fastened to truss through slotted holes to allow for truss deflection.

        - Floor Truss Hangers: U-shaped hangers, full depth of floor truss, with 1-3/4-inch- (44-mm-) long seat; formed from metal strap 0.062 inch (1.6 mm) thick with tabs bent to extend over and be fastened to supporting member.

Description in "Roof Truss Bracing/Spacers" Paragraph below is based on MiTek's "Stabilizer."

        - Roof Truss Bracing/Spacers: U-shaped channels, 1-1/2 inches (38 mm) wide by 1 inch (25 mm) deep by 0.040 inch (1.0 mm) thick, made to fit between two adjacent trusses and accurately space them apart, and with tabs having metal teeth for fastening to trusses.

        - Drag Strut Connectors: Angle clip with one leg extended for fastening to the side of girder truss.

Description in two subparagraphs below are based on Simpson's "DSC2" and "DSC5," respectively.

          - Angle clip is 3 by 3 by 0.179 by 8 inches (76 by 76 by 4.55 by 203 mm) with extended leg 8 inches (203 mm) long. Connector has galvanized finish.

          - Angle clip is 3 by 3 by 0.239 by 10-1/2 inches (76 by 76 by 6.07 by 267 mm) with extended leg 10-1/2 inches (267 mm) long. Connector has painted finish.

      - MISCELLANEOUS MATERIALS

        - Galvanizing Repair Paint: SSPC-Paint 20, with dry film containing a minimum of 92 percent zinc dust by weight.

      - FABRICATION

        - Cut truss members to accurate lengths, angles, and sizes to produce close-fitting joints.

        - Fabricate metal connector plates to sizes, configurations, thicknesses, and anchorage details required to withstand design loads for types of joint designs indicated.

        - Assemble truss members in design configuration indicated; use jigs or other means to ensure uniformity and accuracy of assembly, with joints closely fitted to comply with tolerances in TPI 1. Position members to produce design camber indicated.

Manufacturing tolerances permitted by TPI 1 vary according to length and height of trusses as follows. Length: 1/2 inch (13 mm) up to 30 feet (9.14 m) long, thereafter 3/4 inch (19 mm). Height: 1/4 inch (6.4 mm) up to 60 inches (1524 mm) high, thereafter 1/2 inch (13 mm).

          - Fabricate wood trusses within manufacturing tolerances in TPI 1.

        - Connect truss members by metal connector plates located and securely embedded simultaneously in both sides of wood members by air or hydraulic press.

      - SOURCE QUALITY CONTROL

Retain this article if special inspection is required. Consider deleting if requiring that fabricator participates in a recognized quality-assurance program, which complies with quality-control procedures in TPI 1 and involves third-party inspection by an independent testing and inspecting agency, and if authorities having jurisdiction approve fabrication work without special inspections. Coordinate with "Fabricator Qualifications" Paragraph in "Quality Assurance" Article.

        - Special Inspections: Owner will engage a qualified special inspector to perform special inspections.

          - Provide special inspector with access to fabricator's documentation of detailed fabrication and quality-control procedures that provide a basis for inspection control of the workmanship and the fabricator's ability to conform to approved construction documents and referenced standards.

          - Provide special inspector with access to places where wood trusses are being fabricated to perform inspections.

        - Correct deficiencies in Work that special inspections indicate do not comply with the Contract Documents.

- EXECUTION

      - INSTALLATION

        - Install wood trusses only after supporting construction is in place and is braced and secured.

        - If trusses are delivered to Project site in more than one piece, assemble trusses before installing.

        - Hoist trusses in place by lifting equipment suited to sizes and types of trusses required, exercising care not to damage truss members or joints by out-of-plane bending or other causes.

        - Install and brace trusses according to TPI recommendations and as indicated.

        - Install trusses plumb, square, and true to line and securely fasten to supporting construction.

        - Space trusses [16 inches (406 mm) o.c.] [24 inches (610 mm) o.c.] [as indicated]; adjust and align trusses in location before permanently fastening.

        - Anchor trusses securely at bearing points; use metal truss tie-downs or floor truss hangers as applicable. Install fasteners through each fastener hole in metal framing anchors according to manufacturer's fastening schedules and written instructions.

Retain first paragraph below if built-up girder trusses are required. TPI 1 states it is truss designer's responsibility to design truss-to-girder connection.

        - Securely connect each truss ply required for forming built-up girder trusses.

          - Anchor trusses to girder trusses as indicated.

        - Install and fasten permanent bracing during truss erection and before construction loads are applied. Anchor ends of permanent bracing where terminating at walls or beams.

          - Install bracing to comply with Section 061000 "Rough Carpentry."

Retain subparagraph below if floor trusses are required.

          - Install and fasten strongback bracing vertically against vertical web of parallel-chord floor trusses at centers indicated.

TPI 1 permits out-of-plumb tolerance of the lesser of D/50 or 2 inches (50 mm) maximum. Out-of-plane tolerances or bow is limited to the lesser of L/200 or 2 inches (50 mm) maximum. Location variances of 1/4 inch (6.4 mm) and a top-chord bearing gap of 1/2 inch (13 mm) for parallel-chord trusses are also permitted.

        - Install wood trusses within installation tolerances in TPI 1.

        - Do not alter trusses in field. Do not cut, drill, notch, or remove truss members.

        - Replace wood trusses that are damaged or do not comply with requirements.

          - Damaged trusses may be repaired according to truss repair details signed and sealed by the qualified professional engineer responsible for truss design, when approved by Architect.

      - REPAIRS AND PROTECTION

        - Protect wood that has been treated with inorganic boron (SBX) from weather. If, despite protection, inorganic boron-treated wood becomes wet, apply EPA-registered borate treatment. Apply borate solution by spraying to comply with EPA-registered label.

Retain first paragraph below instead of paragraph above if boron-treated wood is not used, but borate treatment of wood that has become wet is used to help prevent mold and mildew. Delete if site-applied boron treatment is specified in Section 313116 "Termite Control."

        - Protect wood trusses from weather. If, despite protection, wood trusses become wet, apply EPA-registered borate treatment. Apply borate solution by spraying to comply with EPA-registered label.

        - Repair damaged galvanized coatings on exposed surfaces in accordance with ASTM A780/A780M and manufacturer's written instructions.

      - FIELD QUALITY CONTROL

        - Special Inspections: Owner will engage a qualified special inspector to perform special inspections to verify that temporary installation restraint/bracing and the permanent individual truss member restraint/bracing are installed in accordance with the approved truss submittal package.

END OF SECTION 061753