Copyright 2022 by The American Institute of Architects (AIA)

Exclusively published and distributed Deltek, Inc. for the AIA

SECTION 064219 - PLASTIC-LAMINATE-FACED WOOD PANELING

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, LEED 4.1, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

Review requirements of the Architectural Woodwork Institute (AWI), Architectural Woodwork Manufacturers Association of Canada (AWMAC), and Woodwork Institute's (WI) "Architectural Woodwork Standards" (hereafter, AWS) before revising this Section.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Plastic-laminate-faced wood paneling.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 057000 "Decorative Metal" for metal reveals at plastic-laminate-faced wood paneling.

          - Section 061000 "Rough Carpentry" for wood furring, blocking, shims, and hanging strips required for installing paneling that is concealed within other construction before paneling installation.

      - COORDINATION

        - Coordinate sizes and locations of framing, blocking, furring, reinforcements, and other related units of Work specified in other Sections to ensure that paneling can be installed as indicated.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Plastic-laminate-faced wood paneling.

        - Product Data Submittals: For each product.

Retain subparagraph below for fire-retardant-treated materials.

          - Include data for fire-retardant treatment from chemical-treatment manufacturer and certification by treating plant that treated materials comply with requirements.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

          - <Double click to insert sustainable design text for regional materials.>

          - <Double click to insert sustainable design text for certified wood.>

          - <Double click to insert sustainable design text for adhesives.>

          - <Double click to insert sustainable design text for composite wood.>

          - <Double click to insert sustainable design text for installation adhesives.>

        - Shop Drawings: For plastic-laminate-faced wood paneling.

          - Include plans, elevations, sections, and attachment details.

Retain first subparagraph below for ornate or complex work.

          - Show details full size.

          - Show locations and sizes of furring and blocking, including concealed blocking specified in other Sections.

Retain subparagraph below and applicable option if retaining other requirements for AWI's or WI's quality certification program.

          - Apply [AWI Quality Certification] [WI Certified Compliance] Program label to Shop Drawings.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples: For each exposed product and for each color and texture specified, in manufacturer's or fabricator's standard size.

        - Samples for Initial Selection: For each type of plastic laminate.

        - Samples for Verification: For each type of exposed laminate, [8 by 10 inches (200 by 250 mm)] [12 by 12 inches (300 by 300 mm)].

          - Provide one Sample applied to core material[ and with specified edge material applied to one edge].

      - INFORMATIONAL SUBMITTALS

Informational submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data: For [Installer] [fabricator].

Retain "Product Certificates" Paragraph below to require submittal of product certificates from manufacturers.

        - Product Certificates: For each type of product.

        - Quality Standard Compliance Certificates: [AWI Quality Certification Program] [WI Certified Compliance Program].

Retain "Evaluation Reports" Paragraph below for fire-retardant-treated materials.

        - Evaluation Reports: For fire-retardant-treated materials, from ICC-ES.

      - QUALITY ASSURANCE

        - Fabricator Qualifications: Shop that employs skilled workers who custom-fabricate products similar to those required for this Project and whose products have a record of successful in-service performance.

Retain "Shop Certification" Subparagraph below if AWI quality certification or WI compliance certification is required. Both AWI and WI will inspect work and provide certification for work that passes inspection if fabricator is not certified/licensed.

          - Shop Certification: [AWI's Quality Certification Program accredited participant] [WI's Certified Compliance Program licensee].

Retain "Installer Qualifications" Paragraph below if required. If AWI Quality Certification Program labels or certificates are required for installation, Installer must be a certified participant. If paneling must be FSC certified, fabricator must install paneling or be FSC certified for chain of custody.

        - Installer Qualifications: [Fabricator of products] [AWI's Quality Certification Program accredited participant] [WI's Certified Compliance Program licensee].

        - Mockups: Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for materials and execution.

          - Build mockups of [typical paneling as indicated on Drawings] <Insert description>.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - DELIVERY, STORAGE, AND HANDLING

        - Do not deliver paneling until painting and similar operations that might damage paneling have been completed in installation areas. Store paneling in installation areas or in areas where environmental conditions comply with requirements specified in "Field Conditions" Article.

      - FIELD CONDITIONS

Retain "Environmental Limitations without Humidity Control" Paragraph below if humidity will not be controlled after building is occupied.

        - Environmental Limitations without Humidity Control: Do not deliver or install paneling until building is enclosed, wet-work is complete, and HVAC system is operating and will maintain temperature and relative humidity at levels planned for building occupants during the remainder of the construction period.

Retain "Environmental Limitations with Humidity Control" Paragraph below if humidity will be controlled after building is occupied and retain one of three options or insert another range based on local climatological data. Three ranges given are for painted or finished woodwork. First applies to Ontario, Quebec, and most of the United States. Second applies to damp coastal areas of southern United States and Maritime Provinces. Third applies to dry southwestern United States, Alberta, Manitoba, and Saskatchewan. See relative-humidity and moisture-content map in AWS.

        - Environmental Limitations with Humidity Control: Do not deliver or install paneling until building is enclosed, wet-work is complete, and HVAC system is operating and will maintain temperature between 60 and 90 deg F (16 and 32 deg C) and relative humidity between [25 and 55] [43 and 70] [17 and 50] <Insert numbers> percent during the remainder of the construction period.

Retain "Field Measurements" or "Established Dimensions" Paragraph below. Establishing dimensions allows paneling fabrication to begin before installation areas are complete.

        - Field Measurements: Where paneling is indicated to fit to other construction, verify dimensions of other construction by field measurements before fabrication and indicate measurements on Shop Drawings. Coordinate fabrication schedule with construction progress to avoid delaying the Work.

          - Locate concealed framing, blocking, and reinforcements that support paneling by field measurements before being enclosed/concealed by construction and indicate measurements on Shop Drawings.

        - Established Dimensions: Where paneling is indicated to fit to other construction, establish dimensions for areas where woodwork is to fit. Provide allowance for trimming at site, and coordinate construction to ensure that actual dimensions correspond to established dimensions.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - PANELING FABRICATORS

Retain "Fabricators" Paragraph below if a list of preapproved woodworkers is used as a quality-control procedure. See the Evaluations. See AWI's or WI's member list for names of woodworking firms.

        - Fabricators: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available fabricators offering products that may be incorporated into the Work include, but are not limited to, the following]:

          - <Insert, in separate subparagraphs, names and contact information for preapproved woodworking firms>.

      - PANELING, GENERAL

        - Quality Standard: Unless otherwise indicated, comply with the "Architectural Woodwork Standards" for grades of plastic-laminate-faced wood paneling (decorative laminate surfacing) indicated for construction, finishes, installation, and other requirements.

If retaining first subparagraph below, verify applicability of certification for Project's geographic location. Certification programs of AWI and WI organizations are available to member and nonmember firms. Both organizations will inspect work and provide certification for work that passes inspection if fabricator is not certified/licensed. Contact either organization for a list of certified woodworkers in Project's location.

          - Provide inspections including installation together with labels and certificates from [AWI] [WI] certification program indicating that woodwork complies with requirements of grades specified.

Retain subparagraph below if Drawings and the Specifications are more restrictive than AWS.

          - The Contract Documents contain requirements that are more stringent than the referenced woodwork quality standard. Comply with requirements of Contract Documents in addition to those of the referenced quality standard.

        - <Double click to insert sustainable design text for regional wood materials.>

      - PLASTIC-LAMINATE-FACED WOOD PANELING

        - Grade: [Premium] [Custom] [Economy].

        - <Double click to insert sustainable design text for certified wood.>

        - Plastic Laminate: High-pressure decorative laminate complying with ISO 4586-3.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain grade designations from options in "Faces," and "Exposed Edges" subparagraphs below. Grade HGS and flame-retardant Grade HGF have high wear resistance, and Grade VGS and flame-retardant Grade VGF have medium wear resistance. Revise grade if extremely high wear resistance is required.

          - Faces: [Grade HGS] [Grade HGF] [Grade VGS] [Grade VGF] <Insert grade>.

          - Backs: Balance material with thickness matching exposed surface.

          - Exposed Edges: Same as faces[ or Grade VGS].

        - Colors, Patterns, and Finishes: Provide materials and products that result in colors and textures of exposed surfaces complying with the following requirements:

Retain one of first three subparagraphs below. If retaining first, indicate colors and other characteristics in a separate schedule.

          - As indicated by manufacturer's designations.

          - Match Architect's samples.

          - As selected by Architect from laminate manufacturer's full range in the following categories:

            - Solid colors, [gloss] [matte] finish.

            - Solid colors with core same color as surface, [gloss] [matte] finish.

            - Wood grains, [gloss] [matte] finish.

            - Patterns, [gloss] [matte] finish.

          - Grain Direction: [Vertical] [Horizontal].

        - Panel Core: [Particleboard or MDF] [Fire-retardant particleboard or fire-retardant MDF].

          - Thickness: [3/4 inch (19 mm)] [As indicated on Drawings].

Options in "Exposed Panel Edges" and "Panel Reveals" paragraphs below are examples only; revise to suit Project. If retaining metal edges or reveals, consider copying finish information from Section 057000 "Decorative Metal" into descriptions.

        - Exposed Panel Edges: [Plastic-laminate matching faces] [Legs of metal channels forming reveals] [Applied solid-wood banding 11/16 inch (18 mm) thick by depth of panels] [Applied bronze flat bars 1/16 inch (1.6 mm) thick by depth of panels] <Insert description>.

        - Panel Reveals: [Matte black plastic laminate] [Bronze sheet] [Stainless steel sheet] [Bronze channels, 1 by 1 by 1/8 inch (25.4 by 25.4 by 3.2 mm) thick] [Stainless steel channels, 1 by 1 by 1/16 inch (25.4 by 25.4 by 1.6 mm) thick] <Insert description>.

Retain first option in "Adhesive for Bonding Plastic Laminate" Paragraph below for non-climate controlled interior or exterior work.

        - Adhesives for Bonding Plastic Laminate: [Type I, waterproof type] [Type II water-resistant type] as selected by fabricator to comply with requirements .

Retain "Adhesive for Bonding Edges" Subparagraph below unless improved chemical, fire, or temperature resistance is required.

          - Adhesive for Bonding Edges: Hot-melt adhesive[ or adhesive specified above for faces].

        - Fire-Retardant-Treated Paneling: Panels are to consist of fire-retardant plastic laminate and fire-retardant particleboard or fire-retardant, medium-density fiberboard (MDF). Panels are to have a flame-spread index of [25] [75] or less and a smoke-developed index of 450 or less per ASTM E84, and be listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction.

        - Assemble panels by gluing and concealed fastening.

      - MATERIALS

        - Materials, General: Provide materials that comply with requirements of referenced quality standard for each quality grade specified unless otherwise indicated.

Retain one of three options in "Wood Moisture Content" Paragraph below. First range applies to Ontario, Quebec, and most of the United States. Second applies to damp coastal areas of southern United States and Maritime Provinces. Third applies to dry southwestern United States, Alberta, Manitoba, and Saskatchewan. See relative-humidity and moisture-content map in AWS. Some locations have microclimates that differ from the surrounding region.

        - Wood Moisture Content: [5 to 10] [8 to 13] [4 to 9] percent.

        - Composite Wood Products: Provide materials that comply with requirements of referenced quality standard for each quality grade specified unless otherwise indicated.

          - <Double click to insert sustainable design text for recycled content.>

          - <Double click to insert sustainable design text for composite wood products.>

          - MDF: ANSI A208.2, [Grade 130] <Insert grade>.

          - Particleboard (Medium Density): ANSI A208.1, [Grade M-2] [Grade M-2-Exterior Glue].

      - FIRE-RETARDANT-TREATED MATERIALS

If fire-retardant materials are required, verify requirements and acceptability of various materials with authorities having jurisdiction.

        - Fire-Retardant-Treated Materials, General: Where fire-retardant-treated materials are indicated, use materials that are acceptable to authorities having jurisdiction and with fire-test-response characteristics specified as determined by testing identical products per test method indicated by a qualified testing agency.

          - Use treated materials that comply with requirements of referenced quality standard. Do not use materials that are warped, discolored, or otherwise defective.

          - Use fire-retardant-treatment formulations that do not bleed through or otherwise adversely affect finishes. Do not use colorants to distinguish treated materials from untreated materials.

          - Identify fire-retardant-treated materials with appropriate classification marking of qualified testing agency in the form of removable paper label or imprint on surfaces that will be concealed from view after installation.

        - Fire-Retardant-Treated Lumber and Plywood: Products with a flame-spread index of 25 or less when tested according to ASTM E84, with no evidence of significant progressive combustion when the test is extended an additional 20 minutes, and with the flame front not extending more than 10.5 feet (3.2 m) beyond the centerline of the burners at any time during the test.

          - Kiln-dry lumber and plywood after treatment to a maximum moisture content of 19 and 15 percent, respectively.

Retain first subparagraph below if items are to receive a stained or natural finish. Organic resin treatment is more expensive than salt treatments but helps ensure a better stained or natural finish.

          - For items indicated to receive a stained or natural finish, use organic resin chemical formulation.

Retain one of two subparagraphs below. Before retaining first, verify that wood species and treatment process specified are approved for milling after treatment.

          - Mill lumber after treatment within limits set for wood removal that do not affect listed fire-test-response characteristics, using a woodworking shop certified by testing and inspecting agency.

          - Mill lumber before treatment and implement procedures during treatment and drying processes that prevent lumber from warping and developing discolorations from drying sticks or other causes, marring, and other defects affecting appearance of paneling.

Fire-retardant particleboard contains urea formaldehyde. It complies with requirements for Class I (Class A) finish but not with requirements of model codes for use where noncombustible materials are required.

        - Fire-Retardant Particleboard: Made from softwood particles and fire-retardant chemicals mixed together at time of panel manufacture to achieve flame-spread index of 25 or less and smoke-developed index of 25 or less per ASTM E84.

          - For panels 3/4 inch (19 mm) thick and less, comply with ANSI A208.1 for Grade M-2 except for the following minimum properties: modulus of rupture, 1600 psi (11 MPa); modulus of elasticity, 300,000 psi (2070 MPa); internal bond, 80 psi (550 kPa); and screw-holding capacity on face and edge, 250 and 225 lbf (1100 and 1000 N), respectively.

          - For panels 13/16 to 1-1/4 inches (20 to 32 mm) thick, comply with ANSI A208.1 for Grade M-1 except for the following minimum properties: modulus of rupture, 1300 psi (9 MPa); modulus of elasticity, 250,000 psi (1720 MPa); linear expansion, 0.50 percent; and screw-holding capacity on face and edge, 250 and 175 lbf (1100 and 780 N), respectively.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Fiberboard in "Fire-Retardant Fiberboard" Paragraph below complies with requirements for Class I (Class A) finish but not with requirements of model codes for use where noncombustible materials are required.

        - Fire-Retardant Fiberboard: MDF panels complying with ANSI A208.2, made from softwood fibers, synthetic resins, and fire-retardant chemicals mixed together at time of panel manufacture to achieve flame-spread index of 25 or less and smoke-developed index of 200 or less per ASTM E84.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - INSTALLATION MATERIALS

        - Furring, Blocking, Shims, and Hanging Strips: [Softwood or hardwood lumber] [Fire-retardant-treated softwood lumber], kiln-dried to less than 15 percent moisture content.

        - Anchors: Select material, type, size, and finish required for each substrate for secure anchorage. Provide metal expansion sleeves or expansion bolts for post-installed anchors. Use nonferrous-metal or hot-dip galvanized anchors and inserts at inside face of exterior walls.

        - Installation Adhesive: Product recommended by panel fabricator for each substrate for secure anchorage.

          - <Double click to insert sustainable design text for VOC content of adhesive.>

          - <Double click to insert sustainable design text for adhesives.>

      - FABRICATION

        - Complete fabrication, including assembly, to maximum extent possible, before shipment to Project site. Disassemble components only as necessary for shipment and installation. Where necessary for fitting at site, provide ample allowance for scribing, trimming, and fitting.

Retain subparagraph below if Architect will examine paneling in woodwork shop before it is shipped to Project site.

          - Notify Architect seven days in advance of the dates and times paneling fabrication will be complete.

        - Shop cut openings, to maximum extent possible, to receive hardware, appliances, plumbing fixtures, electrical work, and similar items. Locate openings accurately and use templates or roughing-in diagrams to produce accurately sized and shaped openings. Sand edges of cutouts to remove splinters and burrs.

- EXECUTION

      - PREPARATION

        - Before installation, condition paneling to humidity conditions in installation areas.

        - Before installing paneling, examine shop-fabricated work for completion and complete work as required, including removal of packing and backpriming.

      - INSTALLATION

Revise "Grade" Paragraph below if installation grade is different from fabrication grade.

        - Grade: Install paneling to comply with quality standard grade of paneling to be installed.

        - Install paneling level, plumb, true in line, and without distortion. Shim as required with concealed shims. Install level and plumb to a tolerance of 1/8 inch in 96 inches (3 mm in 2400 mm). Install with no more than 1/16 inch in 96-inch (1.6 mm in 2400-mm) vertical cup or bow and 1/8 inch in 96-inch (3 mm in 2400-mm) horizontal variation from a true plane.

          - For flush paneling with revealed joints, install with variations in reveal width, alignment of top and bottom edges, and flushness between adjacent panels not exceeding [1/32 inch (0.8 mm)] [1/16 inch (1.5 mm)].

Retain paragraph below except for economy work that is face nailed or adhesively applied.

        - Anchor paneling to supporting substrate with [concealed panel-hanger clips] [splined connection strips] [blind nailing]. Do not use face fastening unless [covered by trim] [otherwise indicated].

      - ADJUSTING AND CLEANING

        - Repair damaged and defective paneling, where possible, to eliminate defects. Where not possible to repair, replace paneling. Adjust for uniform appearance.

        - Clean paneling on exposed surfaces. Touch up shop-applied finishes to restore damaged or soiled areas.

END OF SECTION 064219