Copyright 2022 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 073216 - CONCRETE ROOF TILES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, LEED v4.1, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Concrete roof tiles.

          - Underlayment materials.

          - Ridge vents.

          - Metal flashing and trim.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections. Retain if snow guards are not inserted in this Section.

          - Section 077253 "Snow Guards" for snow guards.

      - ALLOWANCES

Retain this article if products and Work included in this Section, such as replacement of deteriorated roof decks, are covered by lump-sum, unit-cost, or quantity allowances.

        - See Section 012100 "Allowances" for description of allowances affecting items specified under this Section.

      - UNIT PRICES

Retain this article if products and Work specified in this Section, such as replacement of deteriorated roof decks, are measured and paid for under the provisions of unit prices.

        - See Section 012200 "Unit Prices" for description of unit prices affecting items specified under this Section.

      - ALTERNATES

Retain this article if products and Work specified in this Section are to be added to or deducted from the base bid amount under the provisions of alternates.

        - See Section 012300 "Alternates" for description of alternates affecting items specified under this Section.

      - DEFINITIONS

        - Roofing Terminology: See ASTM D1079 and glossary in TRI/WSRCA's "Concrete and Clay Roof Tile Installation Manual" for definitions of terms related to roofing Work in this Section.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

          - <Insert participant requirements>.

      - ACTION SUBMITTALS

        - Product Data:

          - Concrete roof tiles.

          - Underlayment materials.

          - Ridge vents.

          - Metal flashing and trim.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for roofing.>

          - <Double click to insert sustainable design text for regional materials.>

Retain "Shop Drawings" Paragraph below if Drawings do not fully detail metal flashing and trim specified in Part 2 or to verify requirements specified in Part 2.

        - Shop Drawings: For metal flashing and trim.

Retain "Samples" Paragraph below for single-stage Samples. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples: For each exposed product and for each color and texture specified, in sizes indicated.

          - Concrete Roof Tiles: Full size, showing full range of color values and blends.

          - Accessory Tiles: Full size, each type.

          - Metal Flashing: 12 inches (305 mm) square.

          - Ridge Vents: 12-inch- (305-mm-) long Sample.

          - Eave Closures: In manufacturer's standard size.

        - Samples for Initial Selection: For each type of concrete roof tile and accessory tile.

          - Include Samples of accessories involving color selection.

        - Samples for Verification: For the following products, in sizes indicated:

          - Concrete Roof Tiles: Full size, showing full range of color values and blends.

          - Accessory Tiles: Full size, each type.

          - Metal Flashing: 12 inches (305 mm) square.

          - Ridge Vents: 12-inch- (305-mm-) long Sample.

          - Eave Closures: In manufacturer's standard size.

      - INFORMATIONAL SUBMITTALS

Retain "Material Test Reports" Paragraph below for material test reports that are Contractor's responsibility.

        - Material Test Reports: For each type of concrete roof tile, based on evaluation of comprehensive tests performed by a qualified testing agency.

        - Research Reports: From [an agency acceptable to authorities having jurisdiction] [ICC-ES] <Insert evaluation agency>, indicating that product is suitable for intended use under applicable building codes for the following:

Retain applicable subparagraphs below to suit Project. The IBC and the IRC do not address mortar- or adhesive-set tile, wire-tie tile-attachment systems, and polymer-modified bitumen sheet and synthetic underlayments.

          - [Mortar] [Adhesive] tile-attachment systems.

          - Wire-tie tile-attachment systems.

          - Polymer-modified bitumen sheet underlayment.

          - Synthetic underlayment.

        - Sample Warranty: For manufacturer's materials warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For roofing to include in maintenance manuals.

        - Materials warranties.

        - Roofing Installer's warranty.

      - MAINTENANCE MATERIAL SUBMITTALS

        - Furnish extra materials, from the same product run, that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Revise "Concrete Roof Tiles" Subparagraph below if different quantities of different types of tiles are required; for example, special shapes or colors.

          - Concrete Roof Tiles: [100 sq. ft. (9.3 sq. m)] <Insert area> of each type, in unbroken bundles.

      - MOCKUPS

        - Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for materials and execution.

          - Build mockups for concrete roof tiles including related roofing materials.

            - Size: [48 inches (1219 mm) long by 48 inches (1219 mm) wide] <Insert dimensions>.

Retain first subparagraph below if required; insert other mockup requirements to suit Project.

            - Include gutter and downspout complying with requirements in [Section 076200 "Sheet Metal Flashing and Trim."] [Section 077100 "Roof Specialties."]

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - DELIVERY, STORAGE, AND HANDLING

        - Store underlayment rolls in a dry, well-ventilated location protected from weather, sunlight, and moisture in accordance with manufacturer's written instructions.

          - Store on end, on pallets or other raised surfaces. Do not double-stack rolls.

        - Protect unused underlayment from weather, sunlight, and moisture when left overnight or when roofing work is not in progress.

        - Handle, store, and place roofing materials in a manner to prevent damage to roof deck or structural supporting members.

      - FIELD CONDITIONS

        - Environmental Limitations: Proceed with installation only when existing and forecasted weather conditions permit product installation and related work to be performed in accordance with manufacturer's written instructions and warranty requirements.

Retain subparagraph below for self-adhering, polymer-modified bitumen sheet used as water protection, an ice barrier, or underlayment.

          - Install self-adhering, polymer-modified bitumen sheet underlayment within the range of ambient and substrate temperatures recommended in writing by manufacturer.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Materials Warranty: Manufacturer agrees to repair or replace concrete roof tiles that fail in materials within specified warranty period.

Verify available warranties and warranty periods.

          - Warranty Period: [50] <Insert number> years from date of Substantial Completion.

Retain "Roofing Installer's Warranty" Paragraph below, with "Roofing Installer's Warranty" Article, if required.

        - Roofing Installer's Warranty: On warranty form at end of this Section, signed by Installer, in which Installer agrees to repair or replace components of concrete-tile roofing that fail in materials or workmanship within specified warranty period.

          - Warranty Period: [Two] [Five] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SOURCE LIMITATIONS

        - Obtain each type of product from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

Retain "Exterior Fire-Test Exposure" Paragraph below for classified roof assemblies on combustible decks and revise to suit Project. Roof tiles are only exempt from testing if installed on noncombustible roof decks. Verify requirements of authorities having jurisdiction for roof assembly.

        - Exterior Fire-Test Exposure: Provide concrete roof tiles and related roofing materials identical to those of assemblies tested for Class A fire resistance in accordance with ASTM E108 or UL 790 by Underwriters Laboratories or another testing and inspecting agency acceptable to authorities having jurisdiction. Identify products with appropriate markings of applicable testing agency.

Retain "Energy Performance, Energy Star" Paragraph below if required. To bear the Energy Star label, roof tiles must have initial solar reflectance of 0.25 or greater and maintain a solar reflectance of 0.15 or greater three years after installation under normal conditions.

        - Energy Performance, Energy Star: Provide roof tiles that are listed on the DOE's "Energy Star Roof Product List" for steep-slope roof products.

        - <Double click to insert sustainable design text for solar reflectance index.>

      - CONCRETE ROOF TILES

        - Concrete Roof Tiles: ASTM C1492, molded- or extruded-concrete roof tile units of shape and configuration indicated, with integral color, and free of surface imperfections. Provide with fastening holes prepunched at factory.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Weight: [Normal] [Medium] [Light].

Retain "High-Profile Shape," "Medium-Profile Shape," or "Low-Profile Shape" Subparagraph below to suit Project.

          - High-Profile Shape: Type I, [Spanish or "S"] [Spanish or "S," interlocking] <Insert shape>.

            - Accessory Tiles: Type IV, [ridge] [ridge vent] [ridge end] [hip and hip starter] [header course] [L-shaped rake edge] [roll rake edge] [starter] [end band] [and] [terminal] <Insert accessory> units.

          - Medium-Profile Shape: Type II, [French interlocking] <Insert shape>.

            - Accessory Tiles: Type IV, [ridge] [ridge vent] [ridge end] [hip and hip starter] [header course] [L-shaped rake edge] [roll rake edge] [starter] [end band] [and] [terminal] <Insert accessory> units.

          - Low-Profile Shape: Type III, [flat shingle] [flat interlocking] <Insert shape>.

            - Accessory Tiles: Type IV, [Ridge and closed ridge end] [hip and hip starter] [header course] [L-shaped rake edge] [starter] [end band] [and] [terminal] <Insert accessory> units.

          - Size: <Insert length and width dimensions>.

          - Finish and Texture: [Matte, smooth] [Matte, striated] [Glazed, smooth] <Insert finish and texture>.

If retaining first option in "Colors, Blends, and Patterns" Subparagraph below, indicate colors, blends, and patterns in a separate schedule.

          - Colors, Blends, and Patterns: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert description>.

          - <Double click to insert sustainable design text for regional materials.>

      - UNDERLAYMENT MATERIALS

See "Underlayment," "Model-Code Underlayment Requirements," "NRCA Underlayment Recommendations," "Felt and Roll-Roofing Underlayment," "Synthetic Underlayment," "Polymer-Modified Bitumen Sheet Underlayment," and "Self-Adhering, Polymer-Modified Bitumen Sheet Underlayment" articles in the Evaluations for a discussion of various underlayment options.

Retain one of three options in "Felt" Paragraph below to suit Project. The IBC and the IRC recognize ASTM D226/D226M Type II and ASTM D2626/D2626M felts as acceptable single-layer roof-tile underlayments for slopes equal to or greater than 4:12. The IBC and the IRC require a double-layer installation for slopes from 2-1/2:12 to 4:12. NRCA does not recommend using ASTM D226/D226M or ASTM D2626/D2626M felts as underlayment for roof tile. NRCA only recommends using ASTM D4869/D4869M Type IV felt as a top layer installed over an anchor layer of polymer-modified bitumen sheet in a two-layer installation. ASTM D226/D226M Type II and ASTM D4869/D4869M Type IV felts both weigh 26 lb/100 sq. ft. (1270 g/sq. m). ASTM D2626/D2626M felts weigh 37 lb/100 sq. ft. (1806 g/sq. m).

        - Felt: [ASTM D226/D226M Type II, asphalt saturated] [ASTM D4869/D4869M Type IV, asphalt saturated] [ASTM D2626/D2626M, asphalt saturated and coated, mineral-granule surfaced on weather (top) side], unperforated.

Retain "Asphalt Roll-Roofing" Paragraph below if required. ASTM D6380/D6380M, Class M, Type II, felts weigh 71.5 lb/100 sq. ft. (3490 g/sq. m).

        - Asphalt Roll-Roofing: ASTM D6380/D6380M, Class M, Type II, asphalt-saturated and -coated organic felt; mineral-granule surfaced on weather (top) side.

If retaining "Synthetic Underlayment" Paragraph below, verify that products comply with requirements of authorities having jurisdiction and are recommended for use under roof tile. NRCA only recommends using synthetic underlayment as a top layer installed over an anchor layer of mechanically fastened or self-adhering, polymer-modified bitumen sheet in a two-layer installation. See "NRCA Underlayment Recommendations" Article in the Evaluations.

        - Synthetic Underlayment: UV-resistant polypropylene, polyolefin, or polyethylene polymer fabric with surface coatings or treatments to improve traction underfoot and abrasion resistance; recommended, in writing, by manufacturer for use under roof tile; and evaluated and documented to be suitable for use as a roof underlayment under applicable codes by a testing and inspecting agency acceptable to authorities having jurisdiction.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Polymer-Modified Bitumen Sheet" Paragraph below for products marketed as underlayment or valley flashing. Revise to suit Project. NRCA recommends using polymer-modified bitumen sheet, including products used as base sheets in low-slope membrane roof systems and products marketed as underlayment, for roof-tile underlayment. Sheets are mechanically fastened or, where used as an adhered top layer in a two-layer installation, set in a continuous layer of hot roofing asphalt or cold adhesive. See "NRCA Underlayment Recommendations" and "Polymer-Modified Bitumen Sheet Underlayment" articles in the Evaluations.

        - Polymer-Modified Bitumen Sheet: Styrene-butadiene-styrene- (SBS) modified asphalt, glass-fiber-mat-reinforced sheet; minimum [55-mil (1.4-mm)] [40-mil (1.0-mm)] <Insert dimension> nominal thickness; recommended in writing by manufacturer and acceptable to authorities having jurisdiction for use as underlayment in tile steep-slope roofing systems; and designed for mechanical fastening or adhesive attachment using roofing asphalt or cold-applied adhesive.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

If required, insert specific testing requirements for polymer-modified bitumen sheet products here. Product testing varies among manufacturers.

          - <Insert testing requirements>.

Retain "Self-Adhering, Polymer-Modified Bitumen Sheet" or "Self-Adhering, Polymer-Modified Bitumen Sheet, High Temperature" Paragraph below to suit Project. Self-adhering, polymer-modified bitumen sheet commonly covers the entire roof deck under concrete tile and is also used for water and ice-dam protection in roof areas vulnerable to leakage. NRCA suggests covering self-adhering sheet with a top layer of a different type of underlayment. See "Self-Adhering, Polymer-Modified Bitumen Sheet Underlayment" Article in the Evaluations.

Retain thickness option in "Self-Adhering, Polymer-Modified Bitumen Sheet " Paragraph to suit Project. ASTM D1970/D1970M requires a minimum thickness of 40 mils (1.0 mm); however, product thicknesses vary.

        - Self-Adhering, Polymer-Modified Bitumen Sheet: ASTM D1970/D1970M, minimum [55-mil- (1.4-mm-)] [50-mil- (1.3-mm-)] [40-mil- (1.0-mm-)] <Insert dimension> thick sheet; glass-fiber-mat-reinforced, polymer-modified asphalt; with slip-resistant top surface and release backing; cold applied; and recommended in writing by manufacturer for use in tile roofing system required.[ Provide primer for adjoining concrete, masonry, and metal surfaces to receive underlayment.]

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Top Surface" Subparagraph below if a specific surface is required. Polymer-film- and polyester-surfaced products cost more and might form better-quality laps than sand- and granule-surfaced products. Tiles might stick to sand- and granule-surfaced products where they contact, which makes roof repairs more difficult. To prevent this problem, cover self-adhering sheet with another type of underlayment. Installer preference based on experience and the slip-resistance characteristics of the various surface options might be a good reason to choose one type of surface over another.

          - Top Surface: [Sand] [Granule] [Textured polymer film] [Polyester].

        - Self-Adhering, Polymer-Modified Bitumen Sheet, High Temperature: ASTM D1970/D1970M, minimum [55-mil- (1.4-mm-)] [50-mil- (1.3-mm-)] [40-mil- (1.0-mm-)] <Insert thickness> thick sheet; glass-fiber-mat-reinforced, polymer-modified asphalt; with slip-resistant top surface and release backing; cold applied; and recommended in writing by manufacturer for use in tile roofing system required.[ Provide primer for adjoining concrete, masonry, and metal surfaces to receive underlayment.]

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Thermal Stability: Stable after testing at 240 deg F (116 deg C) in accordance with ASTM D1970/D1970M.

Retain "Top Surface" Subparagraph below if a specific surface is required. Polymer-film- and polyester-surfaced products cost more and might form better-quality laps than granule-surfaced products. Tiles might stick to granule-surfaced products where they contact, which makes roof repairs more difficult. To prevent this problem, cover self-adhering sheet with another type of underlayment. Installer preference based on experience and the slip-resistance characteristics of the various surface options might be a good reason to choose one type of surface over another.

          - Top Surface: [Granule] [Textured polymer film] [Polyester].

      - RIDGE VENTS

Retain this article for rigid-plastic or flexible ridge vents installed under ridge tiles. Sheet metal ridge vents are specified in "Metal Flashing and Trim" Article. If retaining, verify that products can withstand the weight of concrete tile over several decades and indicate details of ridge vents on Drawings.

        - Rigid-Plastic Ridge Vent: Manufacturer's standard, rigid section high-density polypropylene or other UV-stabilized plastic ridge vent for use under ridge tiles.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Minimum Net Free Area: <Insert area>.

          - Width: <Insert dimension>.

          - Thickness: <Insert dimension>.

        - Flexible Ridge Vent: Manufacturer's standard roll-form ridge vent that protects against driven rain and snow and is recommended in writing by manufacturer for installation with roof tile indicated.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Width: <Insert dimension>.

          - Features: [Butyl adhesive strips at sides] [Corrugated aluminum strips at sides].

      - ACCESSORIES

        - Asphalt Roofing Cement: ASTM D4586/D4586M Type II, asbestos free.

        - Butyl Sealant: ASTM C1311, single-component, solvent-release butyl rubber sealant; polyisobutylene plasticized; heavy bodied.

Revise "Elastomeric Sealant" Paragraph below if a specific polymer, class, and use are required.

        - Elastomeric Sealant: ASTM C920, Type S, Grade NS, one-part, non-sag, elastomeric polymer sealant of class and use classifications required to seal joints in concrete-tile roofing and remain watertight; recommended in writing by manufacturer for applications indicated.

Retain "Roofing Asphalt" Paragraph below if adhering top layer of underlayment to the anchor layer with hot asphalt.

        - Roofing Asphalt: ASTM D312/D312M Type IV.

Retain "Cold-Applied Adhesive" Paragraph below if adhering top layer of underlayment to anchor layer with cold-applied adhesives.

        - Cold-Applied Adhesive: Manufacturer's standard asphalt-based, one- or two-part, asbestos-free, cold-applied adhesive specially formulated for compatibility and use with underlayments.

Retain "Mortar" or "Foam Adhesive" Paragraph below if required for tile closures or an adhesive-set roof in climates not subject to freeze-thaw cycles. Although TRI/FRSA and TRI/WSRCA publications and NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems" include adhesive-set installation, the IBC and the IRC do not. ICC-ES AC152 provides criteria for code acceptance of adhesive attachment of roof tiles. For mortar- or adhesive-set installation, consult manufacturer, require "Research Reports" Paragraph in "Informational Submittals" Article for attachment systems, and verify requirements of authorities having jurisdiction.

        - Mortar: ASTM C270, Type M, [natural color] [with ASTM C979/C979M, pigmented mortar matching the color of concrete roof tiles for exposed-to-view mortar, and natural color for concealed-from-view mortar].

If retaining "Foam Adhesive" Paragraph below, verify years of in-service experience with adhesive recommended by roof-tile manufacturers.

        - Foam Adhesive: Two-component, polyurethane expanding adhesive recommended in writing for application by concrete-roof-tile manufacturer.

Retain "Eave Closure" Paragraph below if not using an accessory tile, mortar, or foam for eave closure.

        - Eave Closure: Manufacturer's standard [EPDM] [copper] [stainless steel] [galvanized-steel] [aluminum, mill finish] <Insert material> eave closure formed to shape of concrete roof tiles.

        - Ridge Closure: Manufacturer's standard EPDM ridge closure, formed to shape of concrete roof tiles.

Retain "Wood Nailers" Paragraph below if above-roof-deck wood components, including cants and battens, are required. Revise to insert material requirements if preferred.

        - Wood Nailers: Comply with requirements for pressure-preservative-treated wood in Section 061000 "Rough Carpentry."

Retain "Mesh Fabric" Paragraph below if using insect-screen materials to seal roof-deck openings at sheet metal ridge vents.

        - Mesh Fabric: 18-by-14 (1.1-by-1.4-mm) mesh of PVC-coated, glass-fiber thread.

      - FASTENERS

Retain "Roofing Nails" Paragraph below for direct deck installation method.

        - Roofing Nails: ASTM F1667, [hot-dip galvanized-steel, 0.120-inch- (3.05-mm-)] [stainless steel, 0.120-inch- (3.05-mm-)] [copper, 0.135-inch- (3.43-mm-)] [silicon-bronze, 0.120-inch- (3.05-mm-)] diameter shank, sharp-pointed, conventional roofing nails with barbed shanks; minimum 3/8-inch- (10-mm-) diameter head; of sufficient length to penetrate 3/4 inch (19 mm) into substrate or extend at least 1/8 inch (3 mm) through thickness of the sheathing, whichever is less.

          - Where nails are in contact with metal flashing, use nails made from same metal as flashing.

        - Underlayment Nails: Aluminum, stainless steel, or hot-dip galvanized-steel wire nails with low-profile metal or plastic caps, 1-inch- (25-mm-) minimum diameter.

Retain subparagraph below to comply with IBC requirements for locations where the basic design wind speed (three-second gust per ASCE/SEI 7) is equal to or greater than 140 mph (63 m/s) or with IRC requirements for locations where the ultimate design wind speed (as defined in the IRC) is equal to or greater than 140 mph (63 m/s).

          - Provide with minimum 0.0134-inch- (0.34-mm-) thick metal cap, 0.010-inch- (0.25-mm-) thick power-driven metal cap, or 0.035-inch- (0.89-mm-) thick plastic cap; and with minimum 0.083-inch- (2.11-mm-) thick ring shank or 0.091-inch- (2.31-mm-) thick smooth shank of length to penetrate at least 3/4 inch (19 mm) into roof sheathing or to penetrate through roof sheathing less than 3/4 inch (19 mm) thick.

        - Nails for Wood Nailers: ASTM F1667; common or box, steel wire, flat head, and smooth shank.

Retain "Wire Ties" Paragraph below for miscellaneous wiring of valley or other cut concrete roof tiles.

        - Wire Ties: [Copper] [Brass] [Stainless steel], 0.083-inch- (2.11-mm-) minimum diameter.

Retain "Twisted-Wire-Tie System" Paragraph below if required. This system is an alternative to the direct deck installation method. Revise twisted-wire tie to flat metal strip if required. Verify applicability for roof slope and tile shapes required with manufacturers.

        - Twisted-Wire-Tie System: Continuously twisted, two-wire unit with loops formed 6 inches (152 mm) apart, minimum [0.101-inch- (2.57-mm-) diameter copper wire and 0.064-inch- (1.63-mm-) diameter copper tie wires] [0.090-inch- (2.29-mm-) diameter stainless steel wire and 0.063-inch- (1.60-mm-) diameter stainless steel tie wires] [0.105-inch- (2.67-mm-) diameter galvanized-steel wire and 0.063-inch- (1.60-mm-) diameter galvanized-steel tie wires] <Insert requirements>, with matching-metal folding clip anchors.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Hook Nails" Paragraph below in high-wind areas and on roofs steeper than 7:12. Hook nails are also called "wind locks."

        - Hook Nails: One-piece wind lock and concrete-roof-tile fastener system, minimum [0.135-inch- (3.43-mm-) diameter brass] [0.135-inch- (3.43-mm-) diameter copper] [0.120-inch- (3.05-mm-) diameter stainless steel] [0.120-inch- (3.05-mm-) diameter galvanized-steel] <Insert requirements> wire, for direct deck nailing.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Tile Locks" Paragraph below if required for use with direct-deck or twisted-wire-tie system fastening methods. Tile locks, also called "butt hooks," "nose clips," "nose hooks," and "wind locks," help reduce wind-induced chatter of concrete roof tiles and limit stress on the primary fastener at top of the concrete roof tile.

        - Tile Locks: [Brass] [Copper] [Stainless steel] [Hot-dip galvanized-steel], nominal 0.1-inch- (2.5-mm-) diameter wire device designed to secure butt edges of overlaid concrete roof tiles.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Storm Clips" Paragraph below if additional wind-uplift resistance is required. Storm clips are also called "hurricane clips," "storm lock side clips," or "grip clips."

        - Storm Clips: [Brass, minimum 0.048-by-1/2-inch (1.22-by-13-mm)] [Stainless steel, minimum 0.048-by-1/2-inch (1.22-by-13-mm)] [Hot-dip galvanized-steel, minimum 0.048-by-1/2-inch (1.22-by-13-mm)] <Insert requirements> strap-type, L-shaped retainer clips designed to secure side edges of concrete roof tiles. Provide with two fastener holes in base flange.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - METAL FLASHING AND TRIM

        - General: Comply with requirements in Section 076200 "Sheet Metal Flashing and Trim."

          - Sheet Metal: [Copper] [Stainless steel] [Zinc-tin alloy coated copper] [Anodized aluminum] [Aluminum, mill finished] <Insert requirements>.

        - Fabricate sheet metal flashing and trim to comply with recommendations in SMACNA's "Architectural Sheet Metal Manual" for design, dimensions, metal, and other characteristics of the item unless otherwise specified in this Section or indicated on Drawings.

Retain "Apron Flashings," "Step Flashings," "Channel Flashings," "Rake Pan Flashings," "Cricket and Backer Flashings," "Counterflashings," "Valley Flashings," and "Drip Edges" subparagraphs below if Drawings do not fully detail these flashing conditions. Review recommendations in SMACNA's "Architectural Sheet Metal Manual" and NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems"; revise to suit Project.

          - Apron Flashings: Fabricate with lower flange extending a minimum of [4 inches (102 mm)] [6 inches (152 mm)] <Insert dimension> over and [4 inches (102 mm)] <Insert dimension> beyond each side of downslope tile roofing and [6 inches (152 mm)] <Insert dimension> up the vertical surface.

Retain "Step Flashings" Subparagraph below for interweaving metal step flashings between succeeding courses of flat roof tile that abut vertical surfaces such as chimneys, sidewalls, and skylights.

          - Step Flashings: Fabricate with a headlap of 4 inches (102 mm) and a minimum extension of [4 inches (102 mm)] [5 inches (127 mm)] <Insert dimension> both horizontally and vertically.

Retain "Channel Flashings" Subparagraph below for metal pan or channel flashings acting as an internal gutter under sloping high-profile roof tile abutting vertical surfaces such as chimneys, sidewalls, and skylights.

          - Channel Flashings: Fabricate with vertical surface extending a minimum of [4 inches (102 mm)] [5 inches (127 mm)] <Insert dimension> above the concrete roof tile and [4 inches (102 mm)] [6 inches (152 mm)] <Insert dimension> beneath the tile roofing, with a [1-inch- (25-mm-)] <Insert dimension> high vertical return to form a runoff channel.

Retain "Rake Pan Flashings" Subparagraph below for metal pan or channel flashings acting as an internal gutter at rake edge fasciae.

          - Rake Pan Flashings: Fabricate with vertical surface extending over fasciae and [6 inches (152 mm)] <Insert dimension> beneath the tile roofing, with a [1-inch- (25-mm-)] <Insert dimension> high vertical return to form a runoff channel.

Retain "Cricket and Backer Flashings" Subparagraph below if required.

          - Cricket and Backer Flashings: Fabricate with concealed flange extending a minimum of [24 inches (610 mm)] <Insert dimension> beneath upslope tile roofing, [6 inches (152 mm)] <Insert dimension> beyond each side of [chimney] [skylight], and [6 inches (152 mm)] <Insert dimension> above the roof plane.

Retain "Counterflashings" Subparagraph below to protect top edges of apron, step, channel, cricket, and backer flashings from water intrusion.

          - Counterflashings: Fabricate to cover [4 inches (102 mm)] <Insert dimension> of base flashing measured vertically; and in lengths required so that no step exceeds [8 inches (203 mm)] <Insert dimension> and overall length is no more than [10 feet (3 m)] <Insert dimension>.

            - Provide metal [reglets] [receivers] for installation.

Retain "Valley Flashings" Subparagraph below if required and revise to suit Project. Both closed and open valleys require metal flashings. The IBC and the IRC include prescriptive requirements for valley flashings; verify requirements of authorities having jurisdiction.

Options in "Valley Flashings" Subparagraph below are coordinated. First option reflects the IBC and the IRC requirement for minimum 24-inch- (610-mm-) wide sheet metal; remaining options reflect NRCA fabrication recommendations for 24-inch- (610-mm-) wide sheet metal. SMACNA recommends a minimum 1-inch- (25-mm-) high, inverted-V profile and a 2-inch- (51-mm-) high, inverted-V profile for slopes greater than 6:12 and for where dissimilar slopes join. Revise subparagraph to suit Project.

          - Valley Flashings: Fabricate from metal sheet not less than [24 inches (610 mm)] <Insert dimension> wide in lengths not exceeding [10 feet (3 m)] <Insert dimension>, with [1-inch- (25-mm-)] <Insert dimension> high, inverted-V profile water diverter at center of valley and equal flange widths of not less than [11 inches (279 mm)] <Insert dimension>.

Retain first subparagraph below if securing flange edges with cleats.

            - Hem flange edges for fastening with metal cleats.

Retain "Drip Edges" Subparagraph below if required at eave or rake edges.

          - Drip Edges: Fabricate in lengths not exceeding [10 feet (3 m)] <Insert dimension>, with minimum 2-inch (51-mm) roof-deck flange and 1-1/2-inch (38-mm) fascia flange with 3/8-inch (10-mm) drip at lower edge.

Retain "Sheet Metal Ridge Vent" Paragraph below if required and revise to suit Project. Indicate details of sheet metal ridge vents on Drawings.

        - Sheet Metal Ridge Vent: Fabricate from 16-oz./sq. ft.- (0.55-mm-) thick copper sheet, terminating each side in V-shaped external baffles with venting holes producing net free ventilation area of 2.65 sq. in./ft. (56 sq. cm/m).

Retain "Vent-Pipe Flashings" Paragraph below for plumbing vents that penetrate roof. Revise if another flashing material is required, or delete paragraph and include requirements in Section 076200 "Sheet Metal Flashing and Trim."

        - Vent-Pipe Flashings: ASTM B749, Type L51121, at least 1/16 inch (1.6 mm) thick. Provide lead sleeve sized to slip over and turn down into pipe, soldered to skirt at slope of roof and extending at least 4 inches (102 mm) from pipe onto roof.

Insert requirements for snow guards using Section 077253 "Snow Guards" as a basis if not specifying them in that Section.

        - <Insert snow-guard requirements>.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

          - Examine roof sheathing to verify that sheathing joints are supported by framing and blocking or metal clips and that installation is within flatness tolerances.

          - Verify that substrate is sound, dry, smooth, clean, sloped for drainage, and completely anchored and that provisions have been made for flashings and penetrations through roofing.

          - Verify that vent stacks and other penetrations through roofing are installed and securely fastened.

        - Prepare written report, endorsed by Installer, listing conditions detrimental to performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION OF CONCRETE ROOF TILES

        - Install concrete roof tiles in accordance with manufacturer's written instructions and recommendations in TRI/WSRCA's "Concrete and Clay Roof Tile Installation Manual" and NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems" unless more stringent requirements are specified in this Section or indicated on Drawings.

Generally, retain first subparagraph below and indicate design wind speeds on Drawings with design wind forces determined by Project's structural engineer.

          - Install tiles to resist wind forces resulting from design wind speeds indicated on Drawings.

          - Maintain uniform exposure and coursing of concrete roof tiles throughout roof.

          - Extend tiles 2 inches (51 mm) over eave fasciae.

Retain one or more of "Nail Fastening," "Wire-Tie Fastening," "Mortar Setting," and "Foam-Adhesive Setting" subparagraphs below to suit Project. Although TRI/FRSA and TRI/WSRCA publications and NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems" include adhesive-set installation for use in climates not subject to freeze-thaw cycles, the IBC and the IRC do not. ICC-ES AC152 provides criteria for code acceptance of adhesive attachment of roof tiles. Verify requirements of authorities having jurisdiction.

          - Nail Fastening: Drive nails to clear the concrete roof tile so the tile hangs from the nail and is not drawn up.

            - Install wire through nail holes of cut tiles that cannot be nailed directly to roof deck, and fasten to nails driven into deck.

          - Wire-Tie Fastening: Install wire-tie systems and fasten concrete roof tiles in accordance with manufacturer's written instructions.

          - Mortar Setting: Install concrete roof tiles in accordance with manufacturer's written instructions and acceptance criteria of authorities having jurisdiction.

          - Foam-Adhesive Setting: Install concrete roof tiles in accordance with adhesive and tile manufacturers' written instructions and acceptance criteria of authorities having jurisdiction.

Retain "Storm Clips" Subparagraph below for additional side fastening of concrete roof tiles.

          - Storm Clips: Install to capture edges of longitudinal sides of concrete roof tiles and securely fasten to roof deck.

Retain "Tile Locks" Subparagraph below to lock butt edges of overlying concrete roof tiles to heads of underlying tiles.

          - Tile Locks: Install to support and lock overlying tile butts to underlying tiles.

          - Cut and fit concrete roof tiles neatly around roof vents, pipes, ventilators, and other projections through roof. Fill voids with mortar.

          - Install concrete roof tiles with color blend approved by Architect.

Retain "Low-Profile, Flat-Shingle Concrete-Roof-Tile Installation" Paragraph below if required.

        - Low-Profile, Flat-Shingle Concrete-Roof-Tile Installation:

          - Maintain 2-inch (51-mm) headlap between succeeding courses of concrete roof tiles.

          - Offset joints by half the concrete-roof-tile width in succeeding courses.

          - Extend concrete roof tiles 1 inch (25 mm) over fasciae at rakes.

          - Install ridge tiles in [V-ridge] [saddle] [mitered] configuration with laps facing away from prevailing wind. Seal laps with [asphalt roofing cement] [butyl sealant] [elastomeric sealant].

Retain first subparagraph below for closing void at junction of ridge tiles with high-profile concrete roof tiles. Special ridge closure tiles are also called "top fixture tiles."

            - Close voids where ridge tiles meet concrete roof tiles with [ridge closure] [mortar struck with face of ridge cover] tiles.

          - Install hip tiles in [V-ridge] [saddle] [mitered] configuration. Seal laps with [asphalt roofing cement] [butyl sealant] [elastomeric sealant].

Retain subparagraph below for mortar-filling voids at junction of hip tiles with high-profile concrete roof tiles.

            - Fill voids with mortar where hip tiles meet concrete roof tiles, and strike mortar flush with face of hip cover tiles.

Retain "Low-Profile, Flat Interlocking Concrete-Roof-Tile Installation" Paragraph below if required.

        - Low-Profile, Flat Interlocking Concrete-Roof-Tile Installation:

          - Provide minimum 3-inch (76-mm) lap between succeeding courses of concrete roof tiles.

          - Offset joints by half the concrete-roof-tile width in succeeding courses.

          - Install L-shaped rake tiles.

          - Install ridge tiles in [V-ridge] [saddle] [mitered] configuration with laps facing away from prevailing wind. Seal laps with [asphalt roofing cement] [butyl sealant] [elastomeric sealant].

Retain first subparagraph below for closing void at junction of ridge tiles with high-profile concrete roof tiles. Special ridge closure tiles are also called "top fixture tiles."

            - Close voids where ridge tiles meet concrete roof tiles with [ridge closure] [mortar struck with face of ridge cover] tiles.

          - Install hip tiles in [V-ridge] [saddle] [mitered] configuration. Seal laps with [asphalt roofing cement] [butyl sealant] [elastomeric sealant].

Retain subparagraph below for mortar-filling voids at junction of hip tiles with high-profile concrete roof tiles.

            - Fill voids with mortar where hip tiles meet concrete roof tiles, and strike mortar flush with face of hip cover tiles.

Retain "Medium-Profile, Interlocking Concrete-Roof-Tile Installation" Paragraph below if required.

        - Medium-Profile, Interlocking Concrete-Roof-Tile Installation:

          - Provide minimum 3-inch (76-mm) lap between succeeding courses of concrete roof tiles.

          - Install rake tiles indicated.

          - Install ridge tiles with laps facing away from prevailing wind. Seal laps with [asphalt roofing cement] [butyl sealant] [elastomeric sealant].

Retain "High-Profile Concrete-Roof-Tile Installation" Paragraph below if required. High-profile tiles include interlocked and overlapped tiles, such as one-piece Spanish or "S" tiles and two-piece barrel mission tiles.

        - High-Profile Concrete-Roof-Tile Installation:

          - Install [tile] [sheet metal] [EPDM] eave closure.

          - Provide minimum 3-inch (76-mm) lap between succeeding courses of concrete roof tiles.

          - Install rake tiles indicated.

          - Install ridge tiles with laps facing away from prevailing wind. Seal laps with [asphalt roofing cement] [butyl sealant] [elastomeric sealant].

Retain "Open Valleys" Paragraph below if required.

        - Open Valleys: Cut concrete roof tiles at open valleys to form straight lines. [Maintain uniform width of exposed open valley] [Widen exposed portion of open valley 1/8 inch in 12 inches (1:96)] from highest to lowest point.

          - Drill or notch cut valley tiles and wire-tie to fastener placed clear of valley metal flashings.

          - Do not nail tiles to metal flashings.

Retain "Closed Valleys" Paragraph below if required.

        - Closed Valleys: Cut concrete roof tiles at closed valleys to form straight lines, trimming upper concealed corners of tiles. Maintain uniform gap of [1/2 to 3/4 inch (13 to 19 mm)] [3/4 to 1 inch (19 to 25 mm)] <Insert dimension> on either side of water diverter at valley centerline.

          - Drill or notch cut valley tiles and wire-tie to fastener placed clear of valley metal flashings.

          - Do not nail tiles to metal flashings.

        - Remove and replace damaged or broken concrete roof tiles.

      - INSTALLATION OF UNDERLAYMENT MATERIALS

Underlayments installed parallel to eaves are installed perpendicular to sloped roof deck. Underlayments installed parallel to the rake are installed parallel to sloped roof deck.

        - Comply with concrete-roof-tile and underlayment manufacturers' written installation instructions and with recommendations in NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems" applicable to products and applications indicated unless more stringent requirements are specified in this Section or indicated on Drawings.

          - Cover [ridge] [hip] wood nailers with underlayment strips.

Retain "Felt" Paragraph below if required. Verify recommendations of local installers and roof-tile manufacturers.

        - Felt: Install parallel with and starting at eaves and fasten with underlayment nails.

Retain "Single-Layer Installation," "Top-Layer Installation," or "Double-Layer Installation" Subparagraph below to suit Project. The IBC and the IRC require two layers of underlayment for slopes from 2-1/2:12 to 4:12. Because of roof tiles' long service lives, NRCA only recommends using felt as a top layer in a two-layer installation over an anchor layer of polymer-modified bitumen sheet.

          - Single-Layer Installation: Install on roof deck.

Usually, retain first option in first subparagraph below for 2-inch- (51-mm-) minimum side laps, except for where the limiting design wind speed is equal to or greater than 140 mph (63 m/s). In that case, the IBC and the IRC require at least 4-inch (102-mm) side laps. Verify requirements of authorities having jurisdiction.

            - Lap sides a minimum of [2 inches (51 mm)] [4 inches (102 mm)] over underlying course.

            - Lap ends a minimum of 4 inches (102 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

          - Top-Layer Installation: Install as second layer over anchor-layer underlayment, with side laps offset halfway between side laps of underlying anchor layer.

Usually, retain first option in first subparagraph below for 2-inch- (51-mm-) minimum side laps, except for where the limiting design wind speed is equal to or greater than 140 mph (63 m/s). In that case, the IBC and the IRC require at least 4-inch (102-mm) side laps. Verify requirements of authorities having jurisdiction.

            - Lap sides a minimum of [2 inches (51 mm)] [4 inches (102 mm)] over underlying course.

            - Lap ends a minimum of 4 inches (102 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

Side lap and starter course dimensions in "Double-Layer Installation" Subparagraph below are based on 36-inch- (914-mm-) wide rolls of felt underlayment and are needed to maintain double-layer felt underlayment coverage with 2 inches (51 mm) to spare. Verify requirements of authorities having jurisdiction.

          - Double-Layer Installation: Install on roof deck.

            - Install a 19-inch- (483-mm-) wide starter course at eaves and completely cover with a 36-inch- (914-mm-) wide second course.

            - Install succeeding 36-inch- (914-mm-) wide courses lapping previous courses 19 inches (483 mm) in shingle fashion.

            - Lap ends a minimum of 6 inches (152 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

Retain first subparagraph below if cemented double layers of felt serve as water and ice-dam membranes or if required for additional protection. Self-adhering, polymer-modified bitumen sheet has largely replaced cemented double layers of felt as water and ice-dam protection.

            - Apply a continuous layer of asphalt roofing cement over starter course and on felt surface to be concealed by succeeding courses as each felt course is installed. Apply [over entire roof] [at locations indicated on Drawings].

Retain first subparagraph below for areas subject to high wind speeds. The IBC requires fastening of underlayment in grid pattern described below where the basic design wind speed is equal to or greater than 140 mph (63 m/s). The IRC requires the same fastening pattern where the ultimate design wind speed is equal to or greater than 140 mph (63 m/s).

          - Install fasteners in a grid pattern of 12 inches (305 mm) between side laps with 6-inch (152-mm) spacing at side and end laps.

Retain first subparagraph below if combining felt with self-adhering, polymer-modified bitumen sheet. NRCA recommends covering water and ice-dam protection with the roof-field underlayment.

          - Install felt over areas protected by self-adhering, polymer-modified bitumen sheet.

Retain first option in subparagraph below if desired for areas where roof deck is covered by self-adhering, polymer-modified bitumen sheet against walls and other roof projections.

          - Terminate felt [flush] [extended up not less than 4 inches (102 mm)] <Insert requirements> against sidewalls, curbs, chimneys, and other roof projections.

Retain "Asphalt Roll-Roofing" Paragraph below if required. Verify recommendations of local installers and roof-tile manufacturers.

        - Asphalt Roll-Roofing: Install parallel with and starting at eaves.

Retain "Single-Layer Installation," "Top-Layer Installation," or "Double-Layer Installation" Subparagraph below to suit Project. The IBC and the IRC require two layers of underlayment for slopes from 2-1/2:12 to 4:12.

          - Single-Layer Installation: Install on roof deck.

Usually, retain first option in first subparagraph below for 2-inch- (51-mm-) minimum side laps, except for where the limiting design wind speed is equal to or greater than 140 mph (63 m/s). In that case, the IBC and the IRC require at least 4-inch (102-mm) side laps. Verify requirements of authorities having jurisdiction.

            - Lap sides a minimum of [2 inches (51 mm)] [4 inches (102 mm)] over underlying course.

            - Lap ends a minimum of 4 inches (102 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

            - Fasten with underlayment nails.

          - Top-Layer Installation: Install as second layer over anchor-layer underlayment, with side laps offset halfway between side laps of underlying anchor layer.

Usually, retain first option in first subparagraph below for 2-inch- (51-mm-) minimum side laps, except for where the limiting design wind speed is equal to or greater than 140 mph (63 m/s). In that case, the IBC and the IRC require at least 4-inch (102-mm) side laps. Verify requirements of authorities having jurisdiction.

            - Lap sides a minimum of [2 inches (51 mm)] [4 inches (102 mm)] over underlying course.

            - Lap ends a minimum of 4 inches (102 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

            - [Fasten with underlayment nails] [Adhere to anchor layer in solid mopping of hot roofing asphalt, applied within plus or minus 25 deg F (14 deg C) of equiviscous temperature] [Adhere to anchor layer in uniform coating of cold-applied adhesive] [Adhere to anchor layer in uniform coating of asphalt roofing cement].

Side lap and starter course dimensions in "Double-Layer Installation" Subparagraph below are based on 36-inch- (914-mm-) wide rolls of underlayment and are needed to maintain double-layer underlayment coverage with 2 inches (51 mm) to spare. Verify requirements of authorities having jurisdiction.

          - Double-Layer Installation: Install on roof deck.

            - Install a 19-inch- (483-mm-) wide starter course at eaves and completely cover with a 36-inch- (914-mm-) wide second course.

            - Install succeeding 36-inch- (914-mm-) wide courses lapping previous courses 19 inches (483 mm) in shingle fashion.

            - Lap ends a minimum of 6 inches (152 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

            - Fasten with underlayment nails.

Retain first subparagraph below if cemented double layers of roll roofing serve as an ice-dam membrane or if required for additional protection. Self-adhering, polymer-modified bitumen sheet has largely replaced cemented double layers of underlayment as ice-dam protection.

            - Apply a continuous layer of asphalt roofing cement over starter course and on roll-surface to be concealed by succeeding courses as each roll-roofing course is installed. Apply [over entire roof] [at locations indicated on Drawings].

Retain first subparagraph below for areas subject to high wind speeds. The IBC requires fastening of underlayment in grid pattern described below where the basic design wind speed is equal to or greater than 140 mph (63 m/s). The IRC requires the same fastening pattern where the ultimate design wind speed is equal to or greater than 140 mph (63 m/s).

          - Install fasteners in a grid pattern of 12 inches (305 mm) between side laps with 6-inch (152-mm) spacing at side and end laps.

Retain first subparagraph below if combining roll roofing with self-adhering, polymer-modified bitumen sheet. NRCA recommends covering water and ice-dam protection with the roof-field underlayment.

          - Install roll roofing over areas protected by self-adhering, polymer-modified bitumen sheet.

Retain first option in subparagraph below if desired for areas where roof deck is covered by self-adhering, polymer-modified bitumen sheet against walls and other roof projections.

          - Terminate roll roofing [flush] [extended up not less than 4 inches (102 mm)] <Insert requirements> against sidewalls, curbs, chimneys, and other roof projections.

Retain "Synthetic-Underlayment Top Layer" Paragraph below if required as a top layer over an anchor layer of mechanically fastened, polymer-modified bitumen sheet or an anchor layer of self-adhering, polymer-modified bitumen sheet.

        - Synthetic-Underlayment Top Layer: Install in accordance with manufacturer's written installation instructions and as second layer over anchor-layer underlayment.

          - Completely cover anchor-layer underlayment and install parallel with and starting at the eaves, with side laps offset halfway between side laps of underlying anchor layer.

Usually, retain first option in first subparagraph below for 2-inch- (51-mm-) minimum side laps, except for where the limiting design wind speed is equal to or greater than 140 mph (63 m/s). In that case, the IBC and the IRC require at least 4-inch (102-mm) side laps. Verify requirements of manufacturers and authorities having jurisdiction.

          - Lap sides and ends as recommended in writing by manufacturer, but not less than [2 inches (51 mm)] [4 inches (102 mm)] for side laps and 6 inches (152 mm) for end laps.

          - Stagger end laps from anchor-layer end laps and between succeeding top courses at interval recommended in writing by manufacturer, but not less than 72 inches (1829 mm).

          - Fasten with underlayment nails.

Retain subparagraph below for areas subject to high wind speeds. The IBC requires fastening of underlayment in grid pattern described below where the basic design wind speed is equal to or greater than 140 mph (63 m/s). The IRC requires the same fastening pattern where the ultimate design wind speed is equal to or greater than 140 mph (63 m/s).

          - Install fasteners in a grid pattern of 12 inches (305 mm) between side laps with 6-inch (152-mm) spacing at side and end laps.

Retain "Polymer-Modified Bitumen Sheet" Paragraph below if required. NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems" recommends using polymer-modified bitumen sheet for single- and double-layer installations; as an anchor layer under a mechanically fastened top layer of polymer-modified bitumen sheet, felt, or synthetic underlayment; and as an anchor layer under a top layer of polymer-modified bitumen sheet adhered using hot bitumen or cold adhesive or a top layer of self-adhering, polymer-modified bitumen sheet.

        - Polymer-Modified Bitumen Sheet: Install parallel with and starting at eaves.

Retain one of first three subparagraphs below to suit Project.

Retain "Single- or Anchor-Layer Installation" Subparagraph below with "Top-Layer Installation" Subparagraph for both methods of two-layer installation recommended by NRCA. NRCA also recommends using self-adhering, polymer-modified bitumen sheet for the anchor layer. Delete both subparagraphs and retain "Double-Layer Installation" Subparagraph for another installation method recommended by NRCA.

          - Single- or Anchor-Layer Installation: Install on roof deck.

Usually, retain first option in first subparagraph below for 2-inch- (51-mm-) minimum side laps, except for where the limiting design wind speed is equal to or greater than 140 mph (63 m/s). In that case, the IBC and the IRC require at least 4-inch (102-mm) side laps. Verify requirements of authorities having jurisdiction.

            - Lap sides a minimum of [2 inches (51 mm)] [4 inches (102 mm)] over underlying course.

            - Lap ends a minimum of 6 inches (152 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

            - Fasten with underlayment nails.

          - Top-Layer Installation: Install as a second layer over anchor-layer underlayment, with side laps offset halfway between side laps of underlying anchor layer.

Usually, retain first option in first subparagraph below for 2-inch- (51-mm-) minimum side laps, except for where the limiting design wind speed is equal to or greater than 140 mph (63 m/s). In that case, the IBC and the IRC require at least 4-inch (102-mm) side laps. Verify requirements of authorities having jurisdiction.

            - Lap sides a minimum of [2 inches (51 mm)] [4 inches (102 mm)].

            - Lap ends a minimum of 6 inches (152 mm).

            - Stagger end laps from anchor-layer end laps and between succeeding top-layer courses at least 72 inches (1829 mm).

            - [Fasten with underlayment nails] [Adhere to anchor layer in a solid mopping of hot roofing asphalt, applied within plus or minus 25 deg F (14 deg C) of equiviscous temperature] [Adhere to anchor layer in uniform coating of cold-applied adhesive].

          - Double-Layer Installation: Install on roof deck in overlapping layers with a half-width plus 1-inch- (25-mm-) wide starter course at eaves completely covered by full-width second course.

            - Install succeeding courses lapping previous courses by a half-width plus 1 inch (25 mm) in shingle fashion.

            - Lap ends a minimum of 6 inches (152 mm).

            - Stagger end laps between succeeding courses at least 72 inches (1829 mm).

            - Fasten with underlayment nails.

Retain first subparagraph below for areas subject to high wind speeds. The IBC requires fastening of underlayment in grid pattern described below where the basic design wind speed is equal to or greater than 140 mph (63 m/s). The IRC requires the same fastening pattern where the ultimate design wind speed is equal to or greater than 140 mph (63 m/s).

          - Install fasteners in a grid pattern of 12 inches (305 mm) between side laps with 6-inch (152-mm) spacing at side and end laps.

Retain first subparagraph below if combining polymer-modified bitumen sheet with self-adhering, polymer-modified bitumen sheet. NRCA recommends covering water and ice-dam protection with the roof-field underlayment.

          - Install sheets over areas protected by self-adhering, polymer-modified bitumen sheet.

Retain first option in subparagraph below if desired for areas where roof deck is covered by self-adhering, polymer-modified bitumen sheet against walls and other roof projections.

          - Terminate sheets [flush] [extended up not less than 4 inches (102 mm)] <Insert requirements> against sidewalls, curbs, chimneys, and other roof projections.

Retain "Self-Adhering, Polymer-Modified Bitumen Sheet" Paragraph below if required. NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems" recommends using a single layer of self-adhering, polymer-modified bitumen sheet over the entire roof deck, although NRCA suggests covering it with a layer of another type of underlayment, and using it for a top layer over mechanically fastened polymer-modified bitumen sheet.

        - Self-Adhering, Polymer-Modified Bitumen Sheet: Install, wrinkle free.

          - Comply with low-temperature installation restrictions of underlayment manufacturer.

          - Install lapped in direction that sheds water.

          - Lap sides not less than 4 inches (102 mm).

          - Lap ends not less than 6 inches (152 mm), staggered 24 inches (610 mm) between succeeding courses.

          - Roll laps with roller.

Retain first subparagraph below if primer is required to enhance adhesion to concrete and masonry surfaces, such as chimneys or walls, and metal surfaces, such as valley flashing.

          - Prime concrete, masonry, and metal surfaces to receive self-adhering, polymer-modified bitumen sheet.

Retain "Single-Layer Installation" Subparagraph below if self-adhering, polymer-modified bitumen sheet covers the entire roof deck. NRCA suggests installing a second layer of another type of underlayment over self-adhering, polymer-modified bitumen sheet.

          - Single-Layer Installation: Install over entire roof deck.

Retain "Top-Layer Installation" Subparagraph below if required. Verify manufacturers' written instructions for top-layer installation and revise to suit Project.

          - Top-Layer Installation: Install as second layer over anchor-layer underlayment.

            - Completely cover anchor-layer underlayment.

            - Offset side laps halfway between side laps of underlying anchor layer and offset end laps from those of underlying anchor layer at least 72 inches (1829 mm).

Retain "Water and Ice-Dam Protection Installation" Subparagraph below if a layer of self-adhering, polymer-modified bitumen sheet partially covers roof deck in areas vulnerable to moisture penetration; revise to suit Project. Because of tile's long service life, NRCA recommends extending the underlayment covering the roof field over the water and ice-dam protection layer.

          - Water and Ice-Dam Protection Installation: Install on roof deck where indicated [below] [on Drawings].

Retain one or more of first eight subparagraphs below if locations are not indicated on Drawings. Revise to suit Project.

            - Eaves: Extend from edges of eaves [24 inches (610 mm)] [36 inches (914 mm)] <Insert dimension> beyond interior face of exterior wall.

            - Rakes: Extend from edges of rakes [24 inches (610 mm)] [36 inches (914 mm)] <Insert dimension> beyond interior face of exterior wall.

Verify requirements of authorities having jurisdiction for valley underlayment.

            - Valleys: Extend from lowest to highest point [18 inches (457 mm)] <Insert dimension> on each side of centerline.

            - Hips: Extend [18 inches (457 mm)] <Insert dimension> on each side.

            - Ridges: Extend [36 inches (914 mm)] <Insert dimension> on each side[ without obstructing continuous ridge vent slot].

            - Sidewalls: Extend [18 inches (457 mm)] <Insert dimension> beyond sidewalls and return vertically against sidewalls not less than [4 inches (102 mm)] <Insert dimension>.

            - Dormers, Chimneys, Skylights, and Other Roof-Penetrating Elements: Extend [18 inches (457 mm)] <Insert dimension> beyond penetrating elements and return vertically against penetrating elements not less than [4 inches (102 mm)] <Insert dimension>.

            - Roof-Slope Transitions: Extend [18 inches (457 mm)] <Insert dimension> on each roof slope.

          - Cover underlayment within seven days.

Retain "Valley Underlayment" Paragraph below if required and if installing self-adhering, polymer-modified bitumen sheet is not specified for water and ice-dam protection at valleys. Paragraph is applicable if using felt, roll-roofing, or polymer-modified bitumen sheet underlayment. Paragraph is based on NRCA recommendations and IBC and IRC requirements. The IBC and IRC require using "Type I" underlayment on top of roof-field underlayment, presumably ASTM D226/D226M Type I. Verify requirements of authorities having jurisdiction and revise to suit Project.

        - Valley Underlayment: Install one layer of 36-inch- (914-mm-) wide underlayment centered in valley, running full length of valley, and on top of underlayment on field of roof that is woven through valley. Install all layers of underlayment in and through valley tight with no bridging.

Revise first subparagraph below to suit Project.

          - Use [same underlayment as installed on field of roof] <Insert requirements>.

          - Lap ends at least 12 inches (305 mm) in direction that sheds water, and seal with asphalt roofing cement.

          - Fasten to roof deck with underlayment nails located as far from valley center as possible and only to extent necessary to hold underlayment in place until installation of valley flashing.

Subparagraph below is based on IBC and IRC requirements for roof slopes less than 7:12 in areas where there is a possibility of ice forming along eaves causing a backup of water. Verify requirements of authorities having jurisdiction.

          - Solidly cement valley underlayment to roof-field underlayment that is woven through valley using asphalt roofing cement.

      - INSTALLATION OF RIDGE VENTS

        - Rigid-Plastic Ridge Vents: Install continuous ridge vents over concrete roof tiles in accordance with manufacturer's written instructions. Fasten with nails of sufficient length to penetrate substrate.

        - Flexible Ridge Vent: Install continuous-roll ridge vents over concrete roof tiles in accordance with manufacturer's written instructions.

      - INSTALLATION OF METAL FLASHING AND TRIM

        - Install metal flashings and other sheet metal to comply with requirements in Section 076200 "Sheet Metal Flashing and Trim."

          - Install metal flashings in accordance with concrete-roof-tile manufacturer's written instructions and recommendations in NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems."

Retain "Apron Flashings" Paragraph below to provide a weatherproofing transition material where a roof area intersects a head wall. Common locations for apron flashings include the front downslope side of a dormer or chimney, curbed roof penetrations, and clerestory transitions.

        - Apron Flashings: Extend lower flange over and beyond each side of downslope tile roofing and up the vertical surface.

Retain "Step Flashings" Paragraph below for interweaving metal step flashings between succeeding courses of flat roof tile that abut vertical surfaces such as chimneys, sidewalls, and skylights.

        - Step Flashings: Install with a headlap of 4 inches (102 mm) and extend both horizontally and vertically. Install with lower edge of flashing just upslope of, and concealed by, butt of overlying tile. Fasten to roof deck only.

Retain "Cricket and Backer Flashings" Paragraph below if required.

        - Cricket and Backer Flashings: Install against roof-penetrating elements, extending concealed flange beneath upslope tile roofing and beyond each side.

        - Channel Flashings: Install over underlayment materials and fasten to roof deck.

        - Rake Pan Flashings: Install over underlayment materials and fasten to roof deck.

        - Counterflashings: Coordinate with installation of base flashing and fit tightly to base flashing. Lap joints a minimum of 4 inches (102 mm) secured in a waterproof manner.

          - Install in reglets or receivers.

Retain "Valley Flashings" Paragraph below if required. Both open and closed valleys require sheet metal flashings.

        - Valley Flashings: Install centered in valleys, lapping ends at least [8 inches (203 mm)] <Insert dimension> in direction that sheds water. Fasten upper end of each length to roof deck beneath overlap.

Retain one of first two subparagraphs below, or both, if required. NRCA recommends using cleats and stripping-in flashing using self-adhering, polymer-modified bitumen sheet strips adhered to metal flanges and similar valley underlayment in climates prone to heavy accumulations of snow and ice or regular freeze-thaw cycling.

          - Secure hemmed flange edges into metal cleats spaced [12 inches (305 mm)] [24 inches (610 mm)] <Insert dimension> apart and fastened to roof deck.

          - Adhere minimum [9-inch- (229-mm-)] <Insert dimension> wide strips of self-adhering, polymer-modified bitumen sheet to metal flanges and to [underlying self-adhering, polymer-modified bitumen sheet] <Insert requirements>. Place strips parallel to and over flanges so that they will be just concealed by installed tile.

Retain subparagraph below for climates prone to heavy accumulations of snow and ice or regular freeze-thaw cycling.

          - Provide a closure at the end of the inverted-V profile of the valley metal to minimize water and ice infiltration.

        - Rake Drip Edges: Install over underlayment materials and fasten to roof deck.

        - Eave Drip Edges: Install below underlayment materials and fasten to roof deck.

Retain "Sheet Metal Ridge Vents" Paragraph below if required.

        - Sheet Metal Ridge Vents: Install centered on and mechanically fasten to wood ridge. Adhere each side to concrete roof tile with elastomeric sealant.

          - Install fabric mesh over roof-deck air ventilation gaps to prevent insect entry.

        - Pipe Flashings: Form flashing around pipe penetrations and tile roofing. Fasten and seal to tile roofing.

      - INSTALLATION OF WOOD NAILERS

Retain first paragraph below for high-profile concrete roof tiles.

        - Install wood nailers securely fastened to roof deck at the following locations:

          - Hips.

          - Ridges.

          - Rakes.

        - Install beveled wood-cant nailers at eaves and securely fasten to roof deck.

Retain paragraph below if battens are required. Wood battens installed perpendicular to sloped roof deck are installed horizontally or parallel to eaves. Counter battens installed parallel to sloped roof deck are installed vertically or parallel to rakes. This Section specifies conventional concrete-roof-tile installation with battens installed horizontally and counter battens installed vertically.

        - Install nominal 1-by-2-inch (25-by-51-mm) wood-batten nailers horizontally [over 1/2-inch- (13-mm-) high, pressure-preservative-treated wood lath strips] [in 48-inch (1219-mm) lengths with ends separated by 1/2 inch (13 mm)], at spacing required by concrete-roof-tile manufacturer, and securely fasten to roof deck.

Retain subparagraph below if counter battens are required.

          - Install nominal 1-by-2-inch (25-by-51-mm) wood counter battens vertically spaced [24 inches (610 mm)] <Insert dimension> apart and securely fasten to roof deck.

      - ROOFING INSTALLER'S WARRANTY

Retain this article if required. Revise to include another Roofing Installer's Warranty form or as advised by Owner's counsel. Coordinate with "Warranty" Article.

        - WHEREAS <Insert name> of <Insert address>, herein called the "Roofing Installer," has performed roofing and associated work ("the work") on the following project:

          - Owner: <Insert name of Owner>.

          - Owner Address: <Insert address>.

          - Building Name/Type: <Insert information>.

          - Building Address: <Insert address>.

          - Area of the Work: <Insert information>.

          - Acceptance Date: <Insert date>.

          - Warranty Period: <Insert time>.

          - Expiration Date: <Insert date>.

        - AND WHEREAS Roofing Installer has contracted (either directly with Owner or indirectly as a subcontractor) to warrant the work against leaks and faulty or defective materials and workmanship for designated Warranty Period,

        - NOW THEREFORE Roofing Installer hereby warrants, subject to terms and conditions herein set forth, that, during Warranty Period, Roofing Installer will, at Roofing Installer's own cost and expense, make or cause to be made such repairs to or replacements of the work as are necessary to correct faulty and defective work and as are necessary to maintain the work in a watertight condition.

        - This Warranty is made subject to the following terms and conditions:

          - Specifically excluded from this Warranty are damages to the work and other parts of the building, and to building contents, caused by:

            - Lightning;

Insert required wind speed in first subparagraph below.

            - Peak gust wind speed exceeding <Insert mph (m/s)>;

            - Fire;

            - Failure of roofing system substrate, including cracking, settlement, excessive deflection, deterioration, and decomposition;

            - Faulty construction of copings, chimneys, skylights, vents, equipment supports, and other edge conditions and penetrations of the work;

            - Vapor condensation on bottom of roofing; and

            - Activity on roofing by others, including construction contractors, maintenance personnel, other persons, and animals, whether authorized or unauthorized by Owner.

          - When the work has been damaged by any of foregoing causes, Warranty shall be null and void until such damage has been repaired by Roofing Installer and until cost and expense thereof have been paid by Owner or by another responsible party so designated.

          - Roofing Installer is responsible for damage to the work covered by this Warranty but is not liable for consequential damages to building or building contents resulting from leaks or faults or defects of the work.

          - During Warranty Period, if Owner allows alteration of the work by anyone other than Roofing Installer, including cutting, patching, and maintenance in connection with penetrations, attachment of other work, and positioning of anything on roof, this Warranty shall become null and void on date of the alterations, but only to the extent the alterations affect the work covered by this Warranty. If Owner engages Roofing Installer to perform the alterations, Warranty shall not become null and void unless Roofing Installer, before starting the alterations, notified Owner in writing, showing reasonable cause for claim, that the alterations would likely damage or deteriorate the work, thereby reasonably justifying a limitation or termination of this Warranty.

          - During Warranty Period, if original use of roof is changed and it becomes used for, but was not originally specified for, a use or service more severe than originally specified, this Warranty shall become null and void on date of the change, but only to the extent the change affects the work covered by this Warranty.

          - Owner shall promptly notify Roofing Installer of observed, known, or suspected leaks, defects, or deterioration and shall afford reasonable opportunity for Roofing Installer to inspect the work and to examine evidence of such leaks, defects, or deterioration.

          - This Warranty is recognized to be the only warranty of Roofing Installer on the work and shall not operate to restrict or cut off Owner from other remedies and resources lawfully available to Owner in cases of roofing failure. Specifically, this Warranty shall not operate to relieve Roofing Installer of responsibility for performance of the work according to requirements of the Contract Documents, regardless of whether Contract was a contract directly with Owner or a subcontract with Owner's General Contractor.

        - IN WITNESS THEREOF, this instrument has been duly executed this <Insert day> day of <Insert month>, <Insert year>.

          - Authorized Signature: <Insert signature>.

          - Name: <Insert name>.

          - Title: <Insert title>.

END OF SECTION 073216