Copyright 2022 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 081816.13 - SLIDING ALUMINUM-FRAMED GLASS DOORS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED v4, LEED v4.1, IgCC/ASHRAE 189.1, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Exterior sliding aluminum-framed glass doors.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 084113 "Aluminum-Framed Entrances and Storefronts" for coordinating finish among aluminum fenestration units on the building exterior.

          - Section 085113 "Aluminum Windows" for [related aluminum-framed transom and sidelite windows and mullions and for ]coordinating finishes among aluminum fenestration units on the building exterior.

          - [Section 087100 "Door Hardware"] [Section 087111 "Door Hardware (Descriptive Specification)"] for hardware not specified in Section 081816.13.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data:

          - Exterior sliding aluminum-framed glass doors.

        - Product Data Submittals: For each product.

          - Include construction details, material descriptions, fabrication methods, dimensions of individual components and profiles, hardware, finishes, and operating instructions.

        - Sustainable Design Submittals

          - <Double click to insert sustainable design text for recycled content.>

          - <Double click to insert sustainable design text for EPDs and HPDs.>

        - Shop Drawings: For sliding aluminum-framed glass doors.

          - Include plans, elevations, sections, and details.

          - Detail attachments to other work, and between units, if any.

          - Include hardware and required clearances.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples: For each exposed product and for each color specified, 12-inch-long (300-mm-long) section with weather stripping, glazing bead, and factory-applied color finish.

        - Samples for Initial Selection: For each type of sliding aluminum-framed glass door indicated.

          - Include Samples of hardware and accessories involving color selection.

Delete "Samples for Initial Selection" Paragraph above if colors and other characteristics are preselected and specified or scheduled. Retain "Samples for Verification" Paragraph below with or without above.

        - Samples for Verification: For sliding aluminum-framed glass doors and components required, prepared on Samples of size indicated below:

          - Main Framing Member: 12-inch-long (300-mm-long) section with weather stripping, glazing bead, and factory-applied color finish.

          - Hardware: Full-size units with factory-applied finish.

          - <Insert component>: <Insert description>.

        - Product Schedule: For sliding aluminum-framed glass doors.[ Use same designations indicated on Drawings.]

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article. Retain "testing agency" option if Contractor is providing testing in "Field Quality Control" Article.

        - Qualification Data: For [Installer] [manufacturer] [and] [testing agency].

Retain first option in "Product Test Reports" Paragraph below if manufacturers verify and their product literature indicates that sliding doors were tested in accordance with AAMA/WDMA/CSA procedures.

        - Product Test Reports: For each sliding aluminum-framed glass door, for tests performed by [manufacturer and witnessed by a qualified testing agency] [a qualified testing agency], and for each class and performance grade indicated, tested at AAMA gateway size.

Retain "Field quality-control reports" Paragraph below if Contractor is responsible for field quality-control testing and inspecting in "Field Quality Control" Article.

        - Field quality-control reports.

        - Sample Warranty: For manufacturer's special warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For finishes, weather stripping, operable panels, and operating hardware to include in maintenance manuals.

      - QUALITY ASSURANCE

        - Manufacturer Qualifications: A manufacturer capable of fabricating sliding aluminum-framed glass doors that meet or exceed performance requirements indicated and of documenting this performance by inclusion in lists and by labels, test reports, and calculations.

        - Installer Qualifications: An installer acceptable to sliding aluminum-framed glass door manufacturer for installation of units required for this Project.

        - Mockups: Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for materials and execution.

          - Build mockup for sliding aluminum-framed glass doors, as indicated on Drawings.

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Manufacturer's Special Warranty: Manufacturer agrees to repair or replace components of sliding aluminum-framed glass doors that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Failure to meet performance requirements.

            - Structural failures including excessive deflection.

Most manufacturers exclude condensation from their warranties; add requirement for condensation to first subparagraph below if available.

            - Excessive water leakage or air infiltration.

            - Faulty operation of movable panels and hardware.

            - Deterioration of metals, metal finishes, and other materials beyond normal weathering.

            - Failure of insulating glass[ and laminated glass].

            - <Insert failure modes>.

Verify available warranties and warranty periods.

          - Warranty Period:

            - Sliding Door: [Three] [Five] <Insert number> years from date of Substantial Completion.

Warranty period for glazing units varies by type. Insulating-glass units typically have longer warranties than laminated glass. Some manufacturers may insist that warranties begin on date of manufacture or sales.

            - Insulating-Glass Units: [10] [20] <Insert number> years from date of Substantial Completion.

            - Laminated Glass: [Five] <Insert number> years from date of Substantial Completion.

Warranty period for aluminum finish depends on type. Baked-on enamel finishes often have 10-year warranties; high-performance fluoropolymer finishes often have 20-year warranties.

            - Aluminum Finish: [Five] [10] [20] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - PERFORMANCE REQUIREMENTS

See the Evaluations for discussion of establishing performance requirements using AAMA/WDMA/CSA 101/I.S.2/A440.

        - Product Standard: Comply with AAMA/WDMA/CSA 101/I.S.2/A440 for minimum standards of performance, materials, components, accessories, and fabrication unless more stringent requirements are indicated.

Retain "Product Certification" Subparagraph below if AAMA certification and labeling is needed. Not all manufacturers that test products in accordance with AAMA/WDMA/CSA 101/I.S.2/A440 requirements participate in AAMA's third-party certification program for listing and labeling sliding aluminum-framed glass doors.

          - Product Certification: AAMA certified with label attached to each door.

Coordinate performance class and grade with available options and features. Some options affect door performance or may not be available for doors meeting selected performance class and grade.

        - Performance Class and Grade: AAMA/WDMA/CSA 101/I.S.2/A440 as follows:

          - Minimum Performance Class: [Class R] [Class LC] [Class CW] [Class AW] [As indicated on Drawings] <Insert class>.

AAMA/WDMA/CSA 101/I.S.2/A440 establishes a gateway Performance Grade for sliding doors to qualify for each Performance Class. The gateway Performance Grade is 15 for Class R, 25 for Class LC, 30 for Class CW, and 40 for Class AW. For a particular project, the minimum Performance Grade for sliding doors is typically based on the design pressure.

          - Minimum Performance Grade: [Grade 15] [Grade 20] [Grade 25] [Grade 30] [Grade 35] [Grade 40] [Grade 45] [Grade 50] [As indicated on Drawings] <Insert grade>.

Retain "Thermal Transmittance" Paragraph below if required to comply with requirements of authorities having jurisdiction. First option is the Energy Star Requirement for sliding entry doors with glazed area equal to or greater than 29.8 percent of door area (referred to by Energy Star as "1/2 lite") and applies in the entire United States. Requirements of authorities having jurisdiction may exceed the Energy Star minimum.

        - Thermal Transmittance: NFRC 100 maximum total fenestration product U-factor of [0.32 Btu/sq. ft. x h x deg F (1.83 W/sq. m x K)] [0.35 Btu/sq. ft. x h x deg F (2.0 W/sq. m x K)] [0.60 Btu/sq. ft. x h x deg F (3.43 W/sq. m x K)] <Insert value>.

Options in "Solar Heat-Gain Coefficient (SHGC)" Paragraph below are based on Energy Star requirements effective January 2010. First option is the Energy Star Requirement for sliding entry doors with glazing and applies in the entire United States. Requirements of authorities having jurisdiction may exceed the Energy Star minimum.

        - Solar Heat-Gain Coefficient (SHGC): NFRC 200 maximum total fenestration product SHGC of [0.30] [0.27] <Insert value>.

CRF values are typically between 45 and 75. First option in "Condensation-Resistance Factor (CRF)" Paragraph below is suitable for most of the United States at 25 percent relative humidity where outdoor design temperature is below 0 deg F (minus 18 deg C); second option is suitable for same relative humidity at minus 10 deg F (minus 23 deg C). In northern states where winter outdoor design temperature is lower, increase factor. If buildings have mechanically controlled humidification, coordinate CRF with design conditions for humidity level. See AAMA 1503.

        - Condensation-Resistance Factor (CRF): Provide sliding aluminum-framed glass doors tested for thermal performance in accordance with AAMA 1503, showing a CRF of [45] [52] <Insert value>.

        - Thermal Movements: Provide sliding aluminum-framed glass doors, including anchorage, that allow for thermal movements resulting from the following maximum change (range) in ambient and surface temperatures by preventing buckling, opening of joints, overstressing of components, failure of joint sealants, failure of connections, and other detrimental effects. Base engineering calculation on surface temperatures of materials due to both solar heat gain and nighttime-sky heat loss.

Differential values in "Temperature Change (Range)" Subparagraph below (for aluminum in particular) are suitable for most of the United States.

          - Temperature Change (Range): [120 deg F (67 deg C), ambient; 180 deg F (100 deg C)] <Insert temperature range>, material surfaces.

Retain "Sound Transmission Class (STC)" or "Outside-Inside Transmission Class (OITC)" Paragraph below after verifying availability of test data. STC evaluates construction subject to interior, spoken-voice frequencies, while OITC evaluates using a sound-frequency range with lower frequencies more representative of transportation noise, which building envelopes are more likely to experience. OITC is generally a better evaluation method for exterior fenestration, but fewer manufacturers have OITC test data available.

        - Sound Transmission Class (STC): Rated for not less than [28] [32] <Insert rating> STC when tested for laboratory sound transmission loss in accordance with ASTM E90 and determined by ASTM E413.

        - Outside-Inside Transmission Class (OITC): Rated for not less than [23] [27] [30] <Insert rating> OITC when tested for laboratory sound transmission loss in accordance with ASTM E90 and determined by ASTM E1332.

Retain "Windborne-Debris Impact Resistance" Paragraph below to suit Project. The IBC establishes criteria for buildings in hurricane-prone locations. In paragraph, "enhanced" option applies to essential facilities and has additional requirements. Verify requirements of authorities having jurisdiction. Verify which manufacturers have tested products and can demonstrate compliance.

        - Windborne-Debris Impact Resistance: Passes ASTM E1886 missile-impact and cyclic-pressure tests in accordance with ASTM E1996 for Wind Zone [1] [2] [3] [4] for [basic] [enhanced] protection.

Insert increased heights if different from those in "Large-Missile Test" and "Small-Missile Test" subparagraphs below. For enhanced protection, delete "Small-Missile Test" Subparagraph.

          - Large-Missile Test: For glazing located within [30 feet (9.1 m)] <Insert dimension> of grade.

          - Small-Missile Test: For glazing located between 30 feet (9.1 m) and [60 feet (18.3 m)] <Insert dimension> above grade.

      - EXTERIOR SLIDING ALUMINUM-FRAMED GLASS DOORS

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Source Limitations: Obtain sliding aluminum-framed glass doors from single source from single manufacturer.

AAMA/WDMA/CSA 101/I.S.2/A440 includes requirements for aluminum, thermal breaks, and other materials and components. If more stringent requirements apply, insert them in this article.

        - Frames and Door Panels: Fabricated from aluminum extrusions complying with AAMA/WDMA/CSA 101/I.S.2/A440.

          - Thermally Improved Construction: Fabricate frames and door panels with an integral, concealed, low-conductance thermal barrier located between exterior and interior surfaces in a manner that eliminates direct metal-to-metal contact.

Most manufacturers use high-leg, high-performance sills to attain better performance levels for air infiltration and water penetration.

        - Threshold and Sill Cap/Track: Provide extruded-aluminum threshold and track of thickness, dimensions, and profile indicated; designed to comply with performance requirements indicated[ and to drain to the exterior]; with manufacturer's standard finish.

Retain "Low-Profile Floor Track" Subparagraph below for ADA-compliant track offered by some manufacturers and that may be permitted or required by authorities having jurisdiction. Low-profile tracks have lower water infiltration resistance than standard height tracks. Coordinate available AAMA performance grade with manufacturers when using low-profile track.

          - Low-Profile Floor Track: ADA-ABA compliant.

      - GLAZING

Retain third option in "Glass and Glazing" Paragraph below if glass and glazing requirements are indicated in Section 088000 "Glazing." Revise for custom glass or glazing requirements if available from selected manufacturers.

        - Glass and Glazing: Manufacturer's standard glazing system that produces weathertight seal. [Comply with requirements for windborne-debris resistance.] [Comply with requirements indicated in Section 088000 "Glazing."] <Insert glazing requirements.>

Delete subparagraphs below if retaining third option in "Glass and Glazing" Paragraph above.

          - Glass: ASTM C1036, Type 1, q3, Category II safety glass complying with testing requirements in 16 CFR 1201.

Retain "Safety Glazing Labeling" Subparagraph below if applicable and if labeling is required. Not all manufacturers participate in third-party testing programs.

          - Safety Glazing Labeling: Permanently mark safety glazing with certification label of [the SGCC] [the SGCC or another certification agency acceptable to authorities having jurisdiction] [or] [the manufacturer]. Label must indicate manufacturer's name, type of glass, thickness, and safety glazing standard with which glass complies.

          - Tint: [Clear] [Gray] [Bronze] [Green] <Insert tint>.

Delete "Insulating-Glass Units" Subparagraph below if using monolithic glazing. Retain option in "Insulating-Glass Units" Subparagraph below after verifying that manufacturers obtain glass units from glass manufacturers that participate in the Insulating Glass Certification Council program.

          - Insulating-Glass Units: ASTM E2190[, certified through IGCC as complying with requirements of IGCC].

            - Filling: Fill space between glass lites with [air] [argon].

            - Low-E Coating: [Manufacturer's standard] [Pyrolytic on second surface] [Sputtered on second surface] [Sputtered on third surface] [Sputtered on second or third surface] <Insert coating>.

Retain "Integral Louver Blinds" Subparagraph below if required. Verify availability with manufacturers.

            - Integral Louver Blinds: Glass manufacturer's standard horizontal louver blinds with aluminum slats and polyester fiber cords, located in space between glass lites and operated by hardware located on inside face of door panel.

              - Operation: [Tilt only] [Tilt, raising, and lowering].

              - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

      - HARDWARE

Nonmagnetic stainless steel, Series 300, or a superior corrosion-resistant-coated metal may be required to meet specific customer or regional needs and for protection against corrosive environments, such as in urban, coastal, or industrial areas.

        - General: Provide manufacturer's standard hardware, fabricated from a corrosion-resistant material compatible with aluminum complying with AAMA 907 and designed to smoothly operate, tightly close, and securely lock sliding aluminum-framed glass doors.

Some manufacturers may offer door pulls in other designs and materials. Revise "Door Pulls" Paragraph below if selecting non-standard pull.

        - Door Pulls: Provide manufacturer's standard pull.

          - Color and Finish: [As indicated by manufacturer's designations] [Match door frame] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and finish>.

Revise "Lock" Paragraph below to suit Project's locking requirements, or insert others. Multipoint locking devices improve security and performance but are not offered by all manufacturers.

        - Lock: Install manufacturer's keyed cylinder lock and [multipoint] locking device on each movable panel, lockable from the inside [only] [and outside]. Adjust locking device to allow unobstructed movement of the panel across adjacent panel in the direction indicated.

Include key-operated cylinders in "Keying System" Subparagraph below or in Section 087100 "Door Hardware." Sliding aluminum-framed glass doors designed primarily for residential use may not accept key cylinders; confirm with manufacturers.

          - Keying System: [All cylinders keyed alike] [Keyed to match other building entrances] <Insert instructions>.

      - INSECT SCREENS

Manufacturers' designs may incorporate screens on inside or outside of door. Review availability and revise "General" Paragraph below if limiting screen location.

        - General: Design sliding aluminum-framed glass doors to accommodate screens in a tight-fitting, removable arrangement, fully integrated with door frame. Provide screen for each operable door panel.

Revise "Aluminum Frames" Paragraph below if allowing screen frames to be fabricated of roll-formed aluminum, available from some manufacturers. Refer to Evaluations.

        - Aluminum Frames: Manufacturer's standard extruded aluminum profile fabricated from aluminum alloy complying with SMA 1201 and finished to match door frame. Fabricate frames with mitered or coped joints or corner extrusions, concealed fasteners, and removable PVC spline/anchor concealing edge of frame.

        - Screen Fabric: Provide screen mesh of PVC-coated, glass-fiber threads; woven and fused to form a fabric mesh resistant to corrosion, shrinkage, stretch, impact damage, and weather deterioration; and complying with ASTM D3656

          - Fabric Color: [Manufacturer's standard] [Charcoal gray] [Silver gray] <Insert color>.

        - Hardware: Manufacturer's standard noncorrosive metal.

          - Lock: Manufacturer's standard pull and keyless locking device on each movable panel, lockable from inside only, designed to allow unobstructed movement of panel across adjacent panel.

      - ACCESSORIES

        - Fasteners: Noncorrosive and compatible with door members, trim, hardware, anchors, and other components.

Generally, retain "Exposed Fasteners" Subparagraph below. Revise if exposed fasteners are permitted.

          - Exposed Fasteners: Do not use exposed fasteners to the greatest extent possible. For application of hardware, use fasteners that match finish hardware being fastened.

        - Anchors, Clips, and Accessories: Provide anchors, clips, and accessories of aluminum, nonmagnetic stainless steel, or zinc-coated steel or iron for sliding aluminum-framed glass doors, complying with ASTM B456 or ASTM B633 for SC 3 severe service conditions; provide sufficient strength to withstand design pressure indicated.

          - Windborne-Debris Resistance: Provide anchors of same design used in windborne-debris resistance testing.

Insert requirements for false-muntin grilles and simulated divided lites. Verify availability with manufacturer.

      - FABRICATION

        - Fabricate sliding aluminum-framed glass doors in sizes indicated. Include a complete system for assembling components and anchoring doors.

        - Fabricate sliding aluminum-framed glass doors that are reglazable without dismantling panel framing.

        - Weather Stripping: Provide full-perimeter weather stripping for each door panel.

        - Weep Holes: Provide weep holes and internal drainage passages to conduct infiltrating water to exterior.

        - Complete fabrication, assembly, finishing, hardware application, and other work in the factory to greatest extent possible. Disassemble components only as necessary for shipment and installation.

Retain "Factory-Glazed Fabrication" Paragraph below if doors are factory glazed. Verify size limitations of factory glazing with manufacturers.

        - Factory-Glazed Fabrication: Glaze sliding aluminum-framed glass doors in the factory where practical and possible for applications indicated. Comply with requirements in Section 088000 "Glazing" and with AAMA/WDMA/CSA 101/I.S.2/A440.

      - GENERAL FINISH REQUIREMENTS

        - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Appearance of Finished Work: Variations in appearance of abutting or adjacent pieces are acceptable if they are within one-half of the range of approved Samples. Noticeable variations in the same piece are not acceptable. Variations in appearance of other components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - ALUMINUM FINISHES

If retaining more than one finish in paragraphs below, indicate location of each on Drawings or by inserts.

Retain one of two options in "Clear Anodic Finish" Paragraph below. Verify availability with manufacturers.

        - Clear Anodic Finish: AAMA 611, [AA-M12C22A41, Class I, 0.018 mm] [AA-M12C22A31, Class II, 0.010 mm] or thicker.

Retain one of two options in "Color Anodic Finish" Paragraph below. Verify availability with manufacturers.

        - Color Anodic Finish: AAMA 611, [AA-M12C22A42/A44, Class I, 0.018 mm] [AA-M12C22A32/A34, Class II, 0.010 mm] or thicker.

Options in "Color" Subparagraph below are examples only and may vary in color range and availability among manufacturers.

          - Color: [Light bronze] [Medium bronze] [Dark bronze] [Black] [Match Architect's sample] [As selected by Architect from full range of industry colors and color densities] <Insert color>.

"Baked-Enamel or Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

        - Baked-Enamel or Powder-Coat Finish: AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

Retain "High-Performance Organic Finish, Two-Coat PVDF," "Superior-Performance Organic Finish, Three-Coat PVDF," "Superior-Performance Organic Finish, Four-Coat PVDF," "Superior-Performance Organic Finish, Single-Coat FEVE," or "Superior-Performance Organic Finish, Two-Coat FEVE" Paragraph below; if more than one is required, indicate location of each system on Drawings, in schedules, or by inserts. Coordinate finish system selected with special finish warranty period specified in Part 1 "Warranty" Article.

In "High-Performance Organic Finish, Two-Coat PVDF" Paragraph below, retain AAMA 2604 with 50 percent resin content by weight in color coat or AAMA 2605 with 70 percent resin content by weight in color coat for high-performance organic coatings on extrusions and panels. If specific products are required, name coating manufacturers and products.

        - High-Performance Organic Finish, Two-Coat PVDF: Fluoropolymer finish complying with [AAMA 2604] [AAMA 2605] and containing not less than [50] [70] percent PVDF resin by weight in color coat.

          - Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions [for seacoast and severe environments].

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

        - Superior-Performance Organic Finish, Three-Coat PVDF: Fluoropolymer finish complying with AAMA 2605 and containing not less than 70 percent PVDF resin by weight in both color coat and clear topcoat.

          - Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions [for seacoast and severe environments].

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

        - Superior-Performance Organic Finish, Four-Coat PVDF: Fluoropolymer finish complying with AAMA 2605 and containing not less than 70 percent PVDF resin by weight in both color coat and clear topcoat.

          - Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions [for seacoast and severe environments].

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

"Superior-Performance Organic Finish, Single-Coat FEVE" Paragraph below is not suitable for seacoast and severe environments.

        - Superior-Performance Organic Finish, Single-Coat FEVE: Fluoropolymer finish complying with AAMA 2605.

          - Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

        - Superior-Performance Organic Finish, Two-Coat FEVE: Fluoropolymer finish complying with AAMA 2605.

          - Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions for seacoast and severe environments.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

- EXECUTION

      - EXAMINATION

        - Examine openings, substrates, structural support, anchorage, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Verify rough opening dimensions, levelness of threshold substrate, and operational clearances.

        - Examine wall flashings, vapor retarders, water and weather barriers, and other built-in components to ensure a coordinated, weathertight sliding aluminum-framed glass door installation.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Comply with Drawings, Shop Drawings, and manufacturer's written instructions for installing doors, hardware, accessories, and other components.

Retain "Windborne-Debris Resistance" Paragraph below for installations requiring windborne-debris resistance. Proper anchorage of units is necessary to maintain performance as tested.

        - Windborne-Debris Resistance: Anchor sliding aluminum-framed glass doors that have been tested for windborne-debris resistance to structure using anchoring method, fastener type, and fastening frequency identical to that used in windborne-debris resistance testing.

        - Install sliding aluminum-framed glass doors level, plumb, square, true to line, without distortion, without warp or rack of frames and panels, and without impeding thermal movement; anchored securely in place to structural support; and in proper relation to wall flashing, vapor retarders, air barriers, water/weather barriers, and other adjacent construction.

        - Set sill members in bed of sealant or with gaskets, as indicated, to provide weathertight construction.

        - Install sliding aluminum-framed glass doors and components to drain condensation, water penetrating joints, and moisture migrating within doors to the exterior.

        - Separate aluminum and other corrodible surfaces from sources of corrosion or electrolytic action at points of contact with other materials.

      - FIELD QUALITY CONTROL

Retain this article if field tests are required. If retaining, indicate number of doors to be tested. Field tests for air and water leakage should be specified for significant projects.

Retain "Testing Agency" Paragraph below to identify who must perform tests and inspections. If retaining second option in "Testing Agency" Paragraph, retain "Field quality-control reports" Paragraph in "Informational Submittals" Article.

        - Testing Agency: [Owner will engage] [Engage] a qualified testing agency to perform tests and inspections.

          - Testing and inspecting agency will interpret tests and state in each report whether tested work complies with or deviates from requirements.

Revise "Testing Services" Paragraph below if more stringent testing is required. AAMA 502 Test Method A is default unless otherwise indicated.

        - Testing Services: Test and inspect installed sliding aluminum-framed glass doors as follows:

          - Testing Methodology: Test sliding aluminum-framed glass doors for air infiltration and water resistance in accordance with AAMA 502.

          - Air-Infiltration Testing:

            - Test Pressure: That required to determine compliance with AAMA/WDMA/CSA 101/I.S.2/A440 performance class indicated.

            - Allowable Air-Leakage Rate: [1.5] <Insert number> times the applicable AAMA/WDMA/CSA 101/I.S.2/A440 rate for product type and performance class rounded down to one decimal place.

          - Water-Resistance Testing:

            - Test Pressure: [Two-thirds] <Insert number> times test pressure required to determine compliance with AAMA/WDMA/CSA 101/I.S.2/A440 performance grade indicated.

            - Allowable Water Infiltration: No water penetration.

          - Testing Extent: [Three] [Three mockup] <Insert number or description> sliding aluminum-framed glass doors of each type as selected by Architect and a qualified independent testing and inspecting agency. Conduct tests after perimeter sealants have cured.

          - Test Reports: Prepared in accordance with AAMA 502.

See Section 014000 "Quality Requirements" for retesting and reinspecting requirements and Section 017300 "Execution" for requirements for correcting the Work.

        - Sliding aluminum-framed glass door will be considered defective if it does not pass tests and inspections.

      - ADJUSTING, CLEANING, AND PROTECTION

        - Lubricate hardware and moving parts.

        - Adjust operating panels and screens to provide a tight fit at contact points and weather stripping for smooth operation, without binding, and a weathertight closure. Adjust hardware for proper alignment, smooth operation, and proper latching without unnecessary force or excessive clearance.

        - Clean exposed surfaces immediately after installing sliding aluminum-framed glass doors. Avoid damaging protective coatings and finishes. Remove nonpermanent labels, excess sealants, glazing materials, dirt, and other substances.

        - Remove and replace glass that has been broken, chipped, cracked, abraded, or damaged during construction period.

        - Protect sliding aluminum-framed glass door surfaces from contact with contaminating substances resulting from construction operations. If contaminating substances contact sliding aluminum-framed glass door surfaces, remove contaminants immediately in accordance with manufacturer's written instructions.

Retain first paragraph below if doors are factory finished.

        - Refinish or replace sliding aluminum-framed glass doors with damaged finishes.

        - Replace damaged components.

END OF SECTION 081816.13