Copyright 2018 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 083113.53 - SECURITY ACCESS DOORS AND FRAMES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Security access doors and frames for walls and ceilings.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 083113 "Access Doors and Frames" for access doors and frames for nonsecurity applications.

      - ALLOWANCES

Retain products and Work included in this Section that are covered by cash or quantity allowance. Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the allowance.

        - Security access doors and frames are part of a security access door and frame allowance.

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details materials, individual components and profiles, and finishes.

        - Samples: For each type of security access door and frame and for each finish specified, complete assembly minimum 6 by 6 inches (150 by 150 mm) in size.

        - Product Schedule: For security access doors and frames.[ Use same designations indicated on Drawings.]

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SECURITY ACCESS DOORS AND FRAMES

Copy first paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Medium-Security Flush Access Doors with Exposed Flanges <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Locations: [Wall] [Ceiling] [Wall and ceiling] <Insert location or substrate>.

          - Door Size: <Insert door size>.

Retain "Uncoated Steel Sheet for Door," "Metallic-Coated Steel Sheet for Door," or "Stainless Steel Sheet for Door" Subparagraph below. Because gage thickness is not defined by industry standards, decimal thickness must be indicated. Most manufacturers provide 12-gage medium-security access doors; verify availability if retaining 14 gage.

          - Uncoated Steel Sheet for Door: [Nominal 0.105 inch (2.66 mm), 12 gage] [Nominal 0.075 inch (1.90 mm), 14 gage]; factory primed.

          - Metallic-Coated Steel Sheet for Door: [Nominal 0.108 inch (2.74 mm), 12 gage] [Nominal 0.079 inch (2.01 mm), 14 gage]; factory primed.

          - Stainless Steel Sheet for Door: [Nominal 0.109 inch (2.78 mm), 12 gage] [Nominal 0.078 inch (1.98 mm), 14 gage]; ASTM A480/A480M No. 4 finish.

          - Frame Material: [Same material, thickness, and finish as door] [Minimum 3/16-by-1-1/2-by-1-1/2-inch (4.7-by-38-by-38-mm) angle welded with joints ground smooth; factory primed] <Insert material, thickness, and finish>.

          - Hinges: [Manufacturer's standard security hinge] <Insert hinge type>.

Revise "Latch and Lock" Subparagraph below for other hardware offered by some manufacturers; options include commonly available choices.

          - Latch and Lock: [Detention deadbolt with parametric key] [Cam latch, cylinder key operated] [Cam latch, hex-head wrench operated] [Cam latch, pinned-hex-head wrench operated] [Cam latch, spanner-head wrench operated] [Prepared for mortise cylinder] [As indicated on Drawings] [As indicated in schedule] <Insert operator>[ with interior release].

Copy first paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - High-Security Flush Access Doors <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Locations: [Wall] [Ceiling] [Wall and ceiling] <Insert location or substrate>.

          - Door Size: <Insert door size>.

Retain "Uncoated Steel Sheet for Door," "Metallic-Coated Steel Sheet for Door," or "Stainless Steel Sheet for Door" Subparagraph below. Because gage thickness is not defined by industry standards, decimal thickness must be indicated.

          - Uncoated Steel Sheet for Door: Nominal 0.134 inch (3.42 mm), 10 gage; factory primed.

          - Metallic-Coated Steel Sheet for Door: Nominal 0.138 inch (3.50 mm), 10 gage; factory primed.

          - Stainless Steel Sheet for Door: Nominal 0.141 inch (3.57 mm), 10 gage; ASTM A480/A480M No. 4 finish.

          - Frame Material: [Same material, thickness, and finish as door] [Minimum 3/16-by-2-by-2-by-3-inch (4.7-by-50-by-50-by-76-mm) angle welded with joints ground smooth; factory prime] <Insert material, thickness, and finish>.

          - Hinges: [Manufacturer's standard security hinge] <Insert hinge type>.

Revise "Latch and Lock" Subparagraph below for other hardware offered by some manufacturers; options include commonly available choices.

          - Latch and Lock: [Detention deadbolt with parametric key] [Cam latch, cylinder key operated] [Cam latch, hex-head wrench operated] [Cam latch, pinned-hex-head wrench operated] [Cam latch, spanner-head wrench operated] [Prepared for mortise cylinder] [As indicated on Drawings] [As indicated in schedule] <Insert operator>[ with interior release].

Copy paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Maximum-Security Flush Access Doors <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Locations: [Wall] [Ceiling] [Wall and ceiling] <Insert location or substrate>.

          - Door Size: <Insert door size>.

Retain "Uncoated Steel Sheet for Door" or "Metallic-Coated Steel Sheet for Door" Subparagraph below. Because gage thickness is not defined by industry standards, decimal thickness must be indicated.

          - Uncoated Steel Sheet for Door: Nominal 0.180 inch (4.55 mm), 7 gage; factory primed.

          - Metallic-Coated Steel Sheet for Door: Nominal 0.183 inch (4.65 mm), 7 gage; factory primed.

          - Frame: Minimum 3/16-by-2-by-2-by-3-inch (4.7-by-50-by-50-by-76-mm) angle welded with joints ground smooth.

          - Hinges: Heavy-duty steel welded to door and frame.

Revise "Latch and Lock" Subparagraph below for other hardware offered by some manufacturers; options include commonly available choices.

          - Latch and Lock: [Detention deadbolt with parametric key] [Prepared for mortise cylinder] [As indicated on Drawings] [As indicated in schedule] <Insert operator>[ with interior release].

      - MATERIALS

Retain materials in this article or revise to suit Project.

        - Steel Plates, Shapes, and Bars: ASTM A36/A36M.

        - Steel Sheet: Uncoated or electrolytic zinc coated, ASTM A879/A879M, with cold-rolled steel sheet substrate complying with ASTM A1008/A1008M, Commercial Steel (CS), exposed.

        - Metallic-Coated Steel Sheet: ASTM A653/A653M, Commercial Steel (CS), Type B; with minimum G60 (Z180) or A60 (ZF180) metallic coating.

Retain the more expensive "Type 316" option in "Stainless Steel Plate, Sheet, and Strip " Paragraph below for more corrosion resistance.

        - Stainless Steel Plate, Sheet, and Strip: ASTM A240/A240M or ASTM A666, [Type 304] [Type 316]. Remove tool and die marks and stretch lines, or blend into finish.

Retain the more expensive "Type 316" option in "Stainless Steel Flat Bars" Paragraph below for more corrosion resistance.

        - Stainless Steel Flat Bars: ASTM A666, [Type 304] [Type 316]. Remove tool and die marks and stretch lines, or blend into finish.

        - Frame Anchors: Same type as door face.

        - Inserts, Bolts, and Anchor Fasteners: Hot-dip galvanized steel according to ASTM A153/A153M or ASTM F2329.

      - FABRICATION

        - General: Provide access door and frame assemblies manufactured as integral units ready for installation.

        - Metal Surfaces: For metal surfaces exposed to view in the completed Work, provide materials with smooth, flat surfaces without blemishes. Do not use materials with exposed pitting, seam marks, roller marks, rolled trade names, or roughness.

        - Doors and Frames: Grind exposed welds smooth and flush with adjacent surfaces. Furnish attachment devices and fasteners of type required to secure access doors to types of supports indicated.

        - Latch and Lock Hardware:

          - Quantity: Furnish number of latches and locks required to hold doors tightly closed.

          - Keys: Furnish two keys per lock and key all locks alike.

Retain "Mortise Cylinder Preparation" Subparagraph below if lock is not provided by access door and frame manufacturer.

          - Mortise Cylinder Preparation: Where indicated, prepare door panel to accept cylinder specified in [Section 087100 "Door Hardware."] [Section 087111 "Door Hardware (Descriptive Specification)."]

      - FINISHES

        - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Appearance of Finished Work: Noticeable variations in same piece are not acceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

        - Factory-Primed Finish: Apply manufacturer's standard lead- and chromate-free, universal primer immediately after surface preparation and pretreatment.

        - Stainless Steel Finishes:

          - Surface Preparation: Remove tool and die marks and stretch lines, or blend into finish.

ASTM A480/A480M No. 4 finish is standard for stainless steel flush access doors; confirm availability of other finishes with manufacturers.

          - Polished Finish: ASTM A480/A480M No. 4 finish. Grind and polish surfaces to produce uniform finish, free of cross scratches.

            - Run grain of directional finishes with long dimension of each piece.

            - When polishing is completed, passivate and rinse surfaces. Remove embedded foreign matter and leave surfaces chemically clean.

Retain "Bright, Cold-Rolled, Unpolished Finish" Subparagraph below for nondirectional finish.

          - Bright, Cold-Rolled, Unpolished Finish: ASTM A480/A480M No. 2B.

- EXECUTION

      - EXAMINATION

        - Examine substrates for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Comply with manufacturer's written instructions for installing access doors and frames.

      - ADJUSTING

        - Adjust doors and hardware, after installation, for proper operation.

Insert Security Access Door Schedule here if required.

END OF SECTION 083113.53