Copyright 2020 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 085123.23 - COLD-ROLLED STEEL WINDOWS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Cold-rolled steel windows.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 081113 "Hollow Metal Doors and Frames" for borrowed-lite interior steel windows.

          - Section 085119 "Stainless Steel Windows" for stainless steel windows fabricated from cold-rolled members.

          - Section 085123.13 "Hot-Rolled Steel Windows" for steel windows fabricated from hot-rolled members.

          - [Section 099113 "Exterior Painting"] [and] [Section 099123 "Interior Painting"] for on-site painting of factory prime-coated windows.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

Retain subparagraph below if additional agenda items specific to Project are required but not mentioned in Section 013100 "Project Management and Coordination."

          - Review requirements for cold-rolled steel windows, including, but not limited to, the following:

            - Coordinating finishing of cold-rolled steel windows with other work where color and finish matching is indicated.

            - Coordinating cold-rolled steel windows with other exterior wall components, including anchorage, glazing, flashing, weeping, air barriers, sealants, and protection of finishes.

            - Sequencing work to construct a watertight and weathertight exterior building enclosure.

            - <Insert agenda items>.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data: For each type of product.

          - Include construction details, material descriptions, fabrication methods, dimensions of individual components and profiles, hardware, finishes, and operating instructions.

        - Shop Drawings:

          - Plans, elevations, sections, and details.

          - Detail attachments to other work, and between units, if any.

          - Hardware and required clearances.

          - Mullion details, including reinforcement and stiffeners.

          - Munton details.

          - Flashing details.

          - Glazing details.

          - Accessories.

          - <Insert requirements>.

Retain "Samples for Initial Selection" or "Samples for Verification" Paragraph below, or both.

        - Samples for Initial Selection: Manufacturer's standard color sheets, showing full range of available colors for units with factory-applied color finishes.

          - Include available color or finish selections for hardware and accessories.

        - Samples for Verification: Actual sample of finished products for each type of cold-rolled steel window including weather stripping, glazing bead, and hardware.

          - Size: [Manufacturers' standard size] <Insert size>.

        - Product Schedule: For cold-rolled steel windows.[ Use same designations indicated on Drawings.]

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

      - INFORMATIONAL SUBMITTALS

Information submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

        - Test and Evaluation Reports:

          - Product Test Reports: For each cold-rolled steel window, for tests performed by a qualified testing agency.

        - Field Quality-Control Submittals:

Retain "Field quality-control reports" Subparagraph below if Contractor is responsible for field quality-control testing and inspecting.

          - Field quality-control reports.

Coordinate "Qualification Statements" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article. If inserting additional entities or specialist, add qualifications to "Quality Assurance" Article.

        - Qualification Statements: For [Installer] [manufacturer] [and] [testing agency].

        - Sample warranties.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For cold-rolled steel windows.

        - Warranty Documentation:

          - Manufacturers' special warranties.

          - Installer's special warranties.

      - QUALITY ASSURANCE

        - Qualifications:

          - Manufacturers: A manufacturer capable of fabricating cold-rolled steel windows that meets performance requirements indicated and of documenting performance by labels, test reports, and calculations.

          - Installers: [Fabricator of products] [Entity that employs installers and supervisors who are trained and approved by manufacturer] [Authorized representative who is trained and approved by manufacturer].

      - MOCKUPS

Coordinate this article with air barrier Sections for building and preconstruction testing of integrated wall mockups that include windows; insert additional requirements if applicable.

        - Build mockups [to verify selections made under Sample submittals] [to demonstrate aesthetic effects] [and] [to set quality standards for fabrication and installation].

          - Build mockup [as indicated on Drawings] <Insert mockup requirements>.

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Owner specifically approves such deviations by Change Order.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - FIELD CONDITIONS

Retain this article if Project includes existing construction that requires custom-fabricated window replacements. Insert requirements for field measurements.

        - <Insert requirements>.

      - WARRANTY

When warranties are required and available, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty: [Manufacturer agrees] [Installer agrees] [Manufacturer and Installer agree] to repair or replace components of cold-rolled steel windows that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures, including <Insert type of failure>.

            - Faulty operation of <Insert components>.

            - Deterioration of metals, metal finishes, and other materials beyond normal [weathering] [use].

            - <Insert conditions relating to specified products>.

            - <Insert failure modes>.

Verify warranty periods for units and components.

          - Warranty Period:

            - Window: [Two] [Three] <Insert number> years from date of Substantial Completion.

Warranty period for finish depends on type.

            - Finish: [Five] [10] [20] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - SOURCE LIMITATIONS

Retain this article to limit sources for the entire Section. Source limitations may also be specified in individual articles if desired.

        - Obtain cold-rolled steel windows from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

Performance requirements in this article include paragraphs for structural, air-leakage, and water-penetration performance based on ASTM standards. A few manufacturers have tested to AAMA's fenestration standard, AAMA/WDMA/CSA101/I.S.2/A440. If using AAMA standards to specify performance, see Section 085113 "Aluminum Windows" for sample text.

        - SWI Standards: Comply with applicable requirements in SWI's "Architect's Guide to Steel Windows and Doors" and "Specifications - Formed Cold Rolled Sections," except where more stringent requirements are indicated.

Usually, indicate on Drawings design loads determined by Project's structural engineer. Verify requirements of authorities having jurisdiction. See the Evaluations for additional information.

        - Structural Wind Loads: As indicated on Drawings.

Limits indicated in "Deflection Limits" Paragraph below are based on IBC requirements for framing members supporting glass.

        - Deflection Limits: Design glass framing system to limit deflection of glass edges in a direction perpendicular to glass plane to less than 1/175 of glass-edge length for each individual glazing light or 3/4 inch (19 mm), whichever is less, at design pressures.

ASTM E330/E330M test method evaluates structural performance of cold-rolled steel windows and not structural performance of contiguous construction.

        - Structural: Test in accordance with ASTM E330/E330M as follows:

          - When tested at positive and negative wind-load design pressures, cold-rolled steel windows do not evidence deflection exceeding specified limits.

          - When tested at [150] <Insert number> percent of positive and negative wind-load design pressures, assemblies, including anchorage, do not evidence material failures, structural distress, or permanent deformation of main framing members exceeding [0.2] <Insert number> percent of span.

Minimum test duration in accordance with ASTM E330/E330M is 10 seconds, which is historically U.S. practice.

          - Test Durations: As required by design wind velocity, but not less than [10] <Insert number> seconds.

The IECC requires limits on air leakage for exterior windows that are expressed in a different unit of measure than is used in options in "Air Leakage for Weather-Stripped Sash" Paragraph below. Consult cold-rolled steel window manufacturers and verify that products comply with requirements of authorities having jurisdiction. See the Evaluations.

        - Air Leakage for Weather-Stripped Sash: Not more than [0.37 cfm/ft. (0.18 L/s/m)] <Insert value> of sash crack length at a differential pressure across the windows of [6.24 lbf/sq. ft. (298 Pa)] <Insert value> when tested in accordance with ASTM E283/E283M.

        - Water Penetration for Weather-Stripped Sash: No leakage for 15 minutes when window is subjected to a rate of flow of 5 gal./h/sq. ft. (0.05 L/s/sq. m) with a differential pressure across the window of [2.86 lbf/sq. ft. (137 Pa)] <Insert value> when tested in accordance with ASTM E331.

First option in "Thermal Transmittance" Paragraph below is IECC default value for U-factor for a window with clear insulating glass; default values must be used for code purposes for products without NFRC certification. Consult cold-rolled steel window manufacturers and verify that products comply with requirements of authorities having jurisdiction. See the Evaluations.

        - Thermal Transmittance: NFRC 100 maximum whole-window U-factor of [0.80 Btu/sq. ft. x h x deg F (4.54 W/sq. m x K)] [0.27 Btu/sq. ft. x h x deg F (1.53 W/sq. m x K)] [0.32 Btu/sq. ft. x h x deg F (1.82 W/sq. m x K)] [0.36 Btu/sq. ft. x h x deg F (2.04 W/sq. m x K)] [0.43 Btu/sq. ft. x h x deg F (2.44 W/sq. m x K)] <Insert value>.

First option in "Solar Heat-Gain Coefficient (SHGC)" Paragraph below is based on IECC default value for SHGC for a window with clear insulating glass; default values must be used for code purposes for products without NFRC certification. Second option is based on ENERGY STAR requirements for North-Central Climate Zone. Third option is based on ENERGY STAR requirements for South-Central and Southern Climate zones. Consult cold-rolled stainless steel window manufacturers and verify that products comply with requirements of authorities having jurisdiction. See the Evaluations.

        - Solar Heat-Gain Coefficient (SHGC): NFRC 200 maximum whole-window SHGC of [0.70] [0.40] [0.25] [0.19] [0.15] <Insert value>.

Retain "Condensation Resistance" Paragraph below for projects where required. "CR" is the designation used by NFRC to rate "condensation resistance." See the Evaluations for further discussion.

        - Condensation Resistance: Provide cold-rolled steel windows with a CR of [33] [34] [35] <Insert number> minimum, determined in accordance with NFRC 500.

        - Thermal Movements: Allow for thermal movements from ambient and surface temperature changes.

Differential values in "Temperature Change" Subparagraph below are suitable for most of the United States.

          - Temperature Change: [120 deg F (67 deg C), ambient; 180 deg F (100 deg C) material surfaces] <Insert temperature change>.

If forced-entry resistance is required, retain "Forced-Entry Resistance" Paragraph below. Verify which manufacturers have tested products and can demonstrate compliance. Grade 10 is lowest level of measured performance.

        - Forced-Entry Resistance: Comply with Performance [Grade 10] <Insert grade> requirements when tested in accordance with ASTM F588.

Retain "Windborne-Debris-Impact Resistance" Paragraph below to suit Project. The IBC establishes criteria for buildings in hurricane-prone locations. In paragraph, "enhanced" option applies to essential facilities and has additional requirements. Verify requirements of authorities having jurisdiction. Verify which manufacturers have tested products and can demonstrate compliance.

        - Windborne-Debris-Impact Resistance: Passes ASTM E1886 missile-impact and cyclic-pressure tests in accordance with ASTM E1996 for Wind Zone [1] [2] [3] [4] for [basic] [enhanced] protection.

Insert increased heights if different from those in "Large-Missile Test" and "Small-Missile Test" subparagraphs below.

          - Large-Missile Test: For glazing located within [30 ft. (9.1 m)] <Insert dimension> of grade.

          - Small-Missile Test: For glazing located between 30 ft. (9.1 m) and [60 ft. (18.3 m)] <Insert dimension> above grade.

Retain "Fire-Test-Response Characteristics" Paragraph below if fire-protection-rated units are required. Fire-rated windows are required to be glazed with labeled fire-protection-rated glazing material and to have automatic closing devices on operable sash; sizes are limited by NFPA 80. Verify which manufacturers have tested products and can demonstrate compliance. Indicate rating and testing agency's design designation on Drawings.

        - Fire-Test-Response Characteristics: Complying with NFPA 80 and listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction for fire-protection ratings indicated, based on testing in accordance with NFPA 257 or UL 9.

Some companies manufacture products with blast resistance or improved noise reduction; insert requirements if needed.

        - <Insert requirements>.

      - COLD-ROLLED STEEL WINDOWS <Insert drawing designation>

Copy this article and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

"Types" Paragraph below may not be needed if window type is indicated by a product name or designation. Verify availability of window types with manufacturers.

        - Types: Provide the following window types in locations indicated on Drawings:

          - Casement: [Outswing] [Inswing].

          - Hung: [Double] [Single].

          - Horizontal sliding.

          - Projected: [Awning] [Hopper].

          - Fixed.

"Cold-Rolled Steel Windows" Paragraph below specifies SWI's weight and depth requirements for cold-rolled steel windows. SWI uses the term "ventilator" instead of "sash."

        - Cold-Rolled Steel Windows: Provide frame and sash members mechanically formed from metallic-coated, low-carbon, cold-rolled steel sheet complying with ASTM A653/A653M. Comply with SWI specifications for combined weight of frame and sash members and front-to-back depth of frame or sash members.

Retain "Thermally (Improved) (or) (Broken) Design" Subparagraph below for cold-rolled steel windows that use thermal spacers or struts to improve the window's U-factor. Designs may differ between manufacturers. Consult manufacturers to determine availability and type of isolation used.

          - Thermally [Improved] [or] [Broken] Design: Provide frame and sash members designed to isolate interior and exterior surfaces for improved thermal performance.

        - Window Finish: [Baked enamel or powder coat] [Primed for field painting].

Retain "Color" Subparagraph below if retaining first option in "Window Finish" Paragraph above.

          - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

        - <Double click to insert sustainable design text for recycled content.>

        - Mullions: Formed of cold-rolled steel matching window units; with anchors for support to structure and for installation of window units and having sufficient strength to withstand design pressure indicated. Provide mullions of profile indicated and with cover plates. Allow for erection tolerances and provide for movement of window units due to thermal expansion and building deflections.

Retain "Muntins" Paragraph below to suit Project. Use of muntins may affect energy performance of window. If retaining, verify that energy performance requirements can still be met.

        - Muntins: Cold-rolled steel T-shaped sections [fully welded] [mechanically fastened] to perimeter frame with cross notched intersections.

Retain "Sill Cap/Track" Paragraph below for horizontal-sliding windows.

        - Sill Cap/Track: Designed to comply with performance requirements indicated and to drain to the exterior.

        - Glazing Stops: Provide [screw-applied] [snap-on] glazing stops; coordinate with Section 088000 "Glazing" and with glazing system indicated. Provide glazing stops to match panel frames. Finish glazing stops to match window units if fabricated of steel; otherwise, provide manufacturer's standard finish.

        - Weather Stripping: Manufacturer's standard compressible weather stripping, complying with AAMA 701/702, ASTM C509, or ASTM C864 and designed for permanently resilient sealing under compression and for complete concealment when sash is closed.

      - GLAZING

Cold-rolled steel windows are typically manufactured for field glazing, although some manufacturers may offer factory glazing. Cold-rolled steel windows are typically glazed using conventional glazing tapes and gaskets. Coordinate requirements with glass and glazing specified in Section 088000 "Glazing." If specifying factory-glazed cold-rolled steel windows, retain "Glass and Glazing System" Paragraph below or add glazing requirements to this article.

        - Glass and Glazing System: See Section 088000 "Glazing" for glass units and glazing requirements for cold-rolled steel windows.

      - HARDWARE

        - General: Provide manufacturer's standard [nonremovable], [solid-bronze] [malleable-iron] [die-cast-metal] hardware, with operating components of stainless steel, carbon steel complying with AAMA 907, brass, bronze, or other corrosion-resistant material designed to smoothly operate, tightly close, and securely lock cold-rolled steel window sash; and sized to accommodate sash weight and dimensions.

Retain "Self-Closing Device for Fire-Rated Windows" Paragraph below for fire-rated, cold-rolled steel windows; verify components with manufacturer. Coordinate with other window hardware to ensure compatibility with self-closing device. Electronically actuated, resettable bimetal thermal link is an alternative device offered by a few manufacturers for some types of operation.

        - Self-Closing Device for Fire-Rated Windows: Manufacturer's standard [heat-activated fusible link] [electronically actuated thermal link] self-closing device, complying with NFPA 80.

Coordinate casement window hardware with "Self-Closing Device for Fire-Rated Windows" Paragraph above to ensure compatibility between self-closing device and other hardware.

Verify hardware requirements and availability with manufacturers.

        - Casement Window Hardware:

          - Operation:

Retain "Gear-Type Rotary Operator" or "Manually operated push/pull" Subparagraph below. Gear-type rotary operator is used for outswing casement windows only and may be used with or without screens.

            - Gear-Type Rotary Operator: Complying with AAMA 901 when tested in accordance with ASTM E405, Test Method A; located on jamb at sill.

Retain first subparagraph below if simultaneous operation of sash pairs is desired.

              - Provide operating device that opens and closes sashes simultaneously, securely closing them at both jambs without use of additional manually controlled locking devices.

              - Roto Operator Handle Type and Style: [As selected by Architect from manufacturer's full range of types and styles] <Insert type and style>.

            - Manually operated push/pull.

          - Hinges:

Retain "Friction Hinges," "Friction Hinges for Emergency Escape and Rescue," "Butt Hinges," or "Extension Hinges or Pivots" Subparagraph below. Friction hinges are required for use with manually operated sash.

            - Friction Hinges: Concealed, four-bar friction hinges with adjustable slide shoes complying with AAMA 904; two per sash.

Retain "Friction Hinges for Emergency Escape and Rescue" Subparagraph below if sashes are intended for emergency-escape and rescue use. If only certain sashes are intended for egress, indicate locations on Drawings. Revise subparagraph if heavy-duty hinges are required.

            - Friction Hinges for Emergency Escape and Rescue: Standard-duty, concealed, four-bar friction egress hinges with adjustable slide shoes; two per sash where indicated. Provide hinge designed to achieve 90-degree sash opening.

Hinge types in "Butt Hinges" and "Extension Hinges or Pivots" subparagraphs below are suitable for use only with rotary operators. Revise if sash size requires more than two hinges.

            - Butt Hinges: Heavy-duty, three-knuckle butt hinges with nylon bushings; two per sash.

            - Extension Hinges or Pivots: Nonfriction type; two per sash.

          - Locks:

Revise "Type" Subparagraph below if sash size requires two or more locks. If outswing casement windows are equipped with insect screens, screen wickets may be required; coordinate with lock type. Concealed multipoint lock with captive locking strike and head in last option below is used in blast or windborne-debris-impact applications.

            - Type: [Cam handle with keeper] [Lift-type, locking handle with keeper] [Custodial cam-action lock with keeper] [Linear operator with standard concealed multipoint lock] [Linear operator with concealed multipoint lock with captive locking strike and head] <Insert type>.

              - Style: [As selected by Architect from manufacturer's full range of styles] <Insert style>.

          - Control Device:

Retain "Limit Device" Subparagraph below for outswing casement windows unless window units are operated only for cleaning. Limit devices should always be used if using butt hinges or extension hinges or pivots. They are optional for inswing casement windows. Limit devices are designed to restrict sash opening and are desirable for low sills, child safety, buffering wind, high-rise applications where high winds and safety are important considerations, and windows used for emergency ventilation only.

            - Limit Device: Adjustable, concealed [friction adjustor/stay-bar] [friction adjustor/stay-bar with release key or tool].

Coordinate hung window hardware with "Self-Closing Device for Fire-Rated Windows" Paragraph to ensure compatibility between self-closing device and other hardware.

        - Hung Window Hardware:

          - Balances:

Retain "Counterbalancing Mechanism" or "Counterbalance and Pulley" Subparagraph below.

            - Counterbalancing Mechanism: Complying with AAMA 902, concealed, of size and capacity to hold sash stationary at any open position.

Verify availability of counterbalance and pulley with manufacturer.

            - Counterbalance and Pulley: Two per sash to operate in unison with stainless steel cable sash cord.

              - Single-Hung Upper Sash Retainer: Manufacturer's standard.

          - Locks and Latches:

            - Type: [Cam-action sweep lock and keeper on meeting rail; one per sash] [Cam-action sweep lock and keeper on meeting rail; two per sash] <Insert type>.

              - Style: [Manufacturer's standard] [As selected by Architect from manufacturer's full range of styles] <Insert style>.

          - Handles:

            - Type: [Lift] [Pull-down] handle; [one] [two] per sash.

              - Style: Manufacturer's standard.

Coordinate horizontal-sliding window hardware with "Self-Closing Device for Fire-Rated Windows" Paragraph to ensure compatibility between self-closing device and other hardware.

        - Horizontal-Sliding Window Hardware:

          - Rollers: Steel, lubricated, ball-bearing rollers.

          - Lock:

            - Type: [Manufacturer's standard, designed to allow unobstructed movement of the sash across adjacent sash in direction indicated and to operate from the inside only] <Insert type>.

          - Control Device:

            - Limit Device: Manufacturer's standard.

          - Handle:

            - Pull Handle: Manufacturer's standard.

Retain "Projected Window Hardware" Paragraph below for projected windows, which include awning windows that project out and hopper windows that project in. Coordinate projected window hardware with "Self-Closing Device for Fire-Rated Windows" Paragraph to ensure compatibility between self-closing device and other hardware.

        - Projected Window Hardware:

          - Operation:

Retain "Operating Device" Subparagraph below if required. Operating device is necessary for screened awning windows unless the screen contains a wicket for access to open.

            - Operating Device: [Gear-type rotary complying with AAMA 901 when tested in accordance with ASTM E405, Test Method A] [Push-bar-type] [, underscreen] sash operator located at sill.

              - Style: [As selected by Architect from manufacturer's full range of styles] <Insert style>.

            - Manually operated push/pull.

          - Hinges:

Retain one of first three subparagraphs below. Welded butt hinges may be used on hopper windows if used in conjunction with a stay bar; insert if required.

            - Concealed, four-bar friction hinges with adjustable slide shoes complying with AAMA 904; two per sash.

            - Balance arms with adjustable, nonabrasive friction pivots; two per sash.

            - Balance arms with adjustable, nonabrasive friction shoes; two per sash.

              - Provide sash operation that permits cleaning of the outside glass face from the interior.

              - Provide jamb-mounted, sliding, brass friction shoes with screw adjusters.

          - Locks:

Revise "Type" Subparagraph below if two locks are needed on wide units. Concealed multipoint lock with captive locking strike and head in fourth option below is used in blast or windborne-debris-impact applications. "Pole-operated spring catch lock" and "Pole-operated ring cam handle with keeper" options are used for window units with sash more than 72 inches (1800 mm) above the floor.

            - Type: [Cam handle with strike] [Cam handle with keeper] [Custodial cam-action lock with keeper] [Linear operator with standard concealed multipoint lock] [Linear operator with concealed multipoint lock with captive locking strike and head] [Pole-operated spring catch lock] [Pole-operated ring cam handle with keeper] <Insert type>.

              - Style: [As selected by Architect from manufacturer's full range of styles] <Insert style>.

Retain "Pole Operators" Subparagraph below if retaining one of two options for pole-operated lock types in "Type" Subparagraph above.

              - Pole Operators: Tubular-shaped, anodized aluminum; with rubber-capped lower end and standard push-pull hook at top to match hardware design; of sufficient length to operate window without reaching more than 60 inches (1500 mm) above floor; one pole operator and pole hanger per room that has operable windows more than 72 inches (1800 mm) above floor.

          - Control Device:

Retain "Limit Device" Subparagraph below if limit devices are required on awning windows. These are optional items designed to restrict sash opening and are desirable for low sills, child safety, buffering wind, high-rise applications where high winds and safety are important considerations, and windows used for emergency ventilation only.

            - Limit Device: Adjustable, concealed [friction adjustor/stay-bar] [friction adjustor/stay-bar with release key or tool] [support arms with adjustable, limited hold-open].

      - INSECT SCREENS

Retain this article if insect screens are needed. Copy and revise article if other types of screens, such as safety protection screens, are required for Project. Coordinate location of screen in first paragraph below with operation of sash.

        - Design windows and hardware to accommodate screens in a tight-fitting, removable arrangement, fully integrated with window. Locate screens on [inside] [outside] of window and provide for each operable exterior sash. Comply with SMA 1201.

Retain "Aluminum Screen Frames" or "Stainless Steel Screen Frames" Paragraph below.

        - Aluminum Screen Frames: Manufacturer's standard [extruded-aluminum] [or] [formed-tubular-aluminum] members; with mitered, coped joints, or corner extrusions; concealed fasteners;[ adjustable rollers;] and removable PVC or PE spline/anchor concealing edge of mesh.

In "Frame Wall Thickness" Subparagraph below, retain first option for frames mounted on outside or inside of window; second option is suitable for frames mounted inside only.

          - Frame Wall Thickness: [0.04 inch (1.0 mm)] [0.03 inch (0.8 mm)] minimum.

          - Finish:

Retain one of two subparagraphs below. Verify available finishes with manufacturers.

            - Anodized aluminum in manufacturer's standard color.

            - [Baked-enamel or powder-coat] [High-performance organic] finish in [manufacturer's standard color] [color selected by the Architect from manufacturer's full range] [color to match steel windows].

        - Stainless Steel Screen Frames: Fabricate frames of tubular-shaped, nonmagnetic stainless steel members of 0.02-inch (0.5-mm) minimum wall thickness; in [ASTM A480/A480M, No. 2B cold-rolled, bright finish] [finish to match cold-rolled steel window].

Retain "Glass-Fiber Mesh Fabric," "Aluminum Wire Fabric," or "Stainless Steel Wire Fabric" Paragraph below.

Usually, retain first option in "Glass-Fiber Mesh Fabric" Paragraph below. Second option is suitable for areas plagued by small insects, such as no-see-ums. Second option is also suitable for use as a solar screen that blocks up to 65 percent of incident solar heat and glare. Some manufacturers offer a screen that uses a finer mesh that transmits more light; confirm availability and revise paragraph if required.

        - Glass-Fiber Mesh Fabric: Complies with ASTM D3656/D3656M, [18-by-14 or 18-by-16] [20-by-20 or 20-by-30] count per sq. in. (645 sq. mm) mesh of PVC-coated, glass-fiber threads; woven and fused to form a fabric mesh resistant to corrosion, shrinkage, stretch, impact damage, and weather deterioration.

          - Mesh Color: [Gray] <Insert color>.

        - Aluminum Wire Fabric: 18-by-16 count per sq. in. (645 sq. mm) mesh of 0.011-inch- (0.28-mm-) diameter, coated aluminum wire.

          - Wire-Fabric Finish: [Natural bright] [Charcoal gray] [Black].

        - Stainless Steel Wire Fabric: [18-by-16] [18-by-18] count per sq. in. (645 sq. mm) mesh of 0.009-inch- (0.2-mm-) minimum diameter, nonmagnetic stainless steel wire, Type 304 or 316.

Retain "Wickets" Paragraph below for screened-window units with outward-opening sash if wickets are necessary for sash operation.

        - Wickets: Provide [sliding] [or] [hinged] wickets, framed and trimmed for a tight fit and durability during handling.

      - ACCESSORIES

Retain items in this article that are needed for fabrication or installation of selected units.

        - Fasteners: Provide fasteners of bronze, brass, stainless steel, or other metal that are warranted by manufacturer to be noncorrosive and compatible with trim, hardware, anchors, and other components of cold-rolled steel windows.

Generally, retain "Exposed Fasteners" Subparagraph below. Revise if exposed fasteners are permitted.

          - Exposed Fasteners: Do not use exposed fasteners to greatest extent possible. For application of hardware, use fasteners that match finish hardware being fastened.

        - Anchors, Clips, and Window Accessories: Provide units of stainless steel, hot-dip zinc-coated steel, bronze, brass, or iron complying with ASTM A123/A123M. Provide units with sufficient strength to withstand design pressure indicated.

          - Windborne-Debris-Impact Resistance: Provide anchors and clips of same design used to pass windborne-debris-impact-resistance testing.

        - Sealant: For sealants required within fabricated windows, provide manufacturer's standard, permanently elastic, nonshrinking, and nonmigrating type recommended by sealant manufacturer for joint size and movement.

      - FABRICATION

        - Fabricate cold-rolled steel windows of type and in sizes indicated to comply with SWI standards. Include a complete system for assembly of components and anchorage of window units.

        - Provide units that are reglazable without dismantling framing.

Cold-rolled steel windows are usually field glazed. Revise first paragraph below if manufacturer offers factory-glazed windows.

        - Prepare windows for site glazing.

Confirm method of fabrication with manufacturers; most manufacturers offer only one method.

        - Subframes and Operable Sash: Formed of cold-rolled steel of profile indicated. Miter or cope corners, and [mechanically fasten and seal joints] [or] [weld and dress joints smooth].

        - Weather Stripping: Provide full-perimeter weather stripping for each operable sash unless otherwise indicated.

        - Provide weep holes and internal water passages to conduct infiltrating water to the exterior.

        - Provide water-shed members above [casement] [horizontal-sliding] sash.

      - METALLIC-COATED STEEL SHEET FINISHES

        - Surface Preparation: Clean surfaces of oil and other contaminants. Use cleaning methods that do not leave residue. After cleaning, apply a conversion coating compatible with the organic coating to be applied over it. Clean welds, mechanical connections, and abraded areas and apply galvanizing repair paint, complying with SSPC-Paint 20, to comply with ASTM A780/A780M.

Retain "Factory Prime Finish" Paragraph below for field-painted, metallic-coated steel sheet fabrications.

        - Factory Prime Finish: After cleaning and pretreating, apply an air-dried primer compatible with the organic coating to be applied over it.

Retain "Baked-Enamel or Powder-Coat Finish" Paragraph below for factory-applied, baked-enamel or powder-coat finish for metallic-coated steel sheet fabrications.

        - Baked-Enamel or Powder-Coat Finish: After cleaning and pretreating, apply manufacturer's standard two-coat, baked-on finish consisting of prime coat and thermosetting topcoat to a minimum dry film thickness of 2 mils (0.05 mm).

- EXECUTION

      - EXAMINATION

        - Examine openings, substrates, structural support, anchorage, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Verify rough-opening dimensions, levelness of sill plate and clearances.

        - Examine wall flashings, vapor retarders, water and weather barriers, and other built-in components to ensure a coordinated, weathertight window installation.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION OF COLD-ROLLED STEEL WINDOWS

        - SWI Publication: Comply with applicable requirements in SWI's "Guidelines on How to Install Steel Windows," except where more stringent requirements are indicated.

        - Comply with manufacturer's written instructions for installing windows, hardware, operators, accessories, and other components.

Retain "Windborne-Debris-Impact Resistance" Paragraph below for installations requiring windborne-debris resistance. Proper anchorage of units is necessary to maintain performance as tested.

        - Windborne-Debris-Impact Resistance: Anchor cold-rolled steel windows required to have windborne-debris resistance to structure using method, anchor type, and anchor spacing identical to that used in windborne-debris-impact-resistance testing.

        - Install windows level, plumb, square, true to line, without distortion or impediment to thermal movement, anchored securely in place to structural support, and in proper relation to wall flashing and other adjacent construction.

        - Set sill members in bed of sealant or with gaskets, as indicated, to provide weathertight construction.

        - Install windows and components to drain condensation, water-penetrating joints, and moisture migrating within windows to the exterior.

        - Separate corrodible surfaces from sources of corrosion or electrolytic action at points of contact with other materials according to ASTM E2112.

      - FIELD QUALITY CONTROL

Retain this article if field tests are required. If retaining, indicate number of windows to be tested. Specify field tests for air leakage and water penetration for significant projects.

Retain first option in "Testing Agency" Paragraph below if Owner hires an independent testing agency.

        - Testing Agency: [Owner will engage] [Engage] a qualified testing agency to perform tests and inspections.

        - Tests and Inspections:

See Section 014000 "Quality Requirements" for retesting and reinspecting requirements and Section 017300 "Execution" for requirements for correcting the Work.

Revise first subparagraph below if more stringent field testing is required for Project. AAMA 502, Test Method A is default unless otherwise indicated.

          - Test windows for air leakage and water penetration in accordance with AAMA 502, Test Method [A] [B], by applying same test pressures required for performance.

          - Testing Extent: [Three] [Three mockup] <Insert number or description> windows as selected by Architect and a qualified independent testing and inspecting agency. Test windows immediately after installation.

          - Window will be considered defective if it does not pass tests and inspections.

        - Prepare test and inspection reports.

      - ADJUSTING, CLEANING, AND PROTECTION

        - Adjust operating sashes, screens, hardware, and accessories for a tight fit at contact points and weather stripping for smooth operation and weathertight closure. Lubricate hardware and moving parts as recommended in writing by manufacturer.

        - Clean factory-finished steel surfaces immediately after installing windows. Comply with manufacturer's written instructions for final cleaning and maintenance. Avoid damaging protective coatings and finishes.

        - Protect window surfaces from contact with contaminating substances resulting from construction operations. Remove contaminants immediately in accordance with manufacturer's written instructions.

Retain paragraph below if windows are factory finished.

        - Refinish or replace windows with damaged finish.

END OF SECTION 085123.23