Copyright 2022 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 087111 - DOOR HARDWARE (DESCRIPTIVE SPECIFICATION)

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Hinges.

          - Self-closing hinges and pivots.

          - Center-hung and offset pivots.

          - Continuous, pin-and-barrel-type hinges.

          - Continuous, gear-type hinges.

          - Concealed hinges.

          - Bored locks.

          - Mortise locks.

          - Interconnected locks.

          - Roller latches.

          - Push-pull latches.

          - Bored auxiliary locks.

          - Mortise auxiliary locks.

          - Narrow-stile auxiliary locks.

          - Push-button combination locks.

          - Electric strikes.

          - Electromagnetic locks.

          - Delayed-egress electromagnetic locks.

          - Electromechanical locks.

          - Self-contained electronic locks.

          - Exit locks and alarms.

          - Surface bolts.

          - Manual flush bolts.

          - Automatic flush bolts.

          - Self-latching flush bolts.

          - Exit devices and auxiliary items.

          - Lock cylinders.

          - Key control cabinet.

          - Key lock boxes.

          - Key control system software.

          - Operating trim.

          - Coordinators.

          - Carry-open bars.

          - Surface closers.

          - Concealed closers.

          - Closer holder release devices.

          - Wall- and floor-mounted stops.

          - Electromagnetic door holders.

          - Overhead stops and holders.

          - Door gasketing.

          - Thresholds.

          - Sliding door hardware.

          - Folding door hardware.

          - Metal protective trim units.

          - Plastic protection plates.

          - Auxiliary door hardware.

          - Auxiliary electrified door hardware.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - [Section 064113 "Wood-Veneer-Faced Architectural Cabinets"] [and] [Section 064116 "Plastic-Laminate-Clad Architectural Cabinets"] for cabinet door hardware provided with cabinets.

Coordinate products specified in Part 2 with Division 08 Sections in subparagraphs below. Astragals, silencers, and cylinders can be specified in this Section or with doors and frames.

          - Section 081113 "Hollow Metal Doors and Frames" [for astragals provided as part of labeled fire-rated assemblies] [and] [for door silencers provided as part of hollow-metal frames].

          - Section 081119 "Stainless-Steel Doors and Frames" [for astragals provided as part of labeled fire-rated assemblies] [and] [for door silencers provided as part of stainless steel frames].

          - Section 081173 "Sliding Metal Fire Doors" for door and track preparation, reinforcement, and motorized operators provided as part of automatic-closing assemblies.

          - Section 081213 "Hollow Metal Frames" [for astragals provided as part of labeled fire-rated assemblies] [and] [for door silencers provided as part of hollow-metal frames].

          - Section 081216 "Aluminum Frames" for door silencers provided as part of aluminum frames.

          - Section 081316.13 "Aluminum Terrace Doors" for entrance door hardware, [except] [including] cylinders.

          - Section 081416 "Flush Wood Doors" for [astragals] [and] [integral intumescent seals] provided as part of labeled fire-rated assemblies.

          - Section 081433 "Stile and Rail Wood Doors" for [astragals] [and] [integral intumescent seals] provided as part of labeled fire-rated assemblies.

          - Section 081713 "Integrated Metal Door Opening Assemblies" for door hardware provided as part of integrated metal door opening assemblies.

          - Section 083113 "Access Doors and Frames" for access door hardware, [except] [including] cylinders.

          - Section 083323 "Overhead Coiling Doors" for door hardware provided as part of overhead coiling door assemblies.

          - Section 083326 "Overhead Coiling Grilles" for door hardware provided as part of overhead coiling grille assemblies.

          - [Section 083473.13 "Metal Sound Control Door Assemblies"] [and] [Section 083473.16 "Wood Sound Control Door Assemblies"] for hinges and gasketing provided as part of sound-rated door assemblies.

          - Section 083513 "Folding Doors" for pulls, latches, hinges, guides, and pivots provided as part of the folding door package.

          - Section 084113 "Aluminum-Framed Entrances and Storefronts" for entrance door hardware, [except] [including] cylinders.

          - Section 084126 "All-Glass Entrances and Storefronts" for entrance door hardware, [except] [including] cylinders.

          - Section 084229.13 "Folding Automatic Entrances" for entrance door hardware, [except] [including] cylinders.

          - Section 084229.23 "Sliding Automatic Entrances" for entrance door hardware, [except] [including] cylinders.

          - Section 084229.33 "Swinging Automatic Entrances" for entrance door hardware, [except] [including] cylinders.

          - Section 084233 "Revolving Door Entrances" for revolving door entrance hardware, [except] [including] cylinders.

          - Section 084243 "Intensive Care Unit/Critical Care Unit (ICU/CCU) Entrances" for entrance door hardware, [except] [including] cylinders.

          - Section 087113 "Power Door Operators" for low-energy power operators and low-energy power-assist operators.

          - Section 102213 "Wire Mesh Partitions" for door hardware for doors in wire mesh partitions, [except] [including] cylinders.

          - Section 102600 "Wall and Door Protection" for plastic door protection units that match wall protection units.

          - Section 119812 "Detention Doors and Frames" for door silencers provided as part of detention frames.

          - Section 119814 "Detention Door Hardware" for hardware for detention doors.

          - Section 133419 "Metal Building Systems" for door hardware, [except] [including] cylinders.

          - Section 134900 "Radiation Protection" for lead-lined astragals provided as part of labeled fire-rated assemblies.

          - Section 281000 "Access Control" for coordination of access control system components.

          - Section 281400 "Access Control System Hardware" for access control system units, power, battery chargers, and computer equipment.

          - Section 283100 "Intrusion Detection" for detection devices installed at door openings and provided as part of an intrusion-detection system.

          - Section 284621.11 "Addressable Fire-Alarm Systems" for connections to building fire-alarm system.

          - Section 284621.13 "Conventional Fire-Alarm Systems" for connections to building fire-alarm system.

      - ALLOWANCES

Retain this article if products and Work included in this Section are covered by lump-sum, unit-cost, quantity, or testing and inspection allowances.

Retain products and Work included in this Section that are covered by cash or quantity allowance. Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the allowance.

        - Door hardware is part of [Door Hardware Allowance] <Insert allowance>.

      - COORDINATION

Retain "Floor-Recessed Door Hardware" Paragraph below if required.

        - Floor-Recessed Door Hardware: Coordinate layout and installation with floor construction.

          - Cast anchoring inserts into concrete.

Retain "Installation Templates" Paragraph below for factory-prepared doors and frames.

        - Installation Templates: Distribute for doors, frames, and other work specified to be factory prepared. Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing door hardware to comply with indicated requirements.

        - Security: Coordinate installation of door hardware, keying, and access control with Owner's security consultant.

        - Electrical System Roughing-In: Coordinate layout and installation of electrified door hardware with connections to power supplies and building safety and security systems.

        - Existing Openings: Where hardware components are scheduled for application to existing construction or where modifications to existing door hardware are required, field-verify existing conditions and coordinate installation of door hardware to suit opening conditions and to provide proper door operation.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

In subparagraph below, insert additional conference participants not mentioned in Section 013100 "Project Management and Coordination" if needed.

          - Conference participants must include Installer's Architectural Hardware Consultant[ and Owner's security consultant].

Retain "Keying Conference" Paragraph below if keying is extensive or complex.

        - Keying Conference: Conduct conference at [Project site] <Insert location>.

In first subparagraph below, insert additional conference participants not mentioned in Section 013100 "Project Management and Coordination" if needed.

          - Conference participants must include Installer's Architectural Hardware Consultant[ and Owner's security consultant].

          - Incorporate conference decisions into keying schedule after reviewing door hardware keying system, including, but not limited to, the following:

            - Flow of traffic and degree of security required.

            - Preliminary key system schematic diagram.

            - Requirements for key control system.

            - Requirements for access control.

            - Address for delivery of keys.

            - <Insert requirements to suit Project>.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Hinges.

          - Self-closing hinges and pivots.

          - Center-hung and offset pivots.

          - Continuous, pin-and-barrel-type hinges.

          - Continuous, gear-type hinges.

          - Concealed hinges.

          - Bored locks.

          - Mortise locks.

          - Interconnected locks.

          - Roller latches.

          - Push-pull latches.

          - Bored auxiliary locks.

          - Mortise auxiliary locks.

          - Narrow-stile auxiliary locks.

          - Push-button combination locks.

          - Electric strikes.

          - Electromagnetic locks.

          - Delayed-egress electromagnetic locks.

          - Electromechanical locks.

          - Self-contained electronic locks.

          - Exit locks and alarms.

          - Surface bolts.

          - Manual flush bolts.

          - Automatic flush bolts.

          - Self-latching flush bolts.

          - Exit devices and auxiliary items.

          - Lock cylinders.

          - Key control cabinet.

          - Key lock boxes.

          - Key control system software.

          - Operating trim.

          - Coordinators.

          - Carry-open bars.

          - Surface closers.

          - Concealed closers.

          - Closer holder release devices.

          - Wall- and floor-mounted stops.

          - Electromagnetic door holders.

          - Overhead stops and holders.

          - Door gasketing.

          - Thresholds.

          - Sliding door hardware.

          - Folding door hardware.

          - Metal protective trim units.

          - Plastic protection plates.

          - Auxiliary door hardware.

          - Auxiliary electrified door hardware.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes.

        - Product Data Submittals: For each product.

Retain "Shop Drawings" Paragraph below for electrified door hardware.

        - Shop Drawings: For electrified door hardware.

          - Include diagrams for power, signal, and control wiring.

          - Include details of interface of electrified door hardware and building safety and security systems.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples. Revise if Samples are only required for certain products.

        - Samples: For each exposed product in each finish specified, in manufacturer's standard size.

          - Tag Samples with full product description to coordinate Samples with door hardware schedule.

        - Samples for Initial Selection: For each type of exposed finish.

        - Samples for Verification: For each type of exposed product, in each finish specified.

          - Sample Size: Full-size units or minimum 2-by-4-inch (51-by-102-mm) Samples for sheet and 4-inch (102-mm) long Samples for other products.

Retain first subparagraph below unless Samples are delivered to Owner as extra materials.

            - Full-size Samples will be returned to Contractor. Units that are acceptable and remain undamaged through submittal, review, and field comparison process may, after final check of operation, be incorporated into the Work, within limitations of keying requirements.

          - Tag Samples with full product description to coordinate Samples with door hardware schedule.

        - Door Hardware Schedule: Prepared by or under the supervision of Installer's Architectural Hardware Consultant. Coordinate door hardware schedule with doors, frames, and related work to ensure proper size, thickness, hand, function, and finish of door hardware.

          - Submittal Sequence: Submit door hardware schedule [after] [or] [concurrent with] submissions of product data, Samples, and Shop Drawings. Coordinate submission of door hardware schedule with scheduling requirements of other work to facilitate the fabrication of other work that is critical in Project construction schedule.

          - Format: Use same scheduling sequence and format[ and use same door numbers] as in door hardware schedule in the Contract Documents.

          - Content: Include the following information:

            - Identification number, location, hand, fire rating, size, and material of each door and frame.

            - Locations of each door hardware set, cross-referenced to Drawings on floor plans and to door and frame schedule.

            - Complete designations, including name and manufacturer, type, style, function, size, quantity, function, and finish of each door hardware product.

Retain first subparagraph below for electrified door hardware.

            - Description of electrified door hardware sequences of operation and interfaces with other building control systems.

            - Fastenings and other installation information.

            - Explanation of abbreviations, symbols, and designations contained in door hardware schedule.

            - Mounting locations for door hardware.

            - List of related door devices specified in other Sections for each door and frame.

        - Keying Schedule: Prepared by or under the supervision of Installer's Architectural Hardware Consultant, detailing Owner's final keying instructions for locks. Include schematic keying diagram and index each key set to unique door designations that are coordinated with the Contract Documents.

      - INFORMATIONAL SUBMITTALS

Informational submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data: For [Installer] [and] [Architectural Hardware Consultant].

Retain "Product Certificates" Paragraph below to require submittal of product certificates from electrified door hardware manufacturers.

        - Product Certificates: For each type of electrified door hardware.

          - Certify that door hardware for use on each type and size of labeled fire-rated doors complies with listed fire-rated door assemblies.

Retain "Product Test Reports" Paragraph below if retaining "Accessibility Requirements" Paragraph in "Performance Requirements" Article.

        - Product Test Reports: For compliance with accessibility requirements, for tests performed by manufacturer and witnessed by a qualified testing agency, for door hardware on doors located in accessible routes.

Retain "Field quality-control reports" Paragraph below if Contractor is responsible for field quality-control inspecting.

        - Field quality-control reports.

        - Sample Warranty: For special warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For each type of door hardware to include in maintenance manuals.

        - Schedules: Final [door hardware] [and] [keying] schedule.

      - MAINTENANCE MATERIAL SUBMITTALS

See "Maintenance Materials" Article in the Evaluations for submission of maintenance material items.

        - Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Door Hardware: <Insert detailed descriptions and specific numbers of units>.

          - Electrical Parts: <Insert detailed descriptions and specific numbers of units>.

      - QUALITY ASSURANCE

        - Installer Qualifications: Supplier of products and an employer of workers trained and approved by product manufacturers and of an Architectural Hardware Consultant who is available during the course of the Work to consult Contractor, Architect, and Owner about door hardware and keying.

          - Warehousing Facilities: In Project's vicinity.

          - Scheduling Responsibility: Preparation of door hardware and keying schedule.

Retain "Engineering Responsibility" Subparagraph below for electrified door hardware.

          - Engineering Responsibility: Preparation of data for electrified door hardware, including Shop Drawings, based on testing and engineering analysis of manufacturer's standard units in assemblies similar to those indicated for this Project.

DHI Architectural Hardware Consultants (AHCs) are not trained in electrified hardware. Electrified Hardware Consultants (EHCs) and Architectural Opening Consultants (AOCs) are trained in electrified hardware; require one of these DHI certifications for projects that include electrified hardware. AOCs are also trained in the construction and application of doors. Door and Hardware Specification Consultants (DHSCs) are trained in specifying all types of door hardware. See the "Door Hardware Consultants" Article in the Evaluations.

        - Architectural Hardware Consultant Qualifications: A person who is experienced in providing consulting services for door hardware installations that are comparable in material, design, and extent to that indicated for this Project and who is currently certified by DHI as [a Door and Hardware Specification Consultant (DHSC)] [an Architectural Hardware Consultant (AHC)] [an Architectural Hardware Consultant (AHC) and an Electrified Hardware Consultant (EHC)] [an Architectural Opening Consultant (AOC)].

      - DELIVERY, STORAGE, AND HANDLING

        - Inventory door hardware on receipt and provide secure lockup for door hardware delivered to Project site.

        - Tag each item or package separately with identification coordinated with the final door hardware schedule, and include installation instructions, templates, and necessary fasteners with each item or package.

Retain one of two paragraphs below or revise to suit Project. Retain second paragraph for high-security cylinders.

        - Deliver keys to manufacturer of key control system for subsequent delivery to Owner.

        - Deliver keys[ and permanent cores] to Owner by registered mail or overnight package service.

      - WARRANTY

When warranties are required and available, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty: Manufacturer agrees to repair or replace components of door hardware that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures, including excessive deflection, cracking, or breakage.

            - Faulty operation of doors and door hardware.

            - Deterioration of metals, metal finishes, and other materials beyond normal weathering and use.

Verify available warranties and warranty periods for door hardware units and components.

          - Warranty Period: [Three] <Insert number> years from date of Substantial Completion unless otherwise indicated below:

Retain applicable subparagraphs below.

            - Electromagnetic and Delayed-Egress Locks: [Five] <Insert number> years from date of Substantial Completion.

            - Exit Devices: [Two] <Insert number> years from date of Substantial Completion.

            - Manual Closers: [10] <Insert number> years from date of Substantial Completion.

            - Concealed Floor Closers: [Five] [10] [25] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SOURCE LIMITATIONS

        - Obtain each type of door hardware from single manufacturer.

Retain subparagraph below if applicable.

          - Provide electrified door hardware from same manufacturer as mechanical door hardware unless otherwise indicated. Manufacturers that perform electrical modifications and that are listed by a testing and inspecting agency acceptable to authorities having jurisdiction are acceptable.

      - PERFORMANCE REQUIREMENTS

Retain "Fire-Rated Door Assemblies" Paragraph below if required. Revise to allow neutral pressure testing if required and as acceptable to authorities having jurisdiction. Revise if some door hardware items do not require listing and labeling.

        - Fire-Rated Door Assemblies: Where fire-rated doors are indicated, provide door hardware complying with NFPA 80 that is listed and labeled by a qualified testing agency, for fire-protection ratings indicated, based on testing at positive pressure in accordance with NFPA 252 or UL 10C.

Retain "Smoke- and Draft-Control Door Assemblies" Paragraph below if required. The International Building Code requires fire door assemblies to comply with smoke- and draft-control requirements in corridors, smoke barriers, and smoke partitions.

        - Smoke- and Draft-Control Door Assemblies: Where smoke- and draft-control door assemblies are required, provide door hardware that complies with requirements of assemblies tested in accordance with UL 1784 and installed in compliance with NFPA 105.

Revise "Air-Leakage Rate" Subparagraph below to suit Project.

          - Air-Leakage Rate: Maximum air leakage of [0.3 cfm per sq. ft. (3 cu. m per minute/sq. m)] <Insert rate> at the tested pressure differential of [0.3 inch wg (75 Pa)] <Insert value> of water.

        - Electrified Door Hardware: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

        - Means of Egress Doors: Latches do not require more than 15 lbf (67 N) to release the latch. Locks do not require use of a key, tool, or special knowledge for operation.

Verify accessibility requirements of authorities having jurisdiction. Some projects may require compliance with multiple accessibility regulations. See "Accessibility Guidelines, Codes, and Standards" Article in the Evaluations.

        - Accessibility Requirements: For door hardware on doors in an accessible route, comply with [the USDOJ's "2010 ADA Standards for Accessible Design"] [the DOT's "ADA Standards for Transportation Facilities"] [the ABA standards of the Federal agency having jurisdiction] [ICC A117.1] [HUD's "Fair Housing Accessibility Guidelines"] [and] <Insert regulation>.

          - Provide operating devices that do not require tight grasping, pinching, or twisting of the wrist and that operate with a force of not more than 5 lbf (22.2 N).

          - Comply with the following maximum opening-force requirements:

            - Interior, Non-Fire-Rated Hinged Doors: 5 lbf (22.2 N) applied perpendicular to door.

            - Sliding or Folding Doors: 5 lbf (22.2 N) applied parallel to door at latch.

            - Fire Doors: Minimum opening force allowable by authorities having jurisdiction.

          - Bevel raised thresholds with a slope of not more than 1:2. Provide thresholds not more than 1/2 inch (13 mm) high.

          - Adjust door closer sweep periods so that, from an open position of 90 degrees, the door will take at least 5 seconds to move to a position of 12 degrees from the latch.

          - Adjust spring hinges so that, from an open position of 70 degrees, the door will take at least 1.5 seconds to move to the closed position.

      - HINGES

Retain option in "Hinges" Paragraph below for hinges installed in hollow-metal doors and frames.

        - Hinges: ANSI/BHMA A156.1.[ Provide template-produced hinges for hinges installed on hollow-metal doors and hollow-metal frames.]

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Use antifriction-bearing hinges on fire-rated doors and doors with closers.

        - Antifriction-Bearing Hinges:

          - Mounting: [Full mortise (butts)] [Half mortise] [Full surface] [Half surface].

ANSI/BHMA A156.1 defines antifriction bearing as having a bearing material between various moving parts of a hinge. Manufacturers use various antifriction bearing materials, including ball bearings, oil-impregnated bearings, and nylon. NFPA 80 requires antifriction bearing surfaces that comply with ANSI/BHMA A156.1 for hinges used in fire-rated door assemblies.

          - Bearing Material: [Manufacturer's standard antifriction bearing] [Ball bearing] <Insert material>.

          - [Grade 1 (heavy weight)] [Grade 2 (standard weight)].

          - Base and Pin Metal:

            - Exterior Hinges: [Stainless steel with stainless steel pin] [Brass with stainless steel pin body and brass protruding heads].

            - Interior Hinges: [Brass with stainless steel pin body and brass protruding heads] [Steel with steel pin] [Stainless steel with stainless steel pin].

            - Hinges for Fire-Rated Assemblies: [Steel with steel pin] [Stainless steel with stainless steel pin].

          - Pins: [Nonrising loose unless otherwise indicated] [Maximum security] [Nonremovable].

Retain "Outswinging Exterior Doors" and "Outswinging Corridor Doors with Locks" subparagraphs below if nonrising loose pins are required in "Pins" Subparagraph above if applicable.

            - Outswinging Exterior Doors: [Maximum security] [Nonremovable].

            - Outswinging Corridor Doors with Locks: [Maximum security] [Nonremovable].

Flat button tips are most commonly used. Hospital tips are less frequently used, and other types of tips are seldom used. See ANSI/BHMA 156.1 for illustrations of each type.

          - Tips: [Flat button] [Hospital] [Oval] [Ball] [Steeple] [Urn] [Acorn].

Square-corner hinges are standard for commercial applications. Radius corners are typically used on prehung doors for residential applications.

          - Corners: [Square] [5/32-inch (4-mm) radius] [1/4-inch (6-mm) radius] [5/8-inch (16-mm) radius].

Safety studs and reverse safety studs are only available on full-mortise hinges.

          - Features: [Raised barrel] [Reverse safety stud] [Safety stud] <Insert special swaging requirements or other features>.

Retain "Electrified Antifriction-Bearing Hinges" Paragraph below if electric function is required for full-mortise antifriction-bearing hinges. Electric functions are generally not available for half-mortise or full- or half-surface hinges.

        - Electrified Antifriction-Bearing Hinges: Full-mortise mounting.

ANSI/BHMA A156.1 defines antifriction bearing as having a bearing material between various moving parts of a hinge. Manufacturers use various antifriction bearing materials, including ball bearings, oil-impregnated bearings, and nylon. NFPA 80 requires antifriction bearing surfaces that comply with ANSI/BHMA A156.1 for hinges used in fire-rated door assemblies.

          - Bearing Material: [Manufacturer's standard antifriction bearing] [Ball bearing] <Insert material>.

          - [Grade 1 (heavy weight)] [Grade 2 (standard weight)].

          - Base and Pin Metal:

            - Exterior Hinges: [Stainless steel with stainless steel pin] [Brass with stainless steel pin body and brass protruding heads].

            - Interior Hinges: [Brass with stainless steel pin body and brass protruding heads] [Steel with steel pin] [Stainless steel with stainless steel pin].

            - Hinges for Fire-Rated Assemblies: [Steel with steel pin] [Stainless steel with stainless steel pin].

          - Pins: [Nonrising loose unless otherwise indicated] [Maximum security] [Nonremovable].

Retain "Outswinging Exterior Doors" and "Outswinging Corridor Doors with Locks" subparagraphs below if nonrising loose pins are required in "Pins" Subparagraph above if applicable.

            - Outswinging Exterior Doors: [Maximum security] [Nonremovable].

            - Outswinging Corridor Doors with Locks: [Maximum security] [Nonremovable].

Flat button tips are most commonly used. Hospital tips are less frequently used, and other types of tips are seldom used. See ANSI/BHMA 156.1 for illustrations of each type.

          - Tips: [Flat button] [Hospital] [Oval] [Ball] [Steeple] [Urn] [Acorn].

Square-corner hinges are standard for commercial applications. Radius corners are typically used on prehung doors for residential applications.

          - Corners: Square.

          - Features: [Raised barrel] [Reverse safety stud] [Safety stud] <Insert special swaging requirements or other features>.

          - Electric Feature: [Concealed electric through wires] [Concealed electric through wires with monitor] [Concealed electric monitor] [Concealed air transfer] [Concealed switch] [Exposed electric switch] [Exposed electric contacts] <Insert feature>.

        - Plain-Bearing Hinges: Grade 3 (standard weight).

          - Mounting: [Full mortise (butts)] [Half mortise] [Full surface] [Half surface].

          - Base and Pin Metal: [Brass with stainless steel pin body and brass protruding heads] [Steel with steel pin].

          - Pins: [Nonrising loose unless otherwise indicated] [Maximum security] [Nonremovable].

Retain "Outswinging Corridor Doors with Locks" Subparagraph below if nonrising loose pins are required in "Pins" Subparagraph above if applicable.

            - Outswinging Corridor Doors with Locks: [Maximum security] [Nonremovable].

Flat button tips are most commonly used. Hospital tips are less frequently used, and other types of tips are seldom used. See ANSI/BHMA 156.1 for illustrations of each type.

          - Tips: [Flat button] [Hospital] [Oval] [Ball] [Steeple] [Urn] [Acorn].

Square-corner hinges are standard for commercial applications. Radius corners are typically used on prehung doors for residential applications.

          - Corners: [Square] [5/32-inch (4-mm) radius] [1/4-inch (6-mm) radius] [5/8-inch (16-mm) radius].

          - Features: [Raised barrel] <Insert special swaging requirements or other features>.

        - Swing-Clear Hinges: Reversible.

          - Mounting: [Full mortise (butts)] [Half mortise] [Full surface] [Half surface].

Only full-mortise, swing-clear hinges are available in Grade 2 and Grade 3.

          - Bearing and Grade: [Antifriction bearing, Grade 1 (heavy weight)] [Antifriction bearing, Grade 2 (standard weight)] [Plain bearing, Grade 3 (standard weight)].

          - Base Metal: [Wrought brass or bronze] [Stainless steel] [Wrought, forged, or cast steel, or malleable iron].

          - Pins: [Nonrising loose unless otherwise indicated] [Maximum security] [Nonremovable].

Retain "Outswinging Exterior Doors" and "Outswinging Corridor Doors with Locks" subparagraphs below if nonrising loose pins are required in "Pins" Subparagraph above if applicable.

            - Outswinging Exterior Doors: [Maximum security] [Nonremovable].

            - Outswinging Corridor Doors with Locks: [Maximum security] [Nonremovable].

          - Tips: [Flat button] [Hospital].

Square-corner hinges are standard for commercial applications. Radius corners are typically used on prehung doors for residential applications.

          - Corners: [Square] [5/32-inch (4-mm) radius] [1/4-inch (6-mm) radius] [5/8-inch (16-mm) radius].

Safety studs and reverse safety studs are only available on full-mortise hinges.

          - Features: [Raised barrel] [Reverse safety stud] [Safety stud] <Insert special swaging requirements or other features>.

Retain "Slip-in-Type Hinges" Paragraph below if applicable. Slip-in-type hinges are used with aluminum doors and frames where hinge leaves are inserted in slot in door or frame.

        - Slip-in-Type Hinges: Full-mortise mounting.

          - Bearing and Grade: [Antifriction, Grade 1 (heavy weight)] [Antifriction, Grade 2 (standard weight)] [Plain, Grade 3 (standard weight)].

          - Base Metal: [Wrought brass or bronze] [Stainless steel] [Wrought, forged, or cast steel, or malleable iron].

          - Swaging: [5/16-inch (7.9-mm) swaging] [3/16-inch (4.8-mm) swaging, handed].

        - Anchor Hinge Set: Grade 1 (heavy weight); consisting of one anchor hinge plus two full-mortise hinges; antifriction bearing; handed; nonremovable pins; flat-button tips.

          - Base Metal: [Wrought brass or bronze] [Stainless steel] [Wrought, forged, or cast steel, or malleable iron].

Retain "Electric Feature for Center Hinge" Subparagraph below if electric function is required.

          - Electric Feature for Center Hinge: [Concealed electric through wires] [Concealed electric switch] <Insert feature>.

      - SELF-CLOSING HINGES AND PIVOTS

Spring hinges for labeled fire-rated assemblies must be single acting and Grade 1 and are available in full-mortise or half-surface mounting.

        - Self-Closing Hinges and Pivots: ANSI/BHMA A156.17.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Spring Hinges: [Grade 1] [Grade 2]; wrought steel, with torsion spring.

          - Type: [Single] [Double] acting.

          - Mounting: [Full mortise (butts)] [Half mortise] [Full surface] [Half surface].

        - Horizontal-Spring Pivot Sets: Grade 3; double acting; nonhanded; consisting of wrought-steel bottom pivot hinge with antifriction bearing and of nylon top pivot and socket.

          - Type: [Hold-open] [Non-hold-open].

          - Tension: [Adjustable] [Fixed].

          - Bottom Pivot Trim: [Steel] [Brass].

          - Bottom Plate: For bottom hinge attachment to [floor] [jamb].

        - Gate-Spring Pivot Sets: Grade 1; double acting; nonhanded; consisting of bottom pivot with door and jamb bracket and of top pivot assembly with jamb bracket.

          - Mounting: [Mortise] [Surface].

          - Tension: [Adjustable] [Fixed].

          - Base Metal: [Cast, forged, or extruded brass or bronze] [Malleable iron].

        - Gravity Pivot Sets: Grade 3; double acting; surface mounting; nonhanded; consisting of bottom pivot with door and jamb bracket and of top pivot assembly with jamb bracket.

          - Tension: [Adjustable] [Fixed].

          - Base Metal: [Wrought brass or bronze] [Steel].

      - CENTER-HUNG AND OFFSET PIVOTS

ANSI/BHMA A156.4 includes requirements for types of pivots specified in this article.

        - Center-Hung and Offset Pivots: ANSI/BHMA A156.4.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

NFPA 80 does not allow use of center-hung pivot sets on labeled fire-rated doors.

        - Center-Hung Pivot Sets: Grade 1.

          - Top Pivots: Walking-beam type with retractable pin and oil-impregnated bronze bearing; mortised into door and frame.

          - Bottom Pivots: [Surface floor mounted,] [Recessed in floor in cement case,] [Mortised into jamb,] and mortised into door.

            - Bearing: [Thrust ball] [Needle].

          - Base Metal: [Brass] [Bronze] [Steel].

        - Offset Pivot Sets: [Grade 1] [Grade 2].

          - Offset: [3/4 inch (19 mm)] [1-1/2 inches (38 mm)].

          - Top Pivot: [Full-mortise] [Half-surface] [Full-surface] mounting; walking-beam type with retractable pin and oil-impregnated bronze bearing.

            - Knuckle: [Standard] [Asylum type].

Retain option in "Feature" Subparagraph below for lead-lined doors.

            - Feature: [With screw holes designed to straddle lead in center of lead-lined door] <Insert feature>.

Recessed-type bottom pivot is unavailable in Grade 2.

          - Bottom Pivot: [Surface floor mounted,] [Recessed in floor in cement case,] [Mortised into jamb,] and mortised into door; with [thrust ball] [needle] bearing.

Model codes require steel pivots for labeled fire-rated doors rated 1-1/2 hours or more.

          - Base Metal: [Brass] [Bronze] [Stainless steel] [Steel].

Offset intermediate pivots are typically required for offset pivots and are not used with center pivot sets. For labeled fire-rated door assemblies, NFPA 80 requires intermediate pivots for every 30 inches (760 mm) of door height greater than 60 inches (1520 mm).

Coordinate manufacturer of offset intermediate pivots with manufacturer of offset-hung concealed floor closers if any.

        - Offset Intermediate Pivots: Grade 1; for use with offset pivot sets; with oil-impregnated bronze bearings.

Half-mortise, half-surface, and full-surface hinges are available only with 3/4-inch (19-mm) offset.

          - Mounting: [Full mortise, 3/4-inch (19-mm) offset] [Full mortise, 1-1/2-inch (38-mm) offset] [Half mortise] [Half surface] [Full surface].

Asylum-type knuckle is available only for full-mortise pivots.

          - Knuckle: [Standard] [Asylum type].

Retain option in "Feature" Subparagraph below for lead-lined doors.

          - Feature: [With screw holes designed to straddle lead in center of lead-lined door] <Insert feature>.

Retain "Electric Feature" Subparagraph below for electrified function.

          - Electric Feature: [Concealed monitoring] [Concealed power transfer] [Concealed power transfer for use with electrical panic devices and locks] <Insert feature>.

Model codes require steel pivots for labeled fire-rated doors rated 1-1/2 hours or more.

          - Base Metal: [Brass] [Bronze] [Stainless steel] [Steel].

        - Pocket Pivots: Grade 1; full-mortise mounting; nonhanded; allows door to nest in pocket with door surface flush with corridor wall when open; maximum 90-degree swing.

          - Base Metal: [Bronze] [Stainless steel] [Steel].

Retain "Electric Feature" Subparagraph below for electrified function.

          - Electric Feature: [Concealed power transfer in one hinge per door] <Insert feature>.

      - CONTINUOUS HINGES

        - Continuous, Pin-and-Barrel-Type Hinges: ANSI/BHMA A156.26; minimum 0.120-inch- (3.0-mm-) thick, hinge leaves with minimum overall width of 4 inches (102 mm); fabricated to full height of door and frame and to template screw locations; with components finished after milling and drilling are complete.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - [Grade 1-150] [Grade 1-300] [Grade 1-600] [Grade 2-150] [Grade 2-300] [Grade 2-600] [Grade 3-150] [Grade 3-300] <Insert grade>.

Retain "Exterior Hinges," "Interior Hinges," and "Hinges for Fire-Rated Assemblies" subparagraphs below if applicable.

          - Exterior Hinges: Stainless steel.

          - Interior Hinges: [Stainless steel] [Steel] [Aluminum].

          - Hinges for Fire-Rated Assemblies: [Stainless steel with steel fire pins to hold fire-rated doors in place if required by tested listing] [Steel].

          - Type: [Concealed leaf] [Swing clear] [Full surface with removable continuous caps over fasteners] [Half mortise, concealed door leaf and with removable continuous cap over fasteners on jamb leaf] [Half surface, concealed jamb leaf and with removable continuous cap over fasteners on door leaf].

Retain "Electric Feature" Subparagraph below if electrical function is required.

          - Electric Feature: [Electric monitoring switch] [Electric through wires and monitor] [Electric through wires] [Concealed power transfer] [Exposed power transfer contact switch] <Insert feature>.

        - Continuous, Gear-Type Hinges: ANSI BHMA A156.26; minimum 0.120-inch- (3.0-mm-) thick, extruded-aluminum, pinless, geared hinge leaves joined by a continuous extruded-aluminum channel cap; with concealed, self-lubricating thrust bearings. Minimum overall width of 4 inches (102 mm); fabricated to full height of door and frame and to template screw locations; with components finished after milling and drilling are complete.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - [Grade 1-150] [Grade 1-300] [Grade 1-600] [Grade 2-150] [Grade 2-300] [Grade 2-600] [Grade 3-150] [Grade 3-300] <Insert grade>.

Retain "Hinges for Fire-Rated Assemblies" Subparagraph below if applicable.

          - Hinges for Fire-Rated Assemblies: With steel fire pins to hold fire-rated doors in place if required by tested listing.

          - Mounting: [Concealed leaf] [Swing clear] [Full surface, with removable continuous caps over fasteners] [Half surface, concealed jamb leaf and with removable continuous cap over fasteners on door leaf].

Retain "Electric Feature" Subparagraph below if electrical function is required.

          - Electric Feature: [Electric monitor] [Electric through wires and monitor] [Electric through wires] [Electric power transfer] [Exposed switch] [Exposed contact] [Removable electric through wires] <Insert feature>.

      - CONCEALED HINGES

        - Concealed Hinges: Fully concealed within mortises in the door edge and frame and allowing door to swing open 180 degrees.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Electric Feature: [Electric through wires] [Electric power transfer] <Insert feature>.

      - MECHANICAL LOCKS AND LATCHES

        - Lock Functions: As indicated in door hardware schedule.

        - Lock Throw: Comply with testing requirements for length of bolts required for labeled fire doors, and as follows:

          - Bored Locks: Minimum 1/2-inch (13-mm) latchbolt throw.

          - Mortise Locks: Minimum 3/4-inch (19-mm) latchbolt throw.

          - Deadbolts: Minimum [1-inch (25-mm)] [1.25-inch (32-mm)] <Insert dimension> bolt throw.

        - Lock Backset: 2-3/4 inches (70 mm) unless otherwise indicated.

Revise "Lock Trim" Paragraph below if more than one type of lock trim is required.

        - Lock Trim:

          - Description: [As indicated on Drawings] <Insert description or manufacturer's design designation>.

Levers are most often cast.

          - Levers: [Wrought] [Forged] [Cast].

            - <Insert model number and description>.

            - Construction: [Solid] [Tube] [Freewheeling vandal resistant] [Breakaway vandal resistant].

          - Escutcheons (Roses): [Wrought] [Forged] [Cast].

          - Dummy Trim: Match lever lock trim and escutcheons.

        - Strikes: Provide manufacturer's standard strike for each lock bolt or latchbolt complying with requirements indicated for applicable lock or latch and with strike box and curved lip extended to protect frame; finished to match lock or latch.

Retain "Flat-Lip Strikes," "Extra-Long-Lip Strikes," "Aluminum-Frame Strike Box," and "Rabbet Front and Strike" subparagraphs below if applicable.

          - Flat-Lip Strikes: For locks with three-piece antifriction latchbolts, as recommended by manufacturer.

          - Extra-Long-Lip Strikes: For locks used on frames with applied wood casing trim.

          - Aluminum-Frame Strike Box: Manufacturer's special strike box fabricated for aluminum framing.

          - Rabbet Front and Strike: Provide on locksets for rabbeted meeting stiles.

Series 4000 locks are installed in round, bored openings in edge and face of door.

        - Bored Locks: ANSI/BHMA A156.2, [Grade 1] [Grade 2], Series 4000.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Mortise Locks: ANSI/BHMA A156.13, [Operational Grade 1] [Security Grade 1] [Operational Grade 2] [Security Grade 2]; stamped steel case with steel or brass parts; Series 1000.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Interconnected Locks: ANSI/BHMA A156.12, [Grade 1] [Grade 2], Series 5000.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Roller Latches: ANSI/BHMA A156.16, Grade 1; rolling plunger that engages socket or catch, with adjustable roller projection.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Material: [Brass] [Bronze].

          - Mounting: [Surface] [Mortise].

        - Push-Pull Latches: [Bored, ANSI/BHMA A156.2, Series 4000] [Mortise, ANSI/BHMA A156.13]; with paddle handles that retract latchbolt; capable of being mounted vertically or horizontally.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - [Grade 1] [Grade 2].

          - Lever and Escutcheon Material: [Brass] [Bronze] [Stainless steel] [Aluminum].

Retain "Lettering" Subparagraph below if engraved paddle handles are required.

          - Lettering: Engrave with the words "Pull" and "Push."

Retain "Lead Lining" Subparagraph below if lead lining is required.

          - Lead Lining: [0.047 inch (1.2 mm)] <Insert dimension> thick for escutcheon plate.

      - AUXILIARY LOCKS

Locks that are typically used in addition to latchbolts that are operated by a knob, lever, or thumbpiece are called "auxiliary locks."

        - Bored Auxiliary Locks: ANSI/BHMA A156.36, [Grade 1] [Grade 2]; with strike that suits frame.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Backset: [2-3/8 inches (60 mm)] [2-3/4 inches (70 mm)].

          - Material: [Aluminum] [Brass] [Bronze] [Stainless steel] [Zinc alloy].

          - Deadlatches: Deadlocking latchbolt operated by [key either side] [key outside and turn inside] [turn inside with no cylinder].

          - Deadlocks: Deadbolt operated by [key either side] [key outside and turn inside] [turn inside with no cylinder] [key outside, no trim inside].

        - Mortise Auxiliary Locks: ANSI/BHMA A156.36, [Grade 1] [Grade 2]; with strike that suits frame.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Backset: [2-3/4 inches (70 mm)] <Insert dimension>.

          - Material: [Aluminum] [Brass] [Bronze] [Stainless steel] [Zinc alloy].

          - Deadlocks: Deadbolt operated by key [either side] [outside and turn inside] [one side].

          - Deadlatches: Latchbolt and auxiliary deadlatch operated by key [either side] [outside and turn inside].

          - Deadlocks for Sliding Doors: Expanding- or interlocking-type deadbolt operated by key [either side] [outside and turn inside] [one side].

          - Deadlatches for Sliding Doors: Hook-type latchbolt operated by key [either side] [outside and handle inside].

        - Narrow-Stile Auxiliary Locks: ANSI/BHMA A156.36, [Grade 1] [Grade 2]; with strike that suits frame.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Backset: [0.98 inch (25 mm)] [1.125 inches (29 mm)] [1.25 inches (32 mm)] [1.5 inches (38 mm)] [1.75 inches (44 mm)] [2 inches (51 mm)] [2.25 inches (57 mm)] [2.5 inches (64 mm)] <Insert dimension>.

          - Strike: [Flat] [Flat with extra-long lip] [Radius] [Radius with weatherstrip] [Bevel].

          - Case Material: [Steel] [Stainless steel].

          - Armored Front and Strike Material: [Aluminum] [Brass] [Bronze] [Stainless steel].

          - Deadlock: Deadlocking bolt.

            - Operation: Key [both sides] [outside and operating trim inside].

            - Door Application: [Swinging] [Sliding] door.

          - Deadlatch: Latchbolt with auxiliary deadlatch operated by key outside and paddle or lever inside; for single swinging doors.

          - Multipoint Lock: Deadlocking bolt for pairs of swinging doors.

            - Operation: Key [both sides] [outside and turn, lever, or knob inside].

            - Type: [Two] [Three] point.

          - Latch/Lock: Deadbolt and latchbolt; both operated by key both sides; inside handle operates only latchbolt.

        - Push-Button Combination Locks: ANSI/BHMA A156.36, cylindrical, Grade 1; lock opens by entering a one- to five-digit code by pushing correct buttons in correct sequence; automatically relocks when door is closed; with strike that suits frame.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Auxiliary Lock Configuration: [Deadbolt] [Deadlocking latch] [Deadlocking rim latch].

Retain "Override" Subparagraph below if key override is required.

          - Override: By key cylinder.

      - ELECTRIC STRIKES

        - Electric Strikes: ANSI/BHMA A156.31, [Grade 1] [Grade 2]; with faceplate to suit lock and frame.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Material: [Steel] [Stainless steel] [Zinc-aluminum alloy].

          - Mounting: [Mortised] [Semirim mounted] [Rim mounted].

Retain "Fire-Rated Door Assemblies" Subparagraph below if applicable.

          - Fire-Rated Door Assemblies: Use fail-secure electric strikes with fire-rated devices.

Retain "Monitoring" and "Features" subparagraphs below if required.

          - Monitoring: [Mechanical latchbolt] [Infrared latchbolt] [Mechanical strike] [Infrared strike].

          - Features: [Lip extension kit] [Open-back strike] <Insert feature>.

      - ELECTROMAGNETIC LOCKS

        - Electromagnetic Locks: ANSI/BHMA A156.23; electrically powered; with electromagnet attached to frame and armature plate attached to door; full-exterior or full-interior type, as required by application indicated.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Direct-Hold Type" or "Shear Type" Subparagraph below.

          - Direct-Hold Type: Lock mounted on [bottom of header; strike flush mounted on door push side] [face of header; strike angle mounted on door pull side] [side of jamb; strike flush mounted on door push side].

          - Shear Type: Lock [mounted on face of header; strike angle mounted on door] [mortised in header; strike mortised in top of door] [mortised in jamb; strike mortised in edge of door] [mortised in bottom of door; strike mortised in floor] [mortised in floor; strike mortised in bottom of door].

          - Strength Ranking: [1500 lbf (6672 N)] [1000 lbf (4448 N)] [500 lbf (2224 N)].

Retain one of two options in "Inductive Kickback Peak Voltage" and "Residual Magnetism" subparagraphs below. First value in each subparagraph is from BHMA standard. Second value is available from domestic manufacturers as standard.

          - Inductive Kickback Peak Voltage: Not more than [53] [0] V.

          - Residual Magnetism: Not more than [4 lbf (18 N)] [0 lbf (0 N)] to separate door from magnet.

          - Features:

Retain applicable subparagraphs below.

            - Magnetic bond sensor.

            - Continuous housing for full width of door.

            - Continuous housing for full height of door.

            - Single LED indicators.

            - Double LED indicators.

            - Adjustable time delay with automatic relock.

            - Integral door position switch.

            - <Insert feature>.

        - Delayed-Egress Electromagnetic Locks: ANSI/BHMA A156.24; electrically powered; with electromagnet attached to frame and armature plate attached to door; depressing push bar for more than three seconds initiates irreversible alarm and adjustable time delay for egress. When integrated with fire alarm, fire alarm voids time delay.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Grade: [Security grade, activated from secure side of door by initiating device] [Movement grade, activated by door movement as initiating device].

      - ELECTROMECHANICAL LOCKS

        - Electromechanical Locks: ANSI/BHMA A156.25, [Grade 1] [Grade 2]; motor or solenoid driven; with strike that suits frame.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Type: [Bored] [Mortise latchbolt] [Mortise deadbolt] [Mortise deadlocking latchbolt].

      - SELF-CONTAINED ELECTRONIC LOCKS

        - Self-Contained Electronic Locks: ANSI/BHMA A156.25, [bored] [mortise]; with internal, battery-powered, self-contained electronic locks; consisting of complete lockset, motor-driven lock mechanism, and actuating device; enclosed in zinc-dichromate-plated, wrought-steel case, and strike that suits frame. Provide key override, low-battery detection and warning, LED status indicators, and ability to program at the lock.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Actuating Device: [Digital keypad] [Magnetic-stripe card reader] [Smart-card reader] <Insert device>.

Retain "Card" and "Accessory" subparagraphs below if retaining either card-reader option in "Actuating Device" Subparagraph above.

            - Card: [Manufacturer's standard] [0.030-inch- (0.76-mm-) thick PVC or polyester] [Custom].

            - Accessory: Card encoder and software.

          - Faceplate Material: [Wrought brass] [Wrought bronze] [Stainless steel].

          - Lock Trim: [Lever] [Knob] [Match trim specified for mechanical locks].

          - Function: [Latch with] [Deadbolt with] [Latch without] [Deadbolt without] key.

      - EXIT LOCKS AND EXIT ALARMS

        - Exit Locks and Alarms: ANSI/BHMA A156.29, Grade 1.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Exit Locks: Surface mounted; battery powered, housed in metal case; with manufacturer's standard strike that suits frame; with red-and-white lettering reading "EMERGENCY EXIT PUSH TO OPEN - ALARM WILL SOUND."

          - Activated by [arm, push plate, or paddle] [horizontal bar].

          - Features:

Retain applicable subparagraphs below.

            - Low-battery alert.

            - Outside key control.

            - Audible alarm that sounds when unauthorized use of door occurs.

            - Silent alarm with remote signal capability for connection to remote indicating panel.

            - Strike: [Surface] [Mortise].

            - <Insert feature>.

        - Standalone Exit Alarms: [Surface mounted on door] [Mounted separate from door and activated by door movement switch].

          - Features:

Retain applicable subparagraphs below.

            - Low-battery alert.

            - Outside key control.

            - Audible alarm that sounds when unauthorized use of door occurs.

            - Automatic rearming after authorized use[, with adjustable time delay].

            - Remote signal capability for connection to remote indicating panel.

            - <Insert feature>.

      - SURFACE BOLTS

        - Surface Bolts: ANSI/BHMA A156.16.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Half-Round Surface Bolts: [Grade 1] [Grade 2], 6-inch (152-mm) polished-brass or burnished-steel, half-round rod and knob; minimum 7/8-inch (22-mm) throw; with universal strike.

        - Interlocking Surface Bolts: [Grade 1] [Grade 2], 6-inch (152-mm) extruded-brass or aluminum, interlocking track and rod; minimum 15/16-inch (24-mm) throw; with universal or mortise strike.

        - Fire-Rated Surface Bolts: [Grade 1] [Grade 2], 8-inch (203-mm) steel bolt with two steel guides; minimum 1-inch (25-mm) throw; listed and labeled for use in fire-rated assemblies; with universal strike.

Revise "Dutch-Door Surface Bolts" Paragraph below to require a mortise strike for door application with a shelf.

        - Dutch-Door Surface Bolts: [Grade 1] [Grade 2], polished-brass bolt and knob, minimum 3/4-inch (19-mm) throw, with standard strike.

      - MANUAL FLUSH BOLTS

Manual flush bolts are for inactive leaf of a pair of doors. They are available for labeled fire-rated doors but do not meet model code requirements for doors used as a means of egress.

        - Manual Flush Bolts: ANSI/BHMA A156.16; minimum 3/4-inch (19-mm) throw; designed for mortising into door edge.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Manual-Extension Flush Bolts: [Grade 1] [Grade 2], fabricated from extruded brass or aluminum, with 12-inch (305-mm) rod actuated by flat lever.

          - Strike: [Matching] [Dustproof].

          - Fire Rated: Listed and labeled for use in fire-rated assemblies.

        - Slide Flush Bolts: [Grade 1] [Grade 2], cast brass, with rod actuated by slide.

          - Strike: [Matching] [Dustproof].

        - Tubular Bolts: [Grade 1] [Grade 2], polished-brass or polished-bronze, oval turn knob and escutcheon; minimum 9/16-inch (14-mm) steel bolt with 1/2-inch (13-mm) throw.

          - Strike: [Matching] [Dustproof].

        - Top-Bolt Extension Rod Length: As required to locate operating mechanism at not more than 72 inches (1829 mm) above the finished floor at all times.

Retain "Dustproof Strikes" Paragraph below if required.

        - Dustproof Strikes: [Locking type, ]Grade 1, polished wrought brass, with 3/4-inch- (19-mm-) diameter, spring-tension plunger.

      - AUTOMATIC AND SELF-LATCHING FLUSH BOLTS

Automatic and self-latching flush bolts are for inactive leaf of a pair of doors. Self-latching bolts are also called "constant-latching bolts."

        - Automatic Flush Bolts: ANSI/BHMA A156.3, Type 25; minimum 3/4-inch (19-mm) throw; designed for mortising into door edge.[ Include wear plates.]

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Self-Latching Flush Bolts: ANSI/BHMA A156.3, Type 27; minimum 3/4-inch (19-mm) throw; with dust-proof strikes; designed for mortising into door edge.[ Include wear plates.]

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Dustproof Strikes" Paragraph below if required.

        - Dustproof Strikes: [Locking type,] Grade 1, polished wrought brass, with 3/4-inch- (19-mm-) diameter, spring-tension plunger.

      - EXIT DEVICES AND AUXILIARY ITEMS

        - Exit Devices and Auxiliary Items: ANSI/BHMA A156.3.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Only fire exit devices may be used for fire-rated assemblies. NFPA 80 distinguishes between panic exit hardware and fire exit hardware. See the Evaluations.

        - Panic Exit Devices: Listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction, for panic protection, based on testing in accordance with UL 305.

UL test in "Fire Exit Devices" Paragraph below includes operational test of 100,000 cycles. ANSI/BHMA A156.3 requires 500,000 cycles for Grade 1 and 250,000 cycles for Grade 2.

        - Fire Exit Devices: Devices complying with NFPA 80 that are listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction, for fire and panic protection, based on testing in accordance with UL 305 and NFPA 252.

        - Rim Exit Devices: [Grade 1] [Grade 2].

          - [Type 1, rim] [Type 4, narrow stile] [Type 28, incorporating a deadbolt].

          - Actuating Bar: [Cross bar] [Push pad] [Narrow-stile push pad].

          - Material: [Brass] [Bronze] [Stainless steel] [Aluminum] [Wrought steel].

          - Electrified Features:

Retain applicable subparagraphs below. Verify availability of products with manufacturers.

            - Push pad monitor switch.

            - Double push pad monitor switch.

            - Electric locking and unlocking.

            - Fail-secure electric latch retraction (dogging) that engages latch when fire-alarm system is activated.

            - Delayed egress.

            - Alarm.

            - <Insert feature>.

        - Mortise Exit Devices: [Grade 1] [Grade 2].

          - [Type 3] [Type 10, narrow stile].

          - Actuating Bar: [Cross bar] [Push pad] [Narrow-stile push pad].

          - Material: [Brass] [Bronze] [Stainless steel] [Aluminum] [Wrought steel].

          - Electrified Features:

Retain applicable subparagraphs below. Verify availability of products with manufacturers.

            - Push pad monitor switch.

            - Double push pad monitor switch.

            - Electric locking and unlocking.

            - Fail-secure electric latch retraction (dogging) that engages latch when fire-alarm system is activated.

            - Delayed egress.

            - Alarm.

            - <Insert feature>.

        - Surface Vertical-Rod Exit Devices: [Grade 1] [Grade 2].

          - [Type 2] [Type 5, narrow stile].

          - Actuating Bar: [Cross bar] [Push pad] [Narrow-stile push pad].

          - Material: [Brass] [Bronze] [Stainless steel] [Aluminum] [Wrought steel].

          - Configuration: [Top and bottom rods] [Top rod].

          - Electrified Features:

Retain applicable subparagraphs below. Verify availability of products with manufacturers.

            - Push pad monitor switch.

            - Double push pad monitor switch.

            - Electric locking and unlocking.

            - Fail-secure electric latch retraction (dogging) that engages latch when fire-alarm system is activated.

            - Delayed egress.

            - Alarm.

            - <Insert feature>.

        - Concealed Vertical-Rod Exit Devices: [Grade 1] [Grade 2].

          - [Type 6, narrow stile] [Type 7, for wood doors] [Type 8, for metal doors].

          - Actuating Bar: [Cross bar] [Push pad] [Narrow-stile push pad].

          - Material: [Brass] [Bronze] [Stainless steel] [Aluminum] [Wrought steel].

          - Configuration: [Top and bottom rods] [Top rod].

          - Electrified Features:

Retain applicable subparagraphs below. Verify availability of products with manufacturers.

            - Push pad monitor switch.

            - Double push pad monitor switch.

            - Electric locking and unlocking.

            - Fail-secure electric latch retraction (dogging) that engages latch when fire-alarm system is activated.

            - Delayed egress.

            - Alarm.

            - <Insert feature>.

        - Combination Exit Devices: [Grade 1] [Grade 2].

          - [Type 9, rim and surface vertical rod] [Type 11, mortise and surface vertical rod] [Type 12, mortise and concealed vertical rod].

          - Actuating Bar: [Cross bar] [Push pad] [Narrow-stile push pad].

          - Material: [Brass] [Bronze] [Stainless steel] [Aluminum] [Wrought steel].

          - Electrified Features:

Retain applicable subparagraphs below. Verify availability of products with manufacturers.

            - Push pad monitor switch.

            - Double push pad monitor switch.

            - Electric locking and unlocking.

            - Fail-secure electric latch retraction (dogging) that engages latch when fire-alarm system is activated.

            - Delayed egress.

            - Alarm.

            - <Insert feature>.

        - Automatic Latching Two-Point Bolts: Grade 1.

          - [Type 23, concealed] [Type 24, surface].

          - Material: [Brass] [Bronze] [Stainless steel].

        - Extension Flush Bolt Sets: ANSI/BHMA A156.3, Grade 1.

          - Type: [Type 25, automatic] [Type 27, self-latching].

          - Material: [Brass] [Bronze] [Stainless steel].

Electronic exit bars can be used only with electrified locks.

        - Electronic Exit Bars: Nonlatching electronic actuating (releasing) device activated by an adjustable capacitance sensor and with no moving parts; listed and labeled as panic exit hardware. Fabricate bar from extruded aluminum, and provide door and frame transfer device and 16 ft. (4.9 m) of cord to route wiring off the door frame.

Retain "Extruded-Aluminum Removable Mullions," "Tube-Steel Removable Mullions," or "Fire-Exit Removable Mullions" paragraphs below if required and removable mullions are not specified with door frames in other Sections.

        - Extruded-Aluminum Removable Mullions: With malleable-iron top and bottom retainers, and prepared for strikes as follows:

          - Strikes: [Two standard recessed strikes] [Two monitor strikes] [One standard and one electric strike with disconnect for wiring harness at top of mullion].

        - Tube-Steel Removable Mullions: With malleable-iron top and bottom retainers, and prepared for strikes as follows:

          - Strikes: [Two standard recessed strikes] [Two monitor strikes] [One standard and one electric strike with disconnect for wiring harness at top of mullion].

        - Fire-Exit Removable Mullions: Provide removable mullions for use with fire-exit devices complying with NFPA 80 that are listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction, for fire and panic protection, based on testing in accordance with UL 305 and NFPA 252. Use mullions only with exit devices for which they have been tested.

        - Dummy Push Bar: Nonfunctioning push bar matching functional push bar.

          - Operation: [Rigid] [Movable] [Movable with monitor switch].

Retain "Exit Device Outside Trim" Paragraph below if exit device levers, knobs, and pulls are required to match lockset and latchset designs. Matching locksets to exit device trim and levers limit the design options available; coordinate with lock design retained in "Lock Trim" Paragraph in "Mechanical Locks and Latches" Article and "Lock Trim" Subparagraph in "Self-Contained Electronic Locks" Article.

        - Exit Device Outside Trim: [Lever] [Lever with cylinder] [Knob] [Knob with cylinder] [Pull] [Pull with cylinder] [Thumb turn with cylinder]; material and finish to match locksets unless otherwise indicated.

          - Match design for lock trim unless otherwise indicated.

Retain "Through-Bolt Fasteners" Paragraph below for doors subject to physical abuse.

        - Through-Bolt Fasteners: For exit devices and trim on [metal doors] [non-fire-rated wood doors] [fire-rated wood doors].

      - LOCK CYLINDERS

        - Lock Cylinders: Tumbler type, constructed from brass or bronze, stainless steel, or nickel silver.[ Provide cylinder from same manufacturer of locking devices.]

If retaining option in "Lock Cylinders" Paragraph above, delete "Manufacturers" Subparagraph below.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Standard Lock Cylinders: ANSI/BHMA A156.5, [Grade 1] [Grade 1A] [Grade 2] permanent cores; face finished to match lockset.

          - Core Type: [Interchangeable] [Removable].

Only residential applications typically use five-pin cylinders. Commercial cylinders typically have six or seven pins. Seven-pin cylinders are typically interchangeable cores.

          - Number of Pins: [Five] [Six] [Seven].

          - Lock Type: [Mortise] [Rim] [Bored-lock] type.

        - High-Security Lock Cylinders: ANSI/BHMA A156.30, [Grade 1] [Grade 2] [Grade 3] permanent cores that are removable; face finished to match lockset.

          - [Type M, mechanical] [Type E, electrical].

          - Number of Pins: [Six] [Seven].

          - Lock Type: [Mortise] [Rim] [Bored-lock] type.

Retain "Construction Master Keys" Paragraph below if permanent cores are installed during construction.

        - Construction Master Keys: Provide cylinders with feature that permits voiding of construction keys without cylinder removal. Provide 10 construction master keys.

Retain "Construction Cores" Paragraph below if temporary construction cores are required.

        - Construction Cores: Provide construction cores that are replaceable by permanent cores. Provide 10 construction master keys.

      - KEYING

Retain option in "Keying System" Paragraph below if retaining "Keying Conference" Paragraph in "Preinstallation Meetings" Article.

        - Keying System: Factory registered, complying with guidelines in ANSI/BHMA A156.28, appendix. Provide one extra key blank for each lock.[ Incorporate decisions made in keying conference.]

Retain "No Master Key System," "Master Key System," "Grand Master Key System," or "Great-Grand Master Key System" Subparagraph below.

          - No Master Key System: Only change keys operate cylinders.

            - Provide three cylinder change keys.

          - Master Key System: Change keys and a master key operate cylinders.

            - Provide three cylinder change keys and five master keys.

          - Grand Master Key System: Change keys, a master key, and a grand master key operate cylinders.

            - Provide three cylinder change keys and five each of master and grand master keys.

          - Great-Grand Master Key System: Change keys, a master key, a grand master key, and a great-grand master key operate cylinders.

            - Provide three cylinder change keys and five each of master, grand master, and great-grand master keys.

Retain "Existing System" Subparagraph below if Owner has an existing keying system.

          - Existing System:

            - Master key or grand master key locks to Owner's existing system.

            - Re-key Owner's existing master key system into new keying system.

Retain "Keyed Alike" Subparagraph below if required.

          - Keyed Alike: Key all cylinders to same change key.

        - Keys: [Nickel silver] [Brass].

Retain "Stamping" Subparagraph below if key requires special marking.

          - Stamping: Permanently inscribe each key with a visual key control number and include the following notation:

            - Notation: ["DO NOT DUPLICATE."] [Information to be furnished by Owner.]

      - KEY CONTROL SYSTEM

        - Key Control Cabinet: ANSI/BHMA A156.28; metal cabinet with baked-enamel finish, containing key-holding hooks, labels, two sets of key tags with self-locking key holders, key-gathering envelopes, and temporary and permanent markers; with key capacity of [150] <Insert number> percent of the number of locks.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Multiple-Drawer Cabinet," "Wall-Mounted Cabinet," or "Portable Cabinet" Subparagraph below to suit Project. Multiple-drawer cabinets have minimum of 600 or 700 hooks with expansion capacity; wall-mounted cabinets have minimum of 25, 125, or 150 hooks with expansion capacity; portable cabinets have minimum of 14 hooks with expansion capacity.

          - Multiple-Drawer Cabinet: [Grade 1] [Grade 2] cabinet with drawers equipped with key-holding panels, key envelope storage, and progressive-type ball-bearing suspension slides. Include single cylinder lock to lock all drawers.

          - Wall-Mounted Cabinet: [Grade 1] [Grade 2] cabinet with hinged-panel door equipped with key-holding panels and pin-tumbler cylinder door lock.

          - Portable Cabinet: [Grade 1] [Grade 2] tray for mounting in file cabinet, equipped with key-holding panels, envelopes, and cross-index system.

Key lock boxes are used only to provide controlled access for fire and medical emergency personnel into gated communities and apartment complexes. Coordinate requirements with authorities having jurisdiction.

        - Key Lock Boxes: Designed for storage of [two] [10] <Insert number> keys.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Features:

Retain applicable subparagraphs below.

            - Tamper switches to connect to intrusion detection system.

            - <Insert feature>.

        - Key Control System Software: Multiple-index system for recording and reporting key-holder listings, tracking keys and lock and key history, and printing receipts for transactions. Include instruction manual.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - OPERATING TRIM

        - Operating Trim: ANSI/BHMA A156.6; [aluminum] [brass] [bronze] [stainless steel] unless otherwise indicated.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Flat Push Plates: With square corners and beveled edges; secured with exposed screws.

          - Thickness: [0.050 inch (1.3 mm)] [1/8 inch (3.2 mm)] <Insert dimension>.

          - Size: [4 inches wide by 16 inches high (102 mm wide by 406 mm high)] <Insert dimensions>.

        - Push-Pull Plates: With square corners, beveled edges, and raised integral lip; secured with exposed screws.

          - Thickness: [1/8 inch (3.2 mm)] <Insert dimension>.

          - Size: [3-1/2 inches wide by 15-3/4 inches high (89 mm wide by 400 mm high)] <Insert dimensions>.

        - Straight Door Pulls:

          - Type: [3/4-inch (19-mm) constant-diameter] [variable-diameter] [flattened-round] [hospital-type] pull.

          - Mounting: [Surface applied with concealed fasteners] [Through bolted with oval-head machine screws and countersunk washers] [Back to back with threaded sleeves].

          - Minimum Clearance: [1-1/2 inches (38 mm)] <Insert dimension> from face of door.

          - Overall Length: [9 inches (229 mm)] <Insert dimension>.

        - Offset Door Pulls: [1-inch (25-mm)] <Insert dimension> constant-diameter pull.

          - Mounting: [Surface applied with concealed fasteners] [Through bolted with oval-head machine screws and countersunk washers] [Back to back with threaded sleeves].

          - Offset: [2 inches (51 mm)] <Insert dimension>.

          - Minimum Clearance: [2-1/4 inches (57 mm)] <Insert dimension> from face of door.

          - Overall Length: [9 inches (229 mm)] <Insert dimension>.

Typically, "Flush Door Pulls" and "Straight Pull-Plate Door Pulls" paragraphs below are used for sliding doors.

        - Flush Door Pulls: Mortised 1/2 inch (13 mm) deep; secured with screws.

          - Shape: [Rectangular with rectangular recess] <Insert description>.

          - Size: [3-1/2 inches wide by 4-3/4 inches high (89 mm wide by 121 mm high)] <Insert dimensions>.

        - Straight Pull-Plate Door Pulls: Pull fixed to 0.050-inch- (1.3-mm-) thick plate, [4 inches wide by 16 inches high (102 mm wide by 406 mm high)] <Insert dimensions> with square corners and beveled edges.

          - Type: [3/4-inch (19-mm) constant-diameter] [variable-diameter] [flattened-round] [hospital-type] pull.

          - Mounting: [Surface applied with concealed fasteners] [Through bolted with oval-head machine screws and countersunk washers] [Back to back with threaded sleeves].

          - Minimum Pull Clearance: [1-1/2 inches (38 mm)] <Insert dimension> from face of door.

          - Overall Pull Length: [9 inches (229 mm)] <Insert dimension>.

        - Offset Push-Pull Door Pulls: Pull fixed to 0.050-inch- (1.3-mm-) thick plate, [4 inches wide by 16 inches high (102 mm wide by 406 mm high)] <Insert dimensions> with square corners and beveled edges.

          - Pull Diameter: [1-inch (25-mm)] <Insert dimension> constant diameter.

          - Pull Offset: [2 inches (51 mm)] <Insert dimension>.

          - Minimum Pull Clearance: [2-1/4 inches (57 mm)] <Insert dimension> from face of door.

          - Overall Pull Length: [9 inches (229 mm)] <Insert dimension>.

        - Single Push Bar: Horizontal-bar type.

          - Mounting: [Surface applied with concealed fasteners] [Through bolted with oval-head machine screws and countersunk washers].

          - Shape and Size: [1-inch (25-mm) constant-diameter round bar] [Minimum 3/8-by-1-1/4-inch (10-by-32-mm) flat bar] <Insert shape and size>.

          - Minimum Clearance: [1-1/2 inches (38 mm)] <Insert dimension> from face of door.

        - Double Pull Bar: Two horizontal bars connected by matching vertical pull bar and spaced at [8 inches (200 mm)] <Insert dimension> o.c.

          - Mounting: [Surface applied with concealed fasteners] [Through bolted with oval-head machine screws and countersunk washers].

          - Shape and Size: [1-inch (25-mm) constant-diameter round bars] [Minimum 3/8-by- 1-1/4-inch (10-by-32-mm) flat bars] <Insert shape and size>.

          - Minimum Clearance: [1-1/2 inches (38 mm)] <Insert dimension> from face of door.

      - ACCESSORIES FOR PAIRS OF DOORS

Coordinators on pairs of doors prevent the active leaf from closing before inactive leaf.

        - Coordinators: ANSI/BHMA A156.3; consisting of active-leaf, hold-open lever, and inactive-leaf release trigger; fabricated from steel with nylon-coated strike plates; with built-in, adjustable safety release[; and with internal override].

Retain "Carry-Open Bars" Paragraph below if automatic or self-latching bolts are not used on the inactive leaves of pairs of doors. Carry-open bars push active leaves open when inactive leaves are opened first and prevent damage to astragals.

        - Carry-Open Bars: ANSI/BHMA A156.3; prevent the inactive leaf from opening before the active leaf; provide polished brass or bronze carry-open bars with strike plate for inactive leaves of pairs of doors unless automatic or self-latching bolts are used.

Astragals in three paragraphs below are mounted on one leaf of a pair of doors to protect against weather and to minimize passage of smoke, flame, and gases during a fire. If astragals installed as part of labeled fire-rated assemblies are specified in this Section instead of with doors, retain "Flat Overlapping Astragals" Paragraph below.

        - Flat Overlapping Astragals: ANSI/BHMA A156.22; flat [primed steel] [zinc-plated steel] [aluminum] [stainless steel] [brass] metal bar, surface mounted on face of door with screws; minimum 1/8 inch (3.2 mm) thick by 2 inches (51 mm) wide by full height of door.

        - Rigid, Housed Astragals: ANSI/BHMA A156.22; gasket material held in place by metal housing; fastened to face of door with screws.

          - Gasket Material: [Closed-cell sponge silicone] [Closed-cell sponge neoprene] [Neoprene] [Silicone bulb].

          - Housing Material: [Aluminum] [Copper alloy (brass or bronze)].

        - Overlapping-with-Gasket Astragals: ANSI/BHMA A156.22; T-shaped metal, surface mounted on edge of door with screws.

          - Base Metal: [Primed steel] [Zinc-plated steel] [Aluminum] [Stainless steel].

          - Gasket Material: [Vinyl] [Silicone] [Sponge neoprene] [Brush pile] [Polypropylene].

      - SURFACE CLOSERS

        - Surface Closers: ANSI/BHMA A156.4; rack-and-pinion hydraulic type with adjustable sweep and latch speeds controlled by key-operated valves and forged-steel main arm. Comply with manufacturer's written instructions for size of door closers depending on size of door, exposure to weather, and anticipated frequency of use. Provide factory-sized closers, adjustable to meet field conditions and requirements for opening force.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Cast-Aluminum Surface Closers: [Grade 1] [Grade 2]; traditional type with mechanism enclosed in cast-aluminum alloy shell.

          - Mounting: [Hinge side] [Opposite hinge side] [Parallel arm] [Bracket].

          - Type: [Regular arm] [Fusible-link holder arm] [Two-point hold-open arm] [Delayed-action closing] <Insert type>.

Backcheck feature is not available with delayed action closers.

          - Backcheck: [Factory preset] [Adjustable], effective between 60 and 85 degrees of door opening.

        - Surface Closer without Cover: [Grade 1] [Grade 2]; modern type.

          - Mounting: [Hinge side] [Opposite hinge side] [Parallel arm] [Bracket] [Hinge side top jamb] [Opposite side top jamb].

          - Type: [Regular arm] [Hold-open] [Fusible-link holder arm] [Slide track arm] [Dead stop] [Dead stop hold-open] [Delayed-action closing] <Insert type>.

Backcheck feature is not available with delayed-action closers.

          - Backcheck: [Factory preset] [Adjustable], effective between 60 and 85 degrees of door opening.

          - Closing Power Adjustment: At least [50] [35] [15] percent more than minimum tested value.

        - Surface Closer with Cover: [Grade 1] [Grade 2]; modern type with mechanism enclosed in cover.

          - Mounting: [Hinge side] [Opposite hinge side] [Parallel arm] [Bracket] [Hinge side, top jamb] [Opposite side, top jamb].

          - Type: [Regular arm] [Hold-open] [Fusible-link holder arm] [Slide track arm] [Dead stop] [Dead stop hold-open] [Delayed-action closing] <Insert type>.

Backcheck feature is not available with delayed action closers.

          - Backcheck: [Factory preset] [Adjustable], effective between 60 and 85 degrees of door opening.

          - Cover Material: [Aluminum] [Plated steel] [Molded plastic].

          - Closing Power Adjustment: At least [50] [35] [15] percent more than minimum tested value.

      - CONCEALED CLOSERS

        - Concealed Closers: ANSI/BHMA A156.4; rack-and-pinion hydraulic type with adjustable sweep and latch speeds controlled by key-operated valves. Comply with manufacturer's written instructions for size of door closers depending on size of door, exposure to weather, and anticipated frequency of use. Provide factory-sized closers, adjustable to meet field conditions and requirements for opening force.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Concealed-in-Door Closer: [Grade 1] [Grade 2]; mortised into top rail of minimum 1-3/4-inch- (44-mm-) thick doors and track mortised into head frame; with double-lever arm indicated.

          - Type: [Surface shoe] [Mortised soffit plate].

          - Arm: [Regular] [Hold-open].

          - Closing Power Adjustment: At least [50] [35] [15] percent more than minimum tested value.

        - Concealed Overhead Closer: [Grade 1] [Grade 2]; mortised into head frame; with cast-metal body and exposed cover plate.

          - Type: [Exposed arm with surface shoe, single acting] [Concealed arm and track, butt or pivot hung, single acting] [Concealed arm and track, center pivoted, single acting] [Concealed arm and track, center pivoted, double acting].

          - Arm: [Regular] [Automatic hold-open] [Manually selected hold-open] [Fusible-link holder arm].

          - Track: [Regular] [Automatic hold-open] [Manually selected hold-open].

          - Cover Plate Material: [Aluminum] [Plated steel].

          - Backcheck: [Factory preset] [Adjustable].

          - Closing Power Adjustment: At least [50] [35] percent more than minimum tested value.

        - Concealed Floor Closer: [Grade 1] [Grade 2]; with cement case and cast-iron closer body case and top pivot.

Coordinate manufacturer of intermediate pivot with manufacturer of offset-pivot floor closers. Offset pivots are ordered separately.

          - Closer Type: [Center pivoted; include top pivot] [Offset pivoted; include top pivot] [Independently hung].

          - Door-Swing Type: [Single] [Double] acting.

Retain "Fire Rated" Subparagraph below if required.

          - Fire Rated: Listed for use in labeled fire-rated assemblies where indicated.

          - Function: [Regular] [Automatic hold-open] [Manually selected hold-open] [Delayed-action closing].

          - Backcheck: [Factory preset] [Adjustable].

          - Closing Power Adjustment: At least [50] [35] percent more than minimum tested value.

          - Case Depth: [Regular, 4 inches (100 mm)] [Shallow, 2 inches (50 mm)].

Retain "Floor Plates" Subparagraph below unless saddle-type thresholds are specified for floor closers. For floor closers with thresholds, coordinate requirements of closers with thresholds specified.

          - Floor Plates: Provide [flush cover plates matching door hardware finish] [recessed floor plates with insert of floor finish material and extended closer spindle to accommodate thickness of floor finish] unless thresholds are indicated.

            - Material: [Aluminum] [Plated steel].

      - CLOSER HOLDER RELEASE DEVICES

        - Closer Holder Release Devices: ANSI/BHMA A156.15, Grade 1; closer connected with separate or integral releasing and fire- or smoke-detecting devices. Door must become self-closing on interruption of signal to release device. Automatic release is activated by [smoke detection system] [loss of power].

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Type: [Single-point hold-open] [Multiple-point hold-open] [Free-swinging release].

          - Mounting: [Surface mounted on face of door] [Surface mounted on face of top jamb] [Surface mounted on stop] [Mortised into top rail of door] [Mortised into top jamb] [Recessed into floor].

          - Features:

Retain applicable subparagraphs below.

            - Adjustable backcheck.

            - Integral smoke detector.

            - Adjustable spring power.

            - Adjustable hold-open manual release force.

            - <Insert feature>.

      - MECHANICAL STOPS AND HOLDERS

        - Wall- and Floor-Mounted Stops: ANSI/BHMA A156.16; [polished cast-brass, -bronze, or -aluminum] <Insert metal> base metal.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Rigid-Type Floor Stop: [Grade 1] [Grade 2]; with rubber bumper.

          - Installation: [Surface-screw] [Expansion-shield] installation.

        - Dome-Type Floor Stop: [Grade 1] [Grade 2]; with minimum 1-inch- (25-mm-) high bumper for doors without threshold and 1-3/8-inch- (35-mm-) high bumper for doors with threshold.

Retain subparagraph below if required.

          - Provide with extruded-aluminum riser for carpet installations.

        - Combination Floor Stop and Holder: [Grade 1] [Grade 2].

          - Installation: [Surface-screw] [Expansion-shield] installation.

          - Hold Open: [Semiautomatic] [Automatic with release by pushing door].

        - Manual Combination Floor Stop and Holder: [Grade 1] [Grade 2]; 3-1/2 inches (89 mm) long, with holder, keeper, and rubber bumper.

          - Installation: [Surface-screw] [Expansion-shield] installation.

        - Chain Door Stops: Grade 2; welded chain, each end attached to compression springs, both covered with protective sleeve; for surface-screw application.

Revise "Wall Bumpers" Paragraph below if mounting is by means of screw inserted through small hole in rubber bumper.

        - Wall Bumpers: [Grade 1] [Grade 2]; with rubber bumper; 2-1/2-inch (64-mm) diameter, minimum 3/4-inch (19-mm) projection from wall; with backplate for concealed fastener installation.

          - Bumper Configuration: [Convex] [Concave].

Roller-type wall bumpers are used to prevent interfering doors from hitting.

        - Roller-Type Wall Bumpers: [Grade 1] [Grade 2]; minimum 4-3/8-inch (111-mm) projection from wall; for surface-screw application.

        - Lever-Type Door Holders: [Grade 1] [Grade 2]; minimum 4-inch- (102-mm-) long arm that swings up and remains in vertical position; with replaceable rubber tip; for surface-screw application.

        - Plunger-Type Door Holders: [Grade 1] [Grade 2]; minimum 1-1/8-inch (29-mm) plunger throw; with replaceable rubber tip; for surface-screw application.

      - ELECTROMAGNETIC STOPS AND HOLDERS

        - Electromagnetic Door Holders: ANSI/BHMA A156.15, Grade 1; [wall-mounted electromagnetic single] [floor-mounted electromagnet single] [floor-mounted electromagnet double] unit with strike plate attached to swinging door; coordinated with fire detectors and interfaced with fire-alarm system for labeled fire-rated door assemblies.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - OVERHEAD STOPS AND HOLDERS

        - Overhead Stops and Holders: ANSI/BHMA A156.8.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Overhead Concealed Slide Holders: Type 1, [Grade 1] [Grade 2]; hold-open and release by push and pull of door unless control is set in inactive position; with stop, shock absorber, and adjustable holding pressure; for doors opening 110 degrees.

          - Door Swing Type: [Single] [Double] acting.

        - Overhead Concealed Slide Stops: Type 1, [Grade 1] [Grade 2]; release by push and pull of door unless control is set in inactive position; with stop, shock absorber, and adjustable holding pressure; for doors opening 110 degrees.

          - Door Swing Type: [Single] [Double] acting.

        - Overhead Surface-Mounted Slide Holders: Type 2, [Grade 1] [Grade 2]; hold-open and release by push and pull of door unless control is set in inactive position; with stop, shock absorber, and adjustable holding pressure; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted, Concealed Slide Stops: Type 2, [Grade 1] [Grade 2]; release by push and pull of door unless control is set in inactive position; with stop, shock absorber, and adjustable holding pressure; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted, Jointed-Arm Holders: Type 3, [Grade 1] [Grade 2]; hold-open and release by push and pull of door; control capable of being set in inactive position; with stop and shock absorber; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted, Jointed-Arm Stops: Type 3, [Grade 1] [Grade 2]; release by push and pull of door; control capable of being set in inactive position; with stop and shock absorber; for single-acting doors opening 110 degrees.

        - Overhead Concealed, Friction Slide Holders: Type 4, [Grade 1] [Grade 2]; with frictional element held under adjustable pressure, free-acting shoulder pivots, and shock absorber; for doors opening 110 degrees.

          - Door Swing Type: [Single] [Double] acting.

        - Overhead Concealed, Nonfriction Slide Stops: Type 4, [Grade 1] [Grade 2]; with nonfrictional element held under adjustable pressure and shock absorber; for [single] [double]-acting doors opening 110 degrees.

        - Overhead Concealed, Nonfriction Slide Holders: Type 4, [Grade 1] [Grade 2]; with nonfrictional element held under adjustable pressure, automatic hold-open, and shock absorber; for doors opening 110 degrees.

          - Door Swing Type: [Single] [Double] acting.

        - Overhead Surface-Mounted, Friction Slide Holders: Type 5, [Grade 1] [Grade 2]; with frictional element held under adjustable pressure, free-acting shoulder pivots, and shock absorber; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted, Nonfriction Slide Stops: Type 5, [Grade 1] [Grade 2]; with nonfrictional element held under adjustable pressure and shock absorber; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted, Nonfriction Slide Holders: Type 5, [Grade 1] [Grade 2]; with nonfrictional element held under adjustable pressure, automatic hold-open, and shock absorber; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted Rod Holders: Type 8, [Grade 1] [Grade 2]; hold-open and release by push and pull of door unless roller cam is set in inactive position; with stop, shock absorber, and adjustable spring tension; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted Rod Stops: Type 8, [Grade 1] [Grade 2]; release by push and pull of door unless roller cam is set in inactive position; with stop, shock absorber, and adjustable spring tension; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted Cantilever Holders: Type 9, [Grade 1] [Grade 2]; hold-open and release by push and pull of door or thumb turn; with stop and shock absorber; for single-acting doors opening 110 degrees.

        - Overhead Surface-Mounted Cantilever Stops: Type 9, [Grade 1] [Grade 2]; release by push and pull of door or thumb turn; with stop and shock absorber; for single-acting doors opening 110 degrees.

      - DOOR GASKETING

        - Door Gasketing: ANSI/BHMA A156.22; with resilient or flexible seal strips that are easily replaceable and readily available from stocks maintained by manufacturer.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Maximum Air Leakage: When tested in accordance with ASTM E283/E283M with tested pressure differential of 0.3 inch wg (75 Pa), as follows:

Retail applicable subparagraphs below.

          - Smoke-Rated Gasketing: 0.3 cfm/sq. ft. (3 cu. m per minute/sq. m) of door opening.

          - Gasketing on Single Doors: 0.3 cfm/sq. ft. (3 cu. m per minute/sq. m) of door opening.

          - Gasketing on Double Doors: 0.50 cfm per ft. (0.000774 cu. m/s per m) of door opening.

Adhesive-backed, spring-metal, and rigid and adjustable housed perimeter gasketing is available for smoke-labeled, fire-labeled, and sound-rated doors.

        - Adhesive-Backed Perimeter Gasketing: [Vinyl bulb] [Sponge silicone] [Silicone] [Neoprene bulb] [Sponge neoprene] gasket material applied to frame rabbet with self-adhesive.

        - Spring-Metal Perimeter Gasketing: Minimum [0.008-inch- (0.20-mm-) thick brass or bronze] [0.008-inch- (0.20-mm-) thick stainless steel] [0.012-inch- (0.30-mm-) thick aluminum] gasket material fastened to frame rabbet with nails or screws.

        - Rigid, Housed, Perimeter Gasketing: [Sponge silicone] [Sponge neoprene] [Silicone bulb] [Polyurethane bulb] [Vinyl bulb] [Vinyl brush] [Nylon brush] [Thermoplastic elastomer] gasket material held in place by housing; fastened to frame stop with screws.

          - Housing Material: [Aluminum] [Brass or bronze] [Stainless steel].

        - Adjustable, Housed, Perimeter Gasketing: Screw-adjustable, [sponge silicone] [sponge neoprene] [silicone bulb] [polyurethane bulb] [vinyl bulb] [vinyl brush] [nylon brush] [thermoplastic elastomer] gasket material held in place by housing; fastened to frame stop with screws.

          - Housing Material: [Aluminum] [Brass or bronze] [Stainless steel].

        - Interlocking Perimeter Gasketing: Minimum [0.018-inch- (0.46-mm-) thick zinc] [0.015-inch- (0.38-mm-) thick bronze] gasket material consisting of two pieces, one fastened to door and one fastened to frame, that interlock when door is closed; mounted with screws.

        - Overlapping Astragals for Meeting Stiles: [EPDM strip] [Vinyl strip] [Nylon brush] gasket material held in place by housing and overlapping when doors are closed; mounted to face of meeting stile with screws.

          - Housing Material: [Aluminum] [Bronze].

          - Mounting: Surface mounted on face of [each] [one] door.

        - Meeting Astragals for Meeting Stiles: [Silicone bulb] [Neoprene bulb] [Vinyl bulb] [Nylon brush] [Brush pile] [Thermoplastic elastomer] gasket material held in place by housing; mounted with screws.

          - Housing Material: [Aluminum] [Bronze].

          - Mounting: [Surface mounted on face of each door] [Surface mounted on face of one door] [Semimortised into edge of each door] [Semimortised into edge of one door] [Mortised into edge of each door] [Mortised into edge of one door].

        - Adjustable Astragals for Meeting Stiles: Screw-adjustable, [silicone] [neoprene] [vinyl] [vinyl-covered magnet] [brush pile] [thermoplastic elastomer] gasket material held in place by housing; mounted with screws.

          - Housing Material: [Aluminum] [Bronze].

          - Mounting: [Surface mounted on face] [Semimortised into edge] [Mortised into edge] of each door.

If needed, insert requirements for proprietary intumescent fire-seal astragals.

        - Door Sweeps: [Neoprene] [Vinyl] [Nylon brush] [Polyurethane] [Silicone] gasket material held in place by flat housing or flange; surface mounted to face of door with screws.

          - Housing or Flange Material: [Aluminum] [Bronze].

        - Door Shoes: [Vinyl] [Thermoplastic elastomer] [Neoprene] [Brush pile] gasket material held in place by housing; mounted to bottom edge of door with screws.

          - Housing Material: [Aluminum] [Bronze].

Retain "Extended Housing" Subparagraph below if required.

          - Extended Housing: [One side] [Both sides] of door.

          - Mounting: [Surface mounted on] [Mortised into] bottom edge of door.

Nylon brush option in "Automatic Door Bottoms" Paragraph below is for use on carpet when no threshold is provided.

        - Automatic Door Bottoms: [Sponge neoprene] [Sponge silicone] [Thermoplastic elastomer] [Nylon brush] gasket material held in place by housing that automatically drops to form seal when door is closed; mounted to bottom edge of door with screws.

          - Housing Material: [Aluminum] [Bronze] [Aluminum lined with 0.047-inch (1.2-mm) thick lead].

          - Mounting: [Semimortised into bottom] [Mortised into bottom] of door.

Retain "Type" Subparagraph below if low-closing force is required.

          - Type: Low-closing-force type for doors required to meet accessibility requirements.

      - THRESHOLDS

        - Thresholds: ANSI/BHMA A156.21; fabricated to full width of opening indicated.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Compressing-Top Thresholds: Metal member with compressible vinyl seal on top of threshold that seals against bottom of door; and base metal of [aluminum] [extruded bronze] [stainless steel].

        - Saddle Thresholds:

          - Type: [Smooth top] [Fluted top] [Fluted top and offset] [Thermal break and fluted top] [Applied gasketed stop and fluted top] [Carpet separator with fluted top] [Fluted top, barrier free].

          - Base Metal: [Aluminum] [Extruded bronze] [Stainless steel].

        - Half-Saddle Thresholds: Fluted-top metal member; and base metal of [aluminum] [extruded bronze].

        - Interlocking Thresholds: Fluted-top metal member with integral lip designed to engage a hook strip applied to door.

          - Type: [Single lip] [Double lip] [Double-lip water return] [Double-lip water return with aluminum pan] [Single lip with thermal barrier].

          - Base Metal: [Aluminum] [Extruded bronze].

        - Latching/Rabbeted Thresholds:

          - Type: [Fluted] [Smooth] [Offset with fluted] top.

          - Base Metal: [Aluminum] [Extruded bronze].

        - Latching/Rabbeted Thresholds with Gasket: Fluted-top metal member with gasket.

          - Type: [Offset] [Thermal barrier].

          - Base Metal: [Aluminum] [Extruded bronze].

          - Gasket Material: [Vinyl] [Silicone] [Neoprene] [Brush pile] [Closed-cell sponge neoprene].

Latching/rabbeted panic thresholds are for use with vertical rod exit devices.

        - Latching/Rabbeted Panic Thresholds:

          - Type: [Fluted, barrier free] [Fluted with gasket] top.

          - Base Metal: [Aluminum] [Extruded bronze].

        - Plate Thresholds: Solid metal plate.

          - Top Surface: Fluted[ with slip-resistant abrasive].

          - Base Metal: [Aluminum] [Extruded brass or bronze] [Stainless steel].

        - Ramped Thresholds: Modular, interlocking, sloped, fluted-top metal assemblies with closed return ends; 1:12 slope.

          - Top Surface: Fluted[ with slip-resistant abrasive].

          - Base Metal: [Aluminum] [Extruded bronze].

        - Saddle Thresholds for Floor Closers: Fluted top.

          - [Type A, for center-hung doors; ends not mitered] [Type B, for offset-hung doors; ends not mitered] [Type C, for center-hung doors; ends mitered] [Type D, for offset-hung doors; ends mitered].

          - Base Metal: [Aluminum] [Extruded bronze].

      - SLIDING DOOR HARDWARE

        - Sliding Door Hardware: ANSI/BHMA A156.14; consisting of complete sets, including rails, hangers, supports, bumpers, floor guides, and accessories indicated.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Horizontal Sliding Door Hardware: Grade 1; rated for minimum door weight of [240 lb (109 kg)] [320 lb (145 kg)] [450 lb (205 kg)] [560 lb (254 kg)] [640 lb (290 kg)] [800 lb (363 kg)] [1000 lb (455 kg)] [1500 lb (681 kg)] <Insert weight>.

Use galvanized-steel or anodized aluminum hardware for exterior applications.

          - Material: [Wrought steel] [Galvanized steel or anodized aluminum].

          - Rail: [Box without mounting brackets] [Box with attached mounting brackets] [Box with attached flashing] [Round without mounting brackets] [Round with attached mounting brackets].

          - Rail Supports: [Single-sidewall] [Double-sidewall] [Triple-sidewall] [Single-overhead] [Single-overhead parallel] [Single-overhead cross-ear] [Double-overhead cross-ear] [Triple-overhead cross-ear] style.

            - Provide intermediate-, end-, and splice-type track supports as required by rail configuration and door weight indicated.

          - Hanger Configuration: Four-wheel [truck] [hanger assembly with top mounting plate] [hanger assembly with drop bolt] [hanger assembly with single drop strap] [hanger assembly with double drop strap] <Insert hanger configuration>.

            - Wheel Assembly: Steel wheels with ball bearings.

          - Accessories:

Retain applicable subparagraphs below.

            - Continuous bottom guide.

            - Guide rail and guide rail brackets as required by rail configuration.

            - Bow handle, minimum 6 inches (152 mm) in overall length.

            - Flush pull, minimum 4 by 5-1/2 by 3/4 inch (102 by 140 by 19 mm), mortised into door.

            - Cane bolt, minimum 1/2-inch (13-mm) diameter by 12 inches (305 mm) long.

            - Stay roller, minimum 2-inch- (51-mm-) diameter wheel.

            - Floor center stop of cast iron.

            - End guide and stop.

            - Parallel door floor guides.

            - Door stop.

            - Sliding door latch.

            - Bumper shoe, minimum 0.0598-inch (1.5-mm) thickness.

            - Cremone bolt with lever handle, minimum 1/2-inch- (13-mm-) diameter oval or round rod, and rod guides at 24 inches (610 mm) o.c.

            - Top spring bolt, minimum [6 inches (152 mm)] [8 inches (203 mm)]; malleable iron and with angle or surface strike and 24-inch (610-mm) chain.

            - Foot bolt minimum [6 inches (152 mm)] [8 inches (203 mm)]; wrought steel, cast iron, or malleable iron.

        - Bypassing Sliding Door Hardware: Rails and door hardware that allow vertical adjustment and rated for doors weighing up to [120 lb (54 kg); Grade 1] [80 lb (36 kg); Grade 1] [40 lb (18 kg); Grade 2].

          - Rail Material: [Galvanized wrought steel] [Extruded aluminum].

          - Rail Configuration: [V-grooved double leg] [V-grooved double leg with fascia] [I-beam].

          - Mounting: [Top hung] [Bottom supporting with overhead guide].

          - Wheel Assembly: Two wheel or four wheel, with roller bearings.

          - Pulls: [Flush, mortised into door] [Cast-, forged-, or extruded-brass or -bronze surface-applied type] [Wrought-brass or -bronze edge type, mortised into edge of door] [Sliding door latch] [Sliding door lock with emergency release].

          - Accessories:

            - Bumper stops; wrought steel.

            - Floor guides.

        - Pocket Sliding Door Hardware: Grade 1; rated for doors weighing up to [120 lb (54 kg)] [80 lb (36 kg)], overhead box rails and door hardware that allows vertical adjustment.

          - Rail Material: [Galvanized wrought steel] [Extruded aluminum].

          - Door Type: [Single] [Biparting].

          - Rail Configuration: [V-grooved double leg] [I-beam].

          - Wheel Assembly: Two wheel or four wheel, with roller bearings.

          - Pulls: [Flush, mortised into door] [Cast-, forged-, or extruded-brass or -bronze surface-applied type] [Wrought-brass or -bronze edge type, mortised into edge of door] [Sliding door latch] [Sliding door lock with emergency release].

          - Accessories:

            - Bumper stops; wrought steel.

            - Floor guides installed within pocket.

      - FOLDING DOOR HARDWARE

        - Folding Door Hardware: ANS/BHMA A156.14; complete sets, including overhead rails, hangers, supports, bumpers, floor guides, and accessories indicated.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Bifolding Door Hardware: Rated for door panels weighing up to [50 lb (23 kg); Grade 1] [30 lb (14 kg); Grade 2]; with rails and door hardware that allow horizontal and vertical adjustment.

          - Rail Material: [Galvanized wrought steel] [Extruded aluminum].

          - Rail Configuration: [V-grooved double leg] [V-grooved double leg with fascia] [I-beam].

          - Mounting: [Surface mounted overhead] [Top and bottom hung].

          - Wheel Assembly: Two wheel or four wheel, with roller bearings.

        - Multiple Folding Door Hardware: Rated for door panels weighing up to [50 lb (23 kg); Grade 1] [30 lb (14 kg); Grade 2]; with rails and door hardware that allows horizontal and vertical adjustment.

          - Rail Material: [Galvanized wrought steel] [Extruded aluminum].

          - Rail Configuration: [V-grooved double leg] [V-grooved double leg with fascia] [I-beam].

          - Mounting: [Surface mounted overhead] [Top and bottom hung].

          - Wheel Assembly: Two wheel or four wheel, with roller bearing.

      - METAL PROTECTIVE TRIM UNITS

        - Metal Protective Trim Units: ANSI/BHMA A156.6; fabricated from 0.050-inch- (1.3-mm-) thick [aluminum] [brass] [bronze] [stainless steel]; with manufacturer's standard machine or self-tapping screw fasteners.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

NFPA 80 limits height of protection plates on fire-rated doors to 16 inches (406 mm) above bottom of door unless otherwise specifically labeled.

Armor plates protect lower half of doors subject to cart and truck traffic or rough usage and usually are applied to push side of single-acting doors and both sides of double-acting doors.

        - Armor Plates: [36 inches (914 mm)] [40 inches (1016 mm)] [42 inches (1067 mm)] high by door width with allowance for frame stops.

Kick plates protect bottom of push side of doors subject to foot traffic. Standard heights are 8, 10, and 12 inches (203, 254, and 305 mm).

        - Kick Plates: [8 inches (203 mm)] [10 inches (254 mm)] [12 inches (305 mm)] <Insert dimension> high by door width with allowance for frame stops.

Mop plates protect bottom of pull side of doors subject to cleaning.

        - Mop Plates: [4 inches (102 mm)] [6 inches (152 mm)] high by 1 inch (25 mm) less than door width.

Stretcher plates (which are not defined in ANSI/BHMA A156.6) protect doors subject to regular contact with stretchers, service carts, or other equipment and are applied to the middle part of the door surface on both sides of double-acting doors.

        - Stretcher Plates: [6 inches (152 mm)] [8 inches (203 mm)] high by door width with allowance for frame stops.

        - Nonmortise Angle Door Edging: [48 inches (1220 mm)] [42 inches (1067 mm)] high by minimum 0.050-inch- (1.3-mm-) thick metal sheet formed into angle shape; with 1-1/4-inch (32-mm) length of leg on face of door; for surface mounting on door.

          - Leg Offset: To accommodate door protection plate of type indicated.

        - Mortise Angle Door Edging: [48 inches (1220 mm)] [42 inches (1067 mm)] high by minimum 0.050-inch- (1.3-mm-) thick metal sheet formed into angle shape; with 7/8-inch (22-mm) length of leg on face of door; for mortise application into edge of door.

        - Nonmortise Cap Door Edging: [48 inches (1220 mm)] [42 inches (1067 mm)] high by minimum 0.050-inch- (1.3-mm-) thick metal sheet formed into "U" shape; with 1-1/4-inch (32-mm) length of leg on faces of door; for surface mounting on door.

Retain "Leg Offset" Subparagraph below if required.

          - Leg Offset: To accommodate door protection plate of type indicated.

        - Mortise Cap Door Edging: [48 inches (1220 mm)] [42 inches (1067 mm)] high by minimum 0.050-inch- (1.3-mm-) thick metal sheet formed into "U" shape; with 7/8-inch (22-mm) length of leg on faces of door; for mortise application into edge of door.

      - PLASTIC PROTECTION PLATES

        - Plastic Protection Plates: ANSI/BHMA A156.6; fabricated with four sides beveled; [plastic laminate; 1/8 inch (3.2 mm) thick; ISO 4586, Grade HGS] [rigid plastic; 0.060-inch- (1.5-mm-) thick, PVC or acrylic-modified vinyl plastic] [acrylic; 1/8 inch (3.2 mm) thick].

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

NFPA 80 limits height of protection plates on fire-rated doors to 16 inches (406 mm) above bottom of door unless otherwise specifically labeled.

Armor plates protect lower half of doors subject to cart and truck traffic or rough usage and usually are applied to push side of single-acting doors and both sides of double-acting doors.

        - Armor Plates: [36 inches (914 mm)] [40 inches (1016 mm)] [42 inches (1067 mm)] high by door width with allowance for frame stops.

Kick plates protect bottom of push side of doors subject to foot traffic. Standard heights are 8, 10, and 12 inches (203, 254, and 305 mm).

        - Kick Plates: [8 inches (203 mm)] [10 inches (254 mm)] [12 inches (305 mm)] <Insert dimension> high by door width with allowance for frame stops.

Mop plates protect bottom of pull side of doors subject to cleaning.

        - Mop Plates: [4 inches (102 mm)] [6 inches (152 mm)] high by 1 inch (25 mm) less than door width.

Stretcher plates (which are not defined in ANSI/BHMA A156.6) protect doors subject to regular contact with stretchers, service carts, or other equipment and are applied to the middle part of the door surface on both sides of double-acting doors.

        - Stretcher Plates: [6 inches (152 mm)] [8 inches (203 mm)] high by door width with allowance for frame stops.

        - Colors and Textures: [As selected by Architect from manufacturer's full range] [Match Architect's sample] [As indicated by manufacturer's designations in door hardware schedule].

      - AUXILIARY DOOR HARDWARE

        - Auxiliary Door Hardware: ANSI/BHMA A156.16.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Chain Door Guards: [Grade 1] [Grade 2] [Grade 3]; polished cast brass or bronze or extruded brass; with plate slotted to receive 6-inch- (152-mm-) long welded chain secured to an anchor plate. Guard allows door to be opened 3 inches (75 mm) with chain engaged in slotted plate. Equip with chain holder.

        - Rod-Type Door Guards: [Grade 1] [Grade 2] [Grade 3]; straight door-mounted rod that engages U-shaped, jamb-mounted rod. U-shaped rod can swing 180 degrees away from door; rod limits door opening when engaged.

          - Material: [Brass] [Bronze] [Aluminum].

        - Coat Hooks: [Grade 1] [Grade 2] [Grade 3]; two curved hooks with rounded ends; 3-inch (75-mm) projection from wall; for surface-screw application.

          - Material: [Polished cast brass] [Polished cast bronze] [Burnished cast aluminum].

        - Garment Hooks: [Grade 1] [Grade 2] [Grade 3]; one long hat hook and one small coat hook; 3-3/4-inch (95-mm) projection from wall with 7-inch (178-mm) overall height; for surface-screw application.

          - Material: [Polished cast brass] [Burnished cast aluminum].

        - Door Knockers: Grade 1; solid brass[ with engraved number and nameplates].

If needed, add requirements for access control devices, such as proximity sensors, switch devices, and so forth.

        - Wide-Angle Door Viewers: [Grade 1] [Grade 2] [Grade 3]; solid brass with optical glass lenses; adjustable to door thickness and permitting one-way observation with minimum 190-degree viewing angle.

        - Fire-Rated Door Viewers: Solid brass with optical glass lenses; listed and labeled for use in fire-rated assemblies; adjustable to door thickness, and permitting one-way observation with minimum [120] [150] [190]-degree viewing angle.

        - House Numbers: Grade 1; [wrought, cast, or forged brass] <Insert material>; 4 inches (102 mm) high; for screw application.

Retain "Silencers for Wood Door Frames" Paragraph below if silencers are not specified with wood door frames.

        - Silencers for Wood Door Frames: Grade 1; neoprene or rubber; minimum 5/8 by 3/4 inch (16 by 19 mm); fabricated for drilled-in application to frame.

Retain "Silencers for Metal Door Frames" Paragraph below if silencers are not specified with metal door frames in another Section.

        - Silencers for Metal Door Frames: Grade 1; neoprene or rubber; minimum diameter 1/2 inch (13 mm); fabricated for drilled-in application to frame.

      - AUXILIARY ELECTRIFIED DOOR HARDWARE

        - Auxiliary Electrified Door Hardware: ANSI/BHMA A156.35.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Boxed Power Supplies: Modular unit in NEMA ICS 6, Type 4 enclosure; filtered and regulated; voltage rating and type matching requirements of door hardware served; listed and labeled for use with fire-alarm systems.

        - Monitor Strikes: [Cast strike with toggle] [Dustbox monitor for installation under standard strike].

        - Door Position Switches: Magnetically operated reed switch designed for concealed mounting.

        - Door and Frame Transfer Devices: Steel housing for mortise in hinge stile of door, with flexible tube for wiring bundle; accommodating doors that swing open to 120 degrees.

      - FABRICATION

Delete "Manufacturer's Nameplate" Paragraph below to save cost or if visibility of manufacturer's name on items is not objectionable.

        - Manufacturer's Nameplate: Do not provide products that have manufacturer's name or trade name displayed in a visible location except in conjunction with required fire-rating labels and as otherwise approved by Architect.

          - Manufacturer's identification is permitted on rim of lock cylinders only.

        - Base Metals: Produce door hardware units of base metal indicated, fabricated by forming method indicated, using manufacturer's standard metal alloy, composition, temper, and hardness. Furnish metals of a quality equal to or greater than that of specified door hardware units and ANSI/BHMA A156.18.

        - Fasteners: Provide door hardware manufactured to comply with published templates prepared for machine, wood, and sheet metal screws. Provide screws that comply with commercially recognized industry standards for application intended; however, aluminum fasteners are not permitted. Provide Phillips flat-head screws with finished heads to match surface of door hardware unless otherwise indicated.

Retain "Concealed Fasteners" Subparagraph below where visual quality is important. Coordinate with Section 081416 "Flush Wood Doors."

          - Concealed Fasteners: For door hardware units that are exposed when door is closed, except for units already specified with concealed fasteners. Do not use through bolts for installation where bolt head or nut on opposite face is exposed unless it is the only means of securely attaching the door hardware. Where through bolts are used on hollow door and frame construction, provide sleeves for each through bolt.

NFPA 80 requires locks, latches, surface-mounted top and bottom bolts, and fire-exit hardware to be secured with machine screws or through bolts.

          - Fire-Rated Applications:

            - Wood or Machine Screws: For the following:

              - Hinges mortised to doors or frames[; use threaded-to-the-head wood screws for wood doors and frames].

              - Strike plates to frames.

              - Closers to doors and frames.

            - Steel Through Bolts: For the following unless door blocking is provided:

              - Surface hinges to doors.

              - Closers to doors and frames.

              - Surface-mounted exit devices.

          - Spacers or Sex Bolts: For through bolting of hollow-metal doors.

          - Gasketing Fasteners: Provide noncorrosive fasteners for exterior applications and elsewhere as indicated.

      - FINISHES

        - Provide finishes complying with ANSI/BHMA A156.18 as indicated in door hardware schedule.

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Appearance of Finished Work: Variations in appearance of abutting or adjacent pieces are acceptable if they are within one-half of the range of approved Samples. Noticeable variations in the same piece are not acceptable. Variations in appearance of other components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

- EXECUTION

      - EXAMINATION

        - Examine doors and frames, with Installer present, for compliance with requirements for installation tolerances, labeled fire-rated door assembly construction, wall and floor construction, and other conditions affecting performance of the Work.

Retain first paragraph below for electrified hardware.

        - Examine roughing-in for electrical power systems to verify actual locations of wiring connections before electrified door hardware installation.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

Retain this article for on-site door preparation of hardware if any. NFPA 80 restricts on-site door preparation to that required for surface-applied door hardware, function holes for mortise locks, holes for labeled viewers, undercutting of doors, and protection plates. Hardware for aluminum doors and frames generally is supplied to assembly manufacturer for factory preparation of door and frames for hardware.

        - Steel Doors and Frames: For surface-applied door hardware, drill and tap doors and frames in accordance with ANSI/SDI A250.6.

        - Wood Doors: Comply with door and hardware manufacturers' written instructions.

      - INSTALLATION

        - Mounting Heights: Mount door hardware units at heights [indicated on Drawings] [to comply with the following] unless otherwise indicated or required to comply with governing regulations.

If retaining second option in "Mounting Heights" Paragraph above, retain subparagraphs below and revise to suit Project.

          - Standard Steel Doors and Frames: ANSI/SDI A250.8.

          - Custom Steel Doors and Frames: HMMA 831.

          - Wood Doors: DHI's "Recommended Locations for Architectural Hardware for Wood Flush Doors."

        - Install each door hardware item to comply with manufacturer's written instructions. Where cutting and fitting are required to install door hardware onto or into surfaces that are later to be painted or finished in another way, coordinate removal, storage, and reinstallation of surface protective trim units with finishing work. Do not install surface-mounted items until finishes have been completed on substrates involved.

          - Set units level, plumb, and true to line and location. Adjust and reinforce attachment substrates as necessary for proper installation and operation.

          - Drill and countersink units that are not factory prepared for anchorage fasteners. Space fasteners and anchors in accordance with industry standards.

        - Hinges: Install types and in quantities indicated in door hardware schedule, but not fewer than the number recommended by manufacturer for application indicated or one hinge for every 30 inches (760 mm) of door height, whichever is more stringent, unless other equivalent means of support for door, such as spring hinges or pivots, are provided.

        - Intermediate Offset Pivots: Where offset pivots are indicated, provide intermediate offset pivots in quantities indicated in door hardware schedule, but not fewer than one intermediate offset pivot per door and one additional intermediate offset pivot for every 30 inches (760 mm) of door height greater than 90 inches (2286 mm).

        - Lock Cylinders: Install construction cores to secure building and areas during construction period.

Retain one of two subparagraphs below.

          - Replace construction cores with permanent cores as [indicated in keying schedule] [directed by Owner].

          - Furnish permanent cores to Owner for installation.

        - Key Control System:

          - Key Control Cabinet: Tag keys and place them on markers and hooks in key control system cabinet, as determined by final keying schedule.

          - Key Lock Boxes: Install where indicated or approved by Architect to provide controlled access for fire and medical emergency personnel.

          - Key Control System Software: Set up multiple-index system based on final keying schedule.

Retain "Boxed Power Supplies" Paragraph below if electrified door hardware is required. Verify, with authorities having jurisdiction, acceptable location for power supplies.

        - Boxed Power Supplies: Locate power supplies as indicated or, if not indicated, [above accessible ceilings] [in equipment room]. Verify location with Architect.

          - Configuration: Provide [one power supply for each door opening] [least number of power supplies required to adequately serve doors] with electrified door hardware.

If thresholds on doors other than exterior doors are set in full bed of sealant, indicate locations in door hardware schedule or on Drawings or revise "Thresholds" Paragraph below.

        - Thresholds: Set thresholds for exterior doors and other doors indicated in full bed of sealant complying with requirements specified in Section 079200 "Joint Sealants."

Retain "Stops" Paragraph below for default specification of floor stops at every door. Where floor or wall stops are inappropriate, use overhead holders.

        - Stops: Provide floor stops for doors unless wall or other type stops are indicated in door hardware schedule. Do not mount floor stops where they will impede traffic.

        - Perimeter Gasketing: Apply to head and jamb, forming seal between door and frame.

          - Do not notch perimeter gasketing to install other surface-applied hardware.

        - Meeting Stile Gasketing: Fasten to meeting stiles, forming seal when doors are closed.

        - Door Bottoms: Apply to bottom of door, forming seal with threshold when door is closed.

      - FIELD QUALITY CONTROL

Retain first option in "Independent Architectural Hardware Consultant" Paragraph below if Owner will engage an independent Architectural Hardware Consultant to perform field inspections. Retain second option if Contractor must engage consultant for field quality control.

        - Independent Architectural Hardware Consultant: [Owner will engage] [Engage] a qualified independent Architectural Hardware Consultant to perform inspections and to prepare inspection reports.

          - Independent Architectural Hardware Consultant will inspect door hardware and state in each report whether installed work complies with or deviates from requirements, including whether door hardware is properly installed and adjusted.

      - ADJUSTING

        - Initial Adjustment: Adjust and check each operating item of door hardware and each door to ensure proper operation or function of every unit. Replace units that cannot be adjusted to operate as intended. Adjust door control devices to compensate for final operation of heating and ventilating equipment and to comply with referenced accessibility requirements.

          - Door Closers: Adjust sweep period to comply with accessibility requirements and requirements of authorities having jurisdiction.

          - Spring Hinges: Adjust to achieve positive latching when door is allowed to close freely from an open position of 70 degrees and so that closing time complies with accessibility requirements of authorities having jurisdiction.

          - Electric Strikes: Adjust horizontal and vertical alignment of keeper to properly engage lock bolt.

Requirements in "Occupancy Adjustment" Paragraph below might be difficult to monitor and will increase cost; however, owners of substantial projects might determine the cost is justified.

        - Occupancy Adjustment: Approximately [three] [six] <Insert number> months after date of Substantial Completion, Installer's Architectural Hardware Consultant is to examine and readjust each item of door hardware, including adjusting operating forces, as necessary to ensure function of doors, door hardware, and electrified door hardware.

      - CLEANING AND PROTECTION

        - Clean adjacent surfaces soiled by door hardware installation.

        - Clean operating items as necessary to restore proper function and finish.

        - Provide final protection and maintain conditions that ensure that door hardware is without damage or deterioration at time of Substantial Completion.

      - MAINTENANCE SERVICE

        - Maintenance Tools and Instructions: Furnish a complete set of specialized tools and maintenance instructions for Owner's continued adjustment, maintenance, and removal and replacement of door hardware.

Verify with Owner that maintenance service is required for Project.

        - Maintenance Service: Beginning at Substantial Completion, maintenance service is to include [six] [nine] [12] <Insert number> months' full maintenance by skilled employees of door hardware Installer. Include quarterly preventive maintenance, repair or replacement of worn or defective components, lubrication, cleaning, and adjusting as required for proper door and door hardware operation. Parts and supplies are to be manufacturer's authorized replacement parts and supplies.

      - DEMONSTRATION

Retain this article for projects with complex or atypical door hardware.

        - [Engage Installer to train] [Train] Owner's maintenance personnel to adjust, operate, and maintain door hardware.

      - DOOR HARDWARE SCHEDULE

Retain this article if inserting schedule below. Otherwise, delete this article and indicate door hardware schedule on Drawings or in another Division 08 Section.

Schedule a door hardware set for each unique door hardware configuration required for Project. Identify each set with a unique number designation. See "Sample Door Hardware Schedules" Article in the Evaluations for additional information.

        - Hardware Set 1: Each door to have the following:

          - <Insert hardware type>.

END OF SECTION 087111