Copyright 2014 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 098433 - SOUND-ABSORBING WALL UNITS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Access Manufacturer-Provided, AIA MasterSpec-Based Sections:

<Double click here for this Section based on specific manufacturer’s products set as Basis-of-Design at ProductMasterSpec.com.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes shop-fabricated, acoustical panel units tested for acoustical performance, including the following:

          - Sound-absorbing wall panels.

          - Sound-diffusing wall panels.

          - Sound-reflecting wall panels.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 097200 "Wall Coverings" for adhesively applied textile wall coverings[ and for coordinated requirements for fabric].

          - Section 097713 "Stretched-Fabric Wall Systems" for site-upholstered systems applied to walls[ and for coordinated requirements for fabric].

          - Section 097723 "Fabric-Wrapped Panels" for decorative, fabric-wrapped wall panels that are not required to be tested for acoustical performance[ and for coordinated requirements for fabric].

      - DEFINITIONS

Retain definition(s) remaining after this Section has been edited for a project.

        - NRC: Noise Reduction Coefficient.

        - SAA: Sound Absorption Average.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

Generally, retain option in subparagraph below for fabric facing furnished by manufacturer.

          - Include[ fabric facing,] panel edge, core material, and mounting indicated.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

          - <Double click to insert sustainable design text for regional materials.>

          - <Double click to insert sustainable design text for certified wood.>

          - <Double click to insert sustainable design text for certified wood.>

          - <Double click to insert sustainable design text for adhesives.>

          - <Double click to insert sustainable design text for composite wood.>

          - <Double click to insert sustainable design text for walls.>

        - Shop Drawings: For unit assembly and installation.

          - Include plans, elevations, sections, and mounting devices and details.

          - Include details at panel head, base, joints, and corners; and details at ceiling, floor base, and wall intersections. Indicate panel edge profile and core materials.

          - Include details at cutouts and penetrations for other work.

          - Include direction of fabric weave and pattern matching.

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection: For each type of fabric facing.

          - Include Samples of hardware and accessories involving color or finish selection.

        - Samples for Verification: For the following products:

Delete "Fabric" Subparagraph below if fabric is Owner furnished or preselected and specified; revise to suit Project.

          - Fabric: Full-width by approximately [36-inch- (900-mm-)] <Insert dimension> long Sample, but not smaller than required to show complete pattern repeat, from dye lot to be used for the Work, and with specified treatments applied. Mark top and face of fabric.

          - Panel Edge: 12-inch- (300-mm-) long Sample(s) showing each edge profile, corner, and finish.

          - Core Material: 12-inch- (300-mm-) square Sample at corner.

          - Mounting Devices: Full-size Samples.

          - Assembled Panels: Approximately 36 by 36 inches (900 by 900 mm), including joints and mounting methods.

      - INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below if coordination is required for installation of products and materials by separate installers. Coordinate paragraph with other Sections specifying products listed below. Preparation of coordination drawings requires the participation of each trade involved in installations within the limited space.

        - Coordination Drawings: Elevations and other details, drawn to scale, on which the following items are shown and coordinated with each other, using input from installers of the items involved:

          - Electrical outlets, switches, and thermostats.

          - Items penetrating or covered by units including the following:

            - Lighting fixtures.

            - Air outlets and inlets.

            - Speakers.

            - Alarms.

            - Sprinklers.

            - Access panels.

            - <Insert item>.

          - Show operation of hinged and sliding components covered by or adjacent to units.

Retain "Product Certificates" Paragraph below to require submittal of product certificates from manufacturers.

        - Product Certificates: For each type of unit.

        - Sample Warranty: For manufacturer's special warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For each type of unit to include in maintenance manuals. Include fabric manufacturers' written cleaning and stain-removal instructions.

      - MAINTENANCE MATERIAL SUBMITTALS

        - Furnish extra materials from same production run that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Fabric: For each fabric, color, and pattern installed, provide length equal to [10] <Insert number> percent of amount installed, but no fewer than [10 sq. yd. (9 sq. m)] <Insert quantity>, full width of bolt.

          - Mounting Devices: Full-size units equal to [5] <Insert number> percent of amount installed, but no fewer than [five] <Insert number> devices, including unopened adhesives.

      - QUALITY ASSURANCE

        - <Double click to insert sustainable design text for manufacturer qualifications.>

        - <Double click to insert sustainable design text for vendor qualifications.>

        - Mockups: Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for materials, fabrication, and installation.

Indicate portion of wall represented by mockup on Drawings or draw mockup as separate element.

          - Build mockup of typical wall area [as shown on Drawings] [48 inches (1200 mm) wide by full height] <Insert dimensions>.[ Include intersection of wall and ceiling, corners, and perimeters.]

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - DELIVERY, STORAGE, AND HANDLING

        - Comply with fabric and unit manufacturers' written instructions for minimum and maximum temperature and humidity requirements for shipment, storage, and handling.

        - Deliver materials and units in unopened bundles and store in a temperature-controlled dry place with adequate air circulation.

      - FIELD CONDITIONS

        - Environmental Limitations: Do not install units until spaces are enclosed and weathertight, wet-work in spaces is complete and dry, work at and above ceilings is complete, and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

Retain "Lighting" Paragraph below if unit appearance or installation is affected by lighting conditions. This may occur when aligning units over an extensive surface. Revise to suit Project.

        - Lighting: Do not install units until [a permanent level of lighting] [a lighting level of not less than 50 fc (538 lx)] <Insert requirement> is provided on surfaces to receive the units.

Retain "Air-Quality Limitations" Paragraph below to suit Project. See the Evaluations.

        - Air-Quality Limitations: Protect units from exposure to airborne odors, such as tobacco smoke, and install units under conditions free from odor contamination of ambient air.

Retain "Field Measurements" Paragraph below if units are tightly fit to entire wall areas.

        - Field Measurements: Verify unit locations and actual dimensions of openings and penetrations by field measurements before fabrication, and indicate them on Shop Drawings.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty: Manufacturer agrees to repair or replace units and components that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to the following:

            - Acoustical performance.

            - Fabric sagging, distorting, or releasing from panel edge.

            - Warping of core.

Verify available warranties and warranty periods for units and components.

          - Warranty Period: [Two] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in SpecAgent and Masterworks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

Retain option in "Source Limitations" Paragraph below if applicable.

        - Source Limitations: Obtain wall units specified in this Section[ and ceiling units specified in Section 098436 "Sound-Absorbing Ceiling Units] from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

        - <Double click to insert sustainable design text for walls.>

Revise "Fire-Test-Response Characteristics" Paragraph below as required by governing code for unit location, occupancy classification, and whether sprinklered or not. Verify and insert requirements of authorities having jurisdiction; verify whether testing products as whole finish-assemblies complete with fabric is required for Project. Below is based on the 2012 IBC; see the Evaluations.

        - Fire-Test-Response Characteristics: Units shall comply with "Surface-Burning Characteristics" or "Fire Growth Contribution" Subparagraph below, or both, as determined by testing identical products by UL or another testing and inspecting agency acceptable to authorities having jurisdiction:

          - Surface-Burning Characteristics: Comply with ASTM E84 or UL 723; testing by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

Value in "Flame-Spread Index" Subparagraph below is required for textile and expanded vinyl wall and ceiling coverings. Option in "Smoke-Developed Index" Subparagraph below is the maximum allowed.

            - Flame-Spread Index: 25 or less.

            - Smoke-Developed Index: [450] <Insert value> or less.

          - Fire Growth Contribution: Comply with acceptance criteria of local code and authorities having jurisdiction when tested according to NFPA 265 Method B Protocol or NFPA 286.

      - SOUND-ABSORBING WALL UNITS

Copy paragraph below and re-edit for each product. It is intended as a guide if Project requires panels of varying sizes or characteristics.

Insert drawing designation. Use these designations on Drawings to identify each product.

Panel construction, details, mounting, and acoustical performance vary with manufacturer and type of panel; coordinate acoustical performance with Project's acoustical designer.

        - Sound-Absorbing Wall Panel <Insert drawing designation>: Manufacturer's standard panel construction consisting of facing material [laminated to front face, edges, and back edge border of core] [stretched over front face of edge-framed core and bonded or attached to edges and back of frame] <Insert description>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain one or more of 13 subparagraphs below to suit Project; available characteristics, options, and features vary with manufacturer and product.

          - Panel Shape: [Flat] [As indicated on Drawings] <Insert shape>.

Retain one of two "Mounting" subparagraphs below; revise to suit Project.

          - Mounting: Edge mounted with splines secured to substrate.

            - Finish Color at Exposed Edges: [White] [Black] [Match color of facing material] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert requirement>.

Retain one or more options in "Mounting" Subparagraph below to suit Project, depending on weight of panel, substrate, installation appearance and tolerances, ease of replacement, and security considerations. If retaining more than one, indicate locations of each on Drawings or by inserts.

          - Mounting: Back mounted with manufacturer's standard [adhesive] [adhesive tape strips] [hook-and-loop strips] [impaling clips] [magnetic devices] [metal clips or bar hangers], secured to substrate.

If retaining specific core materials in "Core" Subparagraph below, consult unit manufacturers for recommendations on material composition, weight, and acoustical properties. If retaining last option below, indicate size and spacing of nailing strips on Drawings. Although they reduce sound absorption, wood or plywood nailing strips within the core may be available from some manufacturers.

          - Core: [Manufacturer's standard] [Glass-fiber board] [Mineral-fiber board] [Cementitious-fiber board] <Insert requirement>.[ Provide wood or plywood nailing strips in core where indicated.]

Retain "Core-Face Layer" Subparagraph below if different face layer is required for the core material to make the units impact resistant, tackable, or both.

            - Core-Face Layer: Manufacturer's standard [tackable, impact-resistant, high-density board] [impact-resistant, acoustically transparent, copolymer sheet] <Insert requirement>.

First option in "Edge Construction" Subparagraph below is most common and is used with glass- and mineral-fiber board cores. Last three options are sometimes used for panels with facing material stretched over front face of edge-framed core.

          - Edge Construction: Manufacturer's standard [chemically hardened core with no frame] [extruded-aluminum or zinc-coated, rolled-steel frame] [extruded PVC frame] [wood frame, rabbeted, and splined with glued joints and machined corners] <Insert requirement>.

Retain first option in "Edge Profile" Subparagraph below if edge mounting with splines is required; edges of splined panels are generally square. Revise subparagraph if long and short edges are required to have different profiles.

          - Edge Profile: [Long edges kerfed and rabbeted to receive splines] [Chamfered (beveled)] [Eased (small radius)] [Mitered (beveled to a point)] [Radiused (bullnosed)] [Square] [Custom profile as indicated on Drawings] <Insert profile>.

          - Corner Detail in Elevation: [Square] [Round, radius as indicated on Drawings] [Custom as indicated on Drawings] <Insert description> with continuous edge profile indicated.

          - Reveals between Panels: [Recessed] [Flush] [Projecting] reveals [as selected by Architect from manufacturer's full range] [as indicated on Drawings] <Insert requirement>.

Generally, indicate facing material on Drawings or insert, in "Facing Material" Subparagraph below, drawing designation of facing material specified in "Materials" Article.

          - Facing Material: [Owner-furnished material] [As indicated on Drawings] <Insert drawing designation>.

Coordinate options retained in "Acoustical Performance" Subparagraph below with core material and density. Options assume that facing material does not affect typical acoustical performance of core. Verify, with manufacturer, that NRC or SAA range retained is possible and practical for products. Manufacturers generally report NRC values only. Replace range with a single minimum value if required; a single value may be inadequate for Project's acoustical design. Revise mounting type to suit Project if required. See the Evaluations.

          - Acoustical Performance: Sound absorption [NRC] [or] [SAA] of [0.50 to 0.90] [0.60 to 0.70] [0.65 to 0.75] [not less than 0.65] <Insert range or single value> according to ASTM C423 for [Type A] <Insert type> mounting according to ASTM E795.

          - Nominal [Core] [Overall Panel] Thickness: [3/4 inch (19 mm)] [1 inch (25 mm)] [1-1/2 inches (38 mm)] [2 inches (51 mm)] [As indicated on Drawings] <Insert dimension>.

          - Panel Width: [24 inches (610 mm)] [30 inches (762 mm)] [48 inches (1220 mm)] [As indicated on Drawings] <Insert dimension>.

          - Panel Height: [72 inches (1829 mm)] [96 inches (2438 mm)] [108 inches (2743 mm)] [120 inches (3048 mm)] [As indicated on Drawings] <Insert dimension>.

      - SOUND-DIFFUSING AND -REFLECTING WALL UNITS

Copy paragraph below and re-edit for each product. It is intended as a guide if Project requires panels of varying sizes or characteristics.

Insert drawing designation. Use these designations on Drawings to identify each product.

Panel construction, details, mounting, and acoustical performance vary with manufacturer and type of panel; coordinate acoustical performance with Project's acoustical designer.

Generally, retain option in paragraph below unless panel is made of formed plastic without facing material.

        - Sound-[Diffusing] [Reflecting] Wall Panel <Insert drawing designation>: Manufacturer's standard panel construction[ consisting of facing material laminated to front face, edges, and back edge border of core] <Insert description>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain one or more of nine subparagraphs below to suit Project; available characteristics, options, and features vary with manufacturer and product.

Shapes in "Panel Shape" Subparagraph below diffuse or reflect sound striking the surface. Sound-diffusing panels are generally faceted, unlike sound-absorbing panels, or set at angles to each other. Sound-reflecting panels are generally flat or radially curved.

          - Panel Shape: [Barrel] [Pyramidal] [Flat] [Radially curved flat panel] [As indicated on Drawings] <Insert shape>.

Retain one or more options in "Mounting" Subparagraph below to suit Project, depending on weight of panel, substrate, installation appearance and tolerances, ease of replacement, and security considerations. If retaining more than one, indicate locations of each on Drawings or by inserts. Revise subparagraph for custom spline mounting if required.

          - Mounting: Back mounted with manufacturer's standard [adhesive] [adhesive tape strips] [hook-and-loop strips] [impaling clips] [magnetic devices] [metal clips or bar hangers], secured to substrate.

If retaining specific core materials in "Core" Subparagraph below, consult unit manufacturers for recommendations on material composition, weight, and acoustical properties. Fire-retardant formed plastic option is common for barrel- and pyramidal-shape, sound-diffusing panels.

          - Core: [Manufacturer's standard] [Glass-fiber board with a reflective component] [Mineral-fiber board with a reflective component] [Cementitious-fiber board with a reflective component] [Fire-retardant formed plastic] [Medium-density fiberboard] [Particleboard] <Insert requirement>, prepared for required acoustical performance.

Second option in "Edge Construction" Subparagraph below is most common and is used with glass- and mineral-fiber board cores. Last three options are sometimes used for panels with facing material stretched over front face of edge-framed core.

          - Edge Construction: [Manufacturer's standard] [Chemically hardened core with no frame] [Extruded-aluminum or zinc-coated, rolled-steel frame] [Extruded PVC frame] [Wood frame, rabbeted, and splined with glued joints and machined corners] <Insert requirement>.

Insert subparagraphs here if custom edge profile and corner detail in elevation other than square is required. Manufacturers generally do not offer options for these features on sound-diffusing or -reflecting panels.

          - Reveals between Panels: [Recessed] [Flush] [Projecting] reveals [as selected by Architect from manufacturer's full range] [as indicated on Drawings] <Insert requirement>.

Generally, indicate facing material on Drawings or insert, in "Facing Material" Subparagraph below, drawing designation of facing material specified in "Materials" Article. Revise subparagraph if an acoustically reflective coating is required on the fabric.

          - Facing Material: [Owner-furnished material] [As indicated on Drawings] <Insert drawing designation>.

Coordinate options retained in "Acoustical Performance" Subparagraph below with core material and density. Options assume that facing material does not affect typical acoustical performance of core. Verify, with manufacturer, that NRC or SAA range retained is possible and practical for products. Manufacturers generally report NRC values only. Replace range with a single maximum value if required; a single value may be inadequate for Project's acoustical design. Revise mounting type to suit Project if required. See the Evaluations.

          - Acoustical Performance: Sound absorption [NRC] [or] [SAA] of [0.05 to 0.10] [0.15 to 0.25] [0.30 to 0.40] [not more than 0.35] <Insert range or single value> according to ASTM C423 for [Type A] <Insert type> mounting according to ASTM E795.

          - Panel Width: [24 inches (610 mm)] [30 inches (762 mm)] [48 inches (1220 mm)] [As indicated on Drawings] <Insert dimension>.

          - Panel Height: [72 inches (1829 mm)] [96 inches (2438 mm)] [108 inches (2743 mm)] [120 inches (3048 mm)] [As indicated on Drawings] <Insert dimension>.

      - MATERIALS

        - <Double click to insert sustainable design text for composite wood products.>

        - <Double click to insert sustainable design text for recycled content.>

        - <Double click to insert sustainable design text for regional materials.>

        - <Double click to insert sustainable design text for certified wood.>

Core materials vary with manufacturer and required acoustical performance. If retaining "Manufacturer's standard" option in "Core Materials" Paragraph below, usually delete nine subparagraphs specifying core materials. Coordinate with "Sound-Absorbing Wall Units" and "Sound-Diffusing and -Reflecting Wall Units" articles.

        - Core Materials:[ Manufacturer's standard.]

          - Glass-Fiber Board: ASTM C612; of type standard with manufacturer; nominal density of [6 to 7 lb/cu. ft. (96 to 112 kg/cu. m)] <Insert value>, unfaced, and dimensionally stable, molded rigid board; and with maximum flame-spread and smoke-developed indexes of 25 and 50, respectively.

          - Mineral-Fiber Board: Maximum flame-spread and smoke-developed indexes of 25 and 10, respectively; minimum density of [13 lb/cu. ft. (208 kg/cu. m)] [20 lb/cu. ft. (320 kg/cu. m)], and with perforated surface.

          - Cementitious-Fiber Board: Density of not less than [20 lb/cu. ft. (320 kg/cu. m)] <Insert value>.

Materials in "Fire-Retardant Formed Plastic," "Medium-Density Fiberboard," and "Particleboard" subparagraphs below are often used for sound-diffusing or -reflecting panels but not for sound-absorbing panels. Sound-diffusing or -reflecting cores vary with manufacturer and required acoustical performance; they can be made of sound-absorbing, wall-panel core materials with a reflective component or of an entirely different material. Consult manufacturers for material, material weight, and acoustical properties.

          - Fire-Retardant Formed Plastic: Manufacturer's standard formed plastic with flame-spread index of 25 or less and smoke-developed index of 25 or less according to ASTM E84 or UL 723.

          - Medium-Density Fiberboard: Panels complying with ANSI A208.2, [grade to suit performance requirements] [Grade 130] <Insert grade>.

            - Fire-retardant panels made from softwood fibers, synthetic resins, and fire-retardant chemicals mixed together at time of panel manufacture to achieve flame-spread index of 25 or less and smoke-developed index of 200 or less per ASTM E84 or UL 723.

          - Particleboard: Panels complying with ANSI A208.1, [grade to suit performance requirements] [Grade M-2] <Insert grade>.

            - Fire-retardant panels made from softwood particles and fire-retardant chemicals mixed together at time of panel manufacture to achieve flame-spread index of 25 or less and smoke-developed index of 25 or less per ASTM E84 or UL 723.

Retain "Tackable, Impact-Resistant, High-Density Board for Face Layer" Subparagraph below for tackable face layer over glass-fiber board core. This layer provides tackability and some impact resistance and only minimally decreases sound-absorption performance of glass-fiber board core.

          - Tackable, Impact-Resistant, High-Density Board for Face Layer: 1/8-inch- (3.2-mm-) thick layer of compressed molded glass-fiber board with a nominal density of 16 to 18 lb/cu. ft. (256 to 288 kg/cu. m) laminated to face of core.

Retain "Impact-Resistant, Acoustically Transparent, Copolymer Sheet for Face Layer" Subparagraph below for high-abuse locations.

          - Impact-Resistant, Acoustically Transparent, Copolymer Sheet for Face Layer: 1/16- to 1/8-inch- (1.6- to 3.2-mm-) thick layer of perforated, noncombustible, copolymer sheet laminated to face of core.

Retain "Wood and Plywood" Subparagraph below for wood or plywood nailing strips or wood frame and edges; revise to suit Project. Requirements for fire-retardant treatment are based on the 2012 IBC.

          - Wood and Plywood: Manufacturer's standard plywood or clear, vertical grain, straight, kiln-dried hardwood.

            - Fire-retardant treated by pressure process with a flame-spread index of 25 or less when tested according to ASTM E84 or UL 723, and with no evidence of significant progressive combustion when the test is extended an additional 20 minutes, and with the flame front not extending more than 10.5 feet (3.2 m) beyond the centerline of the burners at any time during the test.

              - Treated material shall have a moisture content of 28 percent or less when tested according to ASTM D3201/D3201M at 92 percent relative humidity.

              - Kiln-dry material after treatment to 19 percent or less for lumber and 15 percent or less for plywood.

Copy first paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each facing material.

Owner-furnished facing materials must be acceptable to unit manufacturer; see the Evaluations. For highest acoustical absorption, a lining fabric or core-overlay batting is also generally not recommended by manufacturers.

        - Facing Material <Insert drawing designation>: Fabric from same dye lot; color and pattern [as indicated by manufacturer's designations] [matching Architect's samples] [as selected by Architect from manufacturer's full range] [as indicated on Drawings] <Insert requirement>.

Revise or delete subparagraphs below to coordinate with option retained in last paragraph above.

          - Manufacturer: <Insert manufacturer's name>.

          - Product Line/Pattern: <Insert product name or designation>.

          - Pattern Repeat: <Insert requirement>.

          - Style Number: <Insert number>.

          - Color: <Insert name or number, or both>.

          - Fiber Content: [100] <Insert number> percent [woven polyester] [nonwoven polyester] [polyolefin] [acoustically transparent vinyl] <Insert fiber requirement>.

          - Width: [54 inches (1371 mm)] [66 inches (1676 mm)] <Insert dimension>.

          - Source: <Insert fabric-vendor's name>.

Retain treatments in "Applied Treatments" Subparagraph below to suit Project. Treatment may affect appearance and acoustical performance and may not be offered or recommended by panel manufacturer.

          - Applied Treatments: [Stain resistance] [and] [flame retardant] <Insert treatment>.

        - Mounting Devices: Concealed on back of unit, recommended by manufacturer to support weight of unit, and as follows:

Retain one or more of eight subparagraphs below to suit Project. Combination attachment systems are also available from manufacturers; insert if required.

          - Splines: Manufacturer's standard concealed metal or plastic splines that engage the kerfed edges of the unit, with other moldings and trim for interior corners, exterior corners, and exposed edges, with factory-applied finish on exposed items.

          - <Double click to insert sustainable design text for VOC content of adhesive.>

          - <Double click to insert sustainable design text for adhesives.>

          - Adhesive Tape Strips: [Manufacturer's standard 1/16-inch- (1.6-mm-) thick, double-sided foam tape] <Insert requirement>.

          - Hook-and-Loop Strips: [Manufacturer's standard] <Insert requirement>.

          - Impaling Clips: [Manufacturer's standard] <Insert requirement>.

          - Magnetic Strip or Devices: [Manufacturer's standard] <Insert requirement>.

          - Metal Clips or Bar Hangers: Manufacturer's standard two-part metal "Z" clips, with one part of each clip mechanically attached to back of unit and the other part to substrate, designed to permit unit removal.

      - FABRICATION

        - Standard Construction: Use manufacturer's standard construction unless otherwise indicated; with facing material applied to face, edges, and back border of dimensionally stable core; and with rigid edges to reinforce panel perimeter against warpage and damage.

        - Edge Hardening: For [glass-fiber board] [and] [mineral-fiber board] cores, chemically harden core edges and areas of core where mounting devices are attached.

Retain "Core-Face Layer" Paragraph below if tackable or abuse-resistant face layer is applied over core.

        - Core-Face Layer: Evenly stretched over core face and edges and securely attached to core; free from puckers, ripples, wrinkles, or sags.

        - Facing Material: Apply fabric facing fully covering visible surfaces of unit; with material stretched straight, on the grain, tight, square, and free from puckers, ripples, wrinkles, sags, blisters, seams, adhesive, or other visible distortions or foreign matter.

Retain "Square Corners," "Radius and Other Nonsquare Corners," and "Fabrics with Directional or Repeating Patterns or Directional Weave" subparagraphs below to suit Project; insert requirements for custom corners if required. Indicate location of each corner condition on Drawings or by inserts in "Sound-Absorbing Wall Units" and "Sound-Diffusing and -Reflecting Wall Units" articles.

          - Square Corners: Tailor corners.[ Heat-seal vinyl fabric seams at corners.]

          - Radius and Other Nonsquare Corners: Attach facing material so there are no seams or gathering of material.

          - Fabrics with Directional or Repeating Patterns or Directional Weave: Mark fabric top and attach fabric in same direction so pattern or weave matches in adjacent units.

Dimension in "Dimensional Tolerances of Finished Units" Paragraph below is CISCA and industry consensus; however, some manufacturers fabricate to plus or minus 1/32 inch (0.79 mm).

        - Dimensional Tolerances of Finished Units: Plus or minus 1/16 inch (1.6 mm) for the following:

          - Thickness.

          - Edge straightness.

          - Overall length and width.

          - Squareness from corner to corner.

          - Chords, radii, and diameters.

- EXECUTION

      - EXAMINATION

        - Examine fabric, fabricated units, substrates, areas, and conditions for compliance with requirements, installation tolerances, and other conditions affecting unit performance.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

Revise first paragraph below to suit Project if installation is not squarely aligned to walls.

        - Install units in locations indicated. Unless otherwise indicated, install units with vertical surfaces and edges plumb, top edges level and in alignment with other units, faces flush, and scribed to fit adjoining work accurately at borders and at penetrations.

        - Comply with manufacturer's written instructions for installation of units using type of mounting devices indicated. Mount units securely to supporting substrate.

        - Align fabric pattern and grain [with adjacent units] [as indicated on Drawings].

      - INSTALLATION TOLERANCES

Revise this article to suit Project and tolerances recommended by unit manufacturers. Values given below are examples only and are based on tolerances of finished units. Consult unit manufacturers for recommendations.

        - Variation from Plumb and Level: Plus or minus [1/16 inch (1.6 mm)] <Insert dimension> in 48 inches (1200 mm), noncumulative.

        - Variation of Joint Width: Not more than [1/16-inch (1.6-mm)] [1/32-inch (0.79-mm)] <Insert dimension> variation from [hairline] [reveal line] in 48 inches (1200 mm), noncumulative.

      - CLEANING

        - Clip loose threads; remove pills and extraneous materials.

        - Clean panels on completion of installation to remove dust and other foreign materials according to manufacturer's written instructions.

END OF SECTION 098433