Copyright 2017 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 101146 - VISUAL DISPLAY FABRICS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes dry-erase wall coverings.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 097200 "Wall Coverings" for standard vinyl wall coverings.

          - Section 101100 "Visual Display Units" for manufactured visual display units, including markerboards.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data: For each type of dry-erase wall covering. Include data on physical characteristics, durability, and flame-resistance characteristics.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for adhesives.>

        - Shop Drawings: Show location and extent. Indicate seams and termination points.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples: For each type of dry-erase wall covering.

        - Samples for Initial Selection: For each type of dry-erase wall covering.

        - Samples for Verification: Full width by [36- (914-)] <Insert dimension> inch- (mm-) long section of dry-erase wall covering.

        - Product Schedule: For dry-erase wall coverings.[ Use same designations indicated on Drawings.]

      - INFORMATIONAL SUBMITTALS

        - Product Test Reports: For surface-burning characteristics of each type of dry-erase wall covering, for tests performed by [manufacturer and witnessed by a qualified testing agency] [a qualified testing agency].

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For dry-erase wall coverings to include in maintenance manuals.

      - QUALITY ASSURANCE

        - Mockups: Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for installation.

Retain first subparagraph below for large-scale mockup. Mockups may be especially prudent for floor-to-ceiling installations and for projects with multiple installations of dry-erase wall coverings. Indicate portion of wall represented by mockup on Drawings or draw mockup as separate element.

          - Build mockup of typical dry-erase wall covering as shown on Drawings.

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - FIELD CONDITIONS

        - Environmental Limitations: Do not deliver or install dry-erase wall coverings until spaces are enclosed and weathertight, wet-work in spaces is complete and dry, work above ceilings is complete, and temporary HVAC system is operating and maintaining ambient temperature and humidity conditions at occupancy levels during the remainder of the construction period.

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - PERFORMANCE REQUIREMENTS

Retain "Surface-Burning Characteristics" Paragraph below if dry-erase wall coverings are flammable and testing is required by authorities having jurisdiction. Confirm availability of products complying with surface-burning with manufacturers.

        - Surface-Burning Characteristics: Comply with ASTM E84; testing by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

          - Flame-Spread Index: [25] <Insert value> or less.

          - Smoke-Developed Index: [50] [450] <Insert value> or less.

      - DRY-ERASE WALL COVERINGS

Verify properties of manufacturers' products before revising "Dry-Erase Wall Covering" Paragraph below. Retain second, third, or fourth option if limiting gloss characteristics; confirm availability with manufacturers.

        - Dry-Erase Wall Covering: Intended for use with dry-erase markers[ and as a projection surface] and consisting of [low-gloss] [moderate-gloss] [high-gloss] plastic film bonded to fabric backing.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain "Surface Graphics" Subparagraph below if required.

          - Surface Graphics: 2-inch- (50-mm-) square grid.

          - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

        - Magnetic Dry-Erase Wall Covering: Intended for use with dry-erase markers and magnetic aids and consisting of moderate-gloss plastic film bonded to ferrous-powdered fabric backing.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

Delete "Adhesives for Field Application" Paragraph below if using self-adhesive, dry-erase wall covering.

        - Adhesives for Field Application: Strippable, mildew-resistant, nonstaining adhesive for use with dry-erase wall coverings; and for substrate application; as recommended in writing by dry-erase wall covering manufacturer.

          - <Double click to insert sustainable design text for VOC content of adhesive.>

          - <Double click to insert sustainable design text for low emitting adhesives.>

Delete "Primer/Sealer" Paragraph below if primer/sealer is specified in another Section. If not, retain paragraph or revise Section Text to include primer/sealer here.

        - Primer/Sealer: Mildew-resistant primer/sealer complying with requirements in [Section 099123 "Interior Painting"] <Insert Section number and title> and recommended in writing by dry-erase wall covering manufacturer for intended substrate.

If needed, insert other accessories, such as marker trays.

- EXECUTION

      - EXAMINATION

        - Examine substrates and conditions, with Installer present, for compliance with requirements for installation tolerances, surface conditions of wall, moisture content, and other conditions affecting performance of the Work.

        - Examine walls and partitions for proper preparation for dry-erase wall covering.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Comply with manufacturer's written instructions for surface preparation.

        - Clean substrates of substances, including dirt, mold, and mildew, that could impair the bond of dry-erase wall coverings or affect the smooth, finished surfaces of dry-erase wall coverings.

        - Prepare surfaces to achieve a smooth, dry, clean surface free of flaking, unsound coatings, cracks, defects, projections, and depressions.

        - Prepare substrates indicated to receive dry-erase wall covering as required by manufacturer's written instructions to achieve a smooth, dry, clean, structurally sound surface that is uniform in color.

          - Moisture Content: Maximum of [4] <Insert number> percent when tested with an electronic moisture meter.

          - Metals: If not factory primed, clean and apply metal as recommended in writing by primer/sealer manufacturer and wall-covering manufacturer.

Delete "Plaster" and "Gypsum Board" subparagraphs below if priming of plaster and gypsum board surfaces is performed in another Section.

          - Plaster: Allow new plaster to cure. Neutralize areas of high alkalinity. Prime with primer as recommended in writing by primer/sealer manufacturer and wall-covering manufacturer.

          - Gypsum Board: Prime with primer as recommended in writing by primer/sealer manufacturer and wall-covering manufacturer.

          - Painted Surfaces: Treat areas susceptible to pigment bleeding.

      - INSTALLATION

        - Dry-Erase Wall Covering: Comply with dry-erase wall-covering manufacturers' written installation instructions.

Manufacturers recommend installing with horizontal seams; however, vertical seam installations are possible. Revise first subparagraph below to suit Project.

          - Install seams horizontal and level, with lowest seam [24 (610)] <Insert dimension> inches (mm) above finished floor. Railroad fabric (reverse roll direction) to ensure color matching.

          - Double cut seams, with no gaps or overlaps. Remove air bubbles, wrinkles, blisters, and other defects.

          - After installation, clean dry-erase wall covering in accordance with manufacturer's written instructions. Remove excess adhesive at finished seams, perimeter edges, and adjacent surfaces.

      - CLEANING AND PROTECTION

        - Remove excess adhesive at finished seams, perimeter edges, and adjacent surfaces.

        - Clean dry-erase wall covering in accordance with manufacturer's written instructions. Attach one removable cleaning instructions label to dry-erase wall covering in each room.

        - Reinstall hardware and hardware accessories, electrical plates and covers, light fixture trims, and similar items.

END OF SECTION 101146