Copyright 2021 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 102113.14 - STAINLESS STEEL TOILET COMPARTMENTS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Access Manufacturer-Provided, AIA MasterSpec-Based Sections:

<Double click here for this Section based on specific manufacturer’s products set as Basis-of-Design at ProductMasterSpec.com.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, LEED v4.1, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Stainless steel toilet compartments.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Steel overhead support is generally required for ceiling-hung units; wood construction does not provide adequate support.

          - Section 055000 "Metal Fabrications" for supports that attach [ceiling-hung compartments] [floor-and-ceiling-anchored compartments] [and] [post-to-ceiling screens] to overhead structural system.

Wood may provide adequate overhead support for floor-and-ceiling-anchored compartments and post-to-ceiling screens but is inadequate for supporting ceiling-hung compartments. Consult manufacturers for recommendations.

          - Section 061000 "Rough Carpentry" for [blocking] [overhead support of floor-and-ceiling-anchored compartments] [and] [overhead support of post-to-ceiling screens].

          - Section 092216 "Non-Structural Metal Framing" for blocking.

          - Section 102800 "Toilet, Bath, and Laundry Accessories" for accessories mounted on toilet compartments.

      - COORDINATION

        - Coordinate requirements for [overhead supports, ]blocking, reinforcing, and other supports concealed within wall[ and ceiling] to ensure that toilet compartments can be supported and installed as indicated.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Stainless steel toilet compartments.

            - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for toilet compartments.

        - Shop Drawings:

          - Include plans, elevations, sections, details, and attachment details.

Retain first subparagraph below if required or revise to suit Project.

          - Show locations of cutouts for compartment-mounted toilet accessories.

Retain first subparagraph below if required for stainless steel units; see "Fittings and Hardware" Article in the Evaluations.

          - Show locations of reinforcements for compartment-mounted grab bars and locations of blocking for surface-mounted toilet accessories.

          - Show locations of centerlines of toilet fixtures.

          - Show locations of floor drains.

          - Show[ ceiling grid, ceiling-mounted items, and] overhead support or bracing locations.

          - <Insert requirements>.

Retain "Samples for Initial Selection" or "Samples for Verification" paragraph below, or both.

        - Samples for Initial Selection: Manufacturer's standard color sheets, showing full range of available finishes for each type of toilet compartment.

          - Include Samples of hardware and accessories involving material and color selection.

        - Samples for Verification: Actual sample of finished products for each type of toilet compartment, hardware, and accessory.

          - Size: [Manufacturers' standard size] <Insert size>.

Retain "Product Schedule" Paragraph below for Project if multiple installations with multiple selected colors or patterns are required and Contractor verification is advisable.

        - Product Schedule: For toilet compartments, prepared by or under the supervision of supplier, detailing location and selected colors for toilet compartment material.

The IBC requires grab bars to be able to resist a concentrated load of 250 lbf (1112 N) applied in any direction and at any point. Authorities having jurisdiction might require calculations for grab bars mounted on toilet compartment panels. If required, retain "Delegated Design Submittals" Paragraph below if design services have been delegated to Contractor.

        - Delegated Design Submittals: For grab bars mounted on toilet compartment panels, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

          - Include structural design calculations indicating compliance with specified structural-performance requirements.

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

          - <Double click to insert sustainable design text for regional materials.>

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For toilet compartments.

      - MAINTENANCE MATERIAL SUBMITTALS

See Section 017700 "Closeout Procedures" for submission of maintenance material items.

        - Extra Stock Material: Furnish extra materials to Owner that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Door Hinges: [One] <Insert number> hinge(s) with associated fasteners.

          - Latch and Keeper: [One] <Insert number> latch(es) and keeper(s) with associated fasteners.

          - Door Bumper: [One] <Insert number> door bumper(s) with associated fasteners.

          - Door Pull: [One] <Insert number> door pull(s) with associated fasteners.

          - Fasteners: [10] <Insert number> fasteners of each size and type.

      - FIELD CONDITIONS

        - Field Measurements: Verify actual locations of toilet fixtures, walls, columns, ceilings, and other construction contiguous with toilet compartments by field measurements, and coordinate before fabrication.

Insert article for warranty requirements if required. Availability varies among manufacturers and by type of unit. When special warranties are required and available, verify with Owner's counsel that special warranties are not less than remedies available to Owner under prevailing local laws.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SOURCE LIMITATIONS

Retain this article to limit sources for the entire Section. Source limitations may also be specified in individual articles if desired.

        - Obtain stainless steel toilet compartments from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

        - <Double click to insert sustainable design text for recycled content.>

        - <Double click to insert sustainable design text for recycled content of aluminum.>

        - <Double click to insert sustainable design text for regional materials.>

Toilet partitions are required by the IBC and NFPA 101 to be classified for fire performance and smoke development, tested in accordance with ASTM E84, UL 723, or NFPA 286. Manufacturers of stainless steel toilet partitions typically cite testing in accordance with ASTM E84. If requiring testing to NFPA 286 or UL 723, revise the Section Text accordingly. Flame-spread index options correspond to flame-spread and smoke-developed Classes A, B, and C; verify, with manufacturers, availability of suitable units. See "Fire-Test-Response Characteristics" Article in the Evaluations.

        - Surface-Burning Characteristics: Comply with ASTM E84; testing by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

          - Flame-Spread Index: [25] [75] [200] or less.

          - Smoke-Developed Index: 450 or less.

Retain "Structural Performance" Subparagraph below if required. The IBC requires grab bars to be able to resist a concentrated load of 250 lbf (1112 N) applied in any direction and at any point.

        - Structural Performance: Where grab bars are mounted on toilet compartments, design panels to comply with the following requirements:

          - Panels are able to withstand a concentrated load on grab bar of at least 250 lbf (1112 N) applied at any direction and at any point, without deformation of panel.

Verify accessibility requirements of authorities having jurisdiction. Some projects may require compliance with multiple accessibility regulations.

        - Regulatory Requirements: Comply with applicable provisions in [the USDOJ's "2010 ADA Standards for Accessible Design"] [and] [ICC A117.1] <Insert accessibility regulation> for toilet compartments designated as accessible.

Insert seismic requirements if required for Project.

      - STAINLESS STEEL TOILET COMPARTMENTS <Insert drawing designation>

Copy this article and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain and revise "Toilet-Enclosure Style," "Entrance-Screen Style," and "Urinal-Screen Style" paragraphs below to suit Project.

If retaining option for privacy-type toilet-enclosures, verify availability with manufacturers. For maximized privacy, consider retaining no-sightline system option in "Door, Panel, and Pilaster Construction" Paragraph and full-height (continuous) type brackets in "Brackets (Fittings)" Paragraph.

Indicate heights on Drawings.

        - Toilet-Enclosure Style: [Overhead braced] [Floor anchored] [Ceiling hung] [Floor-and-ceiling anchored][, privacy type].

        - Entrance-Screen Style: [Overhead braced] [Floor anchored] [Ceiling hung] [Floor-and-ceiling anchored].

        - Urinal-Screen Style: [Wall hung flat panel] [Wall hung with integral flanges] [Wall hung, wedge shaped] [Floor anchored] [Overhead braced] [Post to ceiling].

Some manufacturers weld face sheets together at corners; others do not. Revise "Door, Panel, and Pilaster Construction" Paragraph below to suit Project. Before retaining option, verify availability with manufacturers. See the Evaluations.

        - Door, Panel, and Pilaster Construction: Seamless, metal facing sheets pressure laminated to core material; with continuous, interlocking molding strip or lapped-and-formed edge closures; corners secured by welding or clips and exposed welds ground smooth.[ Provide with no-sightline system consisting of a full-height continuous stop on latch side of door and full-height continuous filler strip on hinge side of door (unless continuous hinge is used).] Exposed surfaces shall be free of pitting, seam marks, roller marks, stains, discolorations, telegraphing of core material, or other imperfections.

Revise "Core Material," "Grab-Bar Reinforcement," and "Tapping Reinforcement" subparagraphs below to suit Project. See the Evaluations.

          - Core Material: Manufacturer's standard sound-deadening honeycomb of resin-impregnated kraft paper in thickness required to provide finished thickness of 1 inch (25 mm) for doors and panels and 1-1/4 inches (32 mm) for pilasters.

          - Grab-Bar Reinforcement: Provide concealed internal reinforcement for grab bars mounted on units of size and material adequate for panel to withstand specified structural performance requirements.

If necessary, insert requirements for internal reinforcement where small compartment-mounted accessories, such as toilet tissue dispensers, are attached; see "Fittings and Hardware" Article in the Evaluations.

          - Tapping Reinforcement: Provide concealed reinforcement for tapping (threading) at locations where machine screws are used for attaching items to units.

        - Entrance-Screen Construction: Matching panel construction.

        - Urinal-Screen Construction:

Retain one or more of "Flat-Panel Urinal Screen"; "Integral-Flange, Wall-Hung Urinal Screen"; and "Wedge-Shaped, Wall-Hung Urinal Screen" subparagraphs below. If retaining more than one, indicate locations of each type on Drawings or by inserts. Retain "Integral-Flange, Wall-Hung Urinal Screen" Subparagraph for government-style stainless steel screens; coordinate with manufacturers.

          - Flat-Panel Urinal Screen: Matching panel construction.

          - Integral-Flange, Wall-Hung Urinal Screen: Similar to panel construction, with integral full-height flanges for wall attachment, and maximum 1-1/4 inches (32 mm) thick.

          - Wedge-Shaped, Wall-Hung Urinal Screen: Similar to panels, V-shaped, fabricated for concealed wall attachment, and maximum 6 inches (152 mm) wide at wall and minimum 1 inch (25 mm) wide at protruding end.

        - Facing Sheets and Closures: Stainless steel sheet of nominal thicknesses as follows:

Before revising "Pilasters, Braced at Both Ends"; "Pilasters, Unbraced at One End"; "Panels"; "Doors"; "Entrance Screens"; "Flat-Panel Urinal Screens"; "Integral-Flange, Wall-Hung Urinal Screens"; and "Wedge-Shaped, Wall-Hung Urinal Screens" subparagraphs below, see "Stainless Steel Toilet Compartments" Article and its accompanying table in the Evaluations for discussion of steel sheet thicknesses; the table correlates traditional gage numbers with specified decimal thicknesses.

Retain "Pilasters, Braced at Both Ends" Subparagraph below for overhead-braced and floor-and-ceiling-anchored mounting styles. Retain "Pilasters, Unbraced at One End" Subparagraph for floor-anchored and ceiling-hung mounting styles.

          - Pilasters, Braced at Both Ends: Manufacturer's standard thickness, but not less than 0.038 inch (0.95 mm).

          - Pilasters, Unbraced at One End: Manufacturer's standard thickness, but not less than 0.050 inch (1.27 mm).

Retain first option in "Panels" Subparagraph below to maximize competition among listed manufacturers. Second option eliminates some manufacturers or requires custom fabrications.

          - Panels: [Manufacturer's standard thickness, but not less than 0.031 inch (0.79 mm)] [0.038 inch (0.95 mm)].

          - Doors: Manufacturer's standard thickness, but not less than 0.031 inch (0.79 mm).

          - Entrance Screens: Thickness matching panels.

Retain "Flat-Panel Urinal Screens"; "Integral-Flange, Wall-Hung Urinal Screens"; or "Wedge-Shaped, Wall-Hung Urinal Screens" Subparagraph below if required. Retain "Integral-Flange, Wall-Hung Urinal Screens" Subparagraph for government-style stainless steel screens; coordinate with manufacturers.

          - Flat-Panel Urinal Screens: Thickness matching panels.

          - Integral-Flange, Wall-Hung Urinal Screens: Manufacturer's standard thickness, but not less than 0.031 inch (0.79 mm).

          - Wedge-Shaped, Wall-Hung Urinal Screens: Manufacturer's standard thickness, but not less than 0.038 inch (0.95 mm).

Retain "Pilaster Shoes" Paragraph below if applicable.

        - Pilaster Shoes: Formed from stainless steel sheet, not less than 0.031-inch (0.79-mm) nominal thickness and 3 inches (76 mm) high, finished to match hardware.

Retain "Pilaster Sleeves (Caps)" Paragraph below if applicable.

        - Pilaster Sleeves (Caps): Formed from stainless steel sheet, not less than 0.031-inch (0.79-mm) nominal thickness and 3 inches (76 mm) high, finished to match hardware.

Retain "Urinal-Screen Post" Paragraph below for floor-anchored, overhead-braced, and post-to-ceiling urinal screens.

        - Urinal-Screen Post: Manufacturer's standard post design of [material matching the thickness and construction of pilasters] [or] [1-3/4-inch- (44-mm-) square, aluminum tube with satin finish] <Insert requirement>; with shoe[ and sleeve (cap)] matching that on the pilaster.

        - Brackets (Fittings):

Retain "Stirrup Type" or "Full-Height (Continuous) Type" Subparagraph below, or both; if retaining both, indicate locations of each type of fitting on Drawings or by inserts.

          - Stirrup Type: Ear or U-brackets; [chrome-plated zamac] [stainless steel].

          - Full-Height (Continuous) Type: Manufacturer's standard design; [stainless steel] [aluminum].

Textured finishes are available from some manufacturers, but textures vary.

        - Stainless Steel Finish: [Directional Satin Finish: ASTM A480/A480M, No. 4] [Manufacturer's standard textured finish] <Insert finish> on exposed faces. Protect exposed surfaces from damage by application of strippable, temporary protective covering before shipment.

      - HARDWARE AND ACCESSORIES

Heavy-duty institutional hardware is available from a few manufacturers. If heavy-duty hardware is required for Project, verify availability with manufacturers and retain "Door Hardware and Accessories, Heavy Duty" Paragraph. Otherwise, retain "Door Hardware and Accessories" Paragraph below. Revise retained paragraphs to suit Project, or to add specialty or decorative hardware and accessories not listed here.

Before revising "Door Hardware and Accessories" Paragraph below, verify requirements of authorities having jurisdiction for accessible toilet facilities.

        - Door Hardware and Accessories: Manufacturer's operating hardware and accessories.

Retain "Hinges," "Latch and Keeper," "Coat Hook," "Door Bumper," and "Door Pull" subparagraphs below, or revise to suit Project.

          - Hinges:

Hinge types and materials for stainless steel toilet enclosures vary among manufacturers. Verify availability with manufacturers and retain one of first four subparagraphs below, or insert requirements. Most manufacturers offer gravity-type hinges listed below. Gravity-type hinges can be either preset or adjustable; for preset, insert requirements in first subparagraph below for hold-open angle.

            - Manufacturer's gravity-actuated, cam-action, self-closing type that can be adjusted to hold doors open at any angle up to 90 degrees, allowing emergency access by lifting door.

              - Material, Gravity-Type Hinge: [Chrome-plated zamac] [Stainless steel] [Manufacturer's standard].

            - Manufacturer's continuous, cam type that swings to a closed or partially open position, allowing emergency access by lifting door.

              - Material, Continuous, Cam-Type Hinge: Stainless steel.

            - Manufacturer's continuous, spring-loaded type, allowing emergency access by lifting door.

              - Material, Continuous, Spring-Loaded Hinge: [Stainless steel] [Aluminum] [Manufacturer's standard].

            - Manufacturer's standard hinge.

            - <Insert requirement>.

Recessed latch-operating devices that are accessible to people with disabilities are available. Verify requirements of authorities having jurisdiction.

          - Latch and Keeper: Manufacturer's [recessed] [recessed, with occupancy indicator] [surface-mounted] latch unit designed for emergency access and with combination rubber-faced door strike and keeper. Provide units that comply with regulatory requirements for accessibility at toilet enclosures designated as accessible.

            - Material: [Chrome-plated zamac] [Stainless steel] [Manufacturer's standard].

          - Coat Hook: Manufacturer's combination hook and rubber-tipped bumper, sized to prevent inswinging door from hitting compartment-mounted accessories.

            - Material: [Chrome-plated zamac] [Stainless steel] [Manufacturer's standard].

          - Door Bumper: Manufacturer's rubber-tipped bumper at outswinging doors.

            - Material: Manufacturer's standard.

          - Door Pull: Manufacturer's unit at outswinging doors that complies with regulatory requirements for accessibility. Provide units on both sides of doors at toilet enclosures designated as accessible.

            - Material: [Chrome-plated zamac] [Stainless steel] [Manufacturer's standard].

Before revising "Door Hardware and Accessories, Heavy Duty" Paragraph below, verify availability with manufacturers, and verify requirements of authorities having jurisdiction for accessible toilet facilities.

        - Door Hardware and Accessories, Heavy Duty: Manufacturer's heavy-duty institutional operating hardware and accessories.

Retain "Hinges," "Latch and Keeper," "Coat Hook," "Door Bumper," and "Door Pull" subparagraphs below, or revise to suit Project.

Heavy-duty hinge types for stainless steel toilet enclosures vary in availability among manufacturers. Gravity-type hinges can be either preset or adjustable; for preset, insert requirements in "Hinges" Subparagraph below for hold-open angle.

          - Hinges: Manufacturer's minimum 0.062-inch- (1.59-mm-) thick, stainless steel [gravity-actuated, cam-action, self-closing type that can be adjusted to hold doors open at any angle up to 90 degrees] [continuous, cam type that swings to a closed or partially open position] [continuous, spring-loaded type] <Insert requirement>, allowing emergency access by lifting door. Mount with through bolts.

          - Latch and Keeper: Manufacturer's heavy-duty, surface-mounted, cast stainless steel latch unit designed to resist damage due to slamming, with combination rubber-faced door strike and keeper, and with provision for emergency access. Provide units that comply with regulatory requirements for accessibility at toilet enclosures designated as accessible. Mount with through bolts.

          - Coat Hook: Manufacturer's heavy-duty, combination cast stainless steel hook and rubber-tipped bumper, sized to prevent inswinging door from hitting compartment-mounted accessories. Mount with through bolts.

          - Door Bumper: Manufacturer's heavy-duty, rubber-tipped, cast stainless steel bumper at outswinging doors. Mount with through bolts.

          - Door Pull: Manufacturer's heavy-duty, cast stainless steel pull at outswinging doors that complies with regulatory requirements for accessibility. Provide units on both sides of doors at toilet enclosures designated as accessible. Mount with through bolts.

Retain "Overhead Bracing" Paragraph below for overhead-braced units. Revise if specific finish is required. Generally, clear-anodized finish is provided.

        - Overhead Bracing: Manufacturer's standard continuous, extruded-aluminum head rail with antigrip profile and in manufacturer's standard finish.

Revise "Anchorages and Fasteners" Paragraph below to suit Project.

        - Anchorages and Fasteners: Manufacturer's standard exposed fasteners of stainless steel, finished to match items they are securing, with theft-resistant-type heads. Provide sex-type bolts for through-bolt applications. For concealed anchors, use stainless steel, hot-dip galvanized-steel, or other rust-resistant, protective-coated steel anchors compatible with related materials.

      - MATERIALS

In this article, retain only materials that are referenced in other articles for toilet compartments and accessories.

        - Aluminum Castings: ASTM B26/B26M.

        - Aluminum Extrusions: ASTM B221 (ASTM B221M).

Retain "Stainless Steel Sheet" and "Stainless Steel Castings" paragraphs below for stainless steel brackets (fittings). Manufacturers make brackets (fittings) from either material and often do not state which is used.

        - Stainless Steel Sheet: ASTM A240/A240M or ASTM A666, Type 304, stretcher-leveled standard of flatness.

        - Stainless Steel Castings: ASTM A743/A743M.

Insert specific zamac-alloy number in "Zamac" Paragraph below if required. Zamac alloys are termed "traditional" in ASTM B86 and have a range of mechanical properties. Some manufacturers designate particular zamac-alloy numbers in their specifications.

        - Zamac: ASTM B86, commercial zinc-alloy die castings.

      - FABRICATION

        - Fabricate toilet compartment components to sizes indicated. Coordinate requirements and provide cutouts for through-partition toilet accessories and solid blocking within panel where required for attachment of toilet accessories.

Retain applicable mounting styles in "Overhead-Braced Units," "Floor-Anchored Units," "Ceiling-Hung Units," and "Floor-and-Ceiling-Anchored Units" paragraphs below; coordinate with mounting styles in "Stainless Steel Toilet Compartments" Article.

        - Overhead-Braced Units: Manufacturer's standard corrosion-resistant supports, leveling mechanism, and anchors at pilasters and walls to suit floor and wall conditions. Provide shoes at pilasters to conceal supports and leveling mechanism.

        - Floor-Anchored Units: Manufacturer's standard corrosion-resistant anchoring assemblies at pilasters and walls, with leveling adjustment nuts at pilasters for structural connection to floor. Provide shoes at pilasters to conceal anchorage.

        - Ceiling-Hung Units: Manufacturer's standard corrosion-resistant anchoring assemblies at pilasters and walls, with leveling adjustment nuts at pilasters for connection to structural support above finished ceiling. Provide assemblies that support pilasters from structure without transmitting load to finished ceiling. Provide sleeves (caps) at tops of pilasters to conceal anchorage.

        - Floor-and-Ceiling-Anchored Units: Manufacturer's standard corrosion-resistant anchoring assemblies at pilasters and walls, with leveling adjustment nuts at tops and bottoms of pilasters. Provide shoes and sleeves (caps) at pilasters to conceal anchorage.

        - Urinal-Screen Posts: Manufacturer's standard corrosion-resistant anchoring assemblies at posts and walls, with leveling adjustment nuts at[ tops and] bottoms of posts. Provide shoes[ and sleeves (caps)] at posts to conceal anchorage.

Retain "Door Size and Swings" Paragraph below and revise to suit Project if door sizes and swings are not shown on Drawings; verify requirements of authorities having jurisdiction.

        - Door Size and Swings: Unless otherwise indicated, provide 24-inch- (610-mm-) wide inswinging doors for standard toilet enclosures and 36-inch- (914-mm-) wide outswinging doors with a minimum 32-inch- (813-mm-) wide clear opening for toilet enclosures designated as accessible.

- EXECUTION

      - EXAMINATION

        - Examine areas and conditions, with Installer present, for compliance with requirements for fastening, support, alignment, operating clearances, and other conditions affecting performance of the Work.

          - Confirm location and adequacy of blocking and supports required for installation.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - General: Comply with manufacturer's written installation instructions. Install units rigid, straight, level, and plumb. Secure units in position with manufacturer's recommended anchoring devices.

          - Maximum Clearances:

            - Pilasters and Panels or Screens: 1/2 inch (13 mm).

            - Panels or Screens and Walls: 1 inch (25 mm).

Retain "Stirrup Brackets" or "Full-Height (Continuous) Brackets" Subparagraph below, or both, to suit Project.

          - Stirrup Brackets: Secure panels or screens to walls and to pilasters with no fewer than [two brackets attached] [three brackets attached at midpoint and] near top and bottom of panel.

            - Locate wall brackets so holes for wall anchors occur in masonry or tile joints.

            - Align brackets at pilasters with brackets at walls.

          - Full-Height (Continuous) Brackets: Secure panels or screens to walls and to pilasters with full-height brackets.

            - Locate bracket fasteners so holes for wall anchors occur in masonry or tile joints.

            - Align brackets at pilasters with brackets at walls.

Retain applicable mounting styles in "Overhead-Braced Units," "Floor-Anchored Units," "Ceiling-Hung Units," and "Floor-and-Ceiling-Anchored Units" paragraphs below.

        - Overhead-Braced Units: Secure pilasters to floor and level, plumb, and tighten. Set pilasters with anchors penetrating not less than 1-3/4 inches (44 mm) into structural floor unless otherwise indicated in manufacturer's written instructions. Secure continuous head rail to each pilaster with no fewer than two fasteners. Hang doors to align tops of doors with tops of panels, and adjust so tops of doors are parallel with overhead brace when doors are in closed position.

        - Floor-Anchored Units: Set pilasters with anchors penetrating not less than 2 inches (51 mm) into structural floor unless otherwise indicated in manufacturer's written instructions. Level, plumb, and tighten pilasters. Hang doors and adjust so tops of doors are level with tops of pilasters when doors are in closed position.

        - Ceiling-Hung Units: Secure pilasters to supporting structure and level, plumb, and tighten. Hang doors and adjust so bottoms of doors are level with bottoms of pilasters when doors are in closed position.

        - Floor-and-Ceiling-Anchored Units: Secure pilasters to supporting construction and level, plumb, and tighten. Hang doors and adjust so doors are level and aligned with panels when doors are in closed position.

        - Urinal Screens: Attach with anchoring devices to suit supporting structure. Set units level and plumb, rigid, and secured to resist lateral impact.

      - ADJUSTING

Revise hinge adjustment requirements in this article to suit Project. Verify accessibility requirements of authorities having jurisdiction.

        - Hardware Adjustment: Adjust and lubricate hardware in accordance with hardware manufacturer's written instructions for proper operation. Set hinges on inswinging doors to hold doors open approximately 30 degrees from closed position when unlatched. Set hinges on outswinging doors to return doors to fully closed position.

END OF SECTION 102113.14