Copyright 2020 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 102213 - WIRE MESH PARTITIONS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Access Manufacturer-Provided, AIA MasterSpec-Based Sections:

<Double click here for this Section based on specific manufacturer’s products set as Basis-of-Design at ProductMasterSpec.com.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Standard-duty wire mesh partitions.

          - Heavy-duty wire mesh partitions.

          - Wire mesh ceilings.

          - Wire mesh stairway partitions.

          - Wire mesh equipment barriers.

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project. Include only essential definitions or acronyms not well understood by the affected industry or trade.

"Intermediate crimp" is also called "intercrimped" and "triple crimped."

        - Intermediate Crimp: Wires pass over one and under the next adjacent wire in both directions, with wires crimped before weaving and with extra crimps between the intersections.

Lock crimp has a smoother appearance than intermediate crimp.

        - Lock Crimp: Deep crimps at points of the intersection that lock wires securely in place.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Wire mesh [partitions] [ceilings] [stairway partitions] [and] [equipment barriers].

          - <Insert products>.

        - Shop Drawings:

          - Include plans, elevations, sections, and attachment details.

Retain subparagraph below if wire mesh partitions include service windows, doors, or gates.

          - Indicate clearances required for operation of [service windows] [doors] [and] [gates].

Retain "Samples for Initial Selection" or "Samples for Verification" Paragraph below, or both.

        - Samples for Initial Selection: Manufacturer's standard color sheets, showing full range of available colors for units with factory-applied color finishes.

        - Samples for Verification: Panel constructed of specified frame members and wire mesh. Show method of finishing members at intersections.

          - Size: 12 by 12 inches (300 by 300 mm).

Retain "Delegated Design Submittals" Paragraph below if design services have been delegated to Contractor.

        - Delegated Design Submittals: For wire mesh partitions indicated to comply with performance requirements and design criteria, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

      - INFORMATIONAL SUBMITTALS

Information submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

        - Certificates:

Retain "Welding certificates" Paragraph below if retaining "Welding Qualifications" Subparagraph in "Quality Assurance" Article.

          - Welding certificates.

Coordinate "Qualification Statements" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article. If inserting additional entities or specialist, add qualifications to "Quality Assurance" Article.

        - Qualification Statements: For Installer.

        - Delegated design engineer qualifications.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For wire mesh partition hardware.

      - MAINTENANCE MATERIAL SUBMITTALS

See Section 017700 "Closeout Procedures" for submission of maintenance material items.

        - Extra Stock Material: Furnish extra materials to Owner that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Door Locks: Furnish [5] <Insert number> percent of quantity installed for each type indicated, but no fewer than <Insert number> locks.

      - QUALITY ASSURANCE

        - Qualifications:

          - Installers: [Fabricator of products] [Entity that employs installers and supervisors who are trained and approved by manufacturer] [Authorized representative who is trained and approved by manufacturer].

Retain "Welding Qualifications" Subparagraph below if shop or field welding is required. If retaining, also retain "Welding certificates" Subparagraph in "Informational Submittals" Article.

          - Welding Qualifications: Qualify procedures and personnel in accordance with the following welding codes:

Retain first subparagraph below for structural welding of steel.

            - AWS D1.1/D1.1M.

Retain subparagraph below for structural welding of sheet steel.

            - AWS D1.3/D1.3M.

      - DELIVERY, STORAGE, AND HANDLING

Packaging in first paragraph below may increase cost but may be required for painted items.

        - Deliver wire mesh items [with cardboard protectors on perimeters of panels and doors and with posts wrapped] [palleted] [crated] to provide protection during transit and Project-site storage. Use vented plastic.

        - Inventory wire mesh partition door hardware on receipt, and provide secure lockup for wire mesh partition door hardware delivered to Project site.

          - Tag each item or package separately with identification, and include basic installation instructions with each item or package.

        - Deliver keys to Owner by registered mail or overnight package service[.][, addressed as follows:]

          - <Insert name and address of Owner's representative>.

      - FIELD CONDITIONS

If possible, design wire mesh units so that they do not have to fit other construction and delete this article.

        - Field Measurements: Verify actual dimensions of construction contiguous with wire mesh units by field measurements before fabrication.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - SOURCE LIMITATIONS

Retain this article to limit sources for the entire Section. Source limitations may also be specified in individual articles if desired.

        - For wire mesh products, obtain each color, grade, finish, type, and variety from single source with resources to provide products of consistent quality in appearance and physical properties.

      - PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

        - Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design wire mesh units.

        - Structural Performance: Wire mesh units to withstand the effects of gravity loads and the following loads and stresses within limits and under conditions indicated.

Consult a structural engineer experienced in engineering wire mesh partitions of type indicated to quantify design loads applicable to Project. Verify compliance with codes. See the Evaluations.

Retain one or more of four subparagraphs below if design loads and load combinations are not indicated on Drawings. First subparagraph is an example based on the IBC requirement for guards.

          - Concentrated load of 50 lbf (0.22 kN) applied horizontally on an area of 1 sq. ft. (0.093 sq. m) at any location on a panel.

          - Total load of [200 lbf (0.89 kN)] <Insert value> applied uniformly over each panel.

          - Concentrated load and total load need not be assumed to act concurrently.

          - <Insert loads or load combinations>.

Retain "Seismic Performance" Paragraph below for projects requiring seismic design. Delete paragraph if performance requirements are indicated on Drawings. Model building codes and ASCE/SEI 7 establish criteria for buildings subject to earthquake motions. Coordinate requirements with structural engineer.

        - Seismic Performance: Wire mesh units to withstand the effects of earthquake motions determined in accordance with [ASCE/SEI 7] <Insert requirement>.

Verify accessibility requirements of authorities having jurisdiction. Some projects may require compliance with multiple accessibility regulations.

        - Regulatory Requirements: Comply with applicable provisions in [the United States Access Board's ADA-ABA Accessibility Guidelines] [and] [ICC A117.1] for doors and gates designated as accessible.

      - STANDARD-DUTY WIRE MESH PARTITIONS

Mesh in "Mesh" Paragraph below is industry standard for standard-duty wire mesh. Revise to suit mesh with different wire diameter or different pattern.

        - Mesh: 0.135-inch- (3.5-mm-) diameter, intermediate-crimp steel wire woven into 1-1/2-inch (38-mm) diamond mesh.

        - Vertical Panel Framing: 1-1/4-by-5/8-by-0.080-inch (32-by-16-by-2.0-mm) cold-rolled, C-shaped steel channels with holes for 1/4-inch- (6-mm-) diameter bolts not more than 12 inches (300 mm) o.c.

        - Horizontal Panel Framing: 1-by-1/2-by-1/8-inch (25-by-13-by-3.2-mm) cold-rolled steel channels.

        - Horizontal Panel Stiffeners: Two cold-rolled steel channels, 3/4 by 3/8 by 1/8 inch (19 by 9.5 by 3.2 mm), bolted or riveted toe to toe through mesh; or one 1-by-1/2-by-1/8-inch (25-by-13-by-3.2-mm) cold-rolled steel channel with wire mesh woven through channel.

        - Top Capping Bars: 2-1/4-by-1-inch (57-by-25-mm) cold-rolled steel channels.

        - Posts for 90-Degree Corners: 1-1/4-by-1-1/4-by-1/8-inch (32-by-32-by-3.2-mm) steel angles or square tubes with holes for 1/4-inch- (6-mm-) diameter bolts aligning with bolt holes in vertical framing; with floor anchor clips.

Retain "Posts for Other-Than-90-Degree Corners" or "Adjustable Corner Posts" Paragraph below if other-than-90-degree corners are required.

        - Posts for Other-Than-90-Degree Corners: Steel pipe or tubing with holes for 1/4-inch- (6-mm-) diameter bolts aligning with bolt holes in vertical framing; with floor anchor clips.

          - Partitions up to 12 Ft. (3.7 m) High: 1-1/4-inch (32-mm) OD by 1/8 inch (3.2 mm).

          - Partitions up to 20 Ft. (6.1 m) High: 2-1/2-inch (65-mm) OD by 1/8 inch (3.2 mm).

        - Adjustable Corner Posts: Two 1-1/4-by-5/8-by-0.080-inch (32-by-16-by-2.0-mm) cold-rolled, C-shaped steel channels connected by steel hinges at 36 inches (900 mm) o.c., with holes for 1/4-inch- (6-mm-) diameter bolts aligning with bolt holes in vertical framing.

        - Line Posts: 3-inch-by-4.1-lb (76-mm-by-1.9-kg) or 3-1/2-by-1-1/4-by-0.127-inch (89-by-32-by-3.2-mm) steel channels; with 1/4-inch (6.4-mm) steel base plates.

        - Three-Way Intersection Posts: 1-1/4-by-1-1/4-by-1/8-inch (32-by-32-by-3.2-mm) steel tubes or channels, with holes for 1/4-inch- (6-mm-) diameter bolts aligned for bolting to adjacent panels.

        - Four-Way Intersection Posts: 1-1/4-by-1-1/4-by-1/8-inch (32-by-32-by-3.2-mm) steel tubes, with holes for 1/4-inch- (6-mm-) diameter bolts aligned for bolting to adjacent panels.

        - Floor Shoes: Metal, not less than 2 inches (50 mm) high; sized to suit vertical framing, drilled for attachment to floor, and with setscrews for leveling adjustment.

        - Swinging Doors: Fabricated from same mesh as partitions, with framing fabricated from 1-1/4-by-1/2-by-1/8-inch (32-by-13-by-3.2-mm) steel channels or 1-1/4-by-5/8-by-0.080-inch (32-by-16-by-2.0-mm) cold-rolled, C-shaped steel channels, banded with 1-1/4-by-1/8-inch (32-by-3.2-mm) flat steel bar cover plates on [three] [four] sides, and with 1/8-inch- (3.2-mm-) thick angle strike bar and cover on strike jamb.

          - Hinges: Full-surface type, 3-by-3-inch (76-by-76-mm) steel, three per door; bolted, riveted, or welded to door and jamb framing.

Retain "Padlock Lug" or "Cylinder Lock" Subparagraph below. In "Cylinder Lock" Subparagraph, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Padlock Lug: Mortised into door framing and enclosed with steel cover.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside.

Retain "Inactive Leaf Hardware" Subparagraph below for double doors.

          - Inactive Leaf Hardware: Cane bolt at bottom and chain bolt at top.

        - Swinging Dutch Doors: Fabricated from same mesh as partitions, with framing fabricated from 1-1/4-by-1/2-by-1/8-inch (32-by-13-by-3.2-mm) steel channels or 1-1/4-by-5/8-by-0.080-inch (32-by-16-by-2.0-mm) cold-rolled, C-shaped steel channels, banded with 1-1/4-by-1/8-inch (32-by-3.2-mm) flat steel bar cover plates on [three] [four] sides, and with 1/8-inch- (3.2-mm-) thick angle strike bar and cover on strike jamb.

          - Hinges: Full-surface type, 3-by-3-inch (76-by-76-mm) steel, two per section of door (top and bottom); bolted, riveted, or welded to door and jamb framing.

In "Cylinder Lock" Subparagraph below, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside; mounted in lower section of door.

          - Bolt: Mounted in, securing upper section of door.

          - Shelf: Fabricated from 0.097-inch- (2.5-mm-) thick, steel sheet, [12 inches (300 mm)] [18 inches (450 mm)] deep by full width of door less 4 inches (102 mm); with corners rounded and edges finished smooth; mounted on top of lower section of door and braced with manufacturer's standard brackets.

        - Sliding Doors: Fabricated from same mesh as partitions, with framing fabricated from 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) steel channels, banded with 1-1/2-by-1/8-inch (38-by-3.2-mm) flat steel bar cover plates on four sides.

          - Hardware: Two, four-wheel roller-bearing carriers; box track; and bottom guide channel for each door.

Retain "Padlock Lug" or "Cylinder Lock" Subparagraph below. In "Cylinder Lock" Subparagraph, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Padlock Lug: Mortised into door framing and enclosed with steel cover.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside.

        - Vertically Sliding Service Windows: Fabricated from same mesh and framing as panels and equipped with [spring catch] [slide bolts] on each jamb to lock window in open position and a padlock lug to lock window in closed position. Include opening frame in partition fabricated from 1-1/4-by-1/2-by-1/8-inch (32-by-13-by-3.2-mm) steel channels.

          - Size: [24 inches wide by 18 inches high (600 mm wide by 450 mm high)] [24 inches wide by 21-1/2 inches high (600 mm wide by 546 mm high)] [As indicated] <Insert dimensions>.

          - Shelf: Fabricated from 0.097-inch- (2.5-mm-) thick, steel sheet; with corners rounded and edges finished smooth; braced with manufacturer's standard brackets.

            - Size: [23 inches wide by 12 inches deep (590 mm wide by 300 mm deep)] [As indicated] <Insert dimensions>.

        - Swinging Service Windows: Fabricated from same mesh and framing as panels and equipped with spring catch on strike jamb that locks window in closed position. Include opening frame in partition fabricated from 1-1/4-by-1/2-by-1/8-inch (32-by-13-by-3.2-mm) steel channels.

          - Size: [24 inches wide by 18 inches high (600 mm wide by 450 mm high)] [24 inches wide by 15 inches high (600 mm wide by 380 mm high)] [As indicated] <Insert dimensions>.

          - Shelf: Fabricated from 0.097-inch- (2.5-mm-) thick, steel sheet; with corners rounded and edges finished smooth; braced with manufacturer's standard brackets.

            - Size: [24 inches wide by 12 inches deep (600 mm wide by 300 mm deep)] [24 inches wide by 15-3/4 inches deep (600 mm wide by 400 mm deep)] [As indicated] <Insert dimensions>.

        - Accessories:

"Sheet Metal Base" Subparagraph below is for wainscot panels.

          - Sheet Metal Base: [0.060-inch- (1.5-mm-)] [0.035-inch- (0.9-mm-)] thick, steel sheet.

          - Adjustable Filler Panels: 0.060-inch- (1.5-mm-) thick, steel sheet; capable of filling openings from 2 to 12 inches (50 to 300 mm).

          - Wall Clips: Manufacturer's standard, steel sheet[; allowing up to 1 inch (25 mm) of adjustment].

        - Finish: [Hot-dip galvanized] [Hot-dip galvanized and shop primed for field painting] [Shop primed for field painting] [Enamel finish] [Powder-coated finish] unless otherwise indicated.

          - Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

      - HEAVY-DUTY WIRE MESH PARTITIONS

        - Mesh:

Retain one of two subparagraphs below or revise either to specify a different wire diameter or pattern.

Mesh in first subparagraph is industry standard for heavy-duty wire mesh.

          - 0.192-inch- (4.9-mm-) diameter, intermediate-crimp steel wire woven into 2-inch (50-mm) diamond mesh.

          - 0.192-inch- (4.9-mm-) diameter steel wire, resistance welded into 1-1/2-by-2-1/2-inch (38-mm-by-65-mm) rectangular mesh.

        - Vertical and Horizontal Panel Framing: 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) cold-rolled steel channels; with holes for 3/8-inch- (9.5-mm-) diameter bolts not more than 12 inches (300 mm) o.c.

        - Horizontal Panel Stiffeners: Two cold-rolled steel channels, 1 by 1/2 by 1/8 inch (25 by 13 by 3.2 mm), bolted or riveted toe to toe through mesh.

        - Top Capping Bars: 3-by-1-inch (76-by-25-mm) steel channels.

Option in "Posts for 90-Degree Corners" Paragraph below allows use of an alternative material that is standard with several manufacturers.

        - Posts for 90-Degree Corners: 1-1/2-by-1-1/2-by-1/8-inch (38-by-38-by-3.2-mm) steel angles or tubes[ or 2-by-2-by-0.075-inch (50-by-50-by-1.9-mm) cold-rolled steel angles or tubes], with holes for 3/8-inch- (9.5-mm-) diameter bolts aligning with bolt holes in vertical framing; with 1/4-inch (6.4-mm) steel base plates.

Retain "Posts for Other-Than-90-Degree Corners" or "Adjustable Corner Posts" Paragraph below if other-than-90-degree corners are required.

        - Posts for Other-Than-90-Degree Corners: 2-inch- (50-mm) OD by 1/8-inch (3.2-mm) steel pipe or round tube, with holes for 3/8-inch- (9.5-mm-) diameter bolts aligning with bolt holes in vertical framing; with 1/4-inch (6.4-mm) steel base plates.

        - Adjustable Corner Posts: Two 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) cold-rolled, steel channels or 2-by-2-by-0.075-inch (50-by-50-by-1.9-mm) steel tubes connected by steel hinges at 36 inches (900 mm) o.c. attached to posts; with 1/4-inch- (6-mm-) diameter bolt holes aligning with bolt holes in vertical framing; with 1/4-inch (6.4-mm) steel base plates.

        - Line Posts: 3-inch-by-4.1-lb (76-mm-by-1.9-kg) or 3-1/2-by-1-1/4-by-1/8-inch (89-by-32-by-3.2-mm) steel channels; with 1/4-inch (6.4-mm) steel base plates.

        - [Three-] [and] [Four-]Way Intersection Posts: 2-by-2-by-0.075-inch (50-by-50-by-1.9-mm) steel tubes, with holes for 3/8-inch- (9.5-mm-) diameter bolts aligned for bolting to adjacent panels; with 1/4-inch (6.4-mm) steel base plates.

        - Floor Shoes: Metal, not less than 2 inches (50 mm) high; sized to suit vertical framing, drilled for attachment to floor, and with setscrews for leveling adjustment.

        - Swinging Doors: Fabricated from same mesh as partitions, with framing fabricated from 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) steel channels, banded with 1-1/2-by-1/8-inch (38-by-3.2-mm) flat steel bar cover plates on four sides, and with 1/8-inch- (3.2-mm-) thick angle strike bar and cover on strike jamb.

          - Hinges: Full-surface type, 3-1/2-by-3-1/2-inch (89-by-89-mm) steel, three per door; bolted, riveted, or welded to door and jamb framing.

Retain "Padlock Lug" or "Cylinder Lock" Subparagraph below. In "Cylinder Lock" Subparagraph, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Padlock Lug: Mortised into door framing and enclosed with steel cover.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside; mounted in lower section of door.

Retain "Inactive Leaf Hardware" Subparagraph below for double doors.

          - Inactive Leaf Hardware: Cane bolt at bottom and chain bolt at top.

        - Swinging Dutch Doors: Fabricated from same mesh as partitions, with framing fabricated from 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) steel channels, banded with 1-1/2-by-1/8-inch (38-by-3.2-mm) flat steel bar cover plates on [three] [four] sides, and with 1/8-inch- (3.2-mm-) thick angle strike bar and cover on strike jamb.

          - Hinges: Full-surface type, 3-1/2-by-3-1/2-inch (89-by-89-mm) steel, two per section of door (top and bottom); bolted, riveted, or welded to door and jamb framing.

In "Cylinder Lock" Subparagraph below, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside; mounted in lower section of door.

          - Bolt: Mounted in, securing upper section of door.

          - Shelf: Fabricated from 0.097-inch- (2.5-mm-) thick, steel sheet, [12 inches (300 mm)] [18 inches (450 mm)] deep by full width of door less 4 inches (102 mm); with corners rounded and edges finished smooth; mounted on top of lower section of door and braced with manufacturer's standard brackets.

        - Sliding Doors: Fabricated from same mesh as partitions, with framing fabricated from 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) steel channels, banded with 1-1/2-by-1/8-inch (38-by-3.2-mm) flat steel bar cover plates on four sides.

          - Hardware: Two, four-wheel roller-bearing carriers; box track; and bottom guide channel for each door.

Retain "Padlock Lug" or "Cylinder Lock" Subparagraph below. In "Cylinder Lock" Subparagraph, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Padlock Lug: Mortised into door framing and enclosed with steel cover.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside; mounted in lower section of door.

        - Vertically Sliding Service Windows: Fabricated from same mesh and framing as panels and equipped with [spring catch] [slide bolts] on each jamb to lock window in open and closed positions. Include opening frame in partition fabricated from 1-1/4-by-1/2-by-1/8-inch (32-by-13-by-3.2-mm) steel channels.

          - Size: [24 inches wide by 18 inches high (600 mm wide by 450 mm high)] [24 inches wide by 21-1/2 inches high (600 mm wide by 546 mm high)] [As indicated] <Insert dimensions>.

          - Shelf: Fabricated from 0.097-inch- (2.5-mm-) thick, steel sheet; with corners rounded and edges finished smooth; braced with manufacturer's standard brackets.

            - Size: [24 inches wide by 12 inches deep (600 mm wide by 300 mm deep)] [As indicated] <Insert dimensions>.

        - Swinging Service Windows: Fabricated from same mesh and framing as panels and equipped with spring catch on strike jamb that locks window in closed position. Include opening frame in partition fabricated from 1-1/4-by-1/2-by-1/8-inch (32-by-13-by-3.2-mm) steel channels.

          - Size: [24 inches wide by 18 inches high (600 mm wide by 450 mm high)] [24 inches wide by 15 inches high (600 mm wide by 380 mm high)] [As indicated] <Insert dimensions>.

          - Shelf: Fabricated from 0.097-inch- (2.5-mm-) thick, steel sheet; with corners rounded and edges finished smooth; braced with manufacturer's standard brackets.

            - Size: [24 inches wide by 12 inches deep (600 mm wide by 300 mm deep)] [24 inches wide by 15-3/4 inches deep (600 mm wide by 400 mm deep)] [As indicated] <Insert dimensions>.

        - Accessories:

"Sheet Metal Base" Subparagraph below is for wainscot panels.

          - Sheet Metal Base: 0.060-inch- (1.5-mm-) thick, steel sheet.

          - Adjustable Filler Panels: 0.060-inch- (1.5-mm-) thick, steel sheet; capable of filling openings from 2 to 12 inches (50 to 300 mm).

          - Wall Clips: Manufacturer's standard, cold-rolled steel sheet[; allowing up to 1 inch (25 mm) of adjustment].

        - Finish: [Hot-dip galvanized] [Hot-dip galvanized and shop primed for field painting] [Shop primed for field painting] [Enamel finish] [Powder-coated finish] unless otherwise indicated.

          - Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

      - WIRE MESH CEILINGS

Wire mesh ceilings are typically installed along with wire mesh partitions. Revise this article if wire mesh ceilings are required to suit Project, but wire mesh partitions are not. Consult manufacturers for recommendations.

        - Mesh, Framing, and Stiffeners: Fabricated from same mesh and framing as wire mesh partition panels.

        - Perimeter Partition Supports: [1-1/2-by-1-1/2-by-1/8-inch (38-by-38-by-3.2-mm)] [1-1/2-by-1-1/2-by-0.075-inch (38-by-38-by-1.9-mm)] steel angle, with holes for 1/4-inch- (6-mm-) diameter bolts aligned for bolting to top of wire mesh partitions and to sides of wire mesh ceiling panels.

        - Wall Supports: [1-1/2-by-1-1/2-by-1/8-inch (38-by-38-by-3.2-mm)] [1-1/2-by-1-1/2-by-0.075-inch (38-by-38-by-1.9-mm)] steel angle punched for attachment to wall and wire mesh ceiling panels.

        - Intermediate Supports: Steel I-beams or rectangular tubes, as recommended by manufacturer.

        - Intermediate Support Posts: [2-by-2-by-1/8-inch (50-by-50-by-3.2-mm)] [2-by-2-by-0.075-inch (50-by-50-by-1.9-mm)] steel tubes.

        - Finishes: Match adjacent wire mesh partitions.

      - WIRE MESH STAIRWAY PARTITIONS

        - Standard-Duty Stairway Partitions:

Retain "Diamond Mesh," "Square Mesh," or "Rectangular Mesh" Subparagraph below.

          - Diamond Mesh: 0.135-inch- (3.5-mm-) diameter, intermediate-crimp steel wire woven into 1-1/2-inch (38-mm) diamond pattern and securely clinched to frames.

          - Square Mesh: 0.135-inch- (3.5-mm-) diameter, [intermediate] [lock]-crimp steel wire woven into 1-1/2-inch (38-mm) square pattern, inserted through frame holes and welded into frame. Vertical wires are plumb, and horizontal wires are perpendicular to vertical wires.

          - Rectangular Mesh: 0.135-inch- (3.5-mm-) diameter, [intermediate] [lock]-crimp steel wire woven into 2-by-1-inch (50-by-25-mm) rectangular pattern, inserted through frame holes and welded into frame. Vertical wires are plumb, and horizontal wires are perpendicular to vertical wires.

          - Vertical Panel Framing: 1-1/4-by-5/8-by-0.097-inch (32-by-16-by-2.5-mm) cold-rolled, C-shaped steel channels; with 1/4-inch- (6-mm-) diameter bolt holes spaced not more than 18 inches (450 mm) o.c. along center of framing.

          - Horizontal Panel Framing: 1-by-1/2-by-1/8-inch (25-by-13-by-3.2-mm) cold-rolled steel channels.

          - Horizontal Panel Stiffeners: 1-by-1/2-by-1/8-inch (25-by-13-by-3.2-mm) cold-rolled steel channels with wire woven through, or two 1-by-1/2-by-1/8-inch (25-by-13-by-3.2-mm) cold-rolled steel channels bolted or riveted toe to toe through mesh.

        - Heavy-Duty Stairway Partitions:

Retain "Diamond Mesh," "Square Mesh," or "Rectangular Mesh" Subparagraph below.

          - Diamond Mesh: 0.192-inch- (4.9-mm-) diameter, intermediate-crimp steel wire woven into 2-inch (50-mm) diamond pattern and securely clinched to frames.

          - Square Mesh: 0.192-inch- (4.9-mm-) diameter, [intermediate] [lock]-crimp steel wire woven into 2-inch (50-mm) square pattern, inserted through frame holes and welded into frame. Vertical wires are plumb, and horizontal wires are perpendicular to vertical wires.

          - Rectangular Mesh: 0.192-inch- (4.9-mm-) diameter, [intermediate] [lock]-crimp steel wire woven into 2-by-1-inch (50-by-25-mm) rectangular pattern, inserted through frame holes and welded into frame. Vertical wires are plumb, and horizontal wires are perpendicular to vertical wires.

          - Vertical and Horizontal Panel Framing: 1-1/2-by-3/4-by-0.097-inch (38-by-19-by-2.5-mm) cold-rolled, C-shaped steel channels; with 3/8-inch- (9.5-mm-) diameter bolt holes spaced not more than 18 inches (450 mm) o.c. along center of framing.

          - Horizontal Panel Stiffeners: 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) cold-rolled steel channels with wire woven through, or two 1-by-1/2-by-1/8-inch (25-by-13-by-3.2-mm) cold-rolled steel channels bolted or riveted toe to toe through mesh.

        - Swinging Doors: Fabricated from same mesh as partitions, with framing fabricated from 1-1/2-by-3/4-by-1/8-inch (38-by-19-by-3.2-mm) steel channels, banded with 1-1/2-by-1/8-inch (38-by-3.2-mm) flat steel bar cover plates on [three] [four] sides, and with 1/8-inch- (3.2-mm-) thick angle strike bar and cover on strike jamb.

          - Hinges: Full-surface spring type, 3-1/2-by-3-1/2-inch (89-by-89-mm) steel, one and a half pairs per door; bolted, riveted, or welded to door and jamb framing.

          - Exit Device: As specified in [Section 087100 "Door Hardware."] [Section 087111 "Door Hardware (Descriptive Specification)."]

          - Tamper Shield: Fabricated from 0.097-inch- (2.5-mm-) thick, cold-rolled steel sheet; 15 inches (380 mm) high by width of door.

        - Door Jamb Framing: 2-by-2-by-1/8-inch (50-by-50-by-3.2-mm) steel pipe or tubing.

        - Floor Shoes: Steel, cast iron, or cast aluminum, not less than 2 inches (50 mm) high; sized to suit vertical framing, drilled for attachment to floor, and with setscrews for leveling adjustment.

        - Wall Clips: Manufacturer's standard, cold-rolled steel sheet[; allowing up to 1 inch (25 mm) of adjustment].

        - Finish for Uncoated Ferrous Steel: [Hot-dip galvanized] [Hot-dip galvanized and shop primed for field painting] [Shop primed for field painting] [Enamel finish] [Powder-coated finish] unless otherwise indicated.

          - Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

      - WIRE MESH EQUIPMENT BARRIERS

Revise "Mesh" Paragraph below to suit Project. Manufacturers offer many other sizes of wire mesh, sheet metal, and clear polycarbonate sheet as alternatives.

        - Mesh: 0.135-inch- (3.5-mm-) diameter, intermediate-crimp steel wire woven into [1-1/2-inch (38-mm) diamond] [1-by-2-inch (25-by-50-mm) rectangular] mesh.

        - Panels: 1-1/4-by-1-1/4-by-1/8-inch (32-by-32-by-3.2-mm) steel angle framing on four sides, with wire mesh welded to framing.

          - Horizontal Panel Stiffeners: 1-1/4-by-1-1/4-by-1/8-inch (32-by-32-by-3.2-mm) steel angles or 3/4-by-1/4-inch (19-by-6.4-mm) hot-rolled steel flat bars.

          - Height: [48 inches (1220 mm)] [60 inches (1524 mm)] <Insert dimension>.

        - Line and Corner Posts: 2-by-2-by-0.068-inch (50-by-50-by-1.7-mm) steel tubing with steel base plates welded to bottoms, drilled for attachment to floor, and with steel caps welded to tops.

          - Height: Panel height plus [12-inch- (300-mm-)] <Insert dimension> high, sweep space.

        - Swinging Gates: Fabricated from same mesh as panels, with gate framing fabricated from 1-1/4-by-1-1/4-by-3/16-inch (32-by-32-by-4.7-mm) steel angles on four sides, and with wire mesh welded to framing.

          - Hinges: Full-surface[ spring] type, 3-1/2-by-3-1/2-inch (89-by-89-mm) steel, one pair per door; bolted, riveted, or welded to door and jamb framing.

Retain "Padlock Lug" or "Cylinder Lock" Subparagraph below. In "Cylinder Lock" Subparagraph, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Padlock Lug: Mortised into door framing and enclosed with steel cover.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside.

        - Sliding Gates: Fabricated from same mesh as panels, with framing fabricated from 1-1/4-by-1-1/4-by-3/16-inch (32-by-32-by-4.7-mm) steel angles on four sides, and with wire mesh welded to framing.

          - Hardware: Two, four-wheel roller-bearing carriers; box track; and bottom guide channel for each door.

Retain "Padlock Lug" or "Cylinder Lock" Subparagraph below. In "Cylinder Lock" Subparagraph, "recessed turn knob" option is prevalent but may not comply with accessibility requirements if applicable.

          - Padlock Lug: Mortised into door framing and enclosed with steel cover.

          - Cylinder Lock: Mortise type with [manufacturer's standard cylinder] [cylinder specified in Section 087100 "Door Hardware"] [cylinder specified in Section 087111 "Door Hardware (Descriptive Specification)"]; operated by key outside and [recessed turn knob] [knob] [lever] inside.

If required, insert electrical interlock between gate and equipment to shut down equipment if gate is opened.

        - Finish for Uncoated Ferrous Steel: [Hot-dip galvanized] [Hot-dip galvanized and shop primed for field painting] [Shop primed for field painting] [Enamel finish] [Powder-coated finish] unless otherwise indicated.

          - Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

      - MATERIALS

        - <Double click to insert sustainable design text for recycled content.>

        - Steel Wire: ASTM A510/A510M.

        - Steel Plates, Channels, Angles, and Bars: ASTM A36/A36M.

        - Steel Sheet: Cold-rolled steel sheet, ASTM A1008/A1008M, Commercial Steel (CS), Type B.

        - Steel Pipe: ASTM A53/A53M, Schedule 40, unless another weight is indicated or required by structural loads.

Steel tubing and pipe are sized differently. Pipe is designated by nominal pipe size and weight or schedule number. Tubing is designated by OD and wall thickness.

        - Steel Tubing: ASTM A500/A500M, cold-formed structural-steel tubing or ASTM A513/A513M, Type 5, mandrel-drawn mechanical tubing.

        - Metallic-Coated Steel Sheet: ASTM A653/A653M, Commercial Steel (CS), Type B; with G60 (Z180) zinc (galvanized) or A60 (ZF180) zinc-iron-alloy (galvannealed) coating designation.

        - Panel-to-Panel Fasteners: Manufacturer's standard steel bolts, nuts, and washers.

        - Post-Installed Anchors: Capable of sustaining, without failure, a load equal to 6 times the load imposed when installed in unit masonry and 4 times the load imposed when installed in concrete, as determined by testing in accordance with ASTM E488/E488M, conducted by a qualified independent testing agency.

Material in "Material for Interior Locations" Subparagraph below protects against corrosion in an indoor atmosphere.

          - Material for Interior Locations: Carbon-steel components are zinc plated to comply with ASTM B633 or ASTM F1941/F1941M, Class Fe/Zn 5, unless otherwise indicated.

In "Material for Exterior Locations and Where Stainless Steel Is Indicated" Subparagraph below, Alloy Group 1 (A1) refers to Type 304 and similar alloys, and Alloy Group 2 (A4) refers to Type 316 and similar alloys.

          - Material for Exterior Locations and Where Stainless Steel Is Indicated: Alloy [Group 1 (A1)] [Group 2 (A4)] stainless steel bolts, ASTM F593 (ISO 3506), and nuts, ASTM F594 (ASTM F836M).

Retain "Power-Driven Fasteners" Paragraph below if power-driven fasteners are allowed.

        - Power-Driven Fasteners: ICC-ES AC70.

"Seismic Bracing" Paragraph below is an example only. Revise to suit Project.

        - Seismic Bracing: Angles with legs not less than 1-1/4 inches (32 mm) wide, formed from 0.040-inch- (1.0-mm-) thick, metallic-coated steel sheet; with bolted connections and 1/4-inch- (6-mm-) diameter bolts.

Retain one or more of "Shop Primers," "Universal Shop Primer," and "Zinc-Rich Primer" paragraphs below. Retain first paragraph if primers are specified in another Section. Retain second for a typical shop primer for painted finishes that provide minimum protection to steel. Retain third for a typical primer for high-performance coating. If retaining second or third paragraph, indicate location of each primer on Drawings or in a schedule.

        - Shop Primers: Provide primers that comply with [Section 099113 "Exterior Painting"] [and] [Section 099123 "Interior Painting."]

        - Universal Shop Primer: Fast-curing, lead- and chromate-free, universal modified-alkyd primer, complying with MPI#79.

Retain subparagraph below if zinc-rich primer is also used.

          - Use primer containing pigments that make it easily distinguishable from zinc-rich primer.

        - Zinc-Rich Primer: Compatible with topcoat, complying with SSPC-Paint 20 or SSPC-Paint 29.

Retain "Galvanizing Repair Paint" Paragraph below for galvanized fabrications.

        - Galvanizing Repair Paint: High-zinc-dust-content paint for regalvanizing welds in steel, complying with SSPC-Paint 20.

      - FABRICATION

        - General: Fabricate wire mesh items from components of sizes not less than those indicated. Use larger-sized components as recommended by wire mesh item manufacturer. Furnish bolts, hardware, and accessories required for complete installation with manufacturer's standard finishes.

          - Fabricate wire mesh items to be readily disassembled.

Retain one of four options in "Welding" Subparagraph below; options are listed with best appearance and highest cost first.

          - Welding: Weld corner joints of framing and [grind smooth, leaving no evidence of joint] [finish sand] [remove spatter] [leave as applied].

        - [Standard-] [and] [Heavy-]Duty Wire Mesh Partitions: Fabricate wire mesh partitions with cutouts for pipes, ducts, beams, and other items indicated. Finish edges of cutouts to provide a neat, protective edge.

          - Mesh: [Securely clinch] [Weld] mesh to framing.

          - Framing: Fabricate framing with mortise-and-tenon corner construction.

            - Provide horizontal stiffeners as indicated or, if not indicated, as required by panel height and as recommended by wire mesh partition manufacturer. Weld horizontal stiffeners to vertical framing.

            - Fabricate [three-] [and] [four-]way intersections using [intersection posts] [manufacturer's standard connecting clips and fasteners].

            - Fabricate partition and door framing with slotted holes for connecting adjacent panels.

Retain one of first two subparagraphs below.

          - Fabricate wire mesh partitions with 3 to 4 inches (75 to 100 mm) of clear space between finished floor and bottom horizontal framing.

          - Fabricate wire mesh partitions with bottom horizontal framing flush with finished floor.

          - Doors: Align bottom of door with bottom of adjacent panels.

            - For doors that do not extend full height of partition, provide transom over door, fabricated from same mesh and framing as partition panels.

          - Hardware Preparation: Mortise, reinforce, drill, and tap doors and framing as required to install hardware.

        - Wire Mesh Ceilings: Fabricate wire mesh partitions with cutouts for pipes, ducts, beams, and other items indicated. Finish edges of cutouts to provide a neat, protective edge.

          - Mesh: [Securely clinch] [Weld] mesh to framing.

          - Framing: Fabricate framing with [mortise-and-tenon] [welded] corner construction.

            - Provide stiffeners as indicated or, if not indicated, as required by panel span and as recommended by wire mesh ceiling manufacturer. Weld stiffeners to framing.

        - Wire Mesh Stairway Partitions: Provide door jamb framing on each side of doors. Attach tamper shields centered behind exit devices.

      - STEEL AND IRON FINISHES

        - Galvanizing: Hot-dip galvanize items as indicated to comply with ASTM A153/A153M for steel and iron hardware and with ASTM A123/A123M for other steel and iron products.

Retain subparagraph below if galvanized items are painted.

          - Do not quench or apply post-galvanizing treatments that might interfere with paint adhesion.

        - Preparation for Shop Priming Galvanized Items: After galvanizing, thoroughly clean items of grease, dirt, oil, flux, and other foreign matter, and treat with metallic phosphate process.

        - Preparation for Shop Priming: Prepare surfaces to comply with [SSPC-SP 6/NACE No. 3, "Commercial Blast Cleaning."] [SSPC-SP 3, "Power Tool Cleaning."]

Retain "Shop Priming," "Enamel Finish," or "Powder-Coat Finish" Paragraph below or insert another. Paint finish options for wire mesh units vary widely. For exact finish, insert names of coating manufacturers and products.

        - Shop Priming: Apply shop primer to uncoated surfaces of wire mesh units unless otherwise indicated. Comply with SSPC-PA 1, "Paint Application Specification No. 1: Shop, Field, and Maintenance Painting of Steel," for shop painting.

        - Enamel Finish: Immediately after cleaning and pretreating, apply manufacturer's standard enamel finish, suitable for use indicated, with a minimum dry film thickness of [2 mils (0.05 mm)] <Insert dimension>.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

        - Powder-Coat Finish: Immediately after cleaning and pretreating, apply manufacturer's standard baked-on powder-coat finish, suitable for use indicated, with a minimum dry film thickness of [2 mils (0.05 mm)] <Insert dimension>.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

- EXECUTION

      - EXAMINATION

        - Examine areas, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Examine floors for suitable conditions where wire mesh items will be installed.

        - Examine walls to which wire mesh items will be attached for properly located blocking, grounds, and other solid backing for attachment of support fasteners.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION OF WIRE MESH PARTITIONS

Retain one of first two paragraphs below. Installation method in second paragraph allows vertical adjustment.

        - Anchor wire mesh partitions to floor with 3/8-inch- (9.5-mm-) diameter, postinstalled expansion anchors at 12 inches (300 mm) o.c. through anchor clips located at each post and corner. Shim anchor clips as required to achieve level and plumb installation.

Retain subparagraph below if permitted.

          - Anchors may be set with power-actuated fasteners instead of postinstalled expansion anchors if indicated on Shop Drawings.

        - Anchor wire mesh partitions to floor with 3/8-inch- (9.5-mm-) diameter, postinstalled expansion anchors at 12 inches (305 mm) o.c. through floor shoes located at each post and corner. Adjust wire mesh partition posts in floor shoes to achieve level and plumb installation.

Retain subparagraph below if permitted.

          - Anchors may be set with power-actuated fasteners instead of postinstalled expansion anchors if indicated on Shop Drawings.

        - Anchor wire mesh partitions to walls at 12 inches (305 mm) o.c. through back corner panel framing and as follows:

Retain five subparagraphs below if applicable or revise to suit Project.

          - For concrete and solid masonry anchorage, use expansion anchors.

          - For hollow masonry anchorage, use toggle bolts.

          - For wood stud partitions, use lag bolts set into wood backing between studs. Coordinate with carpentry work to locate backing members.

Retain one of two subparagraphs below if steel studs are used. Revise if plaster is used rather than gypsum board.

          - For steel-framed gypsum board assemblies, use lag bolts set into wood backing between studs. Coordinate with stud installation to locate backing members.

          - For steel-framed gypsum board assemblies, fasten brackets directly to steel framing or concealed reinforcements using self-tapping screws of size and type required to support structural loads.

        - Secure top capping bars to top framing channels with 1/4-inch- (6-mm-) diameter, "U" bolts spaced not more than 28 inches (700 mm) o.c.

        - Provide line posts at locations indicated or, if not indicated, as follows:

Revise subparagraphs below to suit Project. Verify, with manufacturers, location and spacing for line posts, because their recommendations differ.

          - On each side of sliding-door openings.

          - For partitions that are 7 to 9 ft. (2.1 to 2.7 m) high, spaced at 15 to 20 ft. (4.6 to 6.1 m) o.c.

          - For partitions that are 10 to 12 ft. (3.0 to 3.7 m) high, located between every other panel.

          - For partitions that are more than 12 ft. (3.7 m) high, located between each panel.

Retain first paragraph below if required.

        - Provide seismic supports and bracing as indicated or, if not indicated, as recommended by manufacturer and as required for stability, extending and fastening members to supporting structure.

Retain first paragraph below if custom-width wire mesh partition panels are not required and dimensions are not indicated on Drawings.

        - Where standard-width wire mesh partition panels do not fill entire length of run, provide adjustable filler panels to fill openings.

        - Install doors complete with door hardware.

        - Install service windows complete with window hardware.

        - Weld or bolt sheet metal bases to [wire mesh partitions] [and] [doors] [where indicated].

        - Bolt accessories to wire mesh partition framing.

      - INSTALLATION OF WIRE MESH CEILINGS

        - Anchor wall support angle to walls at 12 inches (300 mm) o.c. and as follows:

Retain subparagraphs below if applicable or revise to suit Project.

          - For concrete and solid masonry anchorage, use expansion anchors.

          - For hollow masonry anchorage, use toggle bolts.

          - For wood stud partitions, use lag bolts set into wood backing between studs. Coordinate with carpentry work to locate backing members.

Retain one of two subparagraphs below if steel studs are used. Revise if plaster is used rather than gypsum board.

          - For steel-framed gypsum board assemblies, use lag bolts set into wood backing between studs. Coordinate with stud installation to locate backing members.

          - For steel-framed gypsum board assemblies, fasten brackets directly to steel framing or concealed reinforcements using self-tapping screws of size and type required to support structural loads.

        - Attach wire mesh ceiling panels to wall support angles with bolts at 12 inches (305 mm) o.c.

        - Attach wire mesh ceiling panels to wire mesh partitions with slotted angles bolted to sides of ceiling panels and to top of partitions at 12 inches (305 mm) o.c.

        - Attach wire mesh ceiling panels to intermediate supports as recommended by manufacturer.

Retain paragraph below if required.

        - Provide seismic supports and bracing as indicated or, if not indicated, as recommended by manufacturer and as required for stability, extending and fastening members to supporting structure.

      - INSTALLATION OF WIRE MESH STAIRWAY PARTITIONS

Retain one of first two paragraphs below. Installation method in second paragraph allows vertical adjustment.

        - Anchor wire mesh stairway partitions to floor with 3/8-inch- (9.5-mm-) diameter, postinstalled expansion anchors at 12 inches (305 mm) o.c. through floor shoes located at each post. Adjust wire mesh partition posts in floor shoes to achieve level and plumb installation.

Retain subparagraph below if permitted.

          - Anchors may be set with power-actuated fasteners instead of postinstalled expansion anchors if indicated on Shop Drawings.

        - Anchor angle clips supporting wire mesh stairway partitions at stairs and intermediate landings with 3/8-inch- (9.5-mm-) diameter, postinstalled expansion anchors at 12 inches (305 mm) o.c. Weld stairway partition framing to angle clips.

Retain subparagraph below if permitted.

          - Anchors may be set with power-actuated fasteners instead of postinstalled expansion anchors if indicated on Shop Drawings.

Retain first paragraph below if required.

        - Provide seismic supports and bracing as indicated or, if not indicated, as recommended by manufacturer and as required for stability; extend and fasten members to supporting structure.

        - Install doors complete with door hardware.

      - INSTALLATION OF WIRE MESH EQUIPMENT BARRIERS

        - Anchor wire mesh equipment barriers to floor with 3/8-inch- (9.5-mm-) diameter, expansion anchors through post bases. Shim post bases as required to achieve level and plumb installation.

Retain first paragraph below if permitted.

        - Floor anchors may be set with power-actuated fasteners instead of postinstalled expansion anchors if indicated on Shop Drawings.

        - Install gates complete with gate hardware.

      - REPAIR

        - Repair Painting:

Retain first subparagraph below if touchup painting is included in this Section.

          - Wire brush and clean rust spots, welds, and abraded areas immediately after installation, and apply repair paint with same material as used for shop painting to comply with SSPC-PA 1 requirements for touching up shop-painted surfaces.

            - Apply by brush or spray to provide a minimum 2.0-mil (0.05-mm) dry film thickness.

Retain subparagraph below if repair painting is included in Section 099113 "Exterior Painting" and Section 099123 "Interior Painting." Revise reference if using another Section, such as Section 099600 "High-Performance Coatings."

          - Wire brushing, cleaning, and repair painting of rust spots, welds, and abraded areas are included in Section 099113 "Exterior Painting" and Section 099123 "Interior Painting."

        - Repair of Galvanized Surfaces: Clean field welds, bolted connections, and abraded areas and repair galvanizing to comply with ASTM A780/A780M.

      - ADJUSTING

        - Adjust [doors] [gates] [service windows] to operate smoothly and easily, without binding or warping. Adjust hardware to function smoothly. Verify that latches and locks engage accurately and securely without forcing or binding.

      - PROTECTION

        - Remove and replace defective work, including doors and framing that are warped, bowed, or otherwise unacceptable.

END OF SECTION 102213