Copyright 2022 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 102800 - TOILET, BATH, AND LAUNDRY ACCESSORIES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Access Manufacturer-Provided, AIA MasterSpec-Based Sections:

<Double click here for this Section based on specific manufacturer’s products set as Basis-of-Design at ProductMasterSpec.com.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Public-use washroom accessories.

          - Toilet-compartment occupancy-indicator system.

          - Public-use shower room accessories.

          - Private-use bathroom accessories.

          - Healthcare accessories.

          - Childcare accessories.

          - Underlavatory guards.

          - Custodial accessories.

          - Hand-sanitizer dispensers.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 088300 "Mirrors" for frameless mirrors.

          - Section 093013 "Ceramic Tiling" for ceramic toilet and bath accessories.

          - Section 102813.63 "Detention Toilet Accessories" for accessories designed for installation in detention facilities.

      - ALLOWANCES

Retain this article if products and Work included in this Section are covered by lump-sum, unit-cost, quantity, or testing and inspection allowances.

        - See Section 012100 "Allowances" for description of allowances affecting items specified in this Section.

      - UNIT PRICES

Retain this article if products and Work specified in this Section are measured and paid for under the provisions of unit prices.

        - See Section 012200 "Unit Prices" for description of unit prices affecting items specified in this Section.

      - ALTERNATES

Retain this article if products and Work specified in this Section are proposed to be added to or deducted from the base bid amount under the provisions of alternates.

        - See Section 012300 "Alternates" for description of alternates affecting items specified in this Section.

      - COORDINATION

        - Coordinate accessory locations with other work to prevent interference with clearances required for access by people with disabilities, and for proper installation, adjustment, operation, cleaning, and servicing of accessories.

        - Deliver inserts and anchoring devices set into concrete or masonry as required to prevent delaying the Work.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Public-use washroom accessories.

          - Toilet-compartment occupancy-indicator system.

          - Public-use shower room accessories.

          - Private-use bathroom accessories.

          - Healthcare accessories.

          - Childcare accessories.

          - Underlavatory guards.

          - Custodial accessories.

          - Hand-sanitizer dispensers.

        - Product Data Submittals: For each product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes.

          - Include anchoring and mounting requirements, including requirements for cutouts in other work and substrate preparation.

Retain subparagraph below if units are required that connect to building electrical system.

          - Include electrical characteristics.

If retaining "Samples" Paragraph below, consider revising to limit Sample requirements to specific items.

        - Samples: For each exposed product and for each finish specified, full size.

Retain subparagraph below if applicable.

          - Approved full-size Samples will be returned and may be used in the Work.

        - Product Schedule: Indicating types, quantities, sizes, and installation locations by room of each accessory required.

          - Identify locations using room designations indicated.

Retain subparagraph below if product designations are inserted in the Section Text.

          - Identify accessories using designations indicated.

Retain "Delegated Design Submittals" Paragraph below if design services have been delegated to Contractor. The IBC requires grab bars and shower seats to be able to resist concentrated load of 250 lbf (1112 N) applied in any direction and at any point. Authorities having jurisdiction might require calculations for these accessories, especially if they are mounted on panels of toilet and shower compartments.

        - Delegated Design Submittals: For [grab bars] [and] [shower seats].

          - Include structural design calculations indicating compliance with specified structural-performance requirements.

      - INFORMATIONAL SUBMITTALS

Informational submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

Retain "Sample Warranty" Paragraph below with mirror, hand dryer, or toilet-compartment occupancy-indicator warranty requirements in "Warranty" Article.

        - Sample Warranty: For manufacturer's special warranties.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For accessories to include in maintenance manuals.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Manufacturer's Special Warranty for Mirrors: Manufacturer agrees to repair or replace mirrors that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, visible silver spoilage defects.

Not all manufacturers offer a special mirror warranty. Verify available warranties and warranty periods.

          - Warranty Period: [10] [15] <Insert number> years from date of Substantial Completion.

        - Manufacturer's Special Warranty for Toilet-Compartment Occupancy-Indicator Systems: Manufacturer agrees to repair or replace toilet-compartment occupancy-indicator systems that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods.

          - Warranty Period: [Five] <Insert number> years from date of Substantial Completion.

        - Manufacturer's Special Warranty for Hand Dryers: Manufacturer agrees to repair or replace hand dryers that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods.

          - Warranty Period: [Two] [Five] [10] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - OWNER-FURNISHED MATERIALS

For large commercial projects, campus-type projects with multiple buildings, or similar situations, Owner might have separate service contracts for furnishing various accessories and corresponding supplies, such as towel dispensers and paper towels. In some cases, suppliers install these accessories; this requires coordination with Drawings and accessibility requirements. If Owner will furnish accessories to Contractor for installation, retain this article and insert requirements in "Owner-Furnished Materials" Paragraph below.

        - Owner-Furnished Materials: <Insert products>.

      - PERFORMANCE REQUIREMENTS

Retain this article if accessories that connect to building electrical systems, such as hand dryers, adjustable-height adult changing stations, and toilet-compartment occupancy-indicator systems, are required.

        - Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

        - Structural Performance: Design accessories and fasteners to comply with the following requirements:

Retain "Grab Bars" and "Shower Seats" subparagraphs below if required. The IBC requires grab bars and shower seats to be able to resist concentrated load of 250 lbf (1112 N) applied in any direction and at any point. Shower seats often resist greater loads.

          - Grab Bars: Installed units are able to resist 250 lbf (1112 N) concentrated load applied in any direction and at any point.

          - Shower Seats: Installed units are able to resist [250 lbf (1112 N)] [360 lbf (1601 N)] <Insert load> concentrated load applied in any direction and at any point.

      - PUBLIC-USE WASHROOM ACCESSORIES

Retain one of two options in "Source Limitations" Paragraph below or revise to suit Project. Retain first option if manufacturers can provide all public-use washroom accessories required for Project.

        - Source Limitations: Obtain [public-use washroom accessories] [each type of public-use washroom accessory] from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Toilet Tissue (Roll) Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Roll-in-reserve dispenser with hinged front secured with tumbler lockset] [Single-roll dispenser] [Double-roll dispenser] [Double-roll dispenser with shelf] <Insert description>.

          - Mounting: [Recessed] [Partition mounted, serving two adjacent toilet compartments] [Surface mounted].

Controlled-delivery units described in "Operation" Subparagraph below cannot be used at accessible toilets.

          - Operation: [Noncontrol delivery with standard spindle] [Noncontrol delivery with theft-resistant spindle] [Spindleless with tension-spring controlled delivery] [Spindleless with tension-spring controlled delivery and self-locking device extending through core that prevents core removal until roll is empty] [Eccentric-shaped, molded-plastic spindle revolves one-half revolution per dispensing operation for controlled delivery; core cannot be removed until roll is empty] <Insert description>.

          - Capacity: Designed for [4-1/2- or 5-inch- (114- or 127-mm-)] [5-inch- (127-mm-)] <Insert dimension> diameter tissue rolls.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Chrome-plated zinc alloy (zamac) or steel] [Satin-finish aluminum bracket with plastic spindle] <Insert material and finish>.

Combination toilet tissue dispenser units are available with sanitary-napkin disposal units or seat-cover dispensers, or both.

        - Combination Toilet Tissue Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Combination unit with double-roll toilet tissue dispenser and the following:

            - Removable sanitary-napkin waste receptacle with self-closing, disposal-opening cover.

            - Seat-cover dispenser with minimum capacity of [500] [1000] <Insert number> single or half-fold seat covers.

          - Mounting: [Recessed] [Surface mounted] [Partition mounted, dual access with two tissue rolls per compartment] [Partition mounted, dual access with two tissue rolls per compartment and with one side that mounts flush with partition of accessible compartment].

          - Toilet Tissue Dispenser Capacity: [4-1/2- or 5-inch- (114- or 127-mm-)] <Insert dimension> diameter tissue rolls.

          - Toilet Tissue Dispenser Operation: [Noncontrol delivery with theft-resistant spindles] <Insert description>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Toilet Tissue (Jumbo-Roll) Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [One-roll unit] [Two-roll unit with sliding panel to expose other roll].

          - Mounting: Surface mounted.

          - Capacity: [9- or 10-inch- (228- or 254-mm-)] <Insert dimension> diameter rolls.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray, with translucent front cover] <Insert material and finish>.

          - Lockset: Tumbler type.

          - Refill Indicator: Pierced slots at front.

        - Paper Towel (Folded) Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Recessed] [Semirecessed] [Deck mounted, recessed] [Surface mounted].

Single-fold towels usually require 6-inch- (152-mm-) deep dispensers. C-fold or multifold towels fit in 4-inch- (102-mm-) deep dispensers. C-fold towels are the highest quality folded towels; single-fold towels are the least expensive.

          - Minimum Capacity: [400 C-fold or 525 multifold towels] [600 C-fold or 800 multifold towels] [400 single-fold towels] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

Retain "Refill Indicator" Subparagraph below if required.

          - Refill Indicator: Pierced slots at sides or front.

Roll towel dispensers extend farther from the wall than folded towel dispensers.

        - Paper Towel (Roll) Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Lever-actuated] [Pull-towel-actuated] mechanism that permits controlled delivery of paper rolls in preset lengths.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

          - Minimum Capacity: [8-inch- (203-mm-) wide, 800-foot- (244-m-) long roll] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray, with translucent front cover] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Automatic Paper Towel (Roll) Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Automatic motion-sensing mechanism with user-adjustable delay and paper towel length; [electrically operated, with adapter for 110 to 240 V ac power supply] [battery powered] <Insert description>.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

          - Minimum Capacity: [8-inch- (203-mm-) wide, 800-foot- (244-m-) long roll] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Waste Receptacle <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Open top, recessed] [Self-closing disposal-opening cover, recessed] [Semirecessed] [Surface mounted] [Wall mounted for corner installation] [Freestanding] [Undercounter] <Insert description>.

          - Minimum Capacity: <Insert value>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Liner: [Reusable vinyl liner] <Insert liner description>.

          - Lockset: Tumbler type for waste receptacle.

        - Countertop-Mounted Circular Waste Chute <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - OD: [6 inches (152 mm)] [7 inches (178 mm)] [9 inches (229 mm)] <Insert dimension>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Combination Towel (Folded) Dispenser/Waste Receptacle <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Combination unit for dispensing C-fold or multifold towels, with removable waste receptacle.

          - Mounting: [Surface mounted] [Surface mounted with stainless steel collar] [Recessed] [Recessed with projecting receptacle] [Semirecessed] [Under vanity] <Insert mounting>.

Retain first subparagraph below for recessed or semirecessed units.

            - Designed for nominal [4-inch (100-mm)] [6-inch (150-mm)] wall depth.

          - Minimum Towel-Dispenser Capacity: [600 C-fold or 800 multifold paper towels] <Insert capacity>.

          - Minimum Waste-Receptacle Capacity: [4 gal. (15 L)] [12 gal. (45.4 L)] <Insert value>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Liner: [Reusable, vinyl waste-receptacle liner] <Insert liner description>.

          - Lockset: Tumbler type for towel-dispenser compartment[ and waste receptacle].

        - Combination Towel (Roll) Dispenser/Waste Receptacle <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Combination unit for dispensing preset length of roll paper towels, with removable waste receptacle.

          - Towel Mechanism: [Lever] [Pull towel] [Automatic, electric sensor; with adapter for 110 to 240 V ac power supply] [Automatic, battery-operated sensor] <Insert description>.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

          - Minimum Towel-Dispenser Capacity: [8-inch- (203-mm-) wide, 800-foot- (244-m-) long roll] <Insert capacity>.

          - Minimum Waste Receptacle Capacity: [8 gal. (30 L)] [12 gal. (45.4 L)] [15 gal. (56.8 L)] <Insert value>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Liner: [Reusable, vinyl waste-receptacle liner] <Insert liner description>.

          - Lockset: Tumbler type for towel dispenser compartment[ and waste receptacle].

        - Multipurpose Soap/Towel Dispenser Unit <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Combination unit for dispensing soap in [liquid or lotion] [lather] form and folded towels.

          - Mounting: [Recessed, designed for nominal 4-inch (100-mm) wall depth] [Surface mounted with stainless steel collar and mirror] <Insert mounting>.

          - Minimum Soap-Dispenser Capacity: [80 oz. (2.36 L)] <Insert value>.

          - Minimum Towel-Dispenser Capacity: [600 C-fold or 800 multifold towels] [1000 single-fold towels] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish> for unit body and soap valve.

          - Lockset: Tumbler type.

        - Warm-Air Hand Dryer <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Standard-speed, warm-air hand dryer.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

Retain "Protrusion Limit" Subparagraph below for surface-mounted or semirecessed units that comply with the DOJ's "2010 ADA Standards for Accessible Design" and with ICC A117.1, which limit protrusion of objects in access aisleways with leading edges between 27 and 80 inches (686 and 2032 mm) above the finished floor to 4 inches (102 mm). Recessed units can also comply with the accessibility standards. Verify requirements of authorities having jurisdiction.

            - Protrusion Limit: Installed unit protrudes maximum 4 inches (102 mm) from wall surface.

          - Operation: [Touch-button] [Infrared-sensor] activated with timed power cutoff switch.

            - Automatic Shutoff: At [40] [80] [120] <Insert number> seconds.

          - Maximum Sound Level: [63] [67] [87] <Insert number> dB.

          - Cover Material and Finish: [Steel, with white enamel finish] [Cast iron, with enamel finish in color selected by Architect] [Chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Molded plastic, gray] [Molded plastic, white] <Insert material and finish>.

          - Electrical Requirements: [115 V, 13 A, 1500 W] [115 V, 15 A, 1725 W] [115 V, 20 A, 2300 W] [208 to 240 V, 9 to 10 A, 1900 to 2300 W] <Insert requirements>.

        - High-Speed Air Hand Dryer <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: High-speed, [warm] [unheated]-air hand dryer for rapid hand drying.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

Retain "Protrusion Limit" Subparagraph below for surface-mounted or semirecessed units that comply with the DOJ's "2010 ADA Standards for Accessible Design" and with ICC A117.1, which limit protrusion of objects in access aisleways with leading edges between 27 and 80 inches (686 and 2032 mm) above the finished floor to 4 inches (102 mm). Recessed units can also comply with the accessibility standards. Verify requirements of authorities having jurisdiction.

            - Protrusion Limit: Installed unit protrudes maximum 4 inches (102 mm) from wall surface.

          - Operation: Infrared-sensor activated with timed power cutoff switch.

            - Average Dry Time: [12] <Insert number> seconds.

            - Automatic Shutoff: At [60] <Insert number> seconds.

          - Maximum Sound Level: [69] [75] <Insert number> dB.

          - Cover Material and Finish: [Steel, with white enamel finish] [Cast iron, with enamel finish in color selected by Architect] [Chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Molded plastic, gray] [Molded plastic, white] <Insert material and finish>.

          - Electrical Requirements: [115 V, 13 A, 1500 W] [115 V, 15 A, 1725 W] [115 V, 20 A, 2300 W] [208 to 240 V, 9 to 10 A, 1900 to 2300 W] <Insert requirements>.

        - Multiple-Airflow Hand Dryer <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Multiple-airflow hand dryer, using two or more airstreams for rapid hand drying[ with heating unit on-off control].

Revise "Mounting" Subparagraph below if required. The DOJ's "2010 ADA Standards for Accessible Design" and ICC A117.1 limit protrusion of objects with leading edges between 27 and 80 inches (686 and 2032 mm) above the finished floor to 4 inches (102 mm). Verify requirements of authorities having jurisdiction.

          - Mounting: [Surface mounted] <Insert requirements>.

          - Operation: Electronic-sensor activated with timed power cutoff switch.

            - Average Dry Time: [12] <Insert number> seconds.

            - Automatic Shutoff: At [60] <Insert number> seconds.

          - Maximum Sound Level: [56] [75] <Insert number> dB.

          - Water Collection: Removable reservoir.

          - Filter: HEPA, replaceable.

          - Cover Material and Finish: [Steel, with white enamel finish] [Cast iron, with enamel finish in color selected by Architect] [Chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Molded plastic, gray] [Molded plastic, white] <Insert material and finish>.

          - Electrical Requirements: [115 V, 13 A, 1500 W] [115 V, 15 A, 1725 W] [115 V, 20 A, 2300 W] [208 to 240 V, 9 to 10 A, 1900 to 2300 W] <Insert requirements>.

Coordinate requirements for soap dispensers with other Sections if a central soap storage tank is included.

        - Soap Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Designed for manual operation and dispensing soap in [liquid or lotion] [lather] form.

          - Mounting: [Deck mounted on vanity] [Deck mounted on lavatory] [Horizontally oriented, recessed] [Horizontally oriented, surface mounted] [Vertically oriented, surface mounted] <Insert mounting>.

          - Capacity: <Insert value>.

          - Materials: <Insert requirements>.

Retain "Lockset" and "Refill Indicator" subparagraphs below if required.

          - Lockset: Tumbler type.

          - Refill Indicator: Window type.

        - Automatic Soap Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Automatic dispenser with infrared sensor to detect presence of hands; [electrically operated, with adapter for 110 to 240 V ac power supply] [battery powered]; designed for dispensing soap in [liquid or lotion] [lather] form.

Some manufacturers may provide wall-mounted automatic units.

          - Mounting: [Deck mounted on vanity] [Deck mounted on lavatory] [Surface mounted].

          - Capacity: <Insert value>.

          - Materials: <Insert requirements>.

Retain "Refill Indicator" and "Low-Battery Indicator" subparagraphs below if required.

          - Refill Indicator: LED indicator.

          - Low-Battery Indicator: LED indicator.

        - Grab Bar <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: Flanges with [concealed] [exposed] fasteners.

          - Material: Stainless steel, 0.05 inch (1.3 mm) thick.

According to some manufacturers, a satin finish surface can provide slip-resistance performance similar to that of a textured surface.

            - Finish: Smooth, ASTM A480/A480M No. 4 finish (satin)[ on ends and slip-resistant texture in grip area].

Retain first option in "OD" Subparagraph below for medium-duty applications; second, for heavy duty.

          - OD: [1-1/4 inches (32 mm)] [1-1/2 inches (38 mm)].

          - Configuration and Length: [As indicated on Drawings] [Straight, 36 inches (914 mm) long] <Insert configuration and length>.

        - Sanitary-Napkin and Tampon Vendor <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Fully recessed, designed for 4-inch (100-mm) wall depth] [Semirecessed] [Surface mounted].

Capacity varies among manufacturers. For competitive bidding, consider indicating minimum capacities based on Project requirements.

          - Capacity: <Insert description>.

          - Operation: [No coin (free)] [Single coin (25 cents)] [Two coins (50 cents)] <Insert description>.

          - Exposed Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Lockset: Tumbler type with separate lock and key for coin box.

        - Sanitary-Napkin Disposal Unit <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Recessed] [Partition mounted, dual access] [Surface mounted].

          - Door or Cover: Self-closing, disposal-opening cover[ and hinged face panel with tumbler lockset].

          - Receptacle: Removable.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

        - Seat-Cover Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Surface mounted] [Recessed] [Partition mounted, dual access].

          - Minimum Capacity: [250] [500] <Insert number> seat covers.

          - Exposed Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Purse Shelf <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Fixed rectangular unit] [Hinged unit with spring-loaded shelf that automatically returns to vertical position] <Insert description>.

          - Nominal Size: [15 inches (381 mm) long by 5-1/2 inches (140 mm) wide] <Insert dimensions>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Chrome-plated, cast-zinc alloy (zamac) with stippled finish on tray and bright chrome finish on edges] <Insert material and finish>.

        - Mirror Unit <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

In "Frame" Subparagraph below, options for tilted mirrors and adjustable tilting mirrors are for use by people with disabilities. Adjustable tilting mirrors are prone to vandalism. In lieu of tilted mirrors, standard flat mirrors can be mounted at heights to accommodate users in wheelchairs.

          - Frame: [Stainless steel angle, 0.05 inch (1.3 mm) thick] [Stainless steel channel] [Stainless steel, fixed tilt] [Stainless steel, adjustable tilt].

            - Corners: [Manufacturer's standard] [Mitered and mechanically interlocked] [Welded and ground smooth].

          - Size: [As indicated on Drawings] <Insert dimensions>.

Retain "Shelf" Subparagraph below if required.

          - Shelf:

            - Type: [Integral, welded] [Concealed mounting] [Exposed mounting].

            - Depth: [5 inches (127 mm)] <Insert dimension>.

          - Hangers: [Manufacturer's standard rigid, tamper and theft resistant] <Insert requirements>.

        - Hook <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Double-prong unit] [Single-prong unit] [Combination hat and coat hook] [Combination door bumper and coat hook] <Insert description>.

          - Mounting: [Concealed] [Exposed].

          - Material and Finish: [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Fixed-Height Adult Changing Station <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Horizontal unit that opens by folding down from stored position and with adjustable strap.

            - Engineered to support minimum of [400 lb (181 kg)] <Insert value> static load when opened.

          - Mounting: [Surface mounted, with unit projecting not more than 4 inches (102 mm) from wall when closed] [Semirecessed, with unit projecting not more than 1 inch (25 mm) from wall when closed].

          - Operation: By pneumatic shock-absorbing mechanism.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin), with replaceable insulated polystyrene tray liner and rounded plastic corners] <Insert requirements>.

          - Liner Dispenser: Provide [built-in] [separate, locking] dispenser for disposable sanitary liners.

        - Adjustable-Height Adult Changing Station <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Height-adjustable horizontal unit that is electrically operated with wired hand control and with safety rail[ and receiver tray][ and receiver tray with integrated drain outlet].

            - Engineered to support minimum of [300 lb (136 kg)] [350 lb (159 kg)] [400 lb (181 kg)] <Insert value> static load when opened.

          - Mounting: Surface mounted, foldable by pneumatic shock-absorbing mechanism.

          - Electrical Characteristics: Manufacturer's standard actuator and control system, with integrated 24 V dc transformer, powered by single 120 V electrical receptacle.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin), with PVC mattress] <Insert requirements>.

      - TOILET-COMPARTMENT OCCUPANCY-INDICATOR SYSTEM

Copy paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

Products described in "Toilet-Compartment Occupancy-Indicator System" Paragraph below are unsuitable for wet locations, such as shower compartments.

        - Toilet-Compartment Occupancy-Indicator System <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Battery-powered latch and electrically powered indicator-light system. Latches communicate wirelessly with separate, overhead indicator lights.

          - Latch: Slide operation.

            - Compatibility: Provide latch compatible with compartment doors indicated for Project.

            - Power Source: Two AA batteries.

              - Low-Battery Warning: Indicator light pulses when associated batteries of corresponding latch require replacement.

Retain "Patch Kit" Subparagraph below for installation on existing toilet compartments.

            - Patch Kit: Stainless steel in manufacturer's standard finish, adhesively applied to interior and exterior of compartment door to cover existing hardware locations.

            - Latch Material and Finish: Matte chrome-plated, die-cast zinc alloy (zamac); installed with stainless steel fasteners.

          - Indicator Light: Full-color, low-voltage, LED indicator that displays red when compartment door is latched, green when standard compartment door is unlatched, and blue when accessible compartment door is unlatched.

Retain "Traffic-Management-System Operation" Subparagraph below if required.

            - Traffic-Management-System Operation: Indicator lights also controlled by traffic management system for remote [override of indicator light color] [and] [data collection from indicator lights].

            - Mounting: [Ceiling, surface] [Ceiling, semirecessed] [Ceiling, pendant] [Wall] mounted.

            - Body Material and Finish: Satin white finish ABS plastic; installed with stainless steel fasteners.

            - Lens Material and Finish: [Clear] [Frosted] acrylic.

            - Electrical Requirements: Provide indicator lights with dc adapters and that allow up to 30 lights to be powered by single 115 V, 20 A, electrical receptacle.

      - PUBLIC-USE SHOWER ROOM ACCESSORIES

Retain one of two options in "Source Limitations" Paragraph below or revise to suit Project. Retain first option if manufacturers can provide all public-use shower room accessories required for Project.

        - Source Limitations: Obtain [public-use shower room accessories] [each type of public-use shower room accessory] from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Shower Curtain Rod <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Retain first option in "Description" Subparagraph below for normal-duty applications; second, for heavy duty.

          - Description: [1-inch- (25.4-mm-)] [1-1/4-inch- (32-mm-)] OD, straight rod.

          - Configuration: [As indicated on Drawings] <Insert description>.

          - Mounting Flanges: [Exposed] [Concealed] <Insert requirements> fasteners; in [manufacturer's standard material and finish] [material and finish matching rod] <Insert requirements>.

          - Rod Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Polished chrome-plated brass] <Insert material and finish>.

        - Shower Curtain <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Curtains that are 12 inches (305 mm) wider than opening are recommended for shower openings 48 inches (1219 mm) wide and wider.

          - Size: Minimum [6 inches (152 mm)] [12 inches (305 mm)] wider than opening by 72 inches (1829 mm) high.

          - Material: [Vinyl, minimum 0.006 inch (0.15 mm) thick, opaque, matte] [Duck, minimum 8 oz. (227 g), white, 100 percent cotton] [Nylon-reinforced vinyl, minimum 9 oz. (255 g) or 0.008-inch- (0.2-mm-) thick vinyl, with integral antibacterial and flame-retardant agents] <Insert material>.

Antibacterial shower curtains are available in green.

          - Color: [White] [Beige] [Green] [As selected from manufacturer's full range] <Insert color>.

          - Grommets: Corrosion resistant at minimum 6 inches (152 mm) o.c. through top hem.

          - Shower Curtain Hooks: [Chrome-plated or stainless steel, spring wire curtain hooks with snap fasteners] <Insert description>, sized to accommodate specified curtain rod. Provide one hook per curtain grommet.

        - Folding Shower Seat <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Configuration: [L-shaped seat, designed for wheelchair access] [Rectangular seat] [Triangular, corner-type seat] [Stainless steel seat designed to fold into recessed-mounted, stainless steel wall box] <Insert description>.

          - Seat: [Phenolic or polymeric composite of slat-type or one-piece construction in color as selected by Architect] [White vinyl padded seat] [Stainless steel, ASTM A480/A480M No. 4 finish (satin); 0.05-inch- (1.3-mm-) minimum nominal thickness; with single-piece, pan-type construction and edge seams welded and ground smooth] <Insert description>.

          - Mounting Mechanism: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert description>.

          - Dimensions: <Insert dimensions>.

        - Soap Dish <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Recessed] [Surface] mounted, with the following features:

            - Washcloth bar.

            - <Insert requirements>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

        - Robe Hook <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Double] [Single]-prong unit.

          - Material and Finish: [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

      - PRIVATE-USE BATHROOM ACCESSORIES

Retain one of two options in "Source Limitations" Paragraph below or revise to suit Project. Retain first option if manufacturers can provide all private-use bathroom accessories required for Project.

        - Source Limitations: Obtain [private-use bathroom accessories] [each type of private-use bathroom accessory] from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Private-Use Toilet Tissue Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Single] [Double]-roll dispenser with the following features:

            - Hood.

            - <Insert requirements>.

          - Mounting: [Recessed] [Surface mounted].

          - Capacity: Designed for 4-1/2- or 5-inch- (114- or 127-mm-) diameter tissue rolls.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Private-Use Shower Curtain Rod <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [1-inch- (25.4-mm-)] [1-1/4-inch- (32-mm-)] OD, [straight] [curved] rod.

          - Configuration: [As indicated on Drawings] <Insert description>.

          - Mounting Flanges: [Exposed] [Concealed] <Insert requirements> fasteners; in [manufacturer's standard material and finish] [material and finish matching rod] <Insert requirements>.

          - Rod Material and Finish: [Solid brass, polished] [Polished chrome-plated brass] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

Retain "Features" Subparagraph below if required.

          - Features: Integral chrome-plated brass glide hooks.

        - Private-Use Folding Shower Seat <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Configuration: [L-shaped seat, designed for wheelchair access] [Rectangular seat] [Triangular, corner-type seat] [Stainless steel seat designed to fold into recessed-mounted, stainless steel wall box] <Insert description>.

          - Seat: [Phenolic or polymeric composite of slat-type or one-piece construction in color as selected by Architect] [White vinyl padded seat] [Stainless steel, ASTM A480/A480M No. 4 finish (satin); 0.05-inch- (1.3-mm-) minimum nominal thickness; with single-piece, pan-type construction and edge seams welded and ground smooth] <Insert description>.

          - Mounting Mechanism: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert description>.

          - Dimensions: <Insert dimensions>.

        - Private-Use Soap Dish <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: <Insert description>.

          - Mounting: [Recessed] [Surface mounted].

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Private-Use Medicine Cabinet <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Recessed, for nominal 4-inch (100-mm) wall depth] [Surface mounted].

          - Size: [18 by 24 inches (457 by 610 mm)] <Insert dimensions>.

          - Door: [Framed mirror door concealing storage cabinet equipped with continuous hinge and spring-buffered, rod-type stop and magnetic door catch] <Insert description>.

          - Shelves: [Three, adjustable] <Insert requirements>.

          - Material and Finish:

            - Cabinet: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Steel with corrosion-resistant finish].

            - Mirror Frame: <Insert material and finish>.

            - Door: <Insert material and finish>.

            - Hinge: <Insert material and finish>.

            - Shelves: <Insert material and finish>.

        - Private-Use Facial Tissue Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Recessed] [Surface mounted] <Insert requirements>.

          - Depth: [2-5/8 inches (67 mm)] [4 inches (102 mm)] <Insert dimension>.

          - Material and Finish:

            - Dispenser Face: [Polished chrome-plated brass] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

            - Cabinet: [Steel with corrosion-resistant finish] <Insert material and finish>.

        - Private-Use Robe Hook <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Double] [Single]-prong unit.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Private-Use Toothbrush and Tumbler Holder <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: <Insert description>.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Private-Use Towel Bar <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [3/4-inch- (19-mm-) square tube with rectangular end brackets] [3/4-inch- (19-mm-) round tube with circular end brackets] <Insert description>.

          - Mounting: Flanges with [concealed] [exposed] fasteners.

          - Length: [18 inches (457 mm)] [24 inches (610 mm)] [30 inches (762 mm)] <Insert dimension>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] [Polished aluminum] <Insert material and finish>.

        - Private-Use Towel Pin <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Projecting minimum of [3 inches (76 mm)] [5 inches (127 mm)] <Insert dimension> from wall surface.

          - Material and Finish: [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Private-Use Towel Ring <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Pin projecting approximately [2-1/2 inches (64 mm)] <Insert dimension> from wall with [square] [circular] [oval] [trapezoidal] <Insert requirements> ring.

          - Pin Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

          - Ring Material and Finish: [Matching pin] [Clear plastic] <Insert requirements>.

        - Private-Use Towel Shelf <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Surface-mounted, guest-towel shelf with [four] <Insert number> stainless steel tubes, [3/8 inch (10 mm) in diameter] [5/16 inch (8 mm) square] <Insert requirements>, mounted in support arms.

Retain "Towel Bar" Subparagraph below if required; if retaining, revise to suit Project.

            - Towel Bar: [1/4-inch- (6-mm-) diameter] [5/16-inch- (8-mm-) square] stainless steel towel bar below shelf.

          - Length: [18 inches (457 mm)] [24 inches (610 mm)] <Insert dimension>.

          - Material and Finish: [Polished brass-plated, stainless steel] [Polished chrome-plated, stainless steel] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Private-Use Towel Rack <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Surface-mounted, guest-towel unit with approximately 1/4-inch- (6-mm-) diameter wire rings welded to upright wire bracket] <Insert description>.

          - Capacity: [Two] [Three] [Four] <Insert number> sets of bath towels, hand towels, and washcloths.

          - Nominal Height: [11 inches (279 mm)] [17 inches (432 mm)] [21 inches (533 mm)] <Insert dimension>.

          - Material and Finish: [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated zinc alloy (zamac)] <Insert material and finish>.

        - Private-Use Retractable Clothesline <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Surface-mounted [rectangular] [circular] housing with minimum 72-inch- (1829-mm-) long, retractable, spring-actuated, synthetic clothesline and remote retention bracket.

          - Material and Finish [Chrome-plated brass] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

      - HEALTHCARE ACCESSORIES

Retain one of two options in "Source Limitations" Paragraph below or revise to suit Project. Retain first option if manufacturers can provide all healthcare accessories required for Project.

        - Source Limitations: Obtain [healthcare accessories] [each type of healthcare accessory] from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Specimen Pass-Through Cabinet <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Two-sided type, with self-closing, interlocking doors on both sides, that prevent both from being open at same time, and removable stainless steel tray] [Turntable type, with minimum 12-inch- (305-mm-) diameter removable cylinder that revolves on stainless steel, self-lubricating, ball-bearing plates; with over-rotation prevention mechanism] <Insert description>.

          - Nominal Wall Opening: [12 by 11-1/4 inches (305 by 286 mm)] [13-1/4 by 14 inches (337 by 356 mm)] <Insert dimensions>, width by height.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

Retain "Lockset" Subparagraph below for turnstile type.

          - Lockset: Tumbler type.

        - Bedpan and Urinal Cabinet <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [For storing one conventional-size bedpan and one urinal bottle; with door that produces 1/2-inch (13-mm) opening at top and bottom of cabinet to allow air circulation] <Insert requirements>.

          - Mounting: Recessed.

          - Nominal Wall Opening: [13-1/2 by 26-1/2 by 5 inches (343 by 673 by 127 mm)] <Insert dimensions>, width by height by depth.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

        - Bedpan Rack <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: For storing [one conventional-size bedpan] [two conventional-size bedpans].

          - Mounting: Surface mounted.

          - Size: [12 by 11 inches (305 by 279 mm)] [12 by 27 inches (305 by 686 mm)] <Insert dimensions>, width by height.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

        - Bedpan and Urinal Rack <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: For storing one conventional-size bedpan and one urinal bottle.

          - Mounting: Surface mounted.

          - Size: [12 by 27 inches (305 by 686 mm)] <Insert dimensions>, width by height.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

      - CHILDCARE ACCESSORIES

Retain one of two options in "Source Limitations" Paragraph below or revise to suit Project. Retain first option if manufacturers can provide all childcare accessories required for Project.

        - Source Limitations: Obtain [childcare accessories] [each type of childcare accessory] from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Diaper-Changing Station <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Horizontal] [Vertical] unit that opens by folding down from stored position and with child-protection strap.

            - Engineered to support minimum of [250 lb (113 kg)] <Insert value> static load when opened.

          - Mounting: [Surface mounted, with unit projecting not more than 4 inches (102 mm) from wall when closed] [Semirecessed, with unit projecting not more than 1 inch (25 mm) from wall when closed].

          - Operation: By pneumatic shock-absorbing mechanism.

          - Material and Finish: [HDPE in manufacturer's standard color] [HDPE with plastic-laminate insert in color selected by Architect] [Stainless steel, ASTM A480/A480M No. 4 finish (satin), with replaceable insulated polystyrene tray liner and rounded plastic corners] [Stainless steel, ASTM A480/A480M No. 4 finish (satin), exterior shell with rounded plastic corners; HDPE interior in manufacturer's standard color] <Insert requirements>.

          - Liner Dispenser: Provide [built-in] [separate, locking] dispenser for disposable sanitary liners.

        - Diaper-Changing Station Liner Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: Recessed.

          - Minimum Capacity: [100] <Insert number> liners.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Diaper-Pack Vendor <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Mounting: [Surface mounted] [Recessed].

          - Minimum Capacity: [24] <Insert number> diaper packs.

          - Coin Operation: Coin slot preset for [one U.S. dollar, adjustable up in 25-cent increments] <Insert requirements>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

        - Child-Protection Seat <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Unit that opens by folding down from stored position and with child-protection strap.

            - Engineered to support minimum of [80 lb (36 kg)] [150 lb (68 kg)] <Insert value> static load when opened.

          - Mounting: Surface mounted, with unit projecting not more than [4-1/2 inches (114 mm)] [6 inches (152 mm)] from wall when closed.

          - Material and Finish: [HDPE in manufacturer's standard color] <Insert requirements>.

      - UNDERLAVATORY GUARDS

Delete this article if piping insulation is specified in Section 220719 "Plumbing Piping Insulation" or if architectural woodwork is used to prevent direct contact with piping.

Copy paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Underlavatory Guard <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Insulating pipe covering for supply and drain piping assemblies that prevents direct contact with and burns from piping; allow service access without removing coverings.

          - Material and Finish: Antimicrobial, molded plastic, white.

      - CUSTODIAL ACCESSORIES

Retain one of two options in "Source Limitations" Paragraph below or revise to suit Project. Retain first option if manufacturers can provide all custodial accessories required for Project.

        - Source Limitations: Obtain [custodial accessories] [each type of custodial accessory] from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Custodial Utility Shelf <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: With exposed edges turned down not less than 1/2 inch (13 mm) and supported by two triangular brackets welded to shelf underside.

          - Size: [16 inches (406 mm) long by 6 inches (152 mm) deep] <Insert dimensions>.

          - Material and Finish: Not less than nominal 0.05-inch- (1.3-mm-) thick stainless steel, ASTM A480/A480M No. 4 finish (satin).

        - Custodial Mop and Broom Holder <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: [Unit with shelf, hooks, holders, and rod suspended beneath shelf] <Insert description>.

          - Length: [36 inches (914 mm)] <Insert dimension>.

          - Hooks: [Four] <Insert number>.

          - Mop/Broom Holders: [Three] <Insert number>, spring-loaded, rubber hat, cam type.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

            - Shelf: Not less than nominal 0.05-inch- (1.3-mm-) thick stainless steel.

            - Rod: Approximately 1/4-inch- (6-mm-) diameter stainless steel.

      - HAND-SANITIZER DISPENSERS

Retain one of two options in "Source Limitations" Paragraph below or revise to suit Project. Retain first option if manufacturers can provide all hand dryers required for Project.

        - Source Limitations: Obtain [hand-sanitizer dispensers] [each type of hand-sanitizer dispenser] from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Hand-Sanitizer Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Manually operated dispenser[ with drip tray,] designed for dispensing alcohol-based hand rub in [gel] [foam] form. Designed to not release contents unless manually activated and to not dispense more hand rub than the amount required for hand hygiene consistent with label instructions.

          - Mounting: Wall mounted.

Retain "Protrusion Limit" Subparagraph below for units that comply with the DOJ's "2010 ADA Standards for Accessible Design" and with ICC A117.1, which limit protrusion of objects in access aisleways with leading edges between 27 and 80 inches (686 and 2032 mm) above the finished floor to 4 inches (102 mm). Verify requirements of authorities having jurisdiction.

            - Protrusion Limit: Installed unit protrudes maximum 4 inches (102 mm) from wall surface.

The IFC and NFPA 101 limit capacity depending on dispenser location. Third option in "Capacity" Subparagraph below is maximum capacity for dispensers in corridors or rooms and areas open to corridors. Fourth option is maximum capacity for dispensers in suites of rooms separated from corridors. See the Evaluations.

          - Capacity: [24 oz. (700 mL)] [34 oz. (1000 mL)] [41 oz. (1200 mL)] [68 oz. (2000 mL)] <Insert value> maximum.

          - Materials: [ABS plastic, white] [ABS plastic, black] [ABS plastic, dark gray] <Insert requirements>.

Retain "Lockset" and "Refill Indicator" subparagraphs below if required.

          - Lockset: Tumbler type.

          - Refill Indicator: [Window type] [Translucent top] [Translucent bottom].

        - Automatic Hand-Sanitizer Dispenser <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Automatic, battery powered dispenser with[ drip tray, and] infrared sensor designed to detect presence of hands and dispense alcohol-based hand rub in [gel] [foam] form. Designed to not release contents unless activated and to not dispense more hand rub than the amount required for hand hygiene consistent with label instructions. Designed for activation to only occur when an object is placed within 4 inches (102 mm) of sensing device and so that an object placed and left within the activation zone will not cause more than one activation.

          - Mounting: [Wall mounted] [Floor stand].

Retain "Protrusion Limit" Subparagraph below for wall-mounted units that comply with the DOJ's "2010 ADA Standards for Accessible Design" and with ICC A117.1, which limit protrusion of objects in access aisleways with leading edges between 27 and 80 inches (686 and 2032 mm) above the finished floor to 4 inches (102 mm). Verify requirements of authorities having jurisdiction.

            - Protrusion Limit: Installed unit protrudes maximum 4 inches (102 mm) from wall surface.

The IFC and NFPA 101 limit capacity depending on dispenser location. Second option in "Capacity" Subparagraph below is maximum capacity for dispensers in corridors or rooms and areas open to corridors. See the Evaluations.

          - Capacity: [34 oz. (1000 mL)] [41 oz. (1200 mL)] <Insert value> maximum.

          - Materials: [ABS plastic, white] [ABS plastic, black] [ABS plastic, dark gray] <Insert requirements>.

Retain "Refill Indicator" and "Low-Battery Indicator" subparagraphs below if required.

          - Refill Indicator: [LED indicator] [Window type] [Translucent top] [Translucent bottom].

          - Low-Battery Indicator: LED indicator.

      - MATERIALS

Retain this article if not naming manufacturers' products or if material descriptions are used in part to determine product selection.

        - Stainless Steel: ASTM A240/A240M or ASTM A666, Type 304, 0.031-inch- (0.8-mm-) minimum nominal thickness unless otherwise indicated.

        - Brass: ASTM B19, flat products; ASTM B16/B16M, rods, shapes, forgings, and flat products with finished edges; or ASTM B30, castings.

        - Steel Sheet: ASTM A1008/A1008M, Designation CS (cold rolled, commercial steel), 0.036-inch- (0.9-mm-) minimum nominal thickness.

        - Galvanized-Steel Sheet: ASTM A653/A653M, with G60 (Z180) hot-dip zinc coating.

        - Galvanized-Steel Mounting Devices: ASTM A153/A153M, hot-dip galvanized after fabrication.

        - Fasteners: Screws, bolts, and other devices of same material as accessory unit, unless otherwise recommended by manufacturer or specified in this Section, and tamper and theft resistant where exposed, and of stainless or galvanized steel where concealed.

Retain "Chrome Plating" Paragraph below for chrome finish. Brass, steel, and cast zinc alloy (zamac) are available with chrome finish.

        - Chrome Plating: ASTM B456, Service Condition Number SC 2 (moderate service).

Revise "Mirrors" Paragraph below if tempered or safety glass is required; ASTM C1503 does not cover tempered or safety glass mirrors. Note that reflected images with tempered or safety glass may be distorted.

        - Mirrors: ASTM C1503, Mirror Glazing Quality, clear-glass mirrors, nominal 6.0 mm thick.

      - FABRICATION

        - General: Fabricate units with tight seams and joints, and exposed edges rolled. Hang doors and access panels with full-length, continuous hinges. Equip units for concealed anchorage and with corrosion-resistant backing plates.

        - Keys: Provide universal keys for internal access to accessories for servicing and resupplying. Provide minimum of [six] <Insert number> keys to Owner's representative.

- EXECUTION

      - INSTALLATION

        - Install accessories in accordance with manufacturers' written instructions, using fasteners appropriate to substrate indicated and recommended by unit manufacturer. Install units level, plumb, and firmly anchored in locations and at heights indicated.

          - Remove temporary labels and protective coatings.

        - Grab Bars: Install to comply with specified structural-performance requirements.

        - Shower Seats: Install to comply with specified structural-performance requirements.

      - ADJUSTING AND CLEANING

        - Adjust accessories for unencumbered, smooth operation. Replace damaged or defective items.

        - Clean and polish exposed surfaces in accordance with manufacturer's written instructions.

END OF SECTION 102800