Copyright 2020 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 107343 - TRANSPORTATION STOP SHELTERS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Modular aluminum shelters.

          - Fabricated steel shelters.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for shelters.

        - Shop Drawings: For shelters.

          - Include plans, elevations, sections, and attachment details.

            - Anchor-Bolt Plans: Submit anchor-bolt plans and templates. Include location, diameter, and projection of anchor bolts required to attach shelters to concrete bases. Indicate post reactions at each location.

          - Signed and sealed by the qualified professional engineer responsible for their preparation.

Retain "Samples for Initial Selection" or "Samples for Verification" Paragraph below, or both.

        - Samples for Initial Selection: Manufacturer's standard color sheets, showing full range of available colors for each type of exposed finish.

        - Samples for Verification: Actual sample of finished products for each type of exposed finish.

          - Size: [Manufacturers' standard size] <Insert size>.

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for EPDs and HPDs.>

          - <Double click to insert sustainable design text for recycled content.>

          - <Double click to insert sustainable design text for regional materials.>

      - INFORMATIONAL SUBMITTALS

Information submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

        - Evaluation Reports:

          - Research Reports: For anchors, from ICC-ES.

        - Sample warranties.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For shelters.

      - WARRANTY

When warranties are required and available, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty: Manufacturer agrees to repair finish or replace shelters that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures.

            - Deterioration of metals, metal finishes, and other materials beyond normal weathering.

Verify warranty periods for shelters.

          - Warranty Period: [Five] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - PERFORMANCE REQUIREMENTS

        - Structural Performance: Shelters to withstand the following loads and stresses within limits and under conditions indicated in accordance with [ASCE/SEI 7] <Insert requirement>:

          - Design Loads: [As indicated on Drawings] <Insert loads>.

        - Thermal Movements: Allow for thermal movements from ambient and surface temperature changes. Base calculations on surface temperatures of materials due to both solar heat gain and nighttime-sky heat loss.

Differential values in "Temperature Change" Subparagraph below (for aluminum in particular) are suitable for most of the United States.

          - Temperature Change: [120 deg F (67 deg C), ambient; 180 deg F (100 deg C), material surfaces] <Insert temperature range>.

        - Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

        - Safety Glazing Products: Category II materials complying with testing requirements in 16 CFR 1201.

Retain subparagraph below if applicable. Not all manufacturers participate in third-party testing programs.

          - Subject to compliance with requirements, obtain safety glazing products permanently marked with certification label of [SGCC or another certification agency] [or] [manufacturer] acceptable to authorities having jurisdiction.

      - MODULAR ALUMINUM SHELTERS

Manufacturers' shelters and their components are available in a range of styles, shapes, configurations, and options, in both standard and custom designs. Revise this article, which includes the most common construction, to suit Project.

        - Fabricate shelters as an integrated set of modular components ready for assembly on Project site.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Building Style: [Open front] [Dual entrance] [Single entrance on left] [Single entrance on right] [As indicated on Drawings].

          - Size: [48 inches by 96 inches (1219 mm by 2438 mm)] [60 inches by 120 inches (1524 mm by 3048 mm)] [72 inches by 144 inches (1829 mm by 3658 mm)] [96 inches by 144 inches (2438 mm by 3658 mm)] <Insert dimensions>.

        - Structural Framework: Fabricated from manufacturer's standard 2.5-by-2.5-by-0.125-inch (64-by-64-by-3.18-mm) aluminum tubing, channel, angle, or tee extrusions. Connect framework with exposed mechanical fasteners.

        - Post Bases: Externally adjustable, flanged, aluminum sleeves; minimum 3-inch (76-mm) vertical adjustment. Include shims for leveling.

        - Anchorage: [Cast-in-place anchor bolts] [Postinstalled anchors], fabricated from stainless steel, with allowable load or strength design capacities calculated to be greater than or equal to the design load.

        - Windows: Extruded-aluminum sash frames, factory glazed with [clear tempered float glass] [tempered, tinted float glass] [polycarbonate glazing] [acrylic glazing] <Insert glazing>.

        - Roof: [Flat] [Standing seam hip] [Standing seam gable] [Glazed barrel vault] [Glazed domes] with full perimeter gutter.

Retain "Exterior Roof Panels" Subparagraph below for flat, standing seam hip or standing seam gable roofs.

          - Exterior Roof Panels: Fabricated from 0.032-inch- (0.81-mm-) thick, aluminum sheet with protective plastic sheet finish.

Retain "Glazing" Subparagraph below for glazed barrel vault or dome roofs.

          - Glazing: [Polycarbonate] [Acrylic] <Insert glazing>.

          - Canopy Fascia: Fabricated from 0.063-inch- (1.60-mm-) thick aluminum sheet, of manufacturer's standard design.

            - Height: [6 inches (152 mm)] [8 inches (203 mm)] <Insert dimension>.

            - Overhang: [3 inches (76 mm)] <Insert dimension>.

"Graphics" Subparagraph below is an example only; revise to suit Project. Some manufacturers can provide custom graphics or logos.

            - Graphics: Die-cut vinyl lettering for exterior use.

              - Height: [4 inches (102 mm)] <Insert dimension>.

              - Color: [White] <Insert color>.

              - Font: [Helvetica] <Insert font>.

              - Message: ["Bus Stop"] [As indicated on Drawings] <Insert message>.

          - Downspouts: Integral, extending 3 inches (76 mm) beyond shelter walls.

        - [Integral] [Freestanding] Bench: [Aluminum] [Wood] bench[ with backrest].

          - Length: [Full length of shelter] [Partial length to allow wheelchair access] <Insert dimension>.

Retain "Wood Species" Subparagraph below if retaining "Wood" option in "(Integral) (Freestanding) Bench" Paragraph above.

          - Wood Species: [Oak] [Maple] [Heart redwood] <Insert species>.

          - Armrests: Manufacturer's standard pipe armrests.

Retain "Display Panels" Paragraph below to display transit maps or for advertising.

        - Display Panels: Full width of shelter window by [30 inches (762 mm)] [72 inches (1829 mm)] <Insert dimension> high for [one][two]-sided display. Front-hinged panel with locking device.

          - Location: [As indicated on Drawings] [On left sidewall] <Insert location>.

          - Illumination: Internal [fluorescent] [LED] lighting for [front] [back] [perimeter]-lit display.

        - Shelter Accessories:

          - Ashtray: Wall mounted, stainless steel.

          - Wind Skirts: Aluminum-sheet closures attached to bottom rail and matching shelter finish, full length of walls. Where required for sloping base, overlapping two-piece closures, with bottom anchored to concrete base.

Retain "Electrical Power Service" Paragraph below where power is available and required.

        - Electrical Power Service: 125 A, 120/240 V ac, single phase, three wire. Run copper wiring in 1/2-inch (13-mm) EMT conduit. Exterior weatherproof connection box and lockable interior circuit breaker panel.

Retain "Photovoltaic Power" Paragraph below for remote location where power is required.

        - Photovoltaic Power: Manufacturer's standard photovoltaic power generation system including [pole][roof]-mounted photovoltaic modules, batteries, charge controllers, inverters, and mounting structures.

Coordinate "Lighting Fixtures," "Heating Unit," and "Exhaust Fans" paragraphs below with "Electrical Power Service" or "Photovoltaic Power" Paragraph above.

        - Lighting Fixtures: Activated by [photocell] [motion sensor] [tamper-resistant concealed switch].

Retain one of two subparagraphs below.

          - [One] [Two] ceiling-mounted fluorescent lighting fixture(s), 48 inches (1219 mm) long, with acrylic lens and two 40 W lamps in each fixture.

          - Ceiling-mounted LED lighting.

        - Heating Unit: Wall-mounted [forced-fan operation] [radiant, quartz-tube] heater, 110 V, enclosed in enameled-steel cabinet. Adjustable timer and vandal-resistant switch with engraved sign: "Push for Heat." Heating capacity of not less than [5000 Btu/h (1465 W)] <Insert value>.

        - Exhaust Fans: [7-inch- (178-mm-)] [10-inch- (254-mm-)] [12-inch- (305-mm-)] diameter units, capable of [135 cfm (63 L/s)] [520 cfm (245 L/s)] [980 cfm (462 L/s)] <Insert value> airflow exhaust.

        - Materials:

          - Aluminum: Alloy and temper recommended by aluminum producer and manufacturer for type of use and finish indicated, and as follows:

            - Sheet: ASTM B209 (ASTM B209M).

            - Extruded Shapes: ASTM B221 (ASTM B221M).

Coordinate "Clear Tempered Float Glass," "Tempered, Tinted Float Glass," "Acrylic Glazing," or "Polycarbonate Glazing" Subparagraph below with products retained.

          - Clear Tempered Float Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 1, and Quality q3.

          - Tempered, Tinted Float Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 1, and Quality q3.

            - Tint Color: [Blue] [Blue-green] [Bronze] [Green] [Gray] <Insert color>.

          - Acrylic Glazing: ASTM D4802, thermoformable, monolithic sheet, category as standard with manufacturer, Finish 1 (smooth or polished), Type UVF (formulated with UV absorber).

            - Color: [Colorless, transparent] [White, translucent] [Bronze-tinted, transparent] [Gray-tinted, transparent] <Insert requirements>.

          - Polycarbonate Glazing: Thermoformable, extruded monolithic sheets, UV resistant[, with anti-graffiti coating] and with average impact strength of 12 to 16 ft-lb/in. (640 to 854 J/m) of width when tested in accordance with ASTM D256, Test Method A (Izod).

            - Color: [Colorless, transparent] [White, translucent] [Bronze-tinted, transparent] [Gray-tinted, transparent] <Insert requirements>.

Retain finishes in remaining paragraphs to suit Project. If retaining more than one, indicate location of each on Drawings or by inserts.

Retain one of two options in "Clear Anodic Finish" Paragraph below. Class II finish is standard with many manufacturers; Class I finish is heavy anodized. Verify availability with manufacturers.

        - Clear Anodic Finish: AAMA 611, [AA-M12C22A41, Class I, 0.018 mm] [AA-M12C22A31, Class II, 0.010 mm] or thicker.

Retain one of two options in "Color Anodic Finish" Paragraph below. Verify availability with manufacturers.

        - Color Anodic Finish: AAMA 611, [AA-M12C22A42/A44, Class I, 0.018 mm] [AA-M12C22A32/A34, Class II, 0.010 mm] or thicker.

First four options in "Color" Subparagraph below are examples only and may vary in color range and availability among manufacturers. Retain one or delete all, and retain one of last two options.

          - Color: [Light bronze] [Medium bronze] [Dark bronze] [Black] [Match Architect's sample] [As selected by Architect from full range of industry colors and color densities] <Insert color>.

"Baked-Enamel or Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on aluminum extrusions and panels.

        - Baked-Enamel or Powder-Coat Finish: AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

      - FABRICATED STEEL SHELTERS

Manufacturers' shelters and their components are available in a range of styles, shapes, configurations, and options, in both standard and custom designs. Revise this article, which includes the most common construction, to suit Project.

        - Factory fabricate shelters to form a completed assembly, ready for installation on Project site.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Building Style: [Open front] [Dual entrance] [Single entrance on left] [Single entrance on right] [As indicated on Drawings].

          - Size: [48 inches by 96 inches (1219 mm by 2438 mm)] [60 inches by 120 inches (1524 mm by 3048 mm)] [72 inches by 144 inches (1829 mm by 3658 mm)] [96 inches by 144 inches (2438 mm by 3658 mm)] <Insert dimensions>.

        - Structural Framework: Fabricated from 2-by-2-by-0.075-inch (50-by-50-by-1.90-mm) steel structural or mechanical tubing. Connect framework by welding.

        - Post Bases: Externally adjustable, flanged, steel sleeves; minimum 3-inch (76-mm) vertical adjustment. Include shims for leveling.

        - Anchorage: [Cast-in-place anchor bolts] [Postinstalled anchors], fabricated from stainless steel or corrosion-resistant materials, with allowable load or strength design capacities calculated to be greater than or equal to the design load.

        - Windows: Extruded-aluminum sash frames, factory glazed with [clear tempered float glass] [tempered, tinted float glass] [polycarbonate glazing] [acrylic glazing] <Insert glazing>.

        - Roof: [Flat] [Standing seam hip] [Standing seam gable] [Glazed barrel vault] [Glazed domes] with full perimeter gutter.

Retain "Exterior Roof Panels" Subparagraph below for flat, standing seam hip or standing seam gable roofs.

          - Exterior Roof Panels: Fabricated from [0.079-inch (2.01-mm)] [0.064-inch (1.63-mm)] nominal-thickness, galvanized-steel sheet; with [painted finish] [EPDM membrane], continuously welded seams.

Retain "Glazing "Subparagraph below for glazed barrel vault or dome roofs.

          - Glazing: [Polycarbonate] [Acrylic] <Insert glazing>.

          - Canopy Fascia: Fabricated from 0.079-inch (2.01-mm) nominal-thickness, galvanized-steel sheet, of [manufacturer's standard design] [custom design indicated on Drawings].

            - Height: [6 inches (152 mm)] [8 inches (203 mm)] <Insert dimension>.

            - Overhang: [3 inches (76 mm)] <Insert dimension>.

"Graphics" Subparagraph below is an example only; revise to suit Project. Some manufacturers can provide custom graphics or logos.

            - Graphics: Die-cut vinyl lettering for exterior use.

              - Height: [4 inches (102 mm)] <Insert dimension>.

              - Color: [White] <Insert color>.

              - Font: [Helvetica] <Insert font>.

              - Message: ["Bus Stop"] [As indicated on Drawings] <Insert message>.

          - Downspouts: Integral, extending 3 inches (76 mm) beyond shelter walls.

        - [Integral] [Freestanding] Bench: [Steel] [Wood] bench[ with backrest].

          - Length: [Full length of shelter] [Partial length to allow wheelchair access] <Insert dimension>.

Retain "Wood Species" Subparagraph below if retaining "Wood" option in "(Integral) (Freestanding) Bench" Paragraph above.

          - Wood Species: [Oak] [Maple] [Heart redwood] <Insert species>.

          - Armrests: Manufacturer's standard pipe armrests.

Retain "Display Panels" Paragraph below to display transit maps or for advertising.

        - Display Panels: Full width of shelter window by [30 inches (762 mm)] [72 inches (1829 mm)] <Insert dimension> high for [one][two]-sided display. Front-hinged panel with locking device.

          - Location: [As indicated on Drawings] [On left sidewall] <Insert location>.

          - Illumination: Internal [fluorescent] [LED] lighting for [front] [back] [perimeter]-lit display.

        - Shelter Accessories:

          - Ashtray: Wall mounted, stainless steel.

          - Wind Skirts: Steel-sheet closures attached to bottom rail and matching shelter finish, full length of walls. Where required for sloping base, overlapping two-piece closures, with bottom anchored to concrete base.

Retain "Electrical Power Service" Paragraph below where power is available and required.

        - Electrical Power Service: 125 A, 120/240 V ac, single phase, three wire. Run copper wiring in 1/2-inch (13-mm) EMT conduit. Exterior weatherproof connection box and lockable interior circuit breaker panel.

Retain "Photovoltaic Power" Paragraph below for remote location where power is required.

        - Photovoltaic Power: Manufacturer's standard photovoltaic power generation system including [pole][roof]-mounted photovoltaic modules, batteries, charge controllers, inverters, and mounting structures.

Coordinate "Lighting Fixtures," "Heating Unit," and "Exhaust Fans" paragraphs below with "Electrical Power Service" or "Photovoltaic Power" Paragraph above.

        - Lighting Fixtures: Activated by [photocell] [motion sensor] [tamper-resistant concealed switch].

Retain one of two subparagraphs below.

          - [One] [Two] ceiling-mounted fluorescent lighting fixture(s), 48 inches (1219 mm) long, with acrylic lens and two 40 W lamps in each fixture.

          - Ceiling-mounted LED lighting.

        - Heating Unit: Wall-mounted [forced-fan operation] [radiant, quartz-tube] heater, 110 V, enclosed in enameled-steel cabinet. Adjustable timer and vandal-resistant switch with engraved sign: "Push for Heat." Heating capacity of not less than [5000 Btu/h (1465 W)] <Insert value>.

        - Exhaust Fans: [7-inch- (178-mm-)] [10-inch- (254-mm-)] [12-inch- (305-mm-)] diameter units, capable of [135 cfm (63 L/s)] [520 cfm (245 L/s)] [980 cfm (462 L/s)] <Insert value> airflow exhaust.

        - Materials:

          - Zinc-Coated (Galvanized) Steel Sheet: ASTM A653/A653M, commercial quality, G90 (Z275) coating designation; mill phosphatized.

          - Steel Structural Tubing: ASTM A500/A500M, Grade B.

          - Steel Plates, Shapes, and Bars: ASTM A36/A36M.

          - Steel Mechanical Tubing: ASTM A513/A513M, welded-steel mechanical tubing.

          - Zinc-Coated (Galvanized) Steel: Hot-dip galvanized in accordance with ASTM A123/A123M.

          - Anchorages: Anchor bolts; [hot-dip galvanized in accordance with ASTM A153/A153M or ASTM F2329/F2329M] [stainless steel].

Coordinate "Clear Tempered Float Glass," "Tempered, Tinted Float Glass," "Acrylic Glazing," or "Polycarbonate Glazing" Subparagraph below with products retained.

          - Clear Tempered Float Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 1, and Quality q3.

          - Tempered, Tinted Float Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 1, and Quality q3.

            - Tint Color: [Blue] [Blue-green] [Bronze] [Green] [Gray] <Insert color>.

          - Acrylic Glazing: ASTM D4802, thermoformable, monolithic sheet, category as standard with manufacturer, Finish 1 (smooth or polished), Type UVF (formulated with UV absorber).

            - Color: [Colorless, transparent] [White, translucent] [Bronze-tinted, transparent] [Gray-tinted, transparent] <Insert requirements>.

          - Polycarbonate Glazing: Thermoformable, extruded monolithic sheets, UV resistant[, with anti-graffiti coating] and with average impact strength of 12 to 16 ft-lb/in. (640 to 854 J/m) of width when tested in accordance with ASTM D256, Test Method A (Izod).

            - Color: [Colorless, transparent] [White, translucent] [Bronze-tinted, transparent] [Gray-tinted, transparent] <Insert requirements>.

        - Finish: Immediately after cleaning and pretreating, apply manufacturer's standard two-coat, baked-on enamel or powder-coat finish consisting of prime coat and thermosetting topcoat.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for installation tolerances, including concrete bases; accurate placement, pattern, and orientation of anchor bolts; and critical dimensions; and other conditions affecting performance of the Work.

Retain first paragraph below where power is available and required.

        - Examine roughing-in for electrical system to verify actual locations of connections before shelter installation.

        - Prepare written report, endorsed by Installer, listing conditions detrimental to performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install shelters in accordance with manufacturer's written instructions.

        - Set shelters plumb and aligned with full bearing on concrete bases.

        - Fasten shelters securely to concrete base with anchorage indicated.

Retain paragraph below where power is available and required.

        - Connect to electrical power.

        - After completing installation, inspect exposed finishes and repair damaged finishes.

END OF SECTION 107343