Copyright 2021 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 107516 - GROUND-SET FLAGPOLES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

- GENERAL

      - SUMMARY

        - Section includes ground-set flagpoles made from [aluminum] [copper alloy (bronze)] [stainless steel] [steel] [and] [fiberglass].

Retain "Owner-Furnished Material" Paragraph below if Owner is furnishing flags.

        - Owner-Furnished Material: Flags.

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, operating characteristics, fittings, accessories, and finishes for flagpoles.

Retain "Shop Drawings" Paragraph below if required. Most of below is normally covered in Product Data if not a customized installation.

        - Shop Drawings: For each flagpole.

          - Include the following

            - Plans, elevations, and attachment details. Show general arrangement, jointing, fittings, accessories, grounding, anchoring, and support.

            - Section, and details of foundation system.

Retain "Samples for Verification" Paragraph below if using special finishes.

        - Samples for Verification: For each type of exposed finish, in manufacturer's standard sizes.

Retain "Delegated Design Submittals" Paragraph below if design services have been delegated to Contractor.

        - Delegated Design Submittals: For flagpoles.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For flagpoles to include in operation and maintenance manuals.

      - DELIVERY, STORAGE, AND HANDLING

        - Spiral wrap flagpoles with heavy paper and enclose in a hard fiber tube or other protective container.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

        - Source Limitations: Obtain flagpoles as complete units, including fittings, accessories, bases, and anchorage devices, from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

        - Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design flagpole assemblies.

Retain "Seismic Performance" Paragraph below for projects requiring seismic design. Delete paragraph if performance requirements are indicated on Drawings. Model building codes and ASCE/SEI 7 establish criteria for buildings subject to earthquake motions. Coordinate requirements with structural engineer.

        - Seismic Performance: Flagpole assemblies to withstand the effects of earthquake motions determined according to [ASCE/SEI 7] <Insert requirement>.

        - Structural Performance: Flagpole assemblies, including anchorages and supports, to withstand design loads indicated within limits and under conditions indicated.

Consult a structural engineer experienced in engineering flagpole assemblies of type indicated to quantify design loads applicable to Project. Verify compliance with codes; below is based on the IBC. See Evaluations.

          - Wind Loads: Determine according to NAAMM FP 1001. Basic wind speed for Project location is <Insert wind speed>.

Retain one of two options in subparagraph below. Polyester flags exert approximately 40 percent higher loads on flagpoles than do nylon or cotton flags of same size. Consider future load requirements and liability. Revise if required to suit Project.

          - Base flagpole design on [polyester] [nylon or cotton] flags of maximum standard size suitable for use with flagpole or flag size indicated, whichever is more stringent.

      - ALUMINUM FLAGPOLES

        - Aluminum Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from seamless extruded tubing complying with ASTM B241/B241M, Alloy 6063, with a minimum wall thickness of 3/16 inch (4.8 mm).

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Height" Paragraph below if only one flagpole height is required. For complex flagpoles, such as nautical types, indicate flagpoles on Drawings and delete paragraph.

        - Exposed Height: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] [45 feet (13.5 m)] [50 feet (15 m)] [60 feet (18 m)] [70 feet (21 m)] [80 feet (24 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in remaining paragraphs below to suit flagpole installation.

        - Metal Foundation Tube: Manufacturer's standard corrugated-steel foundation tube, 0.060-inch (1.52-mm) wall thickness with 3/16-inch (4.8-mm) steel bottom plate and support plate; 3/4-inch- (19-mm-) diameter, steel ground spike; and steel centering wedges welded together. Galvanize foundation tube after assembly. Furnish loose hardwood wedges at top of foundation tube for plumbing pole.

Revise "Flashing Collar" Subparagraph below if another base or finish is required.

          - Flashing Collar: Same material and finish as flagpole.

"Sleeve for Aluminum Flagpole" Paragraph below applies primarily to aluminum flagpoles 30 to 40 feet (9 to 12 m) or less in height.

        - Sleeve for Aluminum Flagpole: [Fiberglass] [or] [PVC pipe] foundation sleeve, made to fit flagpole, for casting into concrete foundation.

Revise "Flashing Collar" Subparagraph below if another base or finish is required.

          - Flashing Collar: Same material and finish as flagpole.

        - Cast-Metal Shoe Base: Made from aluminum [with same finish and color as flagpoles] <Insert finish and color> for anchor-bolt mounting; furnish with anchor bolts.

          - Furnish ground spike.

"Hinged Baseplate" Paragraph below is for aluminum flagpoles 30 to 40 feet (9 to 12 m) or less in height.

        - Hinged Baseplate: Cast-metal tilting hinged base and anchor plate joined by permanently secured pivot rod. Furnish with stainless steel screws for securing tilting base to anchor plate when not tilted; furnish with anchor bolts.

          - Finish: Same as flagpole.

          - Furnish aluminum base or aluminum flashing collar finished to match flagpole.

          - Furnish ground spike.

"Pivoting Tilt Base" Paragraph below is for tilting aluminum flagpoles.

        - Pivoting Tilt Base: Steel baseplate with channel or rectangular tube uprights, pivot bolt, and locking device for tilting flagpole. Furnish tilting flagpole with steel counterweight box and weights, or furnish with internal counterweight. Furnish base with anchor bolts.

          - Finish: Same as flagpole.

          - Furnish ground spike.

      - COPPER-ALLOY (BRONZE) FLAGPOLES

        - Copper-Alloy (Bronze) Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from seamless pipe or tube complying with ASTM B43 or ASTM B135 (ASTM B135M), Alloy UNS C23000 (red brass, 85 percent copper).

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Height" Paragraph below if only one flagpole height is required. For complex flagpoles, such as nautical types, indicate flagpoles on Drawings and delete paragraph.

        - Exposed Height: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] [45 feet (13.5 m)] [50 feet (15 m)] [60 feet (18 m)] [70 feet (21 m)] [80 feet (24 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in "Metal Foundation Tube" and "Cast-Metal Shoe Base" paragraphs below to suit flagpole installation.

        - Metal Foundation Tube: Manufacturer's standard corrugated-steel foundation tube, 0.060-inch (1.52-mm) wall thickness with 3/16-inch (4.8-mm) steel bottom plate and support plate; 3/4-inch- (19-mm-) diameter, steel ground spike; and steel centering wedges welded together. Galvanize foundation tube after assembly. Furnish loose hardwood wedges at top of foundation tube for plumbing pole.

Revise "Flashing Collar" Subparagraph below if another base or finish is required.

          - Flashing Collar: Same material and finish as flagpole.

        - Cast-Metal Shoe Base: Made from steel [with finish matching flagpole] <Insert finish and color> for anchor-bolt mounting; furnish with anchor bolts.

          - Furnish ground spike.

      - STAINLESS STEEL FLAGPOLES

Retain "Type 316L" option in "Stainless Steel Flagpoles" Paragraph below if increased corrosion resistance is required for marine environment.

        - Stainless Steel Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from pipe, tube, or plate complying with ASTM A312/A312M, ASTM A269, or ASTM A666, [Type 304] [Type 316L].

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Height" Paragraph below if only one flagpole height is required. For complex flagpoles, such as nautical types, indicate flagpoles on Drawings and delete paragraph.

        - Exposed Height: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] [45 feet (13.5 m)] [50 feet (15 m)] [60 feet (18 m)] [70 feet (21 m)] [80 feet (24 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in "Metal Foundation Tube" and "Cast-Metal Shoe Base" paragraphs below to suit flagpole installation.

        - Metal Foundation Tube: Manufacturer's standard corrugated-steel foundation tube, 0.060-inch (1.52-mm) wall thickness with 3/16-inch (4.8-mm) steel bottom plate and support plate; 3/4-inch- (19-mm-) diameter, steel ground spike; and steel centering wedges welded together. Galvanize foundation tube after assembly. Furnish loose hardwood wedges at top of foundation tube for plumbing pole.

Revise "Flashing Collar" Subparagraph below if another base or finish is required.

          - Flashing Collar: Same material and finish as flagpole.

Revise "Cast-Metal Shoe Base" Paragraph below to specify welded-on baseplate if required.

        - Cast-Metal Shoe Base: Made from steel [with finish matching flagpole] <Insert finish and color> for anchor-bolt mounting; furnish with anchor bolts.

          - Furnish ground spike.

      - STEEL FLAGPOLES

        - Steel Flagpoles: [Cone-tapered] [Stepped-sectional] flagpoles fabricated from standard-weight, seamless steel pipe complying with ASTM A53/A53M, Type S, Grade B, or steel tube complying with ASTM A513.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Height" Paragraph below if only one flagpole height is required. For complex flagpoles, such as nautical types, indicate flagpoles on Drawings and delete paragraph.

        - Exposed Height: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] [45 feet (13.5 m)] [50 feet (15 m)] [60 feet (18 m)] [70 feet (21 m)] [80 feet (24 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

Retain first subparagraph below for tapered flagpoles; retain second for stepped-sectional flagpoles.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

          - Provide self-aligning, snug-fitting joints.

Retain mounting types in "Metal Foundation Tube" and "Cast-Metal Shoe Base" paragraphs below to suit flagpole installation.

        - Metal Foundation Tube: Manufacturer's standard corrugated-steel foundation tube, 0.060-inch (1.52-mm) wall thickness with 3/16-inch (4.8-mm) steel bottom plate and support plate; 3/4-inch- (19-mm-) diameter, steel ground spike; and steel centering wedges welded together. Galvanize foundation tube after assembly. Furnish loose hardwood wedges at top of foundation tube for plumbing pole.

Revise "Flashing Collar" Subparagraph below if another base or finish is required.

          - Flashing Collar: Same material and finish as flagpole.

Retain subparagraph below only if flashing collar is not provided.

          - Furnish steel ground protectors extending 12 inches (300 mm) aboveground and 6 inches (150 mm) belowground for steel flagpoles where flashing collars are not provided.

Revise "Cast-Metal Shoe Base" Paragraph below to specify welded-on baseplate if required.

        - Cast-Metal Shoe Base: Made from steel [with finish matching flagpole] <Insert finish and color> for anchor-bolt mounting; furnish with anchor bolts.

Retain subparagraph below for metal flagpoles or fiberglass flagpoles with metal halyards.

          - Furnish ground spike.

      - FIBERGLASS FLAGPOLES

        - Fiberglass Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from polyester resin reinforced with woven glass-fiber roving with 75 percent of glass fibers parallel to length of flagpole.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Height" Paragraph below if only one flagpole height is required. For complex flagpoles, such as nautical types, indicate flagpoles on Drawings and delete paragraph.

        - Exposed Height: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] [45 feet (13.5 m)] [50 feet (15 m)] [60 feet (18 m)] [70 feet (21 m)] [80 feet (24 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in "Sleeve for Fiberglass Flagpole," "Cast-Metal Shoe Base," and "Hinged Baseplate" paragraphs below to suit flagpole installation.

"Sleeve for Fiberglass Flagpole" Paragraph below applies to fiberglass flagpoles of all heights.

        - Sleeve for Fiberglass Flagpole: [Fiberglass] [or] [PVC pipe] foundation sleeve, made to fit flagpole, for casting into concrete foundation.

Revise "Flashing Collar" Subparagraph below if another base or finish is required.

          - Flashing Collar: Same material and finish as flagpole.

Fiberglass flagpoles typically use cast-aluminum shoe bases; insert the finish and color for fiberglass flagpoles in "Cast-Metal Shoe Base" Paragraph below.

        - Cast-Metal Shoe Base: Made from aluminum [with finish matching flagpole] <Insert finish and color> for anchor-bolt mounting; furnish with anchor bolts.

Retain subparagraph below for fiberglass flagpoles with metal halyards.

          - Furnish ground spike.

"Hinged Baseplate" Paragraph below is for fiberglass flagpoles 30 to 40 feet (9 to 12 m) or less in height.

        - Hinged Baseplate: Cast-metal tilting hinged base and anchor plate joined by permanently secured pivot rod. Furnish with stainless steel screws for securing tilting base to anchor plate when not tilted; furnish with anchor bolts.

          - Finish: Same as flagpole.

          - Furnish aluminum base or aluminum flashing collar finished to match flagpole.

Retain subparagraph below for metal flagpoles or fiberglass flagpoles with metal halyards.

          - Furnish ground spike.

      - FITTINGS

Use manufacturers' catalogs or product data to insert series, type, model, and designations of other characteristics.

Revise fittings in this article to suit Project. Items listed are typical with most manufacturers; other fittings are also available.

        - Finial Ball: Flush-seam ball, sized as indicated or, if not indicated, to match flagpole-butt diameter.

Retain one or more of four subparagraphs below. If retaining more than one, indicate which flagpole type applies to each subparagraph.

          - 0.063-inch (1.6-mm) spun aluminum[, finished to match flagpole][ with gold anodic finish].

          - 20-oz. (0.70-mm) copper with 23-karat, gold-leaf finish.

          - Spun stainless steel, finished to match flagpole.

          - Spun copper alloy, finished to match flagpole.

        - Finial Eagle: Sized [as indicated] [as standard with manufacturer for flagpole size indicated] <Insert size>.

Retain one of two subparagraphs below.

          - Cast aluminum[, finished to match flagpole][ with gold anodic finish].

          - 20-oz. (0.70-mm) copper with 23-karat, gold-leaf finish.

Retain one of three remaining paragraphs below. Revise if another finish is required.

        - Internal Halyard, Winch System: Manually operated winch with control stop device and removable handle, stainless steel cable halyard, and concealed revolving truck assembly with plastic-coated counterweight and sling. Furnish flush access door secured with cylinder lock. Finish truck assembly to match flagpole.

Covers in "Halyard Flag Snaps" Subparagraph below reduce noise of metal snaps hitting flagpole.

          - Halyard Flag Snaps: [Chromium-plated bronze] [Stainless steel] [Bronze] [Nylon] swivel snap hooks[ with neoprene or vinyl covers]. Furnish two per halyard.

Product in "Plastic Halyard Flag Clips for Internal Halyard, Winch System" Subparagraph below is a patented alternative to halyard flag snaps in "Halyard Flag Snaps" Subparagraph above.

          - Plastic Halyard Flag Clips for Internal Halyard, Winch System: Made from injection-molded, UV-stabilized, acetal resin (Delrin). Clips attach to flag and have two eyes for inserting both runs of halyards. Furnish two per halyard.

            - <Double click here to find, evaluate, and insert list of manufacturers and products.>

"Internal Halyard, Cam Cleat System" Paragraph below is available for flagpoles 40 feet (12 m) or less in height. Revise if using stationary truck.

        - Internal Halyard, Cam Cleat System: [5/16-inch- (8-mm-) diameter, braided polypropylene] <Insert type> halyard; cam cleat; and concealed revolving truck assembly with plastic-coated counterweight and sling. Furnish flush access door secured with cylinder lock. Finish truck assembly to match flagpole.

Covers in "Halyard Flag Snaps" Subparagraph below reduce noise of metal snaps hitting flagpole.

          - Halyard Flag Snaps: [Chromium-plated bronze] [Stainless steel] [Bronze] [Nylon] swivel snap hooks[ with neoprene or vinyl covers]. Furnish two per halyard.

Product in "Plastic Halyard Flag Clips for Internal Halyard, Cam Cleat System" Subparagraph below is a patented alternative to halyard flag snaps in "Halyard Flag Snaps" Subparagraph above.

          - Plastic Halyard Flag Clips for Internal Halyard, Cam Cleat System: Made from injection-molded, UV-stabilized, acetal resin (Delrin). Clips attach to flag and have two eyes for inserting both runs of halyards. Furnish two per halyard.

            - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Revise "External Halyard" Paragraph below if using stationary truck.

        - External Halyard: Ball-bearing, nonfouling, revolving truck assembly of cast metal with continuous [5/16-inch- (8-mm-) diameter, braided polypropylene halyard] <Insert type> and 9-inch (228-mm) cast-metal cleats with fasteners. Finish exposed metal surfaces to match flagpole.

          - Halyards and Cleats: [One] [Two] at each flagpole.

"Cleat Covers" and "Halyard Covers" subparagraphs below provide protection against tampering.

          - Cleat Covers: Cast metal, finished to match flagpole, secured with cylinder locks.

          - Halyard Covers: 2-inch (50-mm) channel, 60 inches (1500 mm) long, finished to match flagpole.

Covers in "Halyard Flag Snaps" Subparagraph below reduce noise of metal snaps hitting flagpole (external halyards will still slap).

          - Halyard Flag Snaps: [Chromium-plated bronze] [Stainless steel] [Bronze] [Nylon] swivel snap hooks[ with neoprene or vinyl covers]. Furnish two per halyard.

Product in "Plastic Halyard Flag Clips for External Halyard, Ball-Bearing System" Subparagraph below is a patented alternative to halyard flag snaps in "Halyard Flag Snaps" Subparagraph above.

          - Plastic Halyard Flag Clips for External Halyard, Ball-Bearing System: Made from injection-molded, UV-stabilized, acetal resin (Delrin). Clips attach to flag and have two eyes for inserting both runs of halyards. Furnish two per halyard.

            - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - MISCELLANEOUS MATERIALS

Retain "Nonshrink, Nonmetallic Grout" Paragraph below for baseplate-mounted flagpoles.

        - Nonshrink, Nonmetallic Grout: Factory-packaged, nonstaining, noncorrosive, nongaseous grout complying with ASTM C1107/C1107M.

Retain "Drainage Material" Paragraph below for ground-set flagpoles with foundations.

        - Drainage Material: Crushed stone, or crushed or uncrushed gravel; coarse aggregate.

Retain "Sand" and "Elastomeric Joint Sealant" paragraphs below for ground-set, foundation-tube-mounted flagpoles.

        - Sand: ASTM C33/C33M, fine aggregate.

        - Elastomeric Joint Sealant: [Multicomponent nonsag urethane] [Single-component nonsag urethane] [Single-component neutral-curing silicone] joint sealant complying with requirements in Section 079200 "Joint Sealants."

        - Bituminous Paint: Cold-applied asphalt emulsion complying with ASTM D1187/D1187M.

      - ALUMINUM FINISHES

Retain finishes in paragraphs below to suit Project. If retaining more than one, indicate location of each on Drawings or by inserts.

        - Natural Satin Finish: AA-M32, fine, directional, medium satin polish; buff complying with AA-M20; seal aluminum surfaces with clear, hard-coat wax.

Retain one of two options in "Clear Anodic Finish" Paragraph below. Verify availability with manufacturers.

        - Clear Anodic Finish: AAMA 611, [AA-M12C22A41] [AA-M12C22A31].

Retain one of two options in "Color Anodic Finish" Paragraph below. Verify availability with manufacturers.

        - Color Anodic Finish: AAMA 611, [AA-M12C22A42/A44] [AA-M12C22A32/A34].

First four options in "Color" Subparagraph below are examples of color anodic finishes and may vary in color range and availability among manufacturers.

          - Color: [Light bronze] [Medium bronze] [Dark bronze] [Black] [As indicated by manufacturer's designation] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

          - Appearance of Finished Work: Noticeable variations in same piece are not acceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

"Gold Anodic Finish" Paragraph below is generally available only on flagpoles 20 feet (6 m) or less in height.

        - Gold Anodic Finish: AAMA 611, AA-M32C22A43; gold color.

"Baked-Enamel or Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

        - Baked-Enamel or Powder-Coat Finish: AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

        - High-Performance Organic Finish: Two-coat fluoropolymer finish complying with [AAMA 2604] [AAMA 2605] and containing not less than [50] [70] percent PVDF resin by weight in color coat. Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

      - COPPER-ALLOY FINISHES

        - Hand-Rubbed Finish, Lacquered: Directionally textured, fine satin, mechanical finish coated with lacquer specified for copper alloys, applied by air spray in two coats per manufacturer's written instructions, with interim drying, to a total thickness of 1 mil (0.025 mm).

        - Statuary Conversion Coating over Satin Finish: Directionally textured, fine satin, mechanical finish with sulfide conversion coating.

"Color" Subparagraph below is preferred method of specifying because of variations in color.

          - Color: Match Architect's sample.

          - Appearance of Finished Work: Noticeable variations in same piece are not acceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - STAINLESS STEEL FINISHES

        - Surface Preparation: Remove tool and die marks and stretch lines, or blend into finish.

        - Polished Finishes: Grind and polish surfaces to produce uniform finish, free of cross scratches.

          - When polishing is completed, passivate and rinse surfaces. Remove embedded foreign matter and leave surfaces chemically clean.

          - Directional Satin Finish: No. 4.

      - STEEL FINISHES

        - Flagpole Interior Finish: Apply one coat of bituminous paint on interior of flagpole.

        - Galvanized Finish: Hot-dip galvanize after fabrication to comply with ASTM A123/A123M.

        - Polyurethane Enamel Finish: Remove mill scale and rust, if present, from uncoated steel, complying with [SSPC-SP 5/NACE No. 1, "White Metal Blast Cleaning"] [or] [SSPC-SP 8, "Pickling"] <Insert surface preparation method>. After cleaning, apply manufacturer's standard primer and two-coat, high-gloss, high-build, polyurethane-enamel finish.

          - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

        - Baked-Enamel or Powder-Coat Finish: Remove mill scale and rust, if present, from uncoated steel, complying with [SSPC-SP 5/NACE No. 1, "White Metal Blast Cleaning"] [or] [SSPC-SP 8, "Pickling"] <Insert surface preparation method>. After cleaning, apply a conversion coating suited to the organic coating to be applied over it. After cleaning and pretreating, apply manufacturer's standard two-coat, baked-on finish consisting of prime coat and thermosetting topcoat to a minimum dry film thickness of 2 mils (0.05 mm).

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

For exact finish, insert names of coating manufacturers and products.

      - FIBERGLASS FINISHES

        - Fiberglass: UV-light stable, hard, high-gloss gel coat or high-gloss, high-build polyurethane or polyester coating.

          - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

- EXECUTION

      - PREPARATION

Retain first paragraph below for flagpoles set in foundation tubes.

        - Prepare uncoated metal flagpoles that are set in foundation tubes by painting below-grade portions with a heavy coat of bituminous paint.

        - Foundation Excavation: Excavate to neat clean lines in undisturbed soil. Remove loose soil and foreign matter from excavation and moisten earth before placing concrete. Place and compact drainage material at excavation bottom.

        - Provide forms where required due to unstable soil conditions and for perimeter of flagpole base at grade. Secure and brace forms to prevent displacement during concreting.

        - Foundation Tube: Place foundation tube, center, and brace to prevent displacement during concreting. Place concrete. Plumb and level foundation tube and allow concrete to cure.

Retain "Sleeves" Paragraph below for flagpoles that are mounted in sleeves set in concrete foundations.

        - Sleeves: Locate and secure sleeves in forms by bracing to reinforcement and forms.

Retain "Anchor Bolts" Paragraph below for flagpoles that are mounted on baseplates anchored to concrete foundations.

        - Anchor Bolts: Locate and secure anchor bolts in forms with templates and by tying to reinforcement.

        - Place concrete, as specified in Section 033000 "Cast-in-Place Concrete." Compact concrete in place by using vibrators. Moist-cure exposed concrete for no fewer than seven days or use nonstaining curing compound.

        - Trowel exposed concrete surfaces to a smooth, dense finish, free of trowel marks, and uniform in texture and appearance. Provide positive slope for water runoff to perimeter of concrete base.

      - FLAGPOLE INSTALLATION

Insert specific information in this article to suit Project if required.

        - General: Install flagpoles where indicated and according to[ Shop Drawings and] manufacturer's written instructions.

        - Foundation Tube: Place flagpole in tube, seated on bottom plate between steel centering wedges, and install hardwood wedges to secure flagpole in place. Place and compact sand in foundation tube and remove hardwood wedges. Seal top of foundation tube with a 2-inch (50-mm) layer of elastomeric joint sealant and cover with flashing collar.

        - Baseplate: Cast anchor bolts in concrete foundation. Install baseplate on washers placed over leveling nuts on anchor bolts and adjust until flagpole is plumb. After flagpole is plumb, tighten retaining nuts and fill space under baseplate solidly with nonshrink, nonmetallic grout. Finish exposed grout surfaces smooth and slope 45 degrees away from edges of baseplate.

END OF SECTION 107516