Copyright 2012 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 107523 - WALL-MOUNTED FLAGPOLES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes wall-mounted flagpoles made from [aluminum] [copper alloy (bronze)] [stainless steel] [and] [fiberglass].

Retain "Owner-Furnished Material" Paragraph below if Owner is furnishing flags.

        - Owner-Furnished Material: Flags.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 264113 "Lightning Protection for Structures" for connecting wall-mounted metal flagpoles to lightning protection system.

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, operating characteristics, fittings, accessories, and finishes for flagpoles.

Retain "Shop Drawings" Paragraph below if required. Most of below is normally covered in Product Data if not a customized installation.

        - Shop Drawings: For flagpoles.

          - Include plans, elevations, and details. Show general arrangement, jointing, fittings, accessories, grounding, anchoring, and support.

          - Include details of wall-mounted connections and mountings.

Retain "Samples for Verification" Paragraph below if using special finishes.

        - Samples for Verification: For each type of exposed finish, in manufacturer's standard sizes.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

        - Delegated-Design Submittal: For flagpoles.

          - Include loads, point reactions, and locations for attachment of flagpoles to building's structure.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For flagpoles to include in operation and maintenance manuals.

      - COORDINATION

        - Coordinate installation of anchorages for flagpoles. Furnish setting drawings, templates, and directions for installing anchorages that are to be embedded in concrete or masonry. Deliver such items to Project site in time for installation.

      - DELIVERY, STORAGE, AND HANDLING

        - Spiral wrap flagpoles with heavy paper and enclose in a hard fiber tube or other protective container.

- PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

      - MANUFACTURERS

        - Source Limitations: Obtain flagpoles as complete units, including fittings, accessories, bases, and anchorage devices, from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

        - Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design flagpole assemblies.

Retain "Seismic Performance" Paragraph below for projects requiring seismic design. Delete paragraph if performance requirements are indicated on Drawings. Model building codes and ASCE/SEI 7 establish criteria for buildings subject to earthquake motions. Coordinate requirements with structural engineer.

        - Seismic Performance: Flagpole assemblies shall withstand the effects of earthquake motions determined according to [ASCE/SEI 7] <Insert requirement>.

        - Structural Performance: Flagpole assemblies, including anchorages and supports, shall withstand design loads indicated within limits and under conditions indicated.

Consult a structural engineer experienced in engineering flagpole assemblies of type indicated to quantify design loads applicable to Project. Verify compliance with codes; below is based on the IBC. See Evaluations.

          - Wind Loads: Determine according to NAAMM FP 1001. Basic wind speed for Project location is <Insert wind speed>.

Retain one of two options in subparagraph below. Polyester flags exert approximately 40 percent higher loads on flagpoles than do nylon or cotton flags of same size. Consider future load requirements and liability. Revise if required to suit Project.

          - Base flagpole design on [polyester] [nylon or cotton] flags of maximum standard size suitable for use with flagpole or flag size indicated, whichever is more stringent.

      - ALUMINUM FLAGPOLES

Copy this article and re-edit for each product.

        - Aluminum Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from seamless extruded tubing complying with ASTM B241/B241M, Alloy 6063, with a minimum wall thickness of 3/16 inch (4.8 mm).

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Length" Paragraph below if only one flagpole length is required.

        - Exposed Length: [8 feet (2.5 m)] [10 feet (3 m)] [12 feet (3.7 m)] [15 feet (4.6 m)] [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in "Vertical Wall Mount" and "Outrigger Wall Mount" paragraphs below to suit flagpole installation.

        - Vertical Wall Mount: Cast-aluminum mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: [Same as flagpole] [Gold anodic] [Bronze powder coated] [Black powder coated].

        - Outrigger Wall Mount: Aluminum mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: [Same as flagpole] [Gold anodic] [Bronze powder coated] [Black powder coated].

      - COPPER-ALLOY (BRONZE) FLAGPOLES

Copy this article and re-edit for each product.

        - Copper-Alloy (Bronze) Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from seamless pipe or tube complying with ASTM B43 or ASTM B135 (ASTM B135M), Alloy UNS C23000 (red brass, 85 percent copper).

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Length" Paragraph below if only one flagpole length is required.

        - Exposed Length: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in "Vertical Wall Mount" and "Outrigger Wall Mount" paragraphs below to suit flagpole installation.

        - Vertical Wall Mount: Cast-copper-alloy (bronze) mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: Same as flagpole.

        - Outrigger Wall Mount: Copper-alloy (bronze) mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: Same as flagpole.

      - STAINLESS-STEEL FLAGPOLES

Copy this article and re-edit for each product.

Retain "Type 316L" option in "Stainless-Steel Flagpoles" Paragraph below if increased corrosion resistance is required for marine environment.

        - Stainless-Steel Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from pipe, tube, or plate complying with ASTM A312/A312M, ASTM A269, or ASTM A666, [Type 304] [Type 316L].

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Length" Paragraph below if only one flagpole length is required.

        - Exposed Length: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in "Vertical Wall Mount" and "Outrigger Wall Mount" paragraphs below to suit flagpole installation.

        - Vertical Wall Mount: Cast-aluminum mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: [Clear anodic] [Gold anodic] [Bronze powder-coated] [Black powder-coated] [Silver powder-coated].

        - Outrigger Wall Mount: Aluminum mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: [Clear anodic] [Gold anodic] [Bronze powder-coated] [Black powder-coated] [Silver powder-coated].

      - FIBERGLASS FLAGPOLES

Copy this article and re-edit for each product.

        - Fiberglass Flagpoles: [Cone] [Entasis]-tapered flagpoles fabricated from polyester resin reinforced with woven glass-fiber roving with 75 percent of glass fibers parallel to length of flagpole.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Usually retain one option in "Exposed Length" Paragraph below if only one flagpole length is required.

        - Exposed Length: [20 feet (6 m)] [25 feet (7.5 m)] [30 feet (9 m)] [35 feet (11 m)] [40 feet (12 m)] <Insert dimension>.

        - Construct flagpoles in one piece if possible. If more than one piece is necessary, comply with the following:

          - Fabricate shop and field joints without using fasteners, screw collars, or lead calking.

          - Provide flush hairline joints using self-aligning, snug-fitting, internal sleeves.

Retain mounting types in "Vertical Wall Mount" and "Outrigger Wall Mount" paragraphs below to suit flagpole installation.

        - Vertical Wall Mount: Cast-aluminum mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: [Matching flagpole] [Gold anodic] [Bronze powder-coated] [Black powder-coated].

        - Outrigger Wall Mount: Aluminum mounting bracket complete with[ escutcheon,] mounting plate and through-wall anchorage.

          - Finish: [Matching flagpole] [Gold anodic] [Bronze powder-coated] [Black powder-coated].

      - FITTINGS

Use manufacturers' catalogs or product data to insert series, type, model, and designations of other characteristics.

Revise fittings in this article to suit Project. Items listed are typical with most manufacturers; other fittings are also available.

        - Finial Ball: Flush-seam ball, sized as indicated or, if not indicated, to match flagpole-butt diameter.

Retain one or more of four subparagraphs below. If retaining more than one, indicate which flagpole type applies to each subparagraph.

          - 0.063-inch (1.6-mm) spun aluminum[, finished to match flagpole][ with gold anodic finish].

          - 20-oz. (0.70-mm) copper with 23-karat gold-leaf finish.

          - Spun stainless steel, finished to match flagpole.

          - Spun copper alloy, finished to match flagpole.

        - Finial Eagle: Sized [as indicated] [as standard with manufacturer for flagpole size indicated] <Insert size>.

Retain one of two subparagraphs below.

          - Cast aluminum[, finished to match flagpole][ with gold anodic finish].

          - 20-oz. (0.70-mm) copper with 23-karat gold-leaf finish.

Retain one of three remaining paragraphs below. Revise if another finish is required.

        - Internal Halyard, Winch System: Manually operated winch with control stop device and removable handle, stainless-steel cable halyard, and concealed revolving truck assembly with plastic-coated counterweight and sling. Furnish flush access door secured with cylinder lock. Finish truck assembly to match flagpole.

Covers in "Halyard Flag Snaps" Subparagraph below reduce noise of metal snaps hitting flagpole.

          - Halyard Flag Snaps: [Chromium-plated bronze] [Stainless-steel] [Bronze] [Nylon] swivel snap hooks[ with neoprene or vinyl covers]. Furnish two per halyard.

Product in "Plastic Halyard Flag Clips" Subparagraph below is a patented alternative to halyard flag snaps in subparagraph above.

          - Plastic Halyard Flag Clips: Made from injection-molded, UV-stabilized acetal resin (Delrin). Clips attach to flag and have two eyes for inserting both runs of halyards. Furnish two per halyard.

            - <Double click here to find, evaluate, and insert list of manufacturers and products.>

"Internal Halyard, Cam Cleat System" Paragraph below is available for flagpoles 40 feet (12 m) or less in length. Revise if using stationary truck.

        - Internal Halyard, Cam Cleat System: [5/16-inch- (8-mm-) diameter, braided polypropylene] <Insert type> halyard; cam cleat; and concealed revolving truck assembly with plastic-coated counterweight and sling. Furnish flush access door secured with cylinder lock. Finish truck assembly to match flagpole.

Covers in "Halyard Flag Snaps" Subparagraph below reduce noise of metal snaps hitting flagpole.

          - Halyard Flag Snaps: [Chromium-plated bronze] [Stainless-steel] [Bronze] [Nylon] swivel snap hooks[ with neoprene or vinyl covers]. Furnish two per halyard.

Product in "Plastic Halyard Flag Clips" Subparagraph below is a patented alternative to halyard flag snaps in subparagraph above.

          - Plastic Halyard Flag Clips: Made from injection-molded, UV-stabilized acetal resin (Delrin). Clips attach to flag and have two eyes for inserting both runs of halyards. Furnish two per halyard.

            - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Revise "External Halyard" Paragraph below if using stationary truck.

        - External Halyard: Ball-bearing, nonfouling, revolving truck assembly of cast metal with continuous [5/16-inch- (8-mm-) diameter, braided polypropylene halyard] <Insert type> and 9-inch (228-mm) cast-metal cleats with fasteners. Finish exposed metal surfaces to match flagpole.

          - Halyards and Cleats: [One] [Two] at each flagpole.

Covers in "Halyard Flag Snaps" Subparagraph below reduce noise of metal snaps hitting flagpole (external halyards will still slap).

          - Halyard Flag Snaps: [Chromium-plated bronze] [Stainless-steel] [Bronze] [Nylon] swivel snap hooks[ with neoprene or vinyl covers]. Furnish two per halyard.

Product in "Plastic Halyard Flag Clips" Subparagraph below is a patented alternative to halyard flag snaps in subparagraph above.

          - Plastic Halyard Flag Clips: Made from injection-molded, UV-stabilized acetal resin (Delrin). Clips attach to flag and have two eyes for inserting both runs of halyards. Furnish two per halyard.

            - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - ALUMINUM FINISHES

Retain finishes in paragraphs below to suit Project. If retaining more than one, indicate location of each on Drawings or by inserts.

        - Natural Satin Finish: AA-M32, fine, directional, medium satin polish; buff complying with AA-M20; seal aluminum surfaces with clear, hard-coat wax.

Retain one of two options in "Clear Anodic Finish" Paragraph below. Verify availability with manufacturers.

        - Clear Anodic Finish: AAMA 611, [AA-M12C22A41] [AA-M12C22A31].

Retain one of two options in "Color Anodic Finish" Paragraph below. Verify availability with manufacturers.

        - Color Anodic Finish: AAMA 611, [AA-M12C22A42/A44] [AA-M12C22A32/A34].

Options in first "Color" Subparagraph below are examples only and may vary in color range and availability among manufacturers. Retain one or delete all and retain one of two options in second "Color" Subparagraph below.

          - Color: [Light bronze] [Medium bronze] [Dark bronze] [Black] <Insert color>.

          - Color: [Match Architect's sample] [As selected by Architect from full range of industry colors and color densities].

          - Appearance of Finished Work: Noticeable variations in same piece are not acceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

"Gold Anodic Finish" Paragraph below is generally available only on flagpoles 20 feet (6 m) or less in length.

        - Gold Anodic Finish: AAMA 611, AA-M32C22A43; gold color.

"Baked-Enamel or Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

        - Baked-Enamel or Powder-Coat Finish: AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

        - High-Performance Organic Finish: Two-coat fluoropolymer finish complying with [AAMA 2604] [AAMA 2605] and containing not less than [50] [70] percent PVDF resin by weight in color coat. Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

          - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

      - COPPER-ALLOY FINISHES

        - Hand-Rubbed Finish, Lacquered: Directionally textured, fine satin, mechanical finish coated with lacquer specified for copper alloys, applied by air spray in two coats per manufacturer's written instructions, with interim drying, to a total thickness of 1 mil (0.025 mm).

        - Statuary Conversion Coating over Satin Finish: Directionally textured, fine satin, mechanical finish with sulfide conversion coating.

"Color" Subparagraph below is preferred method of specifying because of variations in color.

          - Color: Match Architect's sample.

          - Appearance of Finished Work: Noticeable variations in same piece are not acceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - STAINLESS-STEEL FINISHES

        - Surface Preparation: Remove tool and die marks and stretch lines, or blend into finish.

        - Polished Finishes: Grind and polish surfaces to produce uniform finish, free of cross scratches.

          - When polishing is completed, passivate and rinse surfaces. Remove embedded foreign matter and leave surfaces chemically clean.

          - Directional Satin Finish: No. 4.

      - FIBERGLASS FINISHES

        - Fiberglass: UV-light stable, hard, high-gloss gel coat or high-gloss, high-build polyurethane or polyester coating.

          - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - FLAGPOLE INSTALLATION

Insert specific information in this article to suit Project if required.

        - General: Install flagpoles where shown and according to[ Shop Drawings and] manufacturer's written instructions.

        - Mounting Brackets and Bases: Anchor brackets and bases securely to structural support with fasteners as indicated on Shop Drawings.

END OF SECTION 107523