Copyright 2014 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 122200 - CURTAINS AND DRAPES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Drapes.

          - Drapery tracks.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 057500 "Decorative Formed Metal" for drapery pockets.

      - ALLOWANCES

Retain products and Work included in this Section that are covered by cash or quantity allowance. Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the allowance.

        - Drapery fabrics are part of <Insert name of allowance>.

      - ACTION SUBMITTALS

        - Product Data: For the following:

          - Drapery Tracks: Include maximum weights of drapes that can be supported.

Retain "Motorized Tracks" Subparagraph below if required.

            - Motorized Tracks: Indicate motor weights, motor-mounting requirements, and electrical requirements.

          - Fabrics.

          - Textile treatments.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

        - Shop Drawings:

          - Drapery Tracks: Show installation and anchorage details and locations of controls.

Retain "Motorized Tracks" Subparagraph below if required.

            - Motorized Tracks: Indicate dimensions, weights, and required clearances for track and motor and differentiate between manufacturer-installed and field-installed wiring.

          - Drapes: Show sizes, locations, and details of installation.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples: As follows:

          - Drapery Tracks: 18 inches (450 mm) long, with carriers, controls, and accessories.

          - Drapery Fabrics: For each color and pattern indicated, full width by 36 inches (1000 mm) long, from dye lot to be used for the Work and with specified textile treatments applied. Show complete pattern repeat if any. Mark top and face of fabric.

          - Textile Trims: For each color and pattern indicated, 18 inches (450 mm) long.

          - Drape Fabrication: For each heading, fabric, color, and pattern indicated, a complete full-size panel to verify details of fabrication and thread colors.

        - Samples for Initial Selection: For each type of product indicated.

        - Samples for Verification: As follows:

          - Drapery Tracks: 18 inches (450 mm) long, with carriers, controls, and accessories.

          - Drapery Fabrics: For each color and pattern indicated, full width by 36 inches (1000 mm) long, from dye lot to be used for the Work and with specified textile treatments applied. Show complete pattern repeat if any. Mark top and face of fabric.

          - Textile Trims: For each color and pattern indicated, 18 inches (450 mm) long.

          - Drape Fabrication: For each heading, fabric, color, and pattern indicated, a complete full-size panel to verify details of fabrication and thread colors.

        - Product Schedule: For [drapes] [and] [drapery tracks]. Use same designations indicated on Drawings.

      - INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below for situations where limited space necessitates maximum utilization for efficient installation of different components or if coordination is required for installation of products and materials by separate installers. Coordinate paragraph with other Sections specifying products listed below. Preparation of coordination drawings requires the participation of each trade involved in installations within the limited space. Retain for drapery track installation if Drawings do not include detailed reflected ceiling plans or if Project involves unusual coordination requirements.

        - Coordination Drawings: For drapery track installation; reflected ceiling plans drawn to scale and coordinating track installation with openings and ceiling-mounted items, on which the following items are shown:

          - Suspended ceiling components.

Retain two subparagraphs below if drapery tracks are motorized.

          - Structural members to which motors are attached.

          - Size and location of motor access panel.

Retain "Product Certificates" Paragraph below if flame-retardant treatments are required for fabrics.

        - Product Certificates: For each drapery fabric treated with flame retardant, signed by fabric supplier and indicating treatment durability and cleaning procedures required to maintain treatment effectiveness.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For products installed to include in maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

        - Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Drapery Track Carriers: For each size indicated, equal to [5] <Insert number> percent of amount installed, but no fewer than [10] <Insert number> of each size.

          - Drapery Track Controls: For each type indicated, equal to [5] <Insert number> percent of amount installed, but no fewer than [10] <Insert number> of each type.

          - Drapery Fabrics: For each fabric, color, and pattern indicated, from the same product run, full-width lengths equal to [5] <Insert number> percent of amount installed, but no fewer than [10] <Insert number> yards of each fabric, color, and pattern.

      - QUALITY ASSURANCE

Retain "Installer Qualifications" Paragraph below if fabricator of drapes is expected to be local to Project site.

        - Installer Qualifications: For drapes and drapery tracks, fabricator of drapes.

        - Mockups: Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for fabrication and installation.

          - Build mockup at location and in size shown on Drawings.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - FIELD CONDITIONS

Retain this article for drapes.

        - Field Measurements: Verify dimensions by field measurements before drape fabrication, and indicate measurements on Shop Drawings.

        - Scheduling: Do not deliver or install drapes until after other finish work, including painting, is complete and spaces are otherwise ready for occupancy.

If drapery fabrics might "grow" over a heating and cooling cycle, consider inserting a "Maintenance Service" Article here that requires temporary hemming of drapes initially and final hemming at end of defined period.

- PRODUCTS

Manufacturers and products listed in SpecAgent and Masterworks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - DRAPERY TRACKS

If more than one type of manually operated track is required, copy first paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Manually Operated Track <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Construction: Extruded aluminum, slotted for mounting at interval of not more than 24 inches (610 mm) o.c.[, and bendable to radii indicated.]

            - Lengths and Configurations: [As indicated on Drawings] <Insert requirements>.

            - Support Capability: [Weight of drape indicated] [30 lb (14 kg)] [45 lb (20 kg)] [60 lb (27 kg)] [80 lb (36 kg)] [140 lb (64 kg)] [210 lb (95 kg)] <Insert weight> mounted on track length indicated.

            - Finish: [Manufacturer's standard] [White baked enamel] [Clear anodic coating] <Insert finish>.

          - Mounting Brackets: Aluminum, of type suitable for fastening track to surface indicated and designed to support weight of track assembly and drape plus force applied to operate track.

            - Mounting Surface: [As indicated on Drawings] [Wall] [Ceiling] [Drapery pocket] <Insert description>.

            - Size: [Adjustable] <Insert dimension>.

          - Installation Fasteners: Sized to support track assembly and drape, and fabricated from metal compatible with track, brackets, and supporting construction. Provide two fasteners to fasten each bracket to supporting construction.

          - Operation: [Baton] [Cord complying with WCMA A 100.1] [Cord tension pulley complying with WCMA A 100.1].

Retain "Pulley Mounting Location" Subparagraph below for cord-tension-pulley operation.

            - Pulley Mounting Location: [Wall] [Baseboard] [Floor].

            - Draw: [One way, stack as indicated on Drawings] [One way, stack left] [One way, stack right] [Two way, center opening].

            - Operating Hardware Location: [On stack side] [Left] [Right] [As indicated on Drawings].

          - Carriers: [Rollers] [Rollers with hooks] [Rollers with snaps] [Slides] [Slides with hooks] [Coordinate with drapery headings indicated] <Insert description>.

In "Master Carriers" Subparagraph below, retain butt type for one-way draw drapes and butt or overlap for two-way, center-opening drapes.

            - Master Carriers: [Butt] [Overlap].

          - End Stops: Manufacturer's standard[ with track end cap].

          - Pulleys: [Standard] [Heavy] duty.

          - Accessories: <Insert accessories>.

If more than one type of motorized track is required, copy paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Motorized Track <Insert drawing designation>:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

          - Construction: Extruded aluminum, slotted for mounting at interval of not more than 24 inches (610 mm) o.c.[, and bendable to radii indicated.]

            - Lengths and Configurations: [As indicated on Drawings] <Insert requirements>.

            - Support Capability: [Weight of drape indicated] [30 lb (14 kg)] [45 lb (20 kg)] [60 lb (27 kg)] [80 lb (36 kg)] [140 lb (64 kg)] [210 lb (95 kg)] <Insert weight> mounted on track length indicated.

            - Finish: [Manufacturer's standard] [White baked enamel] [Clear anodic coating] <Insert finish>.

          - Mounting Brackets: Suitable for fastening track to surface indicated and designed to support weight of track assembly and drape plus force applied to operate track.

            - Mounting Surface: [As indicated on Drawings] [Wall] [Ceiling] [Drapery pocket] <Insert description>.

          - Installation Fasteners: Sized to support track assembly and drape, and fabricated from metal compatible with track, brackets, and supporting construction. Provide two fasteners to fasten each bracket to supporting construction.

          - Motor Operation: Low-voltage motor with built-in low-voltage interface for direct access to control systems, with thermal-overload switch; sized for weight of drape and track length indicated; and equipped with stops to prevent overdrawing.

            - Control: [Wall switch] [Remote, infrared] [Remote, radio controlled] [Digital timer] <Insert description>.

            - Draw: [One way, stack as indicated on Drawings] [One way, stack left] [One way, stack right] [Two way, center opening].

            - Electrical Requirements: [As indicated on Drawings] [115 V/60 Hz/120 W/1.10 A] [115 V/60 Hz/140 W/1.20 A] [110 V/60 Hz/150 W/1.0 A] [110 V/60 Hz/550 W/5.0 A] <Insert requirements>.

            - Travel Speed: [6 inches (152 mm)] [8 inches (203 mm)] [12 inches (305 mm)] per second.

          - Carriers: [Rollers] [Rollers with hooks] [Rollers with snaps] [Slides] [Slides with hooks] [Coordinate with drapery headings indicated] <Insert description>.

In "Master Carriers" Subparagraph below, retain butt type for one-way draw drapes and butt or overlap for two-way, center-opening drapes.

            - Master Carriers: [Butt] [Overlap].

          - End Stops: Manufacturer's standard[ with track end cap].

          - Pulleys: [Standard] [Heavy] duty.

          - Accessories: <Insert accessories>.

      - DRAPES

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Source Limitations: Obtain each color and pattern of drapery fabric and trim from one dye lot.

Retain "Fire-Test-Response Characteristics" Paragraph below if flame-retardant treatments are required for drapery fabrics. Model codes require testing according to NFPA 701, Test Method 1 or 2, as appropriate, for drapes used in certain occupancies. See "Flame-Resistance Testing" Article in the Evaluations for a discussion of the applicability of each test method.

        - Fire-Test-Response Characteristics: For fabrics treated with fire retardants, provide products that pass NFPA 701 as determined by testing of fabrics that were treated using treatment-application method intended for use for this Project by a testing and inspecting agency acceptable to authorities having jurisdiction.

If more than one type of drape is required, copy paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Drape <Insert drawing designation>:

          - Heading:

Retain one of first five subparagraphs below based on heading requirements. See "Drape Construction" Article in the Evaluations for a discussion and illustrations of headings; see "Drape Terminology" Article in the Evaluations for a discussion of fullness. Option for three-fold pleats in "Pinch (French) Pleats" Subparagraph below is most common.

            - Pinch (French) Pleats: [100] [150] [200] [250] percent fullness; [two] [three] [four] [five]-fold (fingers) each pleat.

            - Stack Pleats: [60] [80] [100] [120] percent fullness.

            - Roll Pleats: [60] [80] [100] [120] percent fullness.

            - Accordion Pleats: [40] [80] [100] [120] percent fullness.

            - <Insert heading description>: <Insert number> percent fullness.

Retain "Pleat Spacing" Subparagraph below with any pleat type above. Pleats are often spaced 4 to 6 inches (102 to 152 mm) apart.

            - Pleat Spacing: <Insert dimension>.

            - Heading Accessories:

Coordinate heading accessories with pleat style and track requirements.

              - Nonwoven buckram.

              - Woven snap tape, [7/8 inch (22 mm) wide, with nickel-plated snaps at 4 inches (102 mm) o.c.] <Insert description>.

              - Hooks.

              - <Insert accessory>.

          - Drapery Fabric:

Specific drapery fabrics are usually named to ensure that products satisfy aesthetic and quality requirements. If specific fabric has not been selected, a fabric allowance can be established.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Fiber Content: <Insert description>.

          - <Double click to insert sustainable design text for recycled content.>

"Run right" describes orientation of fabric; see "Drape Terminology" Article in the Evaluations for definition.

          - Orientation: Run right (up the bolt).

Retain "Width" and "Pattern Repeat Distance" subparagraphs below unless it is bidder's responsibility to determine width and pattern repeat; retain to allow bidder to conveniently estimate yardage requirements.

          - Width: <Insert width>.

          - Pattern Repeat Distance: <Insert vertical distance> vertically; <Insert horizontal distance> horizontally.

See "Textile Treatments" and "Flame-Resistance Testing" articles in the Evaluations for a discussion of textile treatments. Inherently flame-resistant fabrics are available.

          - Textile Treatments: [Stain repellent] [Flame retardant, polymer type] [Stain repellent; and flame retardant, polymer type] <Insert requirements>.

          - Lining Fabric:

            - Lining Type: [Blackout] [Thermal reflective] [Water resistant] <Insert description>.

Retain "Product" Subparagraph below if required.

            - Product: [Selected by fabricator for use with drapery fabric indicated] <Insert manufacturer's name; product name or designation>.

            - Color: <Insert description or colorway designation>.

            - Fiber Content: <Insert description>.

            - <Double click to insert sustainable design text for recycled content.>

Retain "Width" Subparagraph below unless it is fabricator's responsibility to determine width; retain to limit number of vertical seams in lining.

            - Width: <Insert width>.

            - Textile Treatments: [Stain repellent] [Flame retardant, polymer type] [Stain repellent; and flame retardant, polymer type] <Insert requirements>.

          - Interlining: [Acoustical] [Blackout] [Thermal] <Insert description>.

Retain "Product" Subparagraph below if required.

            - Product: [Selected by fabricator for use with drapery fabric indicated] <Insert manufacturer's name; product name or designation>.

          - Textile Trim: <Insert description for each type of trim required; include manufacturers, products, and colors>.

          - Tiebacks: <Insert description; include manufacturer's name, product name or designation, and color>.

Retain first option in "Hem Weights" Subparagraph below for opaque fabrics; retain second option for sheer or light-colored fabrics.

          - Hem Weights: [1-inch- (25-mm-) square lead weights] [Tape type (string weights)].

      - DRAPE FABRICATION

Requirements in this article are examples only; revise to suit Project.

        - Fabricate drapes in heading styles and fullnesses indicated. Fabricate headings to stand erect. If less than a full width of fabric is required to produce panel of specified fullness, use equal widths of not less than one-half width of fabric located at ends of panel.

          - One-Way-Stacking Drapes: Add 5 inches (127 mm) to overall width for returns.

          - Center-Opening Drapes: Add 10 inches (254 mm) to overall width for overlap.

        - Seams: Sew vertical seams with twin-needle sewing machine with selvage trimmed and overlocked. Join widths so that patterns match and vertical seams lay flat and straight without puckering. Horizontal seams are unacceptable.

        - Side Hems: Double-turned, 1-1/2-inch- (38-mm-) wide hems consisting of three layers of fabric, and blindstitched so that stitches are invisible on face of drape.

If drapery fabrics might "grow" over a heating and cooling cycle, consider requiring temporary hemming of drapes initially and final hemming at end of defined period in a "Maintenance Service" Article.

        - Bottom Hems: Double-turned, 4-inch- (102-mm-) wide hems consisting of three layers of fabric, and weighted and blindstitched so that weights and stitches are invisible on face of drape.

Retain subparagraph below if square lead weights are required.

          - Sew in square lead weights at each seam and at panel corners.

        - Interlinings: Extend from top of drape to within 1/2 inch (13 mm) of lining's bottom hem and to leading edge of side hems to produce full-shadowed appearance.

        - Linings: Equal to widths of drapery fabric and joined to drapery fabric at top by inside invisible seam, and hand stitched at side hems and shadowed with 1-1/2-inch (38-mm) return of face fabric.

          - Bottom Hem: [Hem separately from] [Blind stitch to] drapery fabric.

- EXECUTION

      - DRAPERY TRACK INSTALLATION

        - Install track systems according to manufacturer's written instructions, level and plumb, and at height and location in relation to adjoining openings as indicated on Drawings.

        - Isolate metal parts of tracks and brackets from concrete, masonry, and mortar to prevent galvanic action. Use tape or another method recommended in writing by track manufacturer.

      - DRAPE INSTALLATION

        - Where drapes abut overhead construction, hang drapes so that clearance between headings and overhead construction is 1/4 inch (6.4 mm).

        - Where drapes extend to floor, install so that bottom hems clear finished floor by not more than 1 inch (25 mm) and not less than 1/2 inch (13 mm).

        - Where drapes extend to windowsill, install so that bottom hems hang above sill line and clear sill line by not more than 1/2 inch (13 mm).

Insert other length requirements if necessary to suit Project.

      - ADJUSTING

        - After hanging drapes, test and adjust each drapery track to produce unencumbered, smooth operation.

        - Steam and dress down drapes as required to produce crease- and wrinkle-free installation.

        - Remove and replace drapes that are stained or soiled.

END OF SECTION 122200