Copyright 2014 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 123661.13 - CULTURED MARBLE COUNTERTOPS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Cultured marble countertops.

          - Integral sinks.

          - Integral backsplashes.

          - Integral end splashes.

          - Loose end splashes.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 224100 "Residential Plumbing Fixtures" for plumbing fittings.

      - ACTION SUBMITTALS

        - Product Data: For countertop materials.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for adhesives.>

        - Shop Drawings: For countertops. Show materials, finishes, edge and backsplash profiles, [integral sinks, ]and methods of joining.

        - Samples for Initial Selection: For each type of material exposed to view.

Delete "Samples for Initial Selection" Paragraph above if colors and other characteristics are preselected and specified or scheduled. Retain "Samples for Verification" Paragraph below with or without paragraph above.

        - Samples for Verification: For the following products:

          - Countertop material, 6 inches (150 mm) square.

          - One full-size cultured marble countertop, with front edge[ and backsplash], 8 by 10 inches (200 by 250 mm), of construction and in configuration specified.

      - COORDINATION

        - Coordinate locations of utilities that will penetrate countertops or backsplashes.

- PRODUCTS

Manufacturers and products listed in SpecAgent and Masterworks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

      - CULTURED MARBLE COUNTERTOPS

        - Cultured Marble: Gel-coated solid fabrication of filled plastic resin complying with CSA B45.5/IAPMO Z124.

          - Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range].

        - Configuration: One-piece units with integral sink bowls and backsplashes unless otherwise indicated, not less than 1/2 inch (12.7 mm) thick.

Retain one front and one backsplash style in first two subparagraphs below or revise to suit Project. Delete all three subparagraphs below if countertop is indicated on Drawings.

          - Front: [No drip (raised marine edge)] [Straight, slightly eased at top] [Radius edge with integral apron, 2 inches (50 mm) high with 3/8-inch (9.5-mm) radius] [No drip (raised marine edge) with separate apron, 6 inches (150 mm) high, recessed 3/8-inch (9.5-mm) behind front edge].

          - Backsplash: [Coved, with 3/8-inch (9.5-mm) radius cove and top] [Straight, with 3/8-inch (9.5-mm) radius cove and slightly eased at top] [Straight, slightly eased at cove and top].

          - End Splash: [Matching backsplash] [None].

      - INSTALLATION MATERIALS

        - Adhesive: Product recommended by cultured marble manufacturer.

          - <Double click to insert sustainable design text for VOC content of adhesive.>

          - <Double click to insert sustainable design text for low emitting adhesives.>

Sealant may be required for joints at walls. If retaining "Sealant for Countertops" Paragraph below, coordinate with Section 079200 "Joint Sealants."

        - Sealant for Countertops: Comply with applicable requirements in Section 079200 "Joint Sealants."

- EXECUTION

      - INSTALLATION

        - Install countertops level to a tolerance of 1/8 inch in 8 feet (3 mm in 2.4 m).

        - Fasten countertops by screwing through corner blocks of base units into underside of countertop. Predrill holes for screws as recommended by manufacturer.

          - Install end splashes to comply with manufacturer's written instructions.

        - Install aprons to backing and countertops with adhesive. Fasten by screwing through backing. Predrill holes for screws as recommended by manufacturer.

        - Apply sealant to gaps at walls; comply with Section 079200 "Joint Sealants." Remove temporary shims before applying sealant.

END OF SECTION 123661.13