Copyright 2015 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 126313 - STADIUM AND ARENA BENCH SEATING

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes fixed, continuous bench- or plank-type metal seating.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 126100 "Fixed Audience Seating" for fixed chair-type seating.

          - Section 126600 "Telescoping Stands" for multiple-tiered benches on interconnected folding platforms that close for storing.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of components, and finishes for seating.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

        - Shop Drawings: For bench seating.

          - Include plans, elevations, sections, details, and attachments to other work.

          - Seating Layout: Show seating layout, aisle widths, aisle-end alignment or stepping, and row-lettering scheme.

        - Samples for Initial Selection: For each type of exposed material, color, finish, and texture indicated.

          - Include Samples of accessories involving color and finish selection.

        - Samples for Verification: For each type of exposed finish, prepared on Samples of size indicated below:

          - Aluminum Finishes: Manufacturer's standard-size unit, not less than 3 inches (75 mm) square.

          - Baked-on Coating Finishes: Manufacturer's standard-size unit, not less than 3 inches (75 mm) square.

          - Row-Letter Plates: Full-size units with characters marked.

          - Seat-Number Plates: Full-size units with characters marked.

          - Donor Plates: Full-size units.

          - Exposed Fasteners: Full-size units of each type.

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data: For [Installer] [and] [manufacturer].

Retain "Product Certificates" and "Material Certificates" paragraphs below to require submittal of certificates from manufacturers.

        - Product Certificates: For each type of bench seating.

        - Sample Warranty: For special warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For bench seating to include in maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

        - Furnish extra materials, from the same production run, that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Revise list below to suit Project. Insert actual quantities rather than percentages when practical.

          - Individual Seat Pads: [5] <Insert number> percent of quantity installed for each color.

          - Individual Backrest Pads: [5] <Insert number> percent of quantity installed for each color.

          - Row-Letter Plates: [100] <Insert number>, [blank] [with characters marked] <Insert requirement>.

          - Seat-Number Plates: [100] <Insert number>, [blank] [with characters marked] <Insert requirement>.

          - Donor Plates: [100] <Insert number>, blank.

      - QUALITY ASSURANCE

Manufacturer may require "Installer Qualifications" Paragraph below as a condition of warranty.

        - Installer Qualifications: An entity that employs installers and supervisors who are trained and approved by manufacturer.

        - Mockups: Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for fabrication and installation.

Indicate portion of seating represented by mockup on Drawings or draw mockup as separate element.

          - Build mockups of [area of bench seating indicated on Drawings] <Insert requirement>, including finishes and accessories.

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty: Manufacturer agrees to repair or replace components of bench seating that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

Revise list below to suit Project.

            - Structural failures including bench seating and attached components.

            - Deterioration of metals, metal finishes, and other materials beyond normal weathering and use.

            - <Insert failure modes>.

Verify available warranties and warranty periods for seating and components.

          - Warranty Periods: [Five] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in SpecAgent and Masterworks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

        - Source Limitations: Obtain each type of bench seating required, including accessories and mounting components, from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

        - Structural Performance: Bench seating shall withstand the effects of gravity and other loads within limits and under conditions indicated.

Subparagraphs below are examples only; revise to suit Project and to comply with requirements of authorities having jurisdiction.

          - Vertical Live Load: [120 lb/ft. (179 kg/m)] [As indicated on Drawings] <Insert load>.

          - Horizontal Sway Loads: [24 lb/ft. (36 kg/m) parallel to bench and 10 lb/ft. (15 kg/m) perpendicular to bench] [As indicated on Drawings] <Insert loads>.

        - Thermal Movements: Allow for thermal movements from ambient and surface temperature changes.

Differential values in "Temperature Change" Subparagraph below (for aluminum, in particular) are suitable for most of the United States.

          - Temperature Change: [120 deg F (67 deg C), ambient; 180 deg F (100 deg C), material surfaces] <Insert temperature change>.

      - BENCH SEATING <Insert drawing designation>

Copy this article and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Description: [Plank-type seating in permanent arrangement] <Insert description> as indicated on Drawings.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Available options in "Seat Planks," "Backrest," "Seat-Plank Support Brackets," and "Backrest Brackets" paragraphs below vary with manufacturer; revise to suit Project.

        - Seat Planks: Horizontal, continuous bench components on which spectators sit.

          - Material: One-piece [fluted] [and] [contoured] aluminum extrusion, approximately [12 inches wide by 1-1/2 inches thick (300 mm wide by 40 mm thick)] [10 inches wide by 1-1/2 inches thick (250 mm wide by 40 mm thick)] <Insert dimensions>.

          - Finish: [Clear anodic] [Baked-enamel or powder-coat] [High-performance organic] finish.

Retain "Color and Gloss" Subparagraph below if baked-enamel, powder-coat, or high-performance organic finish is required.

            - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

          - End-Cap Finish: [Clear anodic finish] [Match seat planks] <Insert requirement>.

        - Backrest: Bracket mounted to underside of the seat planks.

          - Material: One-piece[ contoured] aluminum extrusion, approximately [5-1/2 inches high by 1-1/2 inches thick (140 mm high by 50 mm thick)] <Insert dimensions>.

          - Finish: [Clear anodic] [Baked-enamel or powder-coat] [High-performance organic] finish.

Retain "Color and Gloss" Subparagraph below if baked-enamel, powder-coat, or high-performance organic finish is required.

            - Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

          - End-Cap Finish: [Clear anodic finish] [Match seat planks] <Insert requirement>.

        - Seat-Plank Support Brackets: Manufacturer's standard one-piece, [welded-aluminum] [or] [welded-steel] brackets that support and raise the seating off the supporting substrate, as indicated on Drawings.

Retain one option in "Type" Subparagraph below; revise to suit Project. Availability of each type may vary with manufacturer.

          - Type: [Floor-attached brackets] [L-shaped, riser-mounted brackets to align back of seat plank with riser face] [T-shaped, riser-mounted brackets to center the seat plank over the riser face] [Z-shaped, horizontal-surface brackets] <Insert requirement>.

          - Finish of Visible Surfaces: [Match seat planks] <Insert requirement>.

        - Backrest Brackets: Manufacturer's standard one-piece, extruded- and formed-aluminum brackets that connect the backrest to underside of the seat planks, as indicated on Drawings.

          - Provide with[ tightly fitting end caps] and internal reinforcement, as required for secure attachment to backrest and underside of seat planks.

          - Finish of Visible Surfaces: [Match backrest] [Match seat planks] <Insert requirement>.

        - Individual Seat Pads: Manufacturer's standard molded-plastic units with smoothly rolled edges, reinforcing ribs on underside to prevent deflection and strengthen attachment points, and designed for attachment to seat plank.

        - Individual Backrest Pads: Manufacturer's standard UV-resistant, molded-plastic units with smoothly rolled edges, reinforcing ribs on underside to prevent deflection and strengthen attachment points, and designed for attachment to backrest.

      - ACCESSORIES

Retain "Row-Letter Plates," "Seat-Number Plates," and "Donor Plates" paragraphs below, if required and attached to the seating; revise to suit Project. Availability varies with manufacturer.

        - Row-Letter Plates: [Manufacturer's standard] <Insert description>.

          - Material: Clear-anodized aluminum with black[ embossed] characters .

          - Location: [As indicated on Drawings] [On aisle end of seat plank] <Insert requirement>.

          - Attachment: [Manufacturer's standard method] [Minimum of four tamper-resistant mechanical fasteners] <Insert requirement>.

        - Seat-Number Plates: [Manufacturer's standard] <Insert description>.

          - Material: Clear-anodized aluminum with black[ embossed] characters.

          - Location: [As indicated on Drawings] [On top-front of individual backrest pads] <Insert requirement>.

          - Attachment: [Manufacturer's standard method] [Minimum of four tamper-resistant mechanical fasteners] <Insert requirement>.

        - Donor Plates: [Manufacturer's standard] <Insert description>.

          - Material: Clear-anodized aluminum with black[ embossed] characters.

          - Location: [As indicated on Drawings] [On top-front of individual backrest pads] [On aisle end of seat plank] [On aisle end of backrest] <Insert requirement>.

          - Attachment: [Manufacturer's standard method] [Minimum of four tamper-resistant mechanical fasteners] <Insert requirement>.

      - MATERIALS

        - <Double click to insert sustainable design text for recycled content of aluminum.>

Retain required materials in remaining paragraphs below; revise material requirements or insert other materials to suit Project.

        - Aluminum: Alloy and temper recommended in writing by manufacturer for type of use and finish indicated.

        - Steel: Hot-dip galvanized after fabrication.

        - Molded Plastic: High-density plastic, blow or injection molded, with surface that is mar and dent resistant.

If more than one molded-plastic color or texture is required, indicate locations of each on Drawings or by inserts.

          - Color and Texture: [As indicated by manufacturer's designations] [As selected by Architect from manufacturer's full range] <Insert color and texture>.

          - Provide with UV inhibitors to retard fading.

        - Fasteners: Aluminum, hot-dip galvanized steel, or stainless steel. Use fasteners of the same basic metal as fastened metal unless otherwise indicated. Use metals that are noncorrosive and compatible with each metal joined.

          - Use concealed fasteners for interconnecting metal components and for attaching them to other work unless exposed fasteners are unavoidable.

Revise first subparagraph below if another screw type is required.

          - For exposed fasteners, use tamper-resistant screws of head profile flush with metal surface unless otherwise indicated.

          - Finish heads of exposed fasteners to match finish of metal fastened unless otherwise indicated.

          - Do not use power-actuated fasteners for concrete substrates.

      - GENERAL FINISH REQUIREMENTS

        - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products (AMP 500-06)" for recommendations for applying and designating finishes.

        - Appearance of Finished Work: Noticeable variations in same piece are not acceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - ALUMINUM FINISHES

If retaining more than one finish in paragraphs below, indicate location of each on Drawings or by inserts.

Retain one of two options in "Clear Anodic Finish" Paragraph below. First option is standard with manufacturers; verify availability before retaining second option.

        - Clear Anodic Finish: AAMA 611, [AA-M12C22A41, Class I, 0.018 mm] [AA-M10C22A31, Class II, 0.010 mm] or thicker.

Manufacturers may offer product specified in "Baked-Enamel or Powder-Coat Finish" and "High-Performance Organic Finish" paragraphs below; generally, manufacturers' have incomplete finish descriptions in their product literature. Consult manufacturers for availability.

"Baked-Enamel or Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

        - Baked-Enamel or Powder-Coat Finish: AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

Retain AAMA 2604 or AAMA 2605 in "High-Performance Organic Finish" Paragraph below for high- or superior-performance organic coatings, respectively, on extrusions and panels. If specific products are required, name coating manufacturers and products.

        - High-Performance Organic Finish: Two-coat fluoropolymer finish complying with [AAMA 2604] [AAMA 2605] and containing not less than [50] [70] percent PVDF resin by weight in color coat. Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

      - FABRICATION

Retain and revise paragraphs in this article to suit Project. Fabrication methods vary among manufacturers.

        - Seat-Plank Assembly: Assemble seat plank sections end to end to form continuous benches of lengths indicated on Drawings. Provide tightly fitting aluminum end caps, interior joint sleeves at splices, and welded mounting plates for attaching support[ and backrest] brackets.

        - Backrest Assembly: Assemble backrest sections end to end to form continuous backrest surface of lengths indicated on Drawings. Provide tightly fitting aluminum end caps, interior joint sleeves at splices, and welded mounting plates for attaching to backrest brackets.

        - Shop-Fabricated Connections: Welded according to AWS standards by AWS-certified welders.

- EXECUTION

      - EXAMINATION

        - Examine floors, risers, and other adjacent work and conditions, with Installer present, for compliance with requirements and other conditions affecting performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

Retain applicable paragraphs below; revise to suit Project.

        - Install seating in locations indicated and fasten to substrates according to manufacturer's written installation instructions unless otherwise indicated.

        - Install seating level, with uniform bench height above walking surface, and without sharp edges.

        - Attach end caps tightly abutting ends of seat planks, with exposed end-cap contour aligned with contour of seat planks.

        - Space and attach seat-plank support brackets only to substrate concrete or structural components of metal substrates.

        - Install individual seat and backrest pads equally spaced along length of bench seating.

        - Install accessories in visual alignment.

      - ADJUSTING

        - Repair minor abrasions and imperfections in finishes with coating that matches factory-applied finish.

        - Replace damaged and malfunctioning components that cannot be acceptably repaired.

END OF SECTION 126313