Copyright 2014 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 129200 - INTERIOR PLANTERS AND ARTIFICIAL PLANTS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Freestanding interior planters.

          - Freestanding planters with built-in subirrigation.

          - Site-installed, self-contained subirrigation units.

          - Planter liners.

          - Manufactured interior plants.

        - Owner-Furnished, Contractor-Installed Material: <Insert product>.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 033000 "Cast-in-Place Concrete" for site-built, cast-in-place concrete planters.

          - Section 033300 "Architectural Concrete" for site-built, cast-in-place architectural concrete planters.

          - Section 034500 "Precast Architectural Concrete" for plant-cast architectural concrete planters.

          - Section 042000 "Unit Masonry" for site-built masonry planters.

          - Section 044313.13 "Anchored Stone Masonry Veneer" for site-built natural stone planters.

          - Section 044313.16 "Adhered Stone Masonry Veneer" for site-built natural stone planters.

          - Section 047200 "Cast Stone Masonry" for plant-manufactured cast stone planters.

          - Section 329300 "Plants" for interior live plants and placing planting soil, and for planting accessories.

      - ALLOWANCES

Retain products and Work included in this Section that are covered by cash or quantity allowance. Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the allowance.

        - Work of this Section is part of [Interior Planter and Artificial Plant Allowance] <Insert allowance>.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

        - Shop Drawings:

          - Metal Planter Liners: Show layouts, profiles, shapes, seams, and dimensions.

          - Self-Contained Subirrigation Units: Show fabrication and installation details of units and accessories.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples: For each freestanding planter finish, color, and texture specified, not less than 3 by 3 inches (76 by 76 mm) in size.

        - Samples for Initial Selection: For each type of freestanding planter and each type of exposed finish.

        - Samples for Verification: For each freestanding planter finish, color, and texture, not less than 3 by 3 inches (76 by 76 mm) in size.

        - Product Schedule:[ Use same designations indicated on Drawings.]

          - For freestanding planters, include the following:

            - Finish, color, and texture.

            - Dimensions.

            - Manufacturer's name.

            - Manufacturer's product designation.

          - For manufactured interior plants, include the following:

            - Plant height.

            - Plant width.

            - 35-mm color print or digital photographic record.

            - Manufacturer's name.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For the following to include in maintenance manuals:

          - Freestanding planters.

          - Site-installed, self-contained subirrigation units.

          - Manufactured interior plants.

- PRODUCTS

Manufacturers and products listed in SpecAgent and Masterworks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - FREESTANDING INTERIOR PLANTERS <Insert drawing designation>

Copy this article and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Type: [Hanging] [Tabletop] [Panel/Wall mounted] [Floor set].

        - Material: [Ceramic] [Fiberglass] [Metal] [Plastic] [Cast stone] [GFRC] [Terra cotta] <Insert material>.

Retain "Inside Width," "Outside Width," "Inside Length," and "Outside Length" paragraphs below for rectangular planters.

        - Inside Width: <Insert inches (mm)>.

        - Outside Width: <Insert inches (mm)>.

        - Inside Length: <Insert inches (mm)>.

        - Outside Length: <Insert inches (mm)>.

Retain "Inside Diameter" and "Outside Diameter" paragraphs below for round planters.

        - Inside Diameter: <Insert inches (mm)>.

        - Outside Diameter: <Insert inches (mm)>.

        - Height: <Insert inches (mm)>.

        - Pattern: <Insert manufacturer's designation>.

        - Color: <Insert manufacturer's designation>.

        - Finish: <Insert manufacturer's designation>.

        - Accessories: [Planter pad] [Matching saucer] [Carpet protector disc] [Casters] <Insert accessory>.

      - FREESTANDING PLANTERS WITH BUILT-IN SUBIRRIGATION <Insert drawing designation>

Copy this article and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Planter:

          - Type: [Tabletop] [Floor set] <Insert type>.

          - Material: [Fiberglass] [Plastic] [Cast stone] <Insert material>.

Retain "Inside Width," "Outside Width," "Inside Length," and "Outside Length" subparagraphs below for rectangular planters.

          - Inside Width: <Insert inches (mm)>.

          - Outside Width: <Insert inches (mm)>.

          - Inside Length: <Insert inches (mm)>.

          - Outside Length: <Insert inches (mm)>.

Retain "Inside Diameter" and "Outside Diameter" subparagraphs below for round planters.

          - Inside Diameter: <Insert inches (mm)>.

          - Outside Diameter: <Insert inches (mm)>.

          - Height: <Insert inches (mm)>.

          - Pattern: <Insert manufacturer's designation>.

          - Color: <Insert manufacturer's designation>.

          - Finish: <Insert manufacturer's designation>.

        - Subirrigation Unit: Manufacturer's [reservoir] [container] subirrigation unit sized to fit planter.

        - Accessories: [Tree tie-downs, three per planter] [Security tie-downs] [Root-control fabric] <Insert accessory>.

      - SITE-INSTALLED, SELF-CONTAINED SUBIRRIGATION UNITS

Retain this article if site-installed, self-contained subirrigation units in freestanding planters or site-built interior planters, which are not included in this Section, are required. Indicate on Drawings which freestanding interior planters have built-in subirrigation units and which planters have site-installed, self-contained subirrigation units.

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Size: [Sized to fit planters indicated] <Insert size>.

        - Fittings and Accessories: Provide fittings and accessories necessary for proper functioning of unit as recommended by manufacturer and as follows:

          - <Insert required fittings and accessories, in separate subparagraphs, to suit Project>.

      - PLASTIC PLANTER LINERS

        - Planter Liners: [Seamless] [Welded] plastic planter liners.

          - Size: <Insert size>.

          - Color: [Black] <Insert color>.

      - METAL PLANTER LINERS

        - Materials:

Retain one or more of "Zinc-Coated (Galvanized) Steel Sheet," "Stainless-Steel Sheet," and "Copper Sheet" subparagraphs below.

          - Zinc-Coated (Galvanized) Steel Sheet: ASTM A653/A653M, G90 (Z275) coating designation; structural quality, mill phosphatized for field painting, [0.0276 inch (0.7 mm)] [0.0336 inch (0.85 mm)] <Insert thickness> thick.

          - Stainless-Steel Sheet: ASTM A240/A240M, Type 304, No. 2D finish, [0.040 inch (1.0 mm)] [0.050 inch (1.2 mm)] <Insert thickness> thick.

            - Solder for Stainless Steel: ASTM B32, Grade Sn60, with acid flux of type recommended by stainless-steel sheet manufacturer.

          - Copper Sheet: ASTM B370, Temper H00 or H01, cold-rolled copper sheet, [16 oz./sq. ft. (0.55 mm thick)] [20 oz./sq. ft. (0.7 mm thick)] <Insert weight (thickness)>.

            - Solder for Copper: ASTM B32, Grade Sn50, 50 percent tin and 50 percent lead.

Retain "Elastomeric Sealant" Subparagraph below if joints in galvanized-steel planter liners are made watertight with sealant or for movable joints in stainless-steel or copper sheet planter liners.

          - Elastomeric Sealant: ASTM C920, elastomeric polymer sealant; of type, grade, class, and use classifications required to seal joints in sheet metal and remain watertight.

        - Fabrication:

          - Comply with recommendations in SMACNA's "Architectural Sheet Metal Manual" that apply to design, dimensions, metal, and other characteristics of planter liner indicated.

          - Shop fabricate where practical. Obtain field measurements for accurate fit before shop fabrication.

          - Fabricate sheet metal without excessive oil canning, buckling, and tool marks and true to line and levels indicated, with exposed edges folded back to form hems.

Retain first subparagraph below for stainless-steel or copper planter liners with soldered, flat-lock seams.

          - Fabricate sheet metal joints with soldered, flat-lock seams. Clean surfaces to be soldered, removing oils and foreign matter. Pretin edges of sheets to be soldered to a width of 1-1/2 inches (38 mm).

            - Stainless-Steel Soldering: Pretin edges of uncoated sheets to be soldered using phosphoric acid flux and solder for stainless steel. Promptly wash off acid flux residue from metal after soldering.

            - Copper Soldering: Tin uncoated copper surfaces at edges of sheets using solder for copper.

Retain subparagraph below for galvanized-steel planter liners or if stainless-steel or copper planter liners have movable joints.

          - Fabricate joints in sheet metal to accommodate elastomeric sealant.

            - Seal joints with elastomeric sealant as required for watertight construction. Embed hooked flanges of joint members not less than 1 inch (25 mm) into sealant. Form joints to completely conceal sealant. Prepare joints and apply sealants according to manufacturer's written instructions.

      - MANUFACTURED INTERIOR PLANTS <Insert drawing designation>

Copy this article and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Plant Type: <Insert description>.

        - Treat trunk, branch, and foliage surfaces with flame retardant to comply with NFPA 701.

      - MINERAL MULCH

        - Mineral Mulch: Hard, durable stone, washed free of loam, sand, clay, and other foreign substances, of the following type, size range, and color:

          - Type: [Rounded riverbed gravel or smooth-faced stone] [Crushed stone or gravel] [Marble chips] [Granite chips] <Insert material>.

          - Size Range: [1-1/2 inches (38 mm) maximum, 3/4 inch (19 mm) minimum] [3/4 inch (19 mm) maximum, 1/4 inch (6 mm) minimum].

          - Color: [Uniform tan-beige color range acceptable to Architect] [Readily available natural gravel color range] <Insert color>.

- EXECUTION

      - EXAMINATION

        - Examine areas, with Installer present, for compliance with requirements for environmental conditions, and other conditions affecting performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

Retain paragraph below for complicated or critical installations; delete for small projects or if inspection and approval by Architect are not feasible.

        - Before installation, lay out interior planters in locations indicated. Adjust locations when requested by Architect and obtain Architect's acceptance of layout before installation.

      - FREESTANDING PLANTER PLACEMENT

        - Install freestanding planters level and plumb. Use concealed shims where required for alignment.

        - Remove stickers and other temporary labels from interior planters.

      - INSTALLATION OF SELF-CONTAINED SUBIRRIGATION UNITS

        - Install self-contained subirrigation units into planters according to manufacturer's written instructions, level and plumb.

        - Install drainage overflow adapters before installing self-contained subirrigation units.

Revise paragraph below for units automatically filled using low-pressure irrigation lines.

        - Trim fill tubes to no less than 1 inch (25 mm) higher than level of planting soil after plants have been planted.

      - INSTALLATION OF PLANTER LINERS

        - Set liners in planters [according to freestanding planter manufacturer's written instructions] [as indicated on Drawings].

        - Install liners to allow for liner removal.

      - MANUFACTURED PLANT PLACEMENT

        - Set manufactured plants at locations indicated on Drawings according to manufacturer's written instructions.

        - Place mineral mulch in locations indicated on Drawings.

END OF SECTION 129200