Copyright 2020 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 224233 - WASH FOUNTAINS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Access Manufacturer-Provided, AIA MasterSpec-Based Sections:

<Double click here for this Section based on specific manufacturer’s products set as Basis-of-Design at ProductMasterSpec.com.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provision for LEED 2009, LEED v4, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Precast-terrazzo, circular wash fountains.

          - Solid-surface, circular wash fountains.

          - Stainless steel, circular wash fountains.

          - Precast-terrazzo, semicircular wash fountains.

          - Solid-surface, semicircular wash fountains.

          - Stainless steel, semicircular wash fountains.

          - Precast-terrazzo, corner wash fountains.

          - Solid-surface, corner wash fountains.

          - Stainless steel, corner wash fountains.

          - Solid-surface, linear wash fountains.

          - Stainless steel, linear wash fountains.

          - Wash-fountain carriers.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 224216.13 "Commercial Lavatories."

          - Section 224216.16 "Commercial Sinks."

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for wash fountains.

          - Include rated capacities, operating characteristics,[ electrical characteristics,] and furnished specialties and accessories.

Retain "Shop Drawings" Paragraph below if fixtures or equipment includes wiring.

        - Shop Drawings: For each type of wash fountain.

          - Include plans, elevations, sections, and [mounting] [attachment] details.

          - Include details of equipment assemblies. Indicate dimensions, weights, loads, required clearances, method of field assembly, components, and location and size of each field connection.

          - Include diagrams for power, signal, and control wiring.

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for plumbing fixtures.>

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For wash fountains and components to include in operation and maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

See Section 017700 "Closeout Procedures" for submission of maintenance material items.

        - Extra Stock Materials: Furnish extra materials to Owner that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Faucet Washers and O-Rings: Equal to [5] [10] <Insert number> percent of quantity of each type and size installed.

          - Faucet Cartridges and O-Rings: Equal to [5] <Insert number> percent of quantity of each type and size installed.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - PERFORMANCE REQUIREMENTS

        - Standards:

The U.S. Safe Drinking Water Act (SDWA) requires national compliance with less than or equal to 0.25 percent weighted average lead content at wetted surfaces for pipe, fittings, and devices intended to convey or dispense water for human consumption. The IPC and the UPC have the same requirements. Items in compliance with NSF 61 and NSF 372 also meet this requirement. Some manufacturers choose to meet this requirement through independent testing and have "Certified Lead-Free" products, which may or may not have NSF 61 or NSF 372 certification.

          - Wash fountain spray heads and faucets intended to convey or dispense water for human consumption must comply with the SDWA, requirements of authorities having jurisdiction, and NSF 61 and NSF 372, or must be certified in compliance with NSF 61 and NSF 372 (by an ANSI-accredited third-party certification body) that the weighted average lead content at wetted surfaces is less than or equal to 0.25 percent.

Retain first paragraph below if retaining precast-terrazzo, solid-surface, or stainless steel wash fountains.

        - [Comply with IAPMO IGC 156 for precast-terrazzo, solid-surface, and stainless steel wash fountains.]

Retain first paragraph below if retaining solid-surface, linear wash fountains.

        - [Comply with CSA B45.5/IAPMO Z124 for solid-surface, linear wash fountains.]

        - Comply with ASME A112.18.1/CSA B125.1 for water supply fittings.

        - Comply with ASME A112.18.2/CSA B125.2 for plumbing waste fittings.

        - Comply with ASME A112.19.3/CSA B45.4 for stainless steel wash fountains.

        - Comply with ASSE 1016 for mixing valves.

        - Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by an NRTL, and marked for intended location and application.

      - PRECAST-TERRAZZO, CIRCULAR WASH FOUNTAINS

Caution: These fixtures require a trap with waste and vent connections for installation in the base and vertical vent piping through top of unit, or they may be installed with trap and vent under the floor below the fixture.

Copy "Wash Fountains - Precast Terrazzo, Circular Receptor" Paragraph below and re-edit for each type of precast-terrazzo, circular-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - Precast Terrazzo, Circular Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

            - Nominal Diameter: [39 inches (990 mm)] [54 inches (1372 mm)].

            - Height to Rim: [Standard] [Juvenile].

            - Color: <Insert color>.

            - Drain: Grid with [NPS 2 (DN 50)] <Insert pipe size> tailpiece.

          - Spray Head:

            - Material: Stainless steel.

            - Number of User Stations: <Insert number>.

            - Control: [Foot-control] [Pneumatic push-button] [Infrared sensor] [Low-voltage, mechanical metering] actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>.

          - Pedestal: Manufacturer's standard with access panel.

          - Supply Fittings:

            - Piping: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From [bottom] [top].

          - Waste Fittings:

            - Trap and Drain Piping: [NPS 2 (DN 50)].

            - Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

          - Shroud: [Not required] [Stainless steel of size to cover supply and vent piping].

          - Mounting: Manufacturer's standard floor bracket.

      - SOLID-SURFACE, CIRCULAR WASH FOUNTAINS

Caution: These fixtures require a trap with waste and vent connections for installation in the base and vertical vent piping through top of unit, or they may be installed with trap and vent under the floor below the fixture.

Copy "Wash Fountains - Solid Surface, Circular Receptor" Paragraph below and re-edit for each type of solid-surface, circular-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - Solid Surface, Circular Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

Verify that option retained in "Nominal Diameter" Subparagraph below is available from listed manufacturers for wash fountain required.

            - Nominal Diameter: [36 inches (914 mm)] [or] [39 inches (990 mm)] [54 inches (1370 mm)].

Caution: Selection of "ADA compliant" option in "Height to Rim" Subparagraph below may restrict selection in "Control" Subparagraph. Consult manufacturers.

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Drain: Grid with [NPS 2 (DN 50)] <Insert pipe size> tailpiece.

          - Spray Head:

            - Material: Stainless steel.

            - Number of User Stations: <Insert number>.

Verify that option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

            - Control: [Collective,] [Individual,] [foot-control] [pneumatic push-button] [infrared sensor] [low-voltage, mechanical metering] actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>, for each user station.

          - Pedestal: [Manufacturer's standard] [Painted or coated steel] [Stainless steel] with access panel.

          - Supply Fittings:

            - Piping: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From [bottom] [top].

          - Waste Fittings:

            - Trap and Drain Piping: [NPS 2 (DN 50)].

            - Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

          - Shroud: [Not required] [Stainless steel of size to cover supply and vent piping].

          - Mounting: Manufacturer's standard floor bracket.

      - STAINLESS STEEL, CIRCULAR WASH FOUNTAINS

Caution: These fixtures require a trap with waste and vent connections for installation in the base and vertical vent piping through top of unit, or they may be installed with trap and vent under the floor below the fixture.

Copy "Wash Fountains - Stainless Steel, Circular Receptor" Paragraph below and re-edit for each type of stainless steel, circular-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - Stainless Steel, Circular Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

Verify that option retained in "Nominal Diameter" Subparagraph below is available from listed manufacturers for wash fountain required.

            - Nominal Diameter: [36 inches (914 mm)] [or] [39 inches (990 mm)] [42 inches (1067 mm)] [54 inches (1372 mm)].

Caution: Selection of "ADA compliant" option in "Height to Rim" Subparagraph below may restrict selection in "Control" Subparagraph. Consult manufacturers.

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Drain: Grid with [NPS 2 (DN 50)] <Insert pipe size> tailpiece.

          - Spray Head:

            - Material: Stainless steel.

            - Number of User Stations: <Insert number>.

Verify that option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

            - Control: [Collective,] [Individual,] [foot-control] [pneumatic push-button] [infrared sensor] [low-voltage, mechanical metering] actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type> for each user station.

          - Pedestal: Stainless steel with access panel.

          - Supply Fittings:

            - Piping: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From [bottom] [top].

          - Waste Fittings:

            - Trap and Drain Piping: [NPS 2 (DN 50)].

            - Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

          - Shroud: [Not required] [Stainless steel of size to cover supply and vent piping].

          - Mounting: Manufacturer's standard floor bracket.

      - PRECAST-TERRAZZO, SEMICIRCULAR WASH FOUNTAINS

Copy "Wash Fountains - Precast Terrazzo, Semicircular Receptor" Paragraph below and re-edit for each type of precast-terrazzo, semicircular-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - Precast Terrazzo, Semicircular Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

            - Nominal Diameter: [36 inches (914 mm)] [54 inches (1372 mm)].

            - Height to Rim: [Standard] [Juvenile].

            - Color: <Insert color>.

            - Drain: Grid with [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)] tailpiece.

          - Spray Head:

            - Material: Stainless steel.

            - Number of User Stations: <Insert number>.

            - Control: [Foot-control] [Pneumatic push-button] [Infrared-sensor] [Low-voltage, mechanical metering] actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>.

          - Pedestal: Manufacturer's standard with access panel.

          - Supply Fittings:

            - Piping: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From [bottom] [top] [wall].

          - Waste Fittings:

            - Trap and Drain Piping: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

            - Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

          - Shroud: [Not required] [Stainless steel of size to cover supply and vent piping].

          - Mounting: Manufacturer's standard floor and wall bracket.

      - SOLID-SURFACE, SEMICIRCULAR WASH FOUNTAINS

Copy first paragraph below and re-edit for each type of solid-surface, semicircular-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - [Floor] [Wall] Mounted, Solid Surface, Semicircular Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

Verify that option retained in "Nominal Diameter" Subparagraph below is available from listed manufacturers for wash fountain required.

            - Nominal Diameter: [36 inches (914 mm)] [or] [39 inches (990 mm)] [46 inches (1168 mm)] [54 inches (1372 mm)].

Caution: Selection of "ADA compliant" in "Height to Rim" Subparagraph below may restrict selection in "Control" Subparagraph. Consult manufacturers.

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Drain: Grid with [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)] tailpiece.

          - Spray Head:

            - Material: Stainless steel or integral part of receptor back.

            - Number of User Stations: <Insert number>.

Verify that option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

            - Control: [Collective,] [Individual,] [foot-control] [pneumatic push-button] [infrared sensor] [low-voltage, mechanical metering] <Insert type> actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>, for each user station.

          - Supply Fittings:

            - Piping: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From [bottom] [top] [wall].

          - Waste Fittings:

            - Trap and Drain Piping: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

            - Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

          - Shroud: [Not required] [Stainless steel of size to cover supply and vent piping].

Retain "Wall Mounting" Subparagraph below for wall-mounted wash fountains.

          - Wall Mounting: [Manufacturer's wall-mounting bracket] [and/or] [commercial floor-mounted carrier] <Insert type>.

Retain "Floor Mounting" Subparagraph below for floor-mounted wash fountains.

          - Floor Mounting: Manufacturer's floor bracket and wall bracket attached to [reinforcement in] [concrete or block] wall.

      - STAINLESS STEEL, SEMICIRCULAR WASH FOUNTAINS

Copy first paragraph below and re-edit for each type of stainless steel, semicircular-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - [Floor] [Wall] Mounted, Stainless Steel, Semicircular Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

Verify that option retained in "Nominal Diameter" Subparagraph below is available from listed manufacturers for wash fountain required.

            - Nominal Diameter: [36 inches (914 mm)] [or] [39 inches (990 mm)] [42 inches (1067 mm)] [54 inches (1372 mm)].

Caution: Selection of "ADA compliant" option in "Height to Rim" Subparagraph below may restrict selection in "Control" Subparagraph. Consult manufacturers.

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Drain: Grid with [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)] tailpiece.

          - Spray Head:

            - Material: Stainless steel or integral part of receptor back.

            - Number of User Stations: <Insert number>.

Verify that option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

            - Control: [Collective,] [Individual,] [foot-control] [pneumatic push-button] [infrared sensor] [low-voltage, mechanical metering] <Insert type> actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>, for each user station.

          - Supply Fittings:

            - Piping: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From [bottom] [top] [wall].

          - Waste Fittings:

            - Trap and Drain Piping: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

            - Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

          - Shroud: [Not required] [Stainless steel of size to cover supply and vent piping].

Retain "Wall Mounting" Subparagraph below for wall-mounted wash fountains.

          - Wall Mounting: [Manufacturer's wall-mounting bracket] [and/or] [commercial floor-mounted carrier] <Insert type>.

Retain "Floor Mounting" Subparagraph below for floor-mounted wash fountains.

          - Floor Mounting: Manufacturer's floor bracket and wall bracket attached to [reinforcement in] [concrete or block] wall.

      - PRECAST-TERRAZZO, CORNER WASH FOUNTAINS

Copy "Wash Fountains - Precast Terrazzo, Corner Receptor" Paragraph below and re-edit for each precast-terrazzo, corner-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - Precast Terrazzo, Corner Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

Verify that option retained in "Nominal Side Width" Subparagraph below is available from listed manufacturers for wash fountain required.

            - Nominal Side Width: [27 inches (686 mm)] [37 inches (940 mm)].

            - Height to Rim: [Standard] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Drain: Grid with NPS 1-1/2 (DN 40) tailpiece.

          - Spray Head:

            - Material: Stainless steel or integral part of receptor back.

            - Number of User Stations: <Insert number>.

Verify that option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

            - Control: [Foot-control] [Pneumatic push-button] [Infrared sensor] [Low-voltage, mechanical metering] <Insert type> actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>, for each user station.

          - Supply Fittings:

            - Piping: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From wall.

          - Waste Fittings:

            - Trap and Drain Piping: NPS 1-1/2 (DN 40).

          - Mounting: Manufacturer's standard floor and wall bracket.

      - SOLID-SURFACE, CORNER WASH FOUNTAINS

Copy first paragraph below and re-edit for each type of solid-surface, corner-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - [Floor] [Wall] Mounted, Solid Surface, Corner Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

Verify that option retained in "Nominal Side Width" Subparagraph below is available from listed manufacturers for wash fountain required.

            - Nominal Side Width: [27 inches (686 mm)] [37 inches (940 mm)].

Caution: Selection of "ADA compliant" option in "Height to Rim" Subparagraph below may restrict selection in "Control" Subparagraph.

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Drain: Grid with NPS 1-1/2 (DN 40) tailpiece.

          - Spray Head:

            - Material: Stainless steel or integral part of receptor back.

            - Number of User Stations: <Insert number>.

Verify that option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

            - Control: [Collective,] [Individual,] [foot-control] [pneumatic push-button] [infrared sensor] [low-voltage, mechanical metering] <Insert type> actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>, for each user station.

          - Supply Fittings:

            - Piping: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From wall.

          - Waste Fittings:

            - Trap and Drain Piping: NPS 1-1/2 (DN 40).

Retain "Wall Mounting" Subparagraph below for wall-mounted wash fountains.

          - Wall Mounting: [Manufacturer's wall-mounting bracket] [and/or] [commercial floor-mounted carrier] <Insert type>.

Retain "Floor Mounting" Subparagraph below for floor-mounted wash fountains.

          - Floor Mounting: Manufacturer's floor bracket and wall bracket attached to [reinforcement in] [concrete or block] wall.

      - STAINLESS STEEL, CORNER WASH FOUNTAINS

Copy first paragraph below and re-edit for each type of stainless steel, corner-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - [Floor] [Wall] Mounted, Stainless Steel, Corner Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Receptor:

Verify that option retained in "Nominal Side Width" Subparagraph below is available from listed manufacturers for wash fountain required.

            - Nominal Side Width: [37 inches (940 mm)] [38 inches (965 mm)] [44 inches (1118 mm)].

Caution: Selection of "ADA compliant" option in "Height to Rim" Subparagraph below may restrict selection in "Control" Subparagraph.

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Drain: Grid with NPS 1-1/2 (DN 40) tailpiece.

          - Spray Head:

            - Material: Stainless steel or integral part of receptor back.

            - Number of User Stations: <Insert number>.

Verify that option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

            - Control: [Collective,] [Individual,] [foot-control] [pneumatic push-button] [infrared sensor] [low-voltage, mechanical metering] <Insert type> actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops.

          - Soap Dispensers: [Not required] [Bracket only] [Foam] [Powdered] [Liquid] <Insert type>, for each user station.

          - Supply Fittings:

            - Piping: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] copper tubing.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From wall.

          - Waste Fittings:

            - Trap and Drain Piping: NPS 1-1/2 (DN 40).

Retain "Wall Mounting" Subparagraph below for wall-mounted wash fountains.

          - Wall Mounting: [Manufacturer's wall-mounting bracket] [and/or] [commercial floor-mounted carrier] <Insert type>.

Retain "Floor Mounting" Subparagraph below for floor-mounted wash fountains.

          - Floor Mounting: Manufacturer's floor bracket and wall bracket attached to [reinforcement in] [concrete or block] wall.

      - SOLID-SURFACE, LINEAR WASH FOUNTAINS

Caution: Although included in this Section, this product type may be found under lavatory systems rather than wash fountains, depending on manufacturer. Lavatory systems are completely integrated products that usually include sensor-operated faucets, sensor-operated soap dispensers, and electric deck-mounted hand dryers. Coordinate with Section 224216.13 "Commercial Lavatories" if a complete lavatory system is required.

Copy "Wash Fountains - Solid Surface, Linear (Side-by-Side) Receptor" Paragraph below and re-edit for each type of solid-surface, linear-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - Solid Surface, Linear (Side-by-Side) Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Bowl(s) and Counter:

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Number of Bowls or User Stations: [One] [Two] [Three] [Four] <Insert number>.

            - Bowl Shape: [Edge] [Oval] [Round] [Square] [Rectangular] <Insert shape>.

            - Drain: Grid with NPS 1-1/2 (DN 40) tailpiece, each bowl.

          - Apron: [Not required] [Required, with access panel].

          - Faucets:

            - Type: Manufacturer's standard, chrome-plated solid brass.

            - Control: [Manual] [Hardwired, control-voltage, sensor-actuated] [Battery-powered, sensor-actuated] <Insert type> actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops for each bowl or user station.

          - Soap Dispensers: [Not required] [Bracket only] [Manual] [Hardwired, control-voltage, sensor-actuated] [Battery-powered, sensor-actuated] [, liquid] [, foam] [, powdered] <Insert type> dispenser for each user station.

          - Mounting: [Manufacturer's wall-mounting bracket] [and/or] [commercial floor-mounted carrier] <Insert type>.

          - Supply Fittings:

            - Piping: NPS 1/2 (DN 15) copper tubing, each bowl.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From wall.

          - Waste Fittings:

            - Trap and Drain Piping: NPS 1-1/2 (DN 40), each bowl.

      - STAINLESS STEEL, LINEAR WASH FOUNTAINS

Copy "Wash Fountains - Stainless Steel, Linear (Side-by-Side) Receptor" Paragraph below and re-edit for each type of stainless steel, linear-receptor wash fountain required.

Insert drawing designation. Use these designations on Drawings to identify each wash fountain.

        - Wash Fountains - Stainless Steel, Linear (Side-by-Side) Receptor: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water fountains from single source from single manufacturer.

          - Bowl(s) and Counter:

            - Height to Rim: [Standard] [ADA compliant] [Juvenile].

            - Color or Finish: [Not applicable] <Insert if required>.

            - Number of Bowls or User Stations: [One] [Two] [Three] [Four] <Insert number>.

            - Bowl Shape: [Edge] [Oval] [Round] [Square] [Rectangular] <Insert shape>.

            - Drain: Grid with NPS 1-1/2 (DN 40) tailpiece, each bowl.

          - Apron: [Not required] [Required, with access panel].

          - Faucets:

            - Type: Manufacturer's standard, chrome-plated solid brass.

            - Control: [Manual] [Hardwired, control-voltage, sensor-actuated] [Battery-powered, sensor-actuated] <Insert type> actuation with [pressure-balancing] [thermostatic] mixing valve, and having check stops for each bowl or user station.

          - Soap Dispensers: [Not required] [Bracket only] [Manual] [Hardwired, control-voltage, sensor-actuated] [Battery-powered, sensor-actuated] [, liquid] [, foam] [, powered] <Insert type> dispenser for each user station.

          - Mounting: [Manufacturer's wall-mounting bracket] [and/or] [commercial floor-mounted carrier] <Insert type>.

          - Supply Fittings:

            - Piping: NPS 1/2 (DN 15) copper tubing, each bowl.

            - Valves: Shutoff valve on each supply.

            - Supply Piping: From wall.

          - Waste Fittings:

            - Trap and Drain Piping: NPS 1-1/2 (DN 40), each bowl.

      - WASH-FOUNTAIN CARRIERS

Copy "Wash-Fountain Carrier" Paragraph below and re-edit for each type of wash-fountain carrier required.

        - Wash-Fountain Carrier: <Insert drawing designation>.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain water-fountain carriers from single source from single manufacturer.

          - Standard: ASME A112.6.1M.

- EXECUTION

      - EXAMINATION

        - Examine roughing-in for water-supply piping, and sanitary drainage and vent piping systems to verify actual locations of piping connections before wash-fountain installation.

        - Examine walls and floors for suitable conditions where wash fountains will be installed.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION, GENERAL

        - Install wash fountains level and plumb in accordance with rough-in drawings.

        - Set freestanding wash fountains on floor.

        - Install off-floor carrier supports, affixed to building substrate, for wall-mounted wash fountains.

        - Install accessible, wall-mounted wash fountains at mounting height in accordance with ICC A117.1.

        - Install water-supply piping with shutoff valve on each supply to each wash fountain to be connected to domestic-water distribution piping. Use ball or gate valve. Install valves in locations where they can be easily reached for operation. Valves are specified in Section 220523.12 "Ball Valves for Plumbing Piping" and Section 220523.15 "Gate Valves for Plumbing Piping."

        - Install trap and waste piping on each drain outlet of each wash fountain to be connected to sanitary drainage system.

        - Wall Flange and Escutcheon Installation:

          - Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations.

          - Install deep-pattern escutcheons if required to conceal protruding fittings.

          - Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

Retain "Joint Sealing" Paragraph below if sealants are not specified in Section 079200 "Joint Sealants."

        - Joint Sealing:

          - Seal joints between fixtures and walls using sanitary-type, one-part, mildew-resistant silicone sealant.

          - Match sealant color to fixture color.

          - Comply with sealant requirements specified in Section 079200 "Joint Sealants."

      - PIPING CONNECTIONS

Coordinate piping installations and specialty arrangements with Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

        - Connect wash fountains with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

        - Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

        - Comply with requirements for waste drainage piping[ and vent piping] specified in Section 221316 "Sanitary Waste and Vent Piping."

        - Install protective-shielding pipe covers and enclosures on exposed supplies and waste piping of accessible wash fountains. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

      - ADJUSTING

        - Operate and adjust wash fountains and controls. Replace damaged and malfunctioning wash fountains, fittings, and controls.

        - Adjust water pressure at faucets to produce proper flow.

        - Install new batteries in battery-powered, electronic-sensor mechanisms.

      - CLEANING AND PROTECTION

        - After installing wash fountains, inspect and repair damaged finishes.

        - Clean wash fountains, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.

        - Install protective covering for installed wash fountains and fittings.

        - Do not allow use of wash fountains for temporary facilities unless approved in writing by Owner.

END OF SECTION 224233