Copyright 2021 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 281300 - ACCESS CONTROL SOFTWARE AND DATABASE MANAGEMENT

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Software for access control system units.

          - Software for access control system supplementary computer equipment.

        - Related Requirements:

Always retain first subparagraph below.

          - Section 280010 "Supplemental Requirements for Electronic Safety and Security" for additional abbreviations, definitions, submittals, qualifications, testing agencies, and other Project requirements applicable to Work specified in this Section.

          - Section 281400 "Access Control System Hardware" for hardware required to run software specified in this Section.

          - Section 281500 "Integrated Access Control Hardware Devices" for access control field devices controlled by software specified in this Section.

      - DEFINITIONS

Do not duplicate common acronyms, abbreviations, and terms defined in Section 280010 "Supplemental Requirements for Electronic Safety and Security."

Retain terms that remain after this Section has been edited for a project. Include only essential definitions or acronyms not well understood by the affected industry or trade.

        - RAM: Random-access memory.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

Retain one of two subparagraphs below, or both, as needed to verify compliance of Work with Project requirements.

          - Software for access control system units.

          - Software for access control system supplementary computer equipment.

Retain "Certificates" Paragraph below if certificate from manufacturer or fabricator is required by Architect to verify that product or assembly has been investigated and approved/recognized by third party prior to installation.

        - Certificates:

          - <Insert certificate requirement>.

        - Field Quality-Control Submittals:

          - Field quality-control reports.

      - INFORMATIONAL SUBMITTALS

Information submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

Retain "Manufacturers' Published Instructions" Paragraph below when execution requirements state "in accordance with manufacturer's published instructions," especially if those instructions may require preinstallation review and coordination with Construction Documents to verify constructability and commissioning requirements.

        - Manufacturers' Published Instructions: Record copy of official installation[ and] [testing] [and] [troubleshooting] instructions issued to Installer by manufacturer for the following:

          - Installation and programming instructions for operating system software, including all applicable software add-ons.

          - Manufacturer's recommended setup and testing procedure for operating system software and applicable software add-ons.

          - Troubleshooting instructions for operating system software and applicable software add-ons.

          - Installation and programming instructions for antivirus and security protection software.

          - Manufacturer's recommended setup and testing procedure for antivirus and security protection software.

          - Troubleshooting instructions for antivirus and security protection software.

      - CLOSEOUT SUBMITTALS

Submittals for record documents, operation and maintenance data, software, and software documentation are specified in Section 280010 "Supplemental Requirements for Electronic Safety and Security."

        - Maintenance Contracts:

          - Software and firmware service agreement.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SOFTWARE FOR ACCESS CONTROL SYSTEM UNITS

        - Description: This category covers software products intended for installation on access control system units.

        - Performance Criteria:

          - Regulatory Requirements:

            - Listed and labeled in accordance with NFPA 70 and NFPA 72, by qualified electrical testing laboratory recognized by authorities having jurisdiction, and marked for intended location and application.

            - Comply with NFPA 1, NFPA 730, NFPA 731, and ICC IBC.

            - <Insert requirements>.

          - Listing Criteria:

For assistance with finding guide information for UL Category Control Numbers and identifying UL-approved manufacturers, consult the UL Product iQ database at https://iq.ulprospector.com.

            - Cybersecurity: UL CCN CYBR; including UL 2900-1 and UL 2900-2-3.

            - Access Control System Functionality: UL CCN ALVY; including UL 294.

            - Hardware that Interfaces with Fire Alarm or Mass Notification: UL CCN UOJK; including UL 864.

Copy and re-edit "Access Control System Unit Operating System Software" Paragraph below for each configuration indicated on Drawings.

        - Access Control System Unit Operating System Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Provides event logging, notification, and escalation.

            - Provides threat-level management and response actions.

            - Provides dashboard view of system health and status.

            - Maintains credential-driven user records and access history.

            - Supports network-connected credential readers and controlled doors.

            - Supports add-on software.

            - Supports video-surveillance system integration.

            - Provides email[ and text message] alerts.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows 7] [Windows 10] <Insert operating system>.

            - User Interface: [Web browser] [and] [mobile app].

            - Web Browser: [Microsoft Edge (Chromium)] [or] [Google Chrome] <Insert web browser>.

            - Drive Space: [32 GB] [100 GB] [500 GB] <Insert size>.

            - Memory: [1 GB] [4 GB] [8 GB] <Insert size> RAM.

            - Screen Resolution: [1280×720] [1920×1080] <Insert resolution>.

            - Number of Cardholders: Up to [1000] [10,000] [50,000] [150,000] <Insert number>.

            - Number of Supported Readers: Up to [100] [500] [1000] <Insert number>.

            - Number of Inputs: [128] [256] [1024] <Insert number>.

            - Number of Outputs: [128] [256] [1024] <Insert number>.

            - Simultaneous Users: [Five] [25] [35] <Insert numbers>.

Copy and re-edit "Access Control System Unit Antivirus and Security Protection Software" Paragraph below for each configuration indicated on Drawings.

        - Access Control System Unit Antivirus and Security Protection Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Endpoint detection and response.

            - Anti-ransomware.

            - Managed threat response.

            - Exploit prevention.

            - Active adversary mitigations.

            - Email alerts.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows 7] [Windows 10] <Insert operating system>.

            - User Interface: [Web browser] [and] [mobile app].

            - Web Browser: [Microsoft Edge (Chromium)] [or] [Google Chrome] <Insert web browser>.

            - Drive Space: [1 GB] [4 GB] [8 GB] <Insert size>.

            - Memory: [1 MB] [100 MB] [1 GB] <Insert size> RAM.

Copy and re-edit "Visitor Management Database Software" Paragraph below for each configuration indicated on Drawings.

        - Visitor Management Database Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Visitor registration.

            - Visitor audit trails and reporting.

            - Badge printing.

            - Unassisted visitor registration and badging.

            - Preconfigured security levels of users.

            - Filters out unsolicited visitors.

            - Supports integrated credential readers.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows 7] [Windows 10] <Insert operating system>.

            - User Interface: [Web browser] [and] [mobile app].

            - Web Browser: [Microsoft Edge (Chromium)] [or] [Google Chrome] <Insert web browser>.

            - Drive Space: [100 MB] [500 MB] [1 GB] <Insert size>.

            - Memory: [500 MB] [1 GB] [4 GB] <Insert size> RAM.

            - Automated Alerts:

              - Visitor check-in.

              - Package check-in.

              - Watch list alert.

              - Expired badges.

              - Panic message.

              - Current visitor.

              - Maximum visitor.

              - Frequent visitor.

              - Time based.

Copy and re-edit "Mobile Credential Validation Database Software" Paragraph below for each configuration indicated on Drawings.

        - Mobile Credential Validation Database Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Mobile Validator and Mobile Registration engines.

            - Network-based functionality and database synchronization.

            - Credential audit trail exporting.

            - Contact or contactless verification.

            - Integrated card reader.

            - Biometric finger scanner.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows CE 5.0] [Windows Mobile 6] <Insert operating system>.

            - Multi-Factor Authentication: [One] [Two] [Three] factor authentication.

            - Drive Space: [100 MB] [500 MB] [1 GB] <Insert size>.

            - Memory: [32 MB] [100 MB] [1 GB] <Insert size> RAM.

            - Credentials:

              - PIV.

              - PIV-1.

              - TWIC.

              - FRAC.

              - CAC.

              - CIV.

              - Prox.

              - DESFire.

      - SOFTWARE FOR ACCESS CONTROL SYSTEM SUPPLEMENTARY COMPUTER EQUIPMENT

        - Description: This category covers software products for installation on access control system supplementary computer equipment that do not inhibit safety or intended operation of access control system hardware devices when they are shut off, lose power, or otherwise fail.

        - Performance Criteria:

          - Regulatory Requirements:

            - Listed and labeled in accordance with NFPA 70 and NFPA 72, by qualified electrical testing laboratory recognized by authorities having jurisdiction, and marked for intended location and application.

            - <Insert requirements>.

          - Listing Criteria:

For assistance with finding guide information for UL Category Control Numbers and identifying UL-approved manufacturers, consult the UL Product iQ database at https://iq.ulprospector.com/.

            - Cybersecurity: UL CCN CYBR; including UL 2900-1 and UL 2900-2-3.

Copy and re-edit "Access Control System Supplementary Computer Equipment Operating System Software" Paragraph below for each configuration indicated on Drawings.

        - Access Control System Supplementary Computer Equipment Operating System Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Provides event logging, notification, and escalation.

            - Provides threat-level management and response actions.

            - Provides dashboard view of system health and status.

            - Maintains credential-driven user records and access history.

            - Supports network-connected credential readers and controlled doors.

            - Supports add-on software.

            - Supports video-surveillance system integration.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows 7] [Windows 10] <Insert operating system>.

            - User Interface: [Web browser] [and] [mobile app].

            - Web Browser: [Microsoft Edge (Chromium)] [or] [Google Chrome] <Insert web browser>.

            - Drive Space: [32 GB] [100 GB] [500 GB] <Insert size>.

            - Memory: [1 GB] [4 GB] [8 GB] <Insert size> RAM.

            - Screen Resolution: [1280×720] [1920×1080] <Insert resolution>.

            - Number of Cardholders: Up to [1000] [10,000] [50,000] [150,000] <Insert number>.

            - Number of Supported Readers: Up to [100] [500] [1000] <Insert number>.

            - Number of Inputs: [128] [256] [1024] <Insert number>.

            - Number of Outputs: [128] [256] [1024] <Insert number>.

            - Simultaneous Users: [Five] [25] [35] <Insert numbers>.

Copy and re-edit "Access Control System Supplementary Computer Equipment Antivirus and Security Protection Software" Paragraph below for each configuration indicated on Drawings.

        - Access Control System Supplementary Computer Equipment Antivirus and Security Protection Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Endpoint detection and response.

            - Anti-ransomware.

            - Managed threat response.

            - Exploit prevention.

            - Active adversary mitigations.

            - Email alerts.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows 7] [Windows 10] <Insert operating system>.

            - User Interface: [Web browser] [and] [mobile app].

            - Web Browser: [Microsoft Internet Explorer] [Microsoft Edge (Chromium)] [or] [Google Chrome] <Insert web browser>.

            - Drive Space: [1 GB] [4 GB] [8 GB] <Insert size>.

            - Memory: [1 MB] [100 MB] [1 GB] <Insert size> RAM.

Copy and re-edit "Credential Card Personalization Software" Paragraph below for each configuration indicated on Drawings.

        - Credential Card Personalization Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Credential card design and data-entry customization.

            - Card data fields populated directly from internal or networked database.

            - User-defined privileges.

            - Quick filter for searching internal database.

            - Global find for searching all networked databases.

            - Customized reporting.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows 7] [Windows 10] <Insert operating system>.

            - User Interface: [Web browser] [and] [mobile app].

            - Web Browser: [Microsoft Edge (Chromium)] [or] [Google Chrome] <Insert web browser>.

            - Drive Space: [1 GB] [5 GB] [10 GB] <Insert size>.

            - Memory: [1 GB] [4 GB] [8 GB] <Insert size> RAM.

Copy and re-edit "Credential Card Printer and Encoder Software" Paragraph below for each configuration indicated on Drawings.

        - Credential Card Printer and Encoder Software:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Additional Characteristics:

            - Printer status and diagnostics information.

            - Diagnostics information.

            - Printer integration enables control of printing and encoding through software APIs.

            - Integrates with credential management systems through standard application programming interfaces (APIs).

            - Supports Standard Development Languages:

              - Microsoft Visual Basic.NET.

              - Visual C++.

              - Visual C#.

              - Oracle Java.

            - <Insert characteristics>.

          - Options:

            - Operating System: [Windows 7] [Windows 10] <Insert operating system>.

            - User Interface: [Web browser] [and] [mobile app].

            - Web Browser: [Microsoft Edge (Chromium)] [or] [Google Chrome] <Insert web browser>.

            - Drive Space: [1 GB] [5 GB] [10 GB] <Insert size>.

            - Memory: [1 GB] [4 GB] [8 GB] <Insert size> RAM.

- EXECUTION

      - INSTALLATION OF ACCESS CONTROL SOFTWARE

        - Comply with manufacturer's published instructions.

NFPA 70, NFPA 72, NFPA 731, and NECA NEIS 1 are already specified in Section 280010 "Supplemental Requirements for Electronic Safety and Security." Retain "Reference Standards" Paragraph below to specify other standards applicable to Work specified in this Section.

        - Reference Standards:

          - Installation of Access Control Software: Unless more stringent requirements are specified in Contract Documents or manufacturers' published instructions, comply with requirements in guide information for UL CCN ALVY, UL CCN CYBR, and UL CCN UOJK.

          - Consult Architect for resolution of conflicting requirements.

        - Special Techniques:

          - <Insert more stringent installation requirements that supplement or supersede listed standards and manufacturers' published instructions>.

        - Interfaces with Other Work:

          - Coordinate installation of new access control software with existing equipment and network conditions.

            - <Insert requirements for transition between new and existing>.

          - Coordinate with Section 281000 "Access Control" for physical and functional interfaces with access control system and for compliance with system-wide performance requirements.

          - Coordinate with Section 281400 "Access Control System Hardware" for products that require access control software specified in this Section for operation and compliance with system-wide performance requirements.

          - Coordinate with [Section 284621.11 "Addressable Fire-Alarm Systems"] [Section 284621.13 "Conventional Fire-Alarm Systems"] for interfacing access control system inputs/outputs with fire-alarm system inputs/outputs.

        - Systems Integration:

          - <Insert requirements for integration with other systems>.

      - FIELD QUALITY CONTROL OF ACCESS CONTROL SOFTWARE

        - [Acceptance ]Testing Preparation:

          - <Insert requirements>.

Retain first paragraph below to require that field quality-control tests be witnessed. Local ordinance or custom may require that authorities having jurisdiction witness the testing.

        - Field tests and inspections must be witnessed by [Architect] [Tenant] [authorities having jurisdiction] <Insert names or titles of witnesses>.

        - Tests and Inspections:

          - Perform manufacturer's recommended tests and inspections.

See Section 014000 "Quality Requirements" for retesting and reinspecting requirements and Section 017300 "Execution" for requirements for correcting the Work.

        - Nonconforming Work:

          - Software will be considered defective if it does not pass tests and inspections.

          - Remove and replace defective software and retest.

        - Collect, assemble, and submit test and inspection reports.

Retain "Manufacturer Services" Paragraph below if manufacturer's representative is required to support or supervise the administrant specified in Section 280010 "Supplemental Requirements for Electronic Safety and Security" for field tests and inspections performed by Installer or third-party agencies.

        - Manufacturer Services:

Retain "supervise" option in subparagraph below when third-party tests and inspections must be witnessed and approved by factory-authorized service representative to satisfy special extended-warranty requirements. In some cases, Installer may be factory authorized under warranty provisions to fulfill this role.

          - Engage factory-authorized service representative to [support] [supervise] field tests and inspections.

      - SYSTEM STARTUP

        - [Engage a factory-authorized service representative to perform] [Perform] startup service.

          - Complete software installation and startup checks in accordance with manufacturer's published instructions.

          - <Insert startup steps if any>.

      - MAINTENANCE

Services in "Software and Firmware Service Agreement" Paragraph may not be allowed for publicly funded projects.

        - Software and Firmware Service Agreement:

Retain three subparagraphs below if Owner intends to upgrade software and firmware of products when upgrade is available from manufacturer.

          - Technical Support: Beginning at Substantial Completion, verify that software and firmware service agreement includes software and firmware support for [two] <Insert number> years.

          - Upgrade Service: At Substantial Completion, update software and firmware to latest version. Install and program software and firmware upgrades that become available within [two] <Insert number> years from date of Substantial Completion.[ Verify that upgrading software includes operating system and new or revised licenses for using software.]

            - Upgrade Notice: No fewer than [30] <Insert number> days to allow Owner to schedule and access the system[ and to upgrade computer equipment if necessary].

          - Upgrade Reports: Prepare report after each update, documenting upgrades installed.

END OF SECTION 281300