Copyright 2018 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 321223 - IMPRINTED ASPHALT

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes

          - Imprinting new paving.

          - Imprinting existing paving.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 321216 "Asphalt Paving" for new asphalt paving.

      - UNIT PRICES

Retain this article if Work specified in this Section is measured and paid for under the provisions of unit prices. Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the unit price.

        - Work of this Section is affected by <Insert name of unit price>.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data:

          - Wire templates.

          - Coating system.

          - Thermoplastic marking material.

        - Shop Drawings:

          - Indicate imprinted patterns, colors, and dimensions to adjacent work.

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection: For each type of product requiring color selection.

        - Samples for Verification: For each pattern and color[ with thermoplastic marking material], in manufacturer's standard sizes.

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data: For Installer.

      - QUALITY ASSURANCE

        - Installer Qualifications: Imprinted-asphalt manufacturer's authorized Installer who is trained and approved for installation of imprinted asphalt required for this Project.

Retain "Regulatory Requirements" Paragraph below if imprinted asphalt work is located on public property and must comply with requirements of state or local DOT.

        - Regulatory Requirements: Comply with materials, workmanship, and other applicable requirements of <Insert applicable standards> of <Insert name of state or local DOT> for imprinted asphalt work.

          - Measurement and payment provisions and safety program submittals included in standard specifications do not apply to this Section.

      - FIELD CONDITIONS

        - Environmental Limitations: Proceed with coating imprinted paving only when air temperature is at least 50 deg F (10 deg C) and rising and will not drop below 50 deg F (10 deg C) within eight hours of coating application. Proceed only if no precipitation is expected within two hours after applying the final layer of coating.

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Source Limitations: Obtain imprinted asphalt materials from single source from single manufacturer.

      - MATERIALS

If required, revise materials in this article to suit Project.

        - Wire Templates: Imprinted-asphalt manufacturer's standard, 3/8-inch- (9.5-mm-) diameter, rigid wire rope templates for imprinting pattern into hot asphalt paving.

          - Pattern: [Running bond brick] [Cobblestone] [Custom pattern indicated on Drawings] <Insert pattern>.

Retain "Coating System" Paragraph below only for coated imprinted asphalt.

        - Coating System: Imprinted-asphalt manufacturer's standard system formulated for exterior application on asphalt paving surfaces.

Retain "Base Coating" Subparagraph below only for dry climate zone, such as the Southwest U.S., if recommended by manufacturer.

          - Base Coating: Portland cement and epoxy-modified acrylic polymer blended with sand and aggregate, formulated for exterior application on asphalt paving surfaces.

          - Top Coating: Epoxy-modified acrylic polymer blended with sand and aggregate, formulated for exterior application on asphalt paving surfaces.

          - Colorant: UV-stable pigment blend, added to each coating layer.

          - Color: [White] [Yellow] <Insert color>.

Retain "Thermoplastic Marking Material" Paragraph below only for inlaid, reflectorized, heat-bonded, pavement-marking pattern.

        - Thermoplastic Marking Material: Imprinted-asphalt manufacturer's standard, reflectorized, thermoplastic marking material, 90-mil (2.3-mm) minimum thickness, formulated for exterior application on imprinted, asphalt paving surfaces, and precut to match the imprinted pattern of wire templates.

- EXECUTION

      - EXAMINATION

        - Verify that asphalt paving is dry and in suitable condition to begin imprinting process in accordance with manufacturer's written instructions.

          - Remove existing pavement markings without damaging asphalt paving.

        - Proceed with asphalt imprinting only after unsatisfactory conditions have been corrected.

        - Verify that utilities, traffic loop detectors, and other items requiring a cut and installation beneath the asphalt surface or repairs to the asphalt surface have been completed and that asphalt surface is flush with adjacent asphalt prior to beginning installation of imprinted asphalt.

      - IMPRINTING ASPHALT

If required, revise procedures in this article to suit Project.

        - General: Imprint asphalt in accordance with manufacturer's written instructions, using manufacturer's recommended equipment.

Retain "Freshly Laid Asphalt" Paragraph below if imprinting can immediately follow hot-mix asphalt installation. This sequence, which allows little inspection and correction time after hot-mix asphalt is installed, is uncommon. Coordination between asphalt crews and imprinting crews is essential to maintain adequate surface temperature for imprinting in cool weather and during available daylight.

        - Freshly Laid Asphalt: Immediately after hot-mix asphalt has been laid and compacted but still plastic, begin the surface-imprinting process.

          - Monitor hot-mix asphalt surface temperature to comply with imprinted-asphalt manufacturer's written instructions to ensure required temperature to perform surface imprinting.

          - Reheat hot-mix asphalt if surface temperature drops below that required.

Retain "Reheating Asphalt" Paragraph below for imprinting existing asphalt paving and for reheating new hot-mix asphalt paving.

        - Reheating Asphalt: Soften asphalt paving surface by heating to a depth of at least 1/2 inch (13 mm) without burning asphalt.

          - Heat to a temperature of 300 to 325 deg F (149 to 163 deg C) immediately before applying wire templates.

          - Regularly monitor the pavement temperature to prevent overheating.

          - Direct flame heaters are not permitted.

          - If paving is overheated and begins to emit black smoke, remove damaged paving by milling down 1 inch (25 mm) and replace removed paving with new, compacted hot-mix asphalt surface course before resuming imprinting work.

In "Imprinting Asphalt Paving" Paragraph below, retain first option for coated imprinted asphalt paving. Retain second option for inlaid, reflectorized, heat-bonded, pavement-marking pattern.

        - Imprinting Asphalt Paving: Apply and imprint wire templates to a minimum depth [of 1/4 inch (6 mm)] [as required to embed thermoplastic marking material flush or barely beneath pavement surface].

Retain "Applying Coating" Paragraph below only for coated imprinted asphalt. Retain first option for dry climate zone; second, for other locations.

        - Applying Coating: After imprinted surface has cooled, apply [two layers of base coating followed by two layers] [four layers] of top coating. Do not allow traffic until coating has completely dried and cured.

Retain "Applying Thermoplastic Marking Material" Paragraph below only for inlaid, reflectorized, heat-bonded, pavement-marking pattern.

        - Applying Thermoplastic Marking Material: Position thermoplastic marking material aligned with imprinted pattern and slowly heat to a temperature no higher than 325 deg F (163 deg C) until marking material begins to liquefy and flow. Do not allow traffic until installed marking material has cooled to ambient temperature.

        - Repairs: Perform repairs to defective locations in accordance with imprinted-asphalt manufacturer's written instructions.

END OF SECTION 321223