Copyright 2018 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 321716 - MANUFACTURED TRAFFIC-CALMING DEVICES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Speed bumps.

          - Speed humps.

          - Speed cushions.

      - ACTION SUBMITTALS

        - Product Data:

          - Speed bumps.

          - Speed humps.

          - Speed cushions.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for recycled content.>

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection: For each type of exposed finish requiring color selection.

        - Samples for Verification: Manufactured traffic-calming devices, [6 inches (150 mm) long] <Insert dimension>, showing color and cross section; with fasteners.

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SPEED BUMPS <Insert drawing designation>

Insert drawing designation. Use these designations on Drawings to identify each product.

Revise materials or dimensions of speed bumps, hardware, and adhesive below to suit Project. Available products vary with manufacturer and type of device.

        - General: Solid, integrally colored, preformed [recycled rubber] [or] [polyurethane]; UV stabilized. Provide factory-formed or -drilled vertical holes for anchoring to substrate.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain speed bumps from single source from single manufacturer.

          - <Double click to insert sustainable design text for recycled content for parking bumpers.>

          - Size: [2 inches (51 mm) high by 10 inches (254 mm) wide by 72 inches (1800 mm) long] <Insert dimensions>; with tapered, square, or rounded ends.

Retain required option in "Color" Subparagraph below; verify color availability with manufacturers. If retaining more than one, indicate locations of each on Drawings or by inserts.

          - Color: [Black] [Yellow] <Insert color>.

Retain "Embedded Markings" Subparagraph below if required; revise to suit Project. Availability, shape, and style of markings vary with manufacturer; insert shape or style requirement if required.

          - Embedded Markings: Molded-in, [blue] [white] [yellow] reflective markings, permanently inset in exposed surface.

In "Mounting Hardware" Subparagraph below, first option is generally used for asphalt surfaces and second is generally used for concrete. Consult manufacturer for recommendations.

          - Mounting Hardware: Galvanized-steel [spike or dowel, 1/2-inch (13-mm) diameter, 10-inch (254-mm) minimum length] [lag screw, shield, and washers; 1/2-inch (13-mm) diameter, 8-inch (203-mm) minimum length] [hardware as standard with speed bump manufacturer].

Retain "Adhesive" Subparagraph below where permanently installed speed bumps are required or if traffic load is heavy and mechanical attachment is insufficient.

          - Adhesive: [As recommended in writing by speed bump manufacturer] <Insert requirement> to permanently adhere speed bump to substrate.

      - SPEED HUMPS <Insert drawing designation>

Insert drawing designation. Use these designations on Drawings to identify each product.

Revise materials or dimensions of speed humps, hardware, and adhesive below to suit Project. Available products vary with manufacturer and type of device.

        - General: Solid, integrally colored, preformed recycled rubber; UV stabilized. Provide factory-formed or -drilled vertical holes for anchoring to substrate.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain speed humps from single source from single manufacturer.

          - <Double click to insert sustainable design text for recycled content for parking bumpers.>

          - Size: Modular assembly [3 inches (76 mm) high by 12 feet (3.7 m) in overall width] [4 inches (102 mm) high by 14 feet (4.3 m) in overall width] <Insert dimensions>, with overall length as dimensioned on Drawings; and with tapered, square, or rounded ends.

          - Surface Profile: [Circular] [Sinusoidal] [Trapezoidal].

Retain required option in "Color" Subparagraph below; verify color availability with manufacturers. If retaining more than one, indicate locations of each on Drawings or by inserts.

          - Color: [Black] [Yellow] <Insert color>.

Retain "Embedded Markings" Subparagraph below if required; revise to suit Project. Availability, shape, and style of markings vary with manufacturer; insert shape or style requirement if required.

          - Embedded Markings: Molded-in, [blue] [white] [yellow] reflective markings, permanently inset in exposed surface.

In "Mounting Hardware" Subparagraph below, first option is generally used for asphalt surfaces and second is generally used for concrete. Consult manufacturer for recommendations.

          - Mounting Hardware: Galvanized-steel [spike or dowel, 1/2-inch (13-mm) diameter, 10-inch (254-mm) minimum length] [lag screw, shield, and washers; 1/2-inch (13-mm) diameter, 8-inch (203-mm) minimum length] [hardware as standard with speed hump manufacturer].

Retain "Adhesive" Subparagraph below where permanently installed speed humps are required or if traffic load is heavy and mechanical attachment is insufficient.

          - Adhesive: [As recommended in writing by speed hump manufacturer] <Insert requirement> to permanently adhere speed hump to substrate.

      - SPEED CUSHIONS <Insert drawing designation>

Insert drawing designation. Use these designations on Drawings to identify each product.

Revise materials or dimensions of speed cushions, hardware, and adhesive below to suit Project. Available products vary with manufacturer and type of device.

        - General: Solid, integrally colored, preformed recycled rubber; UV stabilized. Provide factory-formed or -drilled vertical holes for anchoring to substrate.

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Source Limitations: Obtain speed cushions from single source from single manufacturer.

          - <Double click to insert sustainable design text for recycled content for parking bumpers.>

          - Size: Modular assembly [3 inches (76 mm) high by 12 feet (3.7 m) in overall width] [4 inches (102 mm) high by 14 feet (4.3 m) in overall width] <Insert dimensions>, with overall length as dimensioned on Drawings; and with tapered, square, or rounded ends.

Retain required option in "Color" Subparagraph below; verify color availability with manufacturers. If retaining more than one, indicate locations of each on Drawings or by inserts.

          - Color: [Black] [Yellow] <Insert color>.

Retain "Embedded Markings" Subparagraph below if required; revise to suit Project. Availability, shape, and style of markings vary with manufacturer; insert shape or style requirement if required.

          - Embedded Markings: Molded-in, [blue] [white] [yellow] reflective markings, permanently inset in exposed surface.

In "Mounting Hardware" Subparagraph below, first option is generally used for asphalt surfaces and second is generally used for concrete. Consult manufacturer for recommendations.

          - Mounting Hardware: Galvanized-steel [spike or dowel, 1/2-inch (13-mm) diameter, 10-inch (254-mm) minimum length] [lag screw, shield, and washers; 1/2-inch (13-mm) diameter, 8-inch (203-mm) minimum length] [hardware as standard with speed cushion manufacturer].

Retain "Adhesive" Subparagraph below where permanently installed speed cushions are required or if traffic load is heavy and mechanical attachment is insufficient.

          - Adhesive: [As recommended in writing by speed cushion manufacturer] <Insert requirement> to permanently adhere speed cushions to substrate.

- EXECUTION

      - EXAMINATION

        - Verify that pavement is in suitable condition to begin installation according to manufacturer's written instructions.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install manufactured traffic-calming devices according to manufacturer's written instructions unless otherwise indicated.

Retain first paragraph below where permanently installed devices are required or if mechanical attachment is considered insufficient, such as for heavy traffic loads. Consult manufacturers for recommendations.

        - Install devices in bed of adhesive before anchoring.

        - Securely anchor devices to substrate with hardware spaced as recommended in writing by manufacturer for heavy traffic. Recess head of hardware beneath top surface of device.

END OF SECTION 321716