Copyright 2021 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

SECTION 328400 - PLANTING IRRIGATION

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Content Requests:

<Double click here to submit questions, comments, or suggested edits to this Section.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED 2009, LEED v4, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Pipes, tubes, and fittings.

          - Encasement for piping.

          - Manual valves.

          - Pressure-reducing valves.

          - Automatic control valves.

          - Automatic drain valves.

          - Transition fittings.

          - Dielectric fittings.

          - Miscellaneous piping specialties.

          - Sprinklers.

          - Quick couplers.

          - Drip irrigation specialties.

          - Controllers.

          - Boxes for automatic control valves.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 220519 "Meters and Gages for Plumbing Piping" for water metering requirements.

          - Section 230923.14 "Flow Instruments" for water metering equipment.

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project. Include only essential definitions or acronyms not well understood by the affected industry or trade.

        - Circuit Piping: Downstream from control valves to sprinklers, specialties, and drain valves. Piping is under pressure during flow.

Retain "Drain Piping" Paragraph below unless drain valves discharge onto grade without downstream piping.

        - Drain Piping: Downstream from circuit-piping drain valves. Piping is not under pressure.

        - ET Controllers: EvapoTranspiration Controllers. Irrigation controllers, which use some method of weather-based adjustment of irrigation. These adjusting methods include use of historical monthly averages of ET, broadcasting of ET measurements, or use of on-site sensors to track ET.

        - Main Piping: Downstream from point of connection to water distribution piping to, and including, control valves. Piping is under water-distribution-system pressure.

        - Low Voltage: As defined in NFPA 70 for circuits and equipment operating at less than 50 V or for remote-control, signaling power-limited circuits.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data:

          - Pipes, tubes, and fittings.

          - Encasement for piping.

          - Manual valves.

          - Pressure-reducing valves.

          - Automatic control valves.

          - Automatic drain valves.

          - Transition fittings.

          - Dielectric fittings.

          - Miscellaneous piping specialties.

          - Sprinklers.

          - Quick couplers.

          - Drip irrigation specialties.

          - Controllers.

          - Boxes for automatic control valves.

          - Include rated capacities, operating characteristics,[ electrical characteristics,] and furnished specialties and accessories.

Retain "Wiring Diagrams" Paragraph below if equipment includes wiring.

        - Wiring Diagrams: For power, signal, and control wiring.

Retain "Delegated Design Submittals" Paragraph below if design services have been delegated to Contractor.

        - Delegated Design Submittal: For irrigation systems indicated to comply with performance requirements and design criteria, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

          - <Double click to insert sustainable design text for irrigation control system.>

          - <Double click to insert sustainable design text for submetering submittal.>

      - INFORMATIONAL SUBMITTALS

Information submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

Retain "Coordination Drawings" Paragraph below where coordination is required for installation of products and materials by separate installers. Preparation of coordination drawings requires participation of each trade involved; coordinate with other Sections specifying products and materials to be included. See Section 013100 "Project Management and Coordination."

        - Coordination Drawings: Irrigation systems, drawn to scale, on which components are indicated and coordinated with each other, using input from Installers of the items involved. Also include adjustments necessary to avoid plantings and obstructions, such as signs and light standards.

        - Zoning Chart: Indicate each irrigation zone and its control valve.

        - Controller Timing Schedule: Indicate timing settings for each automatic controller zone.

        - Field Quality-Control Submittals:

Retain "Field quality-control reports" Subparagraph below if Contractor is responsible for field quality-control testing and inspecting.

          - Field quality-control reports.

Coordinate "Qualification Statements" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article. If inserting additional entities or specialist, add qualifications to "Quality Assurance" Article.

        - Qualification Statements: For [Installer] <Insert entity or specialist>.

        - Delegated design engineer qualifications.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For [sprinklers] [controllers] [and] [automatic control valves] to include in operation and maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

See Section 017700 "Closeout Procedures" for submission of maintenance material items.

        - Extra Stock Material: Furnish extra materials to Owner that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Impact Sprinklers: Equal to <Insert number> percent of amount installed for each type and size indicated, but no fewer than <Insert number> units.

          - Spray Sprinklers: Equal to <Insert number> percent of amount installed for each type and size indicated, but no fewer than <Insert number> units.

          - Bubblers: Equal to <Insert number> percent of amount installed for each type indicated, but no fewer than <Insert number> units.

          - Emitters: Equal to <Insert number> percent of amount installed for each type indicated, but no fewer than <Insert number> units.

          - Drip-Tube System Tubing: Equal to <Insert number> percent of total length installed for each type and size indicated, but not less than [100 ft. (30 m)] [500 ft. (152 m)] <Insert value>.

          - Soaker Tubes: Equal to <Insert number> percent of total length installed for each type and size indicated, but not less than [50 ft. (15.2 m)] [100 ft. (30 m)] <Insert value>.

        - Schedule of maintenance material items.

      - QUALITY ASSURANCE

        - Qualifications:

          - Installers: Entity that employs a [Certified Irrigation Designer - Landscape qualified by the Irrigation Association] [Professional Class member of the American Society of Irrigation Consultants] [Professional Technical Class member of the American Society of Irrigation Consultants] <Insert qualifications>.

        - Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

      - DELIVERY, STORAGE, AND HANDLING

        - Deliver piping with factory-applied end caps. Maintain end caps through shipping, storage, and handling to prevent pipe-end damage and entrance of dirt, debris, and moisture.

        - Store plastic piping protected from direct sunlight. Support piping to prevent sagging and bending.

      - FIELD CONDITIONS

Retain this article if interruption of existing water service is required.

        - Interruption of Existing Water Service: Do not interrupt water service to facilities occupied by Owner or others unless permitted under the following conditions and then only after arranging to provide temporary water service according to requirements indicated:

          - Notify [Architect] [Construction Manager] [Owner] no fewer than [two] <Insert number> days in advance of proposed interruption of water service.

          - Do not proceed with interruption of water service without [Architect's] [Construction Manager's] [Owner's] written permission.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - PERFORMANCE REQUIREMENTS

Retain one option from "Irrigation Zone Control" Paragraph below. If retaining first option, retain controller, wiring, and automatic control valves. Manual control requires a gate valve for each zone.

        - Irrigation Zone Control: [Automatic operation with controller and automatic control] [Manual operation with manual] valves.

Retain "Location of Sprinklers and Specialties" Paragraph below if complete system design and calculations are in the Contract Documents or "Delegated Design" Paragraph below and Contractor is required to assume responsibility for design.

        - Location of Sprinklers and Specialties: Design location is approximate. Make minor adjustments necessary to avoid plantings and obstructions, such as signs and light standards. Maintain 100 percent irrigation coverage of areas indicated.

        - Delegated Design: Design 100 percent coverage irrigation system, including comprehensive engineering analysis by a qualified professional engineer, using performance requirements and design criteria indicated.

Retain applicable soil conditions in subparagraphs below and coordinate with "Delegated Design" Paragraph above.

          - Available land records indicate the following soil conditions:

            - Type: [Coarse] [Medium] [Fine] <Insert type>.

            - Texture:

              - Sand: <Insert number> percent.

              - Silt: <Insert number> percent.

              - Clay: <Insert number> percent.

            - Particle Size:

              - Sand: <Insert number> mm.

              - Silt: <Insert number> mm.

              - Clay: <Insert number> mm.

            - Structure: [Single grained] [Granular] [Platy] [Blocky] <Insert structure>.

            - Density: <Insert lb/cu. ft. (kg./cu. m)>.

            - Moisture Content: <Insert number> percent.

            - Infiltration Rate: <Insert gph (L/s)>.

        - Minimum Working Pressures: The following are minimum pressure requirements for piping, valves, and specialties unless otherwise indicated:

          - Irrigation Main Piping: [200 psig (1380 kPa)] <Insert value>.

          - Circuit Piping: [150 psig (1035 kPa)] <Insert value>.

      - PIPES, TUBES, AND FITTINGS

See "Writing Guide" Article in the Evaluations for a discussion of how this Section is organized and the most efficient way to edit this Section. See "Piping Materials and Standards" Article in the Evaluations for a discussion of piping materials covered by referenced standards.

        - Comply with requirements in "Piping Schedule" Article for applications of pipe, tube, and fitting materials, and for joining methods for specific services, service locations, and pipe sizes.

Pipe in "Galvanized-Steel Pipe" Paragraph below is available in NPS 1/8 to NPS 26 (DN 6 to DN 650).

        - Galvanized-Steel Pipe: ASTM A53/A53M, Standard Weight, Type E, Grade B.

Nipples in "Galvanized-Steel Pipe Nipples" Subparagraph below are available in NPS 1/8 to NPS 12 (DN 6 to DN 300).

          - Galvanized-Steel Pipe Nipples: ASTM A733, made of ASTM A53/A53M or ASTM A106/A106M, Standard Weight, seamless-steel pipe with threaded ends.

Fittings in "Galvanized, Gray-Iron Threaded Fittings" Subparagraph below are available in NPS 1/4 to NPS 12 (DN 8 to DN 300).

          - Galvanized, Gray-Iron Threaded Fittings: ASME B16.4, Class 125, standard pattern.

Unions in "Malleable-Iron Unions" Subparagraph below are available in NPS 1/8 to NPS 4 (DN 6 to DN 100).

          - Malleable-Iron Unions: ASME B16.39, Class 150, hexagonal-stock body with ball-and-socket, metal-to-metal, bronze seating surface, and female threaded ends.

Flanges in "Cast-Iron Flanges" Subparagraph below are available in NPS 1 to NPS 96 (DN 25 to DN 2400).

          - Cast-Iron Flanges: ASME B16.1, Class 125.

Pipe in "Ductile-Iron Pipe with Mechanical Joints" Paragraph below is available in NPS 3 to NPS 64 (DN 80 to DN 1600).

        - Ductile-Iron Pipe with Mechanical Joints: AWWA C151, with mechanical-joint bell and spigot ends.

Fittings in "Mechanical-Joint, Ductile-Iron Fittings" Subparagraph below are available in NPS 3 to NPS 48 (DN 80 to DN 1200).

          - Mechanical-Joint, Ductile-Iron Fittings: AWWA C110, ductile- or gray-iron standard pattern or AWWA C153, ductile-iron compact pattern.

            - Glands, Gaskets, and Bolts: AWWA C111, ductile- or gray-iron glands, rubber gaskets, and steel bolts.

Pipe in "Ductile-Iron Pipe with Push-on Joint" Paragraph below is available in NPS 3 to NPS 64 (DN 80 to DN 1600).

        - Ductile-Iron Pipe with Push-on Joint: AWWA C151, with push-on-joint bell and spigot ends.

Fittings in "Push-on-Joint, Ductile-Iron Fittings" Subparagraph below are available in NPS 3 to NPS 64 (DN 80 to DN 1600).

          - Push-on-Joint, Ductile-Iron Fittings: AWWA C110, ductile- or gray-iron standard pattern or AWWA C153, ductile-iron compact pattern.

            - Gaskets: AWWA C111 rubber.

Tube in "Soft Copper Tube" Paragraph below is available in NPS 1/8 to NPS 12 (DN 6 to DN 300).

        - Soft Copper Tube: ASTM B88, Type L (ASTM B88M, Type B), water tube, annealed temper.

Cast-copper-alloy fittings in "Copper Pressure Fittings" Subparagraph below are available in NPS 1/4 to NPS 12 (DN 8 to DN 300); wrought-copper fittings are available in NPS 1/4 to NPS 8 (DN 8 to DN 200).

          - Copper Pressure Fittings: ASME B16.18, cast-copper-alloy or ASME B16.22, wrought-copper solder-joint fittings. Furnish wrought-copper fittings if indicated.

Flanges in "Bronze Flanges" Subparagraph below are available in NPS 1/2 to NPS 12 (DN 15 to DN 300).

          - Bronze Flanges: ASME B16.24, Class 150, with solder-joint end.

Unions in "Copper Unions" Subparagraph below are available in NPS 1/4 to NPS 4 (DN 8 to DN 100).

          - Copper Unions: MSS SP-123, cast-copper-alloy, hexagonal-stock body, with ball-and-socket, metal-to-metal seating surfaces and solder-joint or threaded ends.

Tube in "Hard Copper Tube" Paragraph below is available in NPS 1/8 to NPS 12 (DN 6 to DN 300).

        - Hard Copper Tube: [ASTM B88, Type L (ASTM B88M, Type B),] [and] [ASTM B88, Type M (ASTM B88M, Type C),] water tube, drawn temper.

Fittings in "Copper Pressure Fittings" Subparagraph below are available in NPS 1/4 to NPS 12 (DN 8 to DN 300).

          - Copper Pressure Fittings: ASME B16.18, cast-copper-alloy or ASME B16.22, wrought-copper solder-joint fittings. Furnish wrought-copper fittings if indicated.

Flanges in "Bronze Flanges" Subparagraph below are available in NPS 1/2 to NPS 12 (DN 15 to DN 300).

          - Bronze Flanges: ASME B16.24, Class 150, with solder-joint end.

Unions in "Copper Unions" Subparagraph below are available in NPS 1/2 to NPS 4 (DN 15 to DN 100).

          - Copper Unions: MSS SP-123, cast-copper-alloy, hexagonal-stock body, with ball-and-socket, metal-to-metal seating surfaces and solder-joint or threaded ends.

Pipe in "PE Pipe with Controlled ID" Paragraph below is available in NPS 1/2 to NPS 6 (DN 15 to DN 150).

        - PE Pipe with Controlled ID: ASTM D2239, PE 3408 compound; [SIDR 11.5] [and] [SIDR 15].

Fittings in "Insert Fittings for PE Pipe" Subparagraph below are available in NPS 1/2 to NPS 4 (DN 15 to DN 100).

          - Insert Fittings for PE Pipe: ASTM D2609, nylon or propylene plastic with barbed ends. Include bands or other fasteners.

Pipe in "PE Pipe with Controlled OD" Paragraph below is available in NPS 1/2 to NPS 6 (DN 15 to DN 150).

        - PE Pipe with Controlled OD: ASTM D3035, PE 3408 compound, SIDR 11.

Fittings in "PE Butt, Heat-Fusion Fittings" Subparagraph below are available in NPS 1/2 to NPS 48 (DN 15 to DN 1200).

          - PE Butt, Heat-Fusion Fittings: ASTM D3261.

Fittings in "PE Socket-Type Fittings" Subparagraph below are available in NPS 1/2 to NPS 4 (DN 15 to DN 100).

          - PE Socket-Type Fittings: ASTM D2683.

Pipe in "PE Pressure Pipe" Paragraph below is available in NPS 4 to NPS 63 (DN 100 to DN 1575).

        - PE Pressure Pipe: AWWA C906, with DR of 7.3, 9, or 9.3 and PE compound number required to give pressure rating of not less than [160 psig (1100 kPa)] [200 psig (1380 kPa)].

Fittings in "PE Butt, Heat-Fusion Fittings" Subparagraph below are available in NPS 1/2 to NPS 48 (DN 15 to DN 1200).

          - PE Butt, Heat-Fusion Fittings: ASTM D3261.

Fittings in "PE Socket-Type Fittings" Subparagraph below are available in NPS 1/2 to NPS 4 (DN 15 to DN 100).

          - PE Socket-Type Fittings: ASTM D2683.

Pipe in "PVC Pipe" Paragraph below is available in NPS 1/8 to NPS 24 (DN 6 to DN 600). Retain last option if retaining both thicknesses in "Piping Schedule" Article.

        - PVC Pipe: ASTM D1785, PVC 1120 compound, [Schedule 40] [Schedule 80] [Schedules 40 and 80].

Fittings in "PVC Socket Fittings" Subparagraph below are available in NPS 1/2 to NPS 12 (DN 15 to DN 300). Retain last option if retaining both thicknesses in "Piping Schedule" Article.

          - PVC Socket Fittings: ASTM D2466, [Schedule 40] [Schedule 80] [Schedules 40 and 80].

Fittings in "PVC Threaded Fittings" Subparagraph below are available in NPS 1/2 to NPS 2 (DN 15 to DN 50).

          - PVC Threaded Fittings: ASTM D2464, Schedule 80.

Unions in subparagraph below are available in NPS 1/2 to NPS 2 (DN 15 to DN 50).

          - PVC Socket Unions: Construction similar to that of MSS SP-107, except both headpiece and tailpiece shall be PVC with socket ends.

Pipe in "PVC Pipe, Pressure Rated" Paragraph below is available in NPS 3/4 to NPS 36 (DN 20 to DN 900).

        - PVC Pipe, Pressure Rated: ASTM D2241, PVC 1120 compound, [SDR 21] [and] [SDR 26].

Fittings in "PVC Socket Fittings" Subparagraph below are available in NPS 1/8 to NPS 12 (DN 6 to DN 300).

          - PVC Socket Fittings: ASTM D2467, Schedule 80.

Unions in "PVC Socket Unions" Subparagraph below are available in NPS 1/2 to NPS 2 (DN 15 to DN 50).

          - PVC Socket Unions: Construction similar to that of MSS SP-107, except both headpiece and tailpiece shall be PVC with socket or threaded ends.

      - PIPING JOINING MATERIALS

        - Pipe-Flange Gasket Materials: AWWA C110, rubber, flat face, 1/8 inch (3 mm) thick unless otherwise indicated; full-face or ring type unless otherwise indicated.

        - Metal, Pipe-Flange Bolts and Nuts: ASME B18.2.1, carbon steel unless otherwise indicated.

        - Brazing Filler Metals: AWS A5.8/A5.8M, BCuP Series, copper-phosphorus alloys for general-duty brazing unless otherwise indicated.

        - Solder Filler Metals: ASTM B32, lead-free alloys. Include water-flushable flux in accordance with ASTM B813.

        - Solvent Cements for Joining PVC Piping: ASTM D2564. Include primer in accordance with ASTM F656.

        - Plastic, Pipe-Flange Gasket, Bolts, and Nuts: Type and material recommended by piping system manufacturer unless otherwise indicated.

      - ENCASEMENT FOR PIPING

        - Standard: ASTM A674 or AWWA C105.

        - Form: [Sheet] [Sheet or tube] [Tube].

        - Material: [LLDPE film of 0.008-inch (0.20-mm)] [LLDPE film of 0.008-inch (0.20-mm) minimum thickness or high-density, cross-laminated PE film of 0.004-inch (0.10-mm)] [High-density, cross-laminated PE film of 0.004-inch (0.10-mm)] minimum thickness.

        - Color: [Black] [or] [Natural] <Insert color>.

      - MANUAL VALVES

Valves in "Curb Valves" Paragraph below are available in NPS 1/2 to NPS 2 (DN 15 to DN 50).

        - Curb Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: AWWA C800.

            - NPS 1 (DN 25) and Smaller Pressure Rating: [100 psig (690 kPa) minimum] [150 psig (1035 kPa)].

            - NPS 1-1/4 to NPS 2 (DN 32 to DN 50) Pressure Rating: [80 psig (550 kPa) minimum] [150 psig (1035 kPa)].

            - Body Material: Brass or bronze with ball or ground-key plug.

            - End Connections: Matching piping.

            - Stem: With wide-tee head.

Retain "Curb-Valve Casing" Paragraph below with "Curb Valves" Paragraph above. "Casing" is the AWWA term. Most manufacturers simply call them "valve boxes."

        - Curb-Valve Casing:

          - Standard: Similar to AWWA M44 for cast-iron valve casings.

          - Top Section: Telescoping, of length required for depth of burial of curb valve.

          - Barrel: Approximately 3-inch (75-mm) diameter.

          - Plug: With lettering "WATER."

          - Bottom Section: With base of size to fit over valve.

          - Base Support: Concrete collar[ or wood frame].

        - Shutoff Rods for Curb-Valve Casings: Furnish [one] [two] <Insert number> steel, tee-handle shutoff rod(s) with one pointed end, stem of length to operate deepest buried valve, and slotted end matching curb valve for Project.

Valves in "Brass Ball Valves" Paragraph below are available in NPS 1/4 to NPS 4 (DN 8 to DN 100).

        - Brass Ball Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: MSS SP-110.

            - SWP Rating: 150 psig (1035 kPa).

            - CWP Rating: 600 psig (4140 kPa).

            - Body Design: Two piece.

            - Body Material: Forged brass.

            - Ends: Threaded or solder joint if indicated.

            - Seats: PTFE or TFE.

            - Stem: Brass.

            - Ball: Chrome-plated brass.

            - Port: Full[ or regular, but not reduced].

Valves in "Bronze Ball Valves" Paragraph below are available in NPS 1/4 to NPS 4 (DN 8 to DN 100).

        - Bronze Ball Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: MSS SP-110.

            - SWP Rating: 150 psig (1035 kPa).

            - CWP Rating: 600 psig (4140 kPa).

            - Body Design: Two piece.

            - Body Material: Bronze.

            - Ends: Threaded or solder joint if indicated.

            - Seats: PTFE or TFE.

            - Stem: Bronze.

            - Ball: Chrome-plated brass.

            - Port: Full[ or regular, but not reduced].

Valves in "Iron Ball Valves" Paragraph below are available in NPS 1/2 to NPS 36 (DN 15 to DN 900).

        - Iron Ball Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: MSS SP-72.

            - CWP Rating: 200 psig (1380 kPa).

            - Body Design: Split body.

            - Body Material: ASTM A126, gray iron.

            - Ends: Flanged.

            - Seats: PTFE or TFE.

            - Stem: Stainless steel.

            - Ball: Stainless steel.

            - Port: Full.

Valves in "Plastic Ball Valves" Paragraph below are available in NPS 1/2 to NPS 4 (DN 15 to DN 100).

        - Plastic Ball Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: MSS SP-122.

            - Pressure Rating: [125 psig (860 kPa) minimum] [150 psig (1035 kPa)].

            - Body Material: PVC.

            - Type: Union.

            - End Connections: Socket or threaded.

            - Port: Full.

Valves in "Bronze Gate Valves" Paragraph below are available in NPS 1/8 to NPS 3 (DN 6 to DN 80).

        - Bronze Gate Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: MSS SP-80, Type 2.

            - Class: 125.

            - CWP Rating: 200 psig (1380 kPa).

            - Body Material: ASTM B62 bronze with integral seat and screw-in bonnet.

            - Ends: Threaded or solder joint.

            - Stem: Bronze, nonrising.

            - Disc: Solid wedge; bronze.

            - Packing: Asbestos free.

            - Handwheel: Malleable iron, bronze, or aluminum.

Valves in "Iron Gate Valves, Resilient Seated" Paragraph below are available in NPS 3 to NPS 30 (DN 80 to DN 750).

        - Iron Gate Valves, Resilient Seated:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: AWWA C509.

            - Pressure Rating: [200 psig (1380 kPa)] [250 psig (1725 kPa)] minimum.

            - Body Material: Ductile or gray iron with bronze trim.

            - End Connections: Mechanical joint or push-on joint.

            - Interior Coating: Comply with AWWA C550.

            - Body Design: Nonrising stem.

            - Operator: Stem nut.

            - Disc: Solid wedge with resilient coating.

Retain "Iron Gate Valve Casings" Paragraph below with "Iron Gate Valves, Resilient Seated" Paragraph above. "Casing" is the AWWA term. Most manufacturers simply call them "valve boxes."

        - Iron Gate Valve Casings:

          - Standard: AWWA M44 for cast-iron valve casings.

          - Top Section: Adjustable extension of length required for depth of burial of valve.

          - Barrel: Approximately 5-inch (125-mm) diameter.

          - Plug: With lettering "WATER."

          - Bottom Section: With base of size to fit over valve.

          - Base Support: Concrete collar[ or wood frame].

        - Operating Wrenches for Iron Gate Valve Casings: Furnish [one] [two] <Insert number> steel, tee-handle operating wrench(es) with one pointed end, stem of length to operate deepest buried valve, and socket matching valve operating nut for Project.

Valves in "Iron Gate Valves, NRS" Paragraph below are available in NPS 2 to NPS 48 (DN 50 to DN 1200).

        - Iron Gate Valves, NRS:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: MSS SP-70, Type I.

            - CWP Rating: 200 psig (1380 kPa).

            - Body Material: ASTM A126, gray iron with bolted bonnet.

            - Ends: Flanged.

            - Trim: All bronze.

            - Disc: Solid wedge.

            - Packing and Gasket: Asbestos free.

Valves in "Iron Gate Valves, OS&Y" Paragraph below are available in NPS 2 to NPS 48 (DN 50 to DN 1200).

        - Iron Gate Valves, OS&Y:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: MSS SP-70, Type I.

            - CWP Rating: 200 psig (1380 kPa).

            - Body Material: ASTM A126, gray iron with bolted bonnet.

            - Ends: Flanged.

            - Trim: All bronze.

            - Disc: Solid wedge.

            - Packing and Gasket: Asbestos free.

      - PRESSURE-REDUCING VALVES

Copy and revise first paragraph below for each type of water regulator required.

Regulators in "Water Regulators" Paragraph below are available in NPS 1/2 to NPS 3 (DN 15 to DN 80).

        - Water Regulators:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Standard: ASSE 1003.

            - Body Material: Bronze for NPS 2 (DN 50) and smaller; cast iron[ with interior lining complying with AWWA C550 or that is FDA approved] for NPS 2-1/2 and NPS 3 (DN 65 and DN 80).

            - Pressure Rating: Initial pressure of 150 psig (1035 kPa).

            - End Connections: Threaded for NPS 2 (DN 50) and smaller; flanged for NPS 2-1/2 and NPS 3 (DN 65 and DN 80).

If Project has more than one type or configuration of water regulator, delete "Capacities and Characteristics" Subparagraph below and schedule water regulators on Drawings.

          - Capacities and Characteristics:

            - Size: <Insert NPS (DN)>.

            - Design Flow Rate: <Insert gpm (L/s)>.

            - Design Inlet Pressure: <Insert psig (kPa)>.

            - Design Outlet Pressure Setting: <Insert psig (kPa)>.

Valves in "Water-Control Valves" Paragraph below are available in NPS 1-1/4 (DN 32) and larger.

        - Water-Control Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Pilot-operation, diaphragm-type, single-seated main water-control valve. Include small pilot control valve, restrictor device, specialty fittings, and sensor piping.

            - Main Valve Body: Cast- or ductile-iron body with AWWA C550 or FDA-approved, interior epoxy coating; or stainless steel body.

            - Pattern: [Angle] [Globe]-valve design.

            - Trim: Stainless steel.

            - Pressure Rating: Initial pressure of 150 psig (1035 kPa) minimum.

            - End Connections: Threaded for NPS 2 (DN 50) and smaller; [flanged] <Insert type> for NPS 2-1/2 (DN 65) and larger.

If Project has more than one type or configuration of water-control valve, delete "Capacities and Characteristics" Subparagraph below and schedule water control valves on Drawings.

          - Capacities and Characteristics:

            - Size: <Insert NPS (DN)>.

            - Design Flow Rate: <Insert gpm (L/s)>.

            - Design Inlet Pressure: <Insert psig (kPa)>.

            - Design Outlet Pressure Setting: <Insert psig (kPa)>.

      - AUTOMATIC CONTROL VALVES

Valves in this article are typically available as "normally closed" valves that open on signal from the controller and are the recommended type. "Normally open" valves that automatically close if power failure occurs are also available.

        - Bronze, Automatic Control Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Cast-bronze body, normally closed, diaphragm type with manual-flow adjustment, and operated by 24 V ac solenoid.

        - Plastic, Automatic Control Valves:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Molded-plastic body, normally closed, diaphragm type with manual-flow adjustment, and operated by 24 V ac solenoid.

      - AUTOMATIC DRAIN VALVES

        - Description: Spring-loaded-ball type of corrosion-resistant construction and designed to open for drainage if line pressure drops below 2-1/2 to 3 psig (17 to 20 kPa).

      - TRANSITION FITTINGS

        - General Requirements: Same size as, and with pressure rating at least equal to and with ends compatible with, piping to be joined.

Couplings in "Transition Couplings" Paragraph below are available in NPS 1/2 to NPS 144 (DN 15 to DN 3600).

        - Transition Couplings:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: AWWA C219, metal sleeve-type coupling for underground pressure piping.

Fittings in "Plastic-to-Metal Transition Fittings" Paragraph below are available in at least NPS 1/2 to NPS 2 (DN 15 to DN 50).

        - Plastic-to-Metal Transition Fittings:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: PVC one-piece fitting with manufacturer's Schedule 80 equivalent dimensions; one end with threaded brass insert and one solvent-cement-socket[ or threaded] end.

Unions in "Plastic-to-Metal Unions" Paragraph below are available in NPS 1/2 to NPS 2 (DN 15 to DN 50).

        - Plastic-to-Metal Transition Unions:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: MSS SP-107, PVC four-part union. Include one brass[ or stainless steel] threaded end, one solvent-cement-joint[ or threaded] plastic end, rubber O-ring, and union nut.

      - DIELECTRIC FITTINGS

        - General Requirements: Assembly of copper alloy and ferrous materials or ferrous material body with separating nonconductive insulating material suitable for system fluid, pressure, and temperature.

Unions in "Dielectric Unions" Paragraph below are available in at least NPS 1/2 to NPS 2 (DN 15 to DN 50).

        - Dielectric Unions:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Factory-fabricated union, NPS 2 (DN 50) and smaller.

            - Pressure Rating: [150 psig (1035 kPa) minimum] [250 psig (1725 kPa)] at 180 deg F (82 deg C).

            - End Connections: Solder-joint copper alloy and threaded ferrous; threaded ferrous.

Flanges in "Dielectric Flanges" Paragraph below are available in at least NPS 1-1/2 to NPS 4 (DN 40 to DN 100).

        - Dielectric Flanges:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Factory-fabricated, bolted, companion-flange assembly, NPS 2-1/2 to NPS 4 (DN 65 to DN 100) and larger.

            - Pressure Rating: [150 psig (1035 kPa) minimum] [175 psig (1200 kPa) minimum] [300 psig (2070 kPa)].

            - End Connections: Solder-joint copper alloy and threaded ferrous; threaded solder-joint copper alloy and threaded ferrous.

Kits in "Dielectric-Flange Kits" Paragraph below are available in at least NPS 1/2 to NPS 48 (DN 15 to DN 1200).

        - Dielectric-Flange Kits:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Nonconducting materials for field assembly of companion flanges, NPS 2-1/2 (DN 65) and larger.

            - Pressure Rating: [150 psig (1035 kPa) minimum] <Insert value>.

            - Gasket: Neoprene or phenolic.

            - Bolt Sleeves: Phenolic or polyethylene.

            - Washers: Phenolic with steel backing washers.

Couplings in "Dielectric Couplings" Paragraph below are available in at least NPS 1/2 to NPS 3 (DN 15 to DN 80).

        - Dielectric Couplings:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Galvanized-steel coupling.

            - Pressure Rating: 300 psig (2070 kPa) at 225 deg F (107 deg C).

            - End Connections: Female threaded.

            - Lining: Inert and noncorrosive, thermoplastic lining.

Nipples in "Dielectric Nipples" Paragraph below are available in at least NPS 1/2 to NPS 4 (DN 15 to DN 100).

        - Dielectric Nipples:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Electroplated steel nipple complying with ASTM F1545.

            - Pressure Rating: [300 psig (2070 kPa) at 225 deg F (107 deg C)] <Insert values>.

            - End Connections: Male threaded or grooved.

            - Lining: Inert and noncorrosive, propylene.

      - MISCELLANEOUS PIPING SPECIALTIES

        - Water Hammer Arresters: ASSE 1010 or PDI WH 201, with bellows or piston-type pressurized cushioning chamber and in sizes complying with PDI WH 201, Sizes A to F.

        - Pressure Gages: ASME B40.1. Include 4-1/2-inch- (115-mm-) diameter dial, dial range of two times system operating pressure, and bottom outlet.

      - SPRINKLERS

Revise this article to suit Project. Materials and finishes are not uniform among manufacturers. Use a legend on Drawings to indicate type, pressure requirement, and flow capacity of each sprinkler.

        - General Requirements: Designed for uniform coverage over entire spray area indicated at available water pressure.

        - Metal, Exposed, Impact-Drive Rotary Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Construction: Brass and other corrosion-resistant metals.

            - Mounting: Aboveground, exposed on riser.

If Project has more than one type or configuration of metal, exposed, impact-drive rotary sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule metal, exposed, impact-drive rotary sprinklers on Drawings.

          - Capacities and Characteristics:

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)] <Insert value>.

        - Plastic, Exposed, Impact-Drive Rotary Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Construction: ABS and corrosion-resistant metals.

            - Mounting: Aboveground, exposed on riser.

If Project has more than one type or configuration of plastic, exposed, impact-drive rotary sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule plastic, exposed, impact-drive rotary sprinklers on Drawings.

          - Capacities and Characteristics:

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)] <Insert size>.

        - Plastic, Pop-up, Gear-Drive Rotary Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material: ABS.

            - Nozzle: [ABS] [Brass].

            - Retraction Spring: Stainless steel.

            - Internal Parts: Corrosion resistant.

If Project has more than one type or configuration of plastic, pop-up, gear-drive rotary sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule plastic, pop-up, gear-drive rotary sprinklers on Drawings.

IgCC requires a minimum pop-up height of 4 inches (100 mm).

          - Capacities and Characteristics:

            - Flow: <Insert gpm (L/s)>.

            - Pop-up Height: [4 inches (100 mm)] <Insert inches (mm)> aboveground to nozzle.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

        - Metal, Pop-up, Impact-Drive Rotary Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Case: Brass.

            - Body Material: Brass.

            - Pop-up Height: [4 inches (100 mm)] [Approximately 3 inches (75 mm)] <Insert value> aboveground to nozzle.

            - Sprinkler Construction: Brass and other corrosion-resistant metals.

If Project has more than one type or configuration of metal, pop-up, impact-drive rotary sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule metal, pop-up, impact-drive rotary sprinklers on Drawings.

          - Capacities and Characteristics:

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

        - Plastic, Pop-up, Impact-Drive Rotary Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Case: ABS.

            - Pop-up Height: [4 inches (100 mm)] [Approximately 3 inches (75 mm)] <Insert value> aboveground to nozzle.

            - Sprinkler Construction: ABS and other corrosion-resistant metals.

If Project has more than one type or configuration of plastic, pop-up, impact-drive rotary sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule plastic, pop-up, impact-drive rotary sprinklers on Drawings.

          - Capacities and Characteristics:

            - Nozzle: [ABS] [Brass].

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

        - Metal, Surface Spray Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material and Flange: Brass.

            - Nozzle: Brass.

            - Pattern: Fixed, with flow adjustment.

If Project has more than one type or configuration of metal, surface spray sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule metal, surface spray sprinklers on Drawings.

          - Capacities and Characteristics:

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

        - Plastic, Surface Spray Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material and Flange: ABS.

            - Pattern: Fixed, with flow adjustment.

If Project has more than one type or configuration of plastic, surface spray sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule plastic, surface spray sprinklers on Drawings.

          - Capacities and Characteristics:

            - Nozzle: [ABS] [Brass].

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

        - Metal, Surface, Pop-up Spray Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material and Flange: Brass.

            - Nozzle: Brass.

            - Pattern: Fixed, with flow adjustment.

If Project has more than one type or configuration of metal, surface, pop-up spray sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule metal, surface, pop-up spray sprinklers on Drawings.

IgCC requires a minimum pop-up height of 4 inches (100 mm).

          - Capacities and Characteristics:

            - Pop-up Height: [4 inches (100 mm)] [1-1/4 or 1-1/2 inches (32 or 38 mm)] [1-3/4 or 2 inches (44 or 50 mm)] <Insert value>.

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

        - Plastic, Surface, Pop-up Spray Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material and Flange: ABS.

            - Pattern: Fixed, with flow adjustment.

If Project has more than one type or configuration of plastic, surface, pop-up spray sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule plastic, surface, pop-up spray sprinklers on Drawings.

IgCC requires a minimum pop-up height of 4 inches (100 mm).

          - Capacities and Characteristics:

            - Pop-up Height: [4 inches (100 mm)] [1-1/4 or 1-1/2 inches (32 or 38 mm)] [1-3/4 or 2 inches (44 or 50 mm)] <Insert value>.

            - Nozzle: [ABS] [Brass].

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

        - Plastic, Pop-up Spray Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material: ABS.

            - Nozzle: [ABS] [Brass].

            - Retraction Spring: Stainless steel.

            - Internal Parts: Corrosion resistant.

            - Pattern: Fixed, with flow adjustment.

If Project has more than one type or configuration of plastic, pop-up spray sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule plastic, pop-up spray sprinklers on Drawings.

IgCC requires a minimum pop-up height of 4 inches (100 mm).

          - Capacities and Characteristics:

            - Nozzle: [ABS] [Brass].

            - Flow: <Insert gpm (L/s)>.

            - Pop-up Height: [4 inches (100 mm)] <Insert inches (mm)> aboveground to nozzle.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

        - Metal Shrub Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material: Brass.

            - Nozzle: Brass.

            - Pattern: Fixed, with flow adjustment.

If Project has more than one type or configuration of metal shrub sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule metal shrub sprinklers on Drawings.

          - Capacities and Characteristics:

            - Configuration: <Insert description>.

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Mounting Height: <Insert inches (mm)> aboveground to nozzle.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

        - Plastic Shrub Sprinklers:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description:

            - Body Material: ABS or other plastic.

            - Pattern: Fixed, with flow adjustment.

If Project has more than one type or configuration of plastic shrub sprinkler, delete "Capacities and Characteristics" Subparagraph below and schedule plastic shrub sprinklers on Drawings.

          - Capacities and Characteristics:

            - Configuration: <Insert description>.

            - Flow: <Insert gpm (L/s)>.

            - Arc: [Full] [Half] [Quarter] <Insert arc> circle.

            - Radius: <Insert feet (m)>.

            - Mounting Height: <Insert inches (mm)> aboveground to nozzle.

            - Inlet: [NPS 1/2 (DN 15)] [NPS 1/2 or NPS 3/4 (DN 15 or DN 20)] [NPS 3/4 (DN 20)].

      - QUICK COUPLERS

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Description: Factory-fabricated, bronze or brass, two-piece assembly. Include coupler water-seal valve; removable upper body with spring-loaded or weighted, rubber-covered cap; hose swivel with ASME B1.20.7, 3/4-11.5NH threads for garden hose on outlet; and operating key.

Retain "Locking-Top Option" Subparagraph below if required.

          - Locking-Top Option: Vandal-resistant locking feature. Include [one] [two] <Insert number> matching key(s).

      - DRIP IRRIGATION SPECIALTIES

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

Add specific features to paragraphs in this article as required for applications.

        - Freestanding Emitters: Device to deliver water at approximately 20 psig (138 kPa).

          - Body Material: PE or vinyl, with flow control.

          - Riser to Emitter: PE or PVC flexible tubing.

If Project has more than one type or configuration of freestanding emitter, delete "Capacities and Characteristics" Subparagraph below and schedule freestanding emitters on Drawings.

          - Capacities and Characteristics:

            - Flow: [1/2 gph (1.9 L/h)] [1 gph (3.8 L/h)] [2 gph (7.6 L/h)] <Insert value> at approximately 20 psig (138 kPa).

            - Tubing: PE or PVC; 1/8-inch (3-mm) minimum ID.

            - Mounting Height: <Insert dimension> aboveground to nozzle.

        - Manifold Emitter Systems: Manifold with tubing and emitters.

          - Manifold: With multiple outlets to deliver water to emitters.

            - Body Material: Plastic.

            - Outlet Caps: Plastic, for outlets without installed tubing.

            - Operation: Automatic pressure compensating.

          - Tubing: PE or PVC; 1/8-inch (3-mm) minimum ID.

          - Emitter: Device to deliver water at approximately 20 psig (138 kPa).

            - Body Material: PE or vinyl, with flow control.

If Project has more than one type or configuration of manifold emitter system, delete "Capacities and Characteristics" Subparagraph below and schedule manifold emitter systems on Drawings.

          - Capacities and Characteristics:

            - Manifold:

              - Design Flow: <Insert gph (L/h)>.

              - Number of Outlets: [Four] [Six] <Insert number>.

Retain and copy "Emitter No." Subparagraph below for each emitter.

            - Emitter No.: <Insert number>.

              - Flow: [1/2 gph (1.9 L/h)] [1 gph (3.8 L/h)] [2 gph (7.6 L/h)] <Insert value> at approximately 20 psig (138 kPa).

              - Mounting Height: [At ground] [As indicated] <Insert dimension>.

        - Multiple-Outlet Emitter Systems: Emitter with tubing and button-type outlets.

          - Emitter: With multiple outlets to deliver water to remote outlets.

            - Body Material: Plastic, with flow control.

            - Outlet Caps: Plastic, for outlets without installed tubing.

            - Operation: Automatic pressure compensating.

            - Emitters: Devices to deliver water at approximately 20 psig (138 kPa.)

          - Tubing: PE or PVC; 1/8-inch (3-mm) minimum ID.

If Project has more than one type or configuration of multiple-outlet emitter system, delete "Capacities and Characteristics" Subparagraph below and schedule multiple-outlet emitter systems on Drawings.

          - Capacities and Characteristics:

            - Emitter:

              - Flow: [1/2 gph (1.9 L/h)] [1 gph (3.8 L/h)] [2 gph (7.6 L/h)] <Insert value>.

              - Number of Outlets: [Four] [Six] <Insert number>.

Retain and copy "Button-Type Outlet No." Subparagraph below for each emitter.

            - Button-Type Outlet No.: <Insert number>.

              - Mounting Height: [At ground] [As indicated] <Insert dimension>.

        - Drip Tubes with Direct-Attached Emitters:

          - Tubing: Flexible PE or PVC with plugged end.

          - Emitters: Devices to deliver water at approximately 20 psig (138 kPa).

            - Body Material: PE or vinyl, with flow control.

            - Mounting: Inserted into tubing at set intervals.

If Project has more than one type or configuration of drip tube with direct-attached emitter, delete "Capacities and Characteristics" Subparagraph below and schedule drip tubes with direct-attached emitters on Drawings.

          - Capacities and Characteristics:

            - Tubing Size: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] [NPS 1 (DN 25)] <Insert value>.

            - Length: <Insert inches (mm)>.

            - Emitter Spacing: <Insert inches (mm)>.

            - Emitter Flow: [1/2 gph (1.9 L/h)] [1 gph (3.8 L/h)] [2 gph (7.6 L/h)] <Insert value>.

        - Drip Tubes with Remote Discharge:

          - Tubing: Flexible PE or PVC with plugged end.

          - Emitters: Devices to deliver water at approximately 20 psig (138 kPa).

            - Body Material: PE or vinyl, with flow control.

            - Mounting: Inserted into tubing at set intervals.

If Project has more than one type or configuration of drip tube with remote discharge, delete "Capacities and Characteristics" Subparagraph below and schedule drip tubes with remote discharge on Drawings.

          - Capacities and Characteristics:

            - Tubing Size: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] [NPS 1 (DN 25)] <Insert value>.

            - Length: <Insert inches (mm)>.

            - Emitter Spacing: <Insert inches (mm)>.

            - Emitter Flow: [1/2 gph (1.9 L/h)] [1 gph (3.8 L/h)] [2 gph (7.6 L/h)] <Insert value>.

            - Branch Tubing Size: [NPS 1/4 (DN 8)] <Insert value> with button-type outlet.

            - Branch Tubing Length: <Insert inches (mm)>.

        - Off-Ground Supports: Plastic stakes.

        - Application Pressure Regulators: Brass or plastic housing, NPS 3/4 (DN 20), with corrosion-resistant internal parts; capable of controlling outlet pressure to approximately 20 psig (138 kPa).

        - Filter Units: Brass or plastic housing, with corrosion-resistant internal parts; of size and capacity required for devices downstream from unit.

        - Air Relief Valves: Brass or plastic housing, with corrosion-resistant internal parts.

        - Vacuum Relief Valves: Brass or plastic housing, with corrosion-resistant internal parts.

Insert other irrigation specialties and devices here as required to suit Project.

      - CONTROLLERS

        - <Double click here to find, evaluate, and insert list of manufacturers and products.>

        - Description:

Increase station time settings in "Controller Stations for Automatic Control Valves" Subparagraph below as required for drip applications.

          - Controller Stations for Automatic Control Valves: Each station is variable from approximately [5 to 60] <Insert numbers> minutes. Include switch for manual or automatic operation of each station.

Retain "Exterior Control Enclosures" Subparagraph below if exterior controllers are required. Insert special mounting to suit Project.

          - Exterior Control Enclosures: NEMA 250, Type 4, weatherproof, with locking cover and [two] <Insert number> matching keys; include provision for grounding.

            - Body Material: [Enameled-steel sheet metal] [Stainless steel sheet metal] [Molded plastic].

            - Mounting: [Freestanding type for concrete base] [Surface type for wall].

Retain "Interior Control Enclosures" Subparagraph below if interior controllers are required. Insert special mounting to suit Project.

          - Interior Control Enclosures: NEMA 250, Type 12, dripproof, with locking cover and [two] <Insert number> matching keys.

            - Body Material: [Enameled-steel sheet metal] [Stainless steel sheet metal] [Molded plastic].

            - Mounting: [Freestanding type for concrete base] [Surface type for wall].

          - Control Transformer: 24 V secondary, with primary fuse.

          - Timing Device: Adjustable, 24-hour, 14-day clock, with automatic operations to skip operation any day in timer period, to operate every other day, or to operate two or more times daily.

            - Manual or Semiautomatic Operation: Allows this mode without disturbing preset automatic operation.

            - Nickel-Cadmium Battery and Trickle Charger: Automatically powers timing device during power outages.

            - Surge Protection: Metal-oxide-varistor type on each station and primary power.

Retain one of the following two subparagraphs below. Smart controllers are required for compliance with ASHRAE 189.1.

          - Moisture Sensor: Adjustable from one to seven days, to shut off water flow during rain.

          - Smart Controllers: Use ET, tested in accordance with IA SWAT Climatological Based Controllers 8th Draft Testing Protocol and compliant with ASHRAE 189.1.

          - Wiring: UL 493, Type UF multiconductor, with solid-copper conductors; insulated cable; suitable for direct burial.

            - Feeder-Circuit Cables: No. 12 AWG minimum, between building and controllers.

            - Low-Voltage, Branch-Circuit Cables: No. 14 AWG minimum, between controllers and automatic control valves; color-coded different from feeder-circuit-cable jacket color; with jackets of different colors for multiple-cable installation in same trench.

            - Splicing Materials: Manufacturer's packaged kit consisting of insulating, spring-type connector or crimped joint and epoxy resin moisture seal; suitable for direct burial.

Retain "Concrete Base" Subparagraph below for exterior mounting of controller.

          - Concrete Base: Reinforced precast concrete not less than 36 by 24 by 4 inches (900 by 600 by 100 mm) thick, and 6 inches (150 mm) greater in each direction than overall dimensions of controller. Include opening for wiring.

      - BOXES FOR AUTOMATIC CONTROL VALVES

Retain either "Plastic Boxes" or "Polymer-Concrete Boxes" Paragraph below for access from grade to valves and devices. Revise material choices and insert features to suit Project. Do not use in roadways.

Boxes in "Plastic Boxes" Paragraph below should be used only in grass or paved areas subject to pedestrian traffic.

        - Plastic Boxes:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Box and cover, with open bottom and openings for piping; designed for installing flush with grade.

            - Size: As required for valves and service.

            - Shape: [Round] [Square] [Rectangular].

            - Sidewall Material: [PE] [PE, ABS, or FRP] <Insert material>.

            - Cover Material: [PE] [PE, ABS, or FRP] <Insert material>.

Revise "Lettering" Subparagraph below for specific lettering required and verify availability, or delete subparagraph.

              - Lettering: "[VALVE BOX] [IRRIGATION] <Insert lettering>."

Boxes in "Polymer-Concrete Boxes" Paragraph below may be used for applications that include areas subject to infrequent light vehicular traffic but should not be used for locations with roadway traffic.

        - Polymer-Concrete Boxes:

          - <Double click here to find, evaluate, and insert list of manufacturers and products.>

          - Description: Box and cover, with open bottom and openings for piping; designed for installing flush with grade.

            - Size: As required for valves and service.

            - Shape: [Round] [Square] [Rectangular].

            - Sidewall Material: Polymer concrete with lateral and vertical sidewall design loading of [5000 lb (2268 kg)] [10,000 lb (4536 kg)] [15,000 lb (6800 kg)] minimum over 10 by 10 inches (254 by 254 mm) square.

            - Cover Material: [Polymer concrete] [Reinforced polymer concrete] <Insert material> with cover design loading of [5000 lb (2268 kg)] [10,000 lb (4536 kg)] [15,000 lb (6800 kg)] minimum over 10 by 10 inches (254 by 254 mm) square.

Revise "Lettering" Subparagraph below for specific lettering required and verify availability, or delete subparagraph.

              - Lettering: "[VALVE BOX] [IRRIGATION] <Insert lettering>."

Retain "Drainage Backfill" Paragraph below for drain pockets.

        - Drainage Backfill: Cleaned gravel or crushed stone, graded from 3/4 inch (19 mm) minimum to 3 inches (75 mm) maximum.

- EXECUTION

      - EARTHWORK

        - Excavating, trenching, and backfilling are specified in Section 312000 "Earth Moving."

        - Install warning tape directly above pressure piping, [12 inches (300 mm)] <Insert value> below finished grades, except 6 inches (150 mm) below subgrade under pavement and slabs.

        - Drain Pockets: Excavate to sizes indicated. Backfill with cleaned gravel or crushed stone, graded from [3/4 to 3 inches (19 to 75 mm)] <Insert value>, to [12 inches (300 mm)] <Insert value> below grade. Cover gravel or crushed stone with sheet of asphalt-saturated felt and backfill remainder with excavated material.

        - Provide minimum cover over top of underground piping according to the following:

Revise cover depths in subparagraphs below to suit Project and to ensure that pipes are below depth of frost penetration. Depth is not critical if system will be drained during freezing conditions.

          - Irrigation Main Piping: Minimum depth of [36 inches (900 mm)] <Insert value> below finished grade, or not less than [18 inches (450 mm)] <Insert value> below average local frost depth, whichever is deeper.

          - Circuit Piping: [12 inches (300 mm)] <Insert value>.

          - Drain Piping: [12 inches (300 mm)] <Insert value>.

          - Sleeves: [24 inches (600 mm)] <Insert value>.

      - PREPARATION

        - Set stakes to identify locations of proposed irrigation system. Obtain Architect's approval before excavation.

      - INSTALLATION OF PIPING

        - Location and Arrangement: Drawings indicate location and arrangement of piping systems. Install piping as indicated unless deviations are approved on Coordination Drawings.

        - Install piping at minimum uniform slope of 0.5 percent down toward drain valves.

        - Install piping free of sags and bends.

        - Install groups of pipes parallel to each other, spaced to permit valve servicing.

        - Install fittings for changes in direction and branch connections.

        - Install unions adjacent to valves and to final connections to other components with NPS 2 (DN 50) or smaller pipe connection.

        - Install flanges adjacent to valves and to final connections to other components with NPS 2-1/2 (DN 65) or larger pipe connection.

Option in first paragraph below may be withdrawn. If retaining option, verify availability.

        - Install underground thermoplastic piping in accordance with ASTM D2774[ and ASTM ].

        - Install expansion loops in control-valve boxes for plastic piping.

        - Lay piping on solid subbase, uniformly sloped without humps or depressions.

        - Install ductile-iron piping in accordance with AWWA C600.

        - Install PVC piping in dry weather when temperature is above 40 deg F (5 deg C). Allow joints to cure at least 24 hours at temperatures above 40 deg F (5 deg C) before testing.

Revise first two paragraphs below to suit Project; delete if not required.

        - Install water regulators with shutoff valve and strainer on inlet and pressure gage on outlet. Install shutoff valve on outlet. Install aboveground or in control-valve boxes.

        - Water Hammer Arresters: Install between connection to building main and circuit valves aboveground or in control-valve boxes.

        - Install piping in sleeves under parking lots, roadways, and sidewalks.

        - Install sleeves made of [Schedule 40] [Schedule 80], PVC pipe and socket fittings, and solvent-cemented joints.

        - Install transition fittings for plastic-to-metal pipe connections according to the following:

          - Underground Piping:

            - NPS 1-1/2 (DN 40) and Smaller: Plastic-to-metal transition fittings.

            - NPS 2 (DN 50) and Larger: AWWA transition couplings.

          - Aboveground Piping:

            - NPS 2 (DN 50) and Smaller: Plastic-to-metal transition [fittings] [unions].

            - NPS 2 (DN 50) and Larger: Use dielectric flange kits with one plastic flange.

        - Install dielectric fittings for dissimilar-metal pipe connections according to the following:

          - Underground Piping:

            - NPS 2 (DN 50) and Smaller: Dielectric coupling or dielectric nipple.

            - NPS 2-1/2 (DN 65) and Larger: Prohibited except in control-valve box.

          - Aboveground Piping:

            - NPS 2 (DN 50) and Smaller: Dielectric union.

            - NPS 2-1/2 to NPS 4 (DN 65 to DN 100): Dielectric flange.

            - NPS 5 (DN 125) and Larger: Dielectric flange kit.

          - Piping in Control-Valve Boxes:

            - NPS 2 (DN 50) and Smaller: Dielectric union.

            - NPS 2-1/2 to NPS 4 (DN 65 to DN 100): Dielectric flange.

            - NPS 5 (DN 125) and Larger: Dielectric flange kit.

      - JOINT CONSTRUCTION

        - Ream ends of pipes and tubes and remove burrs. Bevel plain ends of steel pipe.

        - Remove scale, slag, dirt, and debris from inside and outside of pipe and fittings before assembly.

        - Threaded Joints: Thread pipe with tapered pipe threads in accordance with ASME B1.20.1. Cut threads full and clean using sharp dies. Ream threaded pipe ends to remove burrs and restore full ID. Join pipe fittings and valves as follows:

          - Apply appropriate tape or thread compound to external pipe threads unless dry seal threading is specified.

          - Damaged Threads: Do not use pipe or pipe fittings with threads that are corroded or damaged. Do not use pipe sections that have cracked or open welds.

        - Flanged Joints: Select rubber gasket material of size, type, and thickness for service application. Install gasket concentrically positioned. Use suitable lubricants on bolt threads.

        - Ductile-Iron Piping Gasketed Joints: Comply with AWWA C600 and AWWA M41.

        - Copper-Tubing Brazed Joints: Construct joints in accordance with CDA's "Copper Tube Handbook," using copper-phosphorus brazing filler metal.

        - Copper-Tubing Soldered Joints: Apply ASTM B813 water-flushable flux to tube end unless otherwise indicated. Construct joints in accordance with ASTM B828 or CDA's "Copper Tube Handbook," using lead-free solder alloy (0.20 percent maximum lead content) complying with ASTM B32.

        - PE Piping Fastener Joints: Join with insert fittings and bands or fasteners in accordance with piping manufacturer's written instructions.

        - PE Piping Heat-Fusion Joints: Clean and dry joining surfaces by wiping with clean cloth or paper towels. Join in accordance with ASTM D2657.

          - Plain-End PE Pipe and Fittings: Use butt fusion.

          - Plain-End PE Pipe and Socket Fittings: Use socket fusion.

        - PVC Piping Solvent-Cemented Joints: Clean and dry joining surfaces. Join pipe and fittings in accordance with the following:

          - Comply with ASTM F402 for safe-handling practice of cleaners, primers, and solvent cements.

          - PVC Pressure Piping: Join schedule number, ASTM D1785, PVC pipe and PVC socket fittings in accordance with ASTM D2672. Join other-than-schedule-number PVC pipe and socket fittings in accordance with ASTM D2855.

          - PVC Nonpressure Piping: Join in accordance with ASTM D2855.

      - INSTALLATION OF VALVES

        - Underground Curb Valves: Install in curb-valve casings with tops flush with grade.

        - Underground Iron Gate Valves, Resilient Seat: Comply with AWWA C600 and AWWA M44. Install in valve casing with top flush with grade.

An alternative to restrained joints in subparagraph below is thrust blocks indicated on Drawings.

          - Install valves and PVC pipe with restrained, gasketed joints.

        - Aboveground Valves: Install as components of connected piping system.

        - Pressure-Reducing Valves: Install in boxes for automatic control valves or aboveground between shutoff valves.[ Install full-size valved bypass.]

        - Throttling Valves: Install in underground piping in boxes for automatic control valves.

        - Drain Valves: Install in underground piping in boxes for automatic control valves.

      - INSTALLATION OF SPRINKLERS

        - Install sprinklers after hydrostatic test is completed.

        - Install sprinklers at manufacturer's recommended heights.

        - Locate part-circle sprinklers to maintain a minimum distance of 4 inches (100 mm) from walls and 2 inches (50 mm) from other boundaries unless otherwise indicated.

      - INSTALLATION OF DRIP IRRIGATION SPECIALTIES

Irrigation specialty type, arrangement, sizes, water-flow data, and mounting heights should be indicated on Drawings; details may be required.

        - Install freestanding emitters on pipe riser to mounting height indicated.

        - Install manifold emitter systems with tubing to emitters. Plug unused manifold outlets. Install emitters on off-ground supports at height indicated.

        - Install multiple-outlet emitter systems with tubing to outlets. Plug unused emitter outlets. Install outlets on off-ground supports at height indicated.

        - Install drip tubes with direct-attached emitters on ground.

        - Install drip tubes with remote discharge on ground with outlets on off-ground supports at height indicated.

        - Install off-ground supports of length required for indicated mounted height of device.

        - Install [application pressure regulators] [and] [filter units] in piping near device being protected, and [aboveground] [in control-valve boxes].

        - Install [air relief valves] [and] [vacuum relief valves] in piping, and [aboveground] [in control-valve boxes].

      - INSTALLATION OF AUTOMATIC IRRIGATION CONTROL SYSTEM

Retain "Equipment Mounting, Interior" or "Equipment Mounting, Exterior" Paragraph below.

        - Equipment Mounting, Interior: Install controllers on interior [floor] [concrete bases] [wall].

          - Place and secure anchorage devices. Use setting drawings, templates, diagrams, instructions, and directions furnished with items to be embedded.

          - Install anchor bolts to elevations required for proper attachment to supported equipment.

        - Equipment Mounting, Exterior: Install exterior freestanding controllers on precast concrete bases.

          - Place and secure anchorage devices. Use setting drawings, templates, diagrams, instructions, and directions furnished with items to be embedded.

          - Install anchor bolts to elevations required for proper attachment to supported equipment.

        - Install control cable in same trench as irrigation piping and at least 2 inches (50 mm) below[ or beside] piping. Provide conductors of size not smaller than recommended by controller manufacturer. Install cable in separate sleeve under paved areas.

      - CONNECTIONS

Coordinate irrigation piping installations and specialty arrangements with schematics on Drawings and with requirements specified. If Drawings are explicit enough, these requirements may be reduced or omitted.

        - Comply with requirements for piping specified in Section 331415 "Site Water Distribution Piping" for water supply from exterior water service piping, water meters, protective enclosures, and backflow preventers. Drawings indicate general arrangement of piping, fittings, and specialties.

        - Install piping adjacent to equipment, valves, and devices to allow service and maintenance.

        - Connect wiring between controllers and automatic control valves.

      - IDENTIFICATION

        - Identify system components. Comply with requirements for identification specified in Section 220553 "Identification for Plumbing Piping and Equipment."

        - Equipment Nameplates and Signs: Install engraved plastic-laminate equipment nameplates and signs on each automatic controller.

          - Text: In addition to identifying unit, distinguish between multiple units, inform operator of operational requirements, indicate safety and emergency precautions, and warn of hazards and improper operations.

        - Warning Tapes: Arrange for installation of continuous, underground, detectable warning tapes over underground piping during backfilling of trenches. See Section 312000 "Earth Moving" for warning tapes.

      - FIELD QUALITY CONTROL

Retain "Manufacturer's Field Service" Paragraph below to require a factory-authorized service representative to perform inspections, tests, and adjustments.

        - Manufacturer's Field Service: Engage a factory-authorized service representative to inspect, test, and adjust components, assemblies, and equipment installations, including connections.

Retain first paragraph below to require Contractor to perform tests and inspections.

        - Perform tests and inspections.

Retain "Manufacturer's Field Service with Test Assistance" Subparagraph below to require a factory-authorized service representative to assist Contractor with inspections, tests, and adjustments.

          - Manufacturer's Field Service with Test Assistance: Engage a factory-authorized service representative to inspect components, assemblies, and equipment installations, including connections, and to assist in testing.

Retain "Tests and Inspections" Paragraph below to describe tests and inspections to be performed.

        - Tests and Inspections:

See Section 014000 "Quality Requirements" for retesting and reinspecting requirements and Section 017300 "Execution" for requirements for correcting the Work.

          - Leak Test: After installation, charge system and test for leaks. Repair leaks and retest until no leaks exist.

Retain "Operational Test" Subparagraph below for controllers and automatic control valves.

          - Operational Test: After electrical circuitry has been energized, operate controllers and automatic control valves to confirm proper system operation.

          - Test and adjust controls and safeties. Replace damaged and malfunctioning controls and equipment.

          - Irrigation system will be considered defective if it does not pass tests and inspections.

        - Prepare test and inspection reports.

      - STARTUP SERVICE

        - [Engage a factory-authorized service representative to perform] [Perform] startup service.

          - Complete installation and startup checks in accordance with manufacturer's written instructions.

Retain both subparagraphs below for controllers and automatic control valves.

          - Verify that controllers are installed and connected in accordance with the Contract Documents.

          - Verify that electrical wiring installation complies with manufacturer's submittal.

      - ADJUSTING

Revise this article to suit Project. Retain first two paragraphs below for controllers and automatic control valves.

        - Adjust settings of controllers.

        - Adjust automatic control valves to provide flow rate at rated operating pressure required for each sprinkler circuit.

        - Adjust sprinklers and devices, except those intended to be mounted aboveground, so they will be flush with, or not more than [1/2 inch (13 mm)] <Insert value> above, finish grade.

      - CLEANING

        - Flush dirt and debris from piping before installing sprinklers and other devices.

      - DEMONSTRATION

        - [Engage a factory-authorized service representative to train] [Train] Owner's maintenance personnel to adjust, operate, and maintain [automatic control valves] [and] [controllers].

      - PIPING SCHEDULE

Retain and revise applicable piping applications. Coordinate with materials specified in Part 2.

        - Install components having pressure rating equal to or greater than system operating pressure.

        - Piping in control-valve boxes and aboveground may be joined with flanges or unions instead of joints indicated.

        - Aboveground Irrigation Main Piping:

          - NPS 4 (DN 100) and Smaller:

            - Galvanized-steel pipe and galvanized-steel pipe nipples; galvanized, gray-iron threaded fittings; and threaded joints.

            - [Type L (Type B)] [Type M (Type C)] hard copper tube, wrought- or cast-copper fittings, and [brazed] [soldered] joints.

            - [Schedule 40] [Schedule 80], PVC pipe; socket-type PVC fittings; and solvent-cemented joints.

            - Schedule 80, PVC pipe; Schedule 80, threaded PVC fittings; and threaded joints.

          - NPS 5 (DN 125) and Larger:

            - Galvanized-steel pipe and galvanized-steel pipe nipples; galvanized, gray-iron threaded fittings; and threaded joints.

            - [Schedule 40] [Schedule 80], PVC pipe and socket fittings; and solvent-cemented joints.

            - Schedule 80, PVC pipe; Schedule 80, threaded PVC fittings; and threaded joints.

        - Underground Irrigation Main Piping:

          - NPS 4 (DN 100) and Smaller:

            - NPS 3 and NPS 4 (DN 80 and DN 100) ductile-iron, mechanical-joint pipe; ductile-iron, mechanical-joint fittings, glands, bolts, and nuts; and gasketed joints.

            - NPS 3 and NPS 4 (DN 80 and DN 100) ductile-iron, push-on-joint pipe; ductile-iron, push-on-joint fittings and gaskets; and gasketed joints.

            - Type L (Type B) soft copper tube, wrought-copper fittings, and brazed joints.

Pipe in first subparagraph below is only available in NPS 4 (DN 100) and larger.

            - NPS 4 (DN 100) PE pressure pipe; PE butt, heat-fusion or socket-type fittings; and heat-fusion joints.

            - [Schedule 40] [Schedule 80], PVC pipe and socket fittings, and solvent-cemented joints.

            - Schedule 80, PVC pipe; Schedule 80, threaded PVC fittings; and threaded joints.

            - SDR 21, PVC, pressure-rated pipe; Schedule 80, PVC socket fittings; and solvent-cemented joints.

          - NPS 5 (DN 125) and Larger:

            - NPS 6 (DN 150) and larger ductile-iron, mechanical-joint pipe; ductile-iron, mechanical-joint fittings, glands, bolts, and nuts; and gasketed joints.

            - NPS 6 (DN 150) and larger ductile-iron, push-on-joint pipe; ductile-iron, push-on-joint fittings and gaskets; and gasketed joints.

            - PE pressure pipe; PE butt, heat-fusion fittings; and heat-fusion joints.

            - [Schedule 40] [Schedule 80], PVC pipe and socket fittings; and solvent-cemented joints.

            - SDR 21, PVC, pressure-rated pipe; Schedule 80, PVC socket fittings; and solvent-cemented joints.

        - Circuit Piping:

          - NPS 2 (DN 50) and Smaller:

            - [SIDR 7] [SIDR 9], PE, controlled ID pipe; insert fittings for PE pipe; and fastener joints.

            - [DR 9] [DR 11], PE, controlled OD pipe; PE butt, heat-fusion, or PE socket-type fittings; and heat-fusion joints.

            - Schedule 40, PVC pipe and socket fittings; and solvent-cemented joints.

            - SDR 26, PVC, pressure-rated pipe; Schedule 40, PVC socket fittings; and solvent-cemented joints.

          - NPS 2-1/2 to NPS 4 (DN 65 to DN 100):

            - [SIDR 7] [SIDR 9], PE, controlled ID pipe; insert fittings for PE pipe; and banded or fastener joints.

            - [DR 9] [DR 11], PE, controlled OD pipe; PE socket or butt-fusion fittings; and heat-fusion joints. NPS 3 (DN 80) pipe and fittings if NPS 2-1/2 (DN 65) pipe and fittings are not available.

            - Schedule 40, PVC pipe and socket fittings; and solvent-cemented joints.

            - SDR 26, PVC, pressure-rated pipe; Schedule 40, PVC socket fittings; and solvent-cemented joints.

        - Underground Branches and Offsets at Sprinklers and Devices: Schedule 80, PVC pipe; threaded PVC fittings; and threaded joints.

          - Option: Plastic swing-joint assemblies, with offsets for flexible joints, manufactured for this application.

        - Risers to Aboveground Sprinklers and Specialties:

Retain one of two subparagraphs below.

          - [Type L (Type B)] [Type M (Type C)] hard copper tube, wrought-copper fittings, and [brazed] [soldered] joints.

          - Schedule 80, PVC pipe and socket fittings; and solvent-cemented joints.

Retain option in paragraph below to allow Contractor to select piping materials from those retained.

        - Drain piping shall be[ one of] the following:

          - SIDR 9, 11.5, or 15; PE, controlled ID pipe; insert fittings for PE pipe; and banded or fastener joints.

          - Schedule 40, PVC pipe and socket fittings; and solvent-cemented joints.

          - SDR 21, 26, or 32.5; PVC, pressure-rated pipe; Schedule 40, PVC socket fittings; and solvent-cemented joints.

      - VALVE SCHEDULE

        - Underground, Shutoff-Duty Valves: Use the following:

          - NPS 2 (DN 50) and Smaller: Curb valve, curb-valve casing, and shutoff rod.

          - NPS 3 (DN 80) and Larger: Iron gate valve, resilient seated; iron gate valve casing; and operating wrench(es).

        - Aboveground, Shutoff-Duty Valves:

          - NPS 2 (DN 50) and Smaller:

Retain one of first two subparagraphs below.

            - [Brass] [Brass or bronze] [Bronze] [Plastic] ball valve.

            - Bronze gate valve.

          - NPS 2-1/2 (DN 65) and Larger:

Retain one of two subparagraphs below.

            - Iron ball valve.

            - Iron gate valve, [NRS] [OS&Y].

Valves in "Throttling-Duty Valves" Paragraph below are to be installed in control-valve boxes.

        - Throttling-Duty Valves:

          - NPS 2 (DN 50) and Smaller:

Retain one of first two subparagraphs below.

            - [Bronze] [Plastic] automatic control valve.

            - [Brass] [Brass or bronze] [Bronze] [Plastic] ball valve.

          - NPS 2-1/2 and NPS 3 (DN 65 and DN 80):

Retain one of two subparagraphs below.

          - [Bronze] [Plastic] automatic control valve.

          - Iron ball valve.

        - Drain Valves:

          - NPS 1/2 and NPS 3/4 (DN 15 and DN 20):

Retain one of first three subparagraphs below.

            - Automatic drain valve.

            - [Brass] [Brass or bronze] [Bronze] [Plastic] ball valve.

            - Bronze gate valve.

          - NPS 1 to NPS 2 (DN 25 to DN 50):

Retain one of two subparagraphs below.

            - [Brass] [Brass or bronze] [Bronze] [Plastic] ball valve.

            - Bronze gate valve.

END OF SECTION 328400